3GPP RAN WG2 Meeting #129bis	R2-2502284

Wuhan, China, April 7th – 11th 2025

Agenda Item:	8.8.3

Source:	InterDigital

Title:	Discussion on Uplink Capacity/Throughput Enhancement for NTN

Document for:	Discussion, Decision

# Introduction

An objective for Rel-19 NR non-terrestrial networks (NTN) is to study impacts and modifications to support uplink capacity/cell throughput improvement. Detailed objectives include the following [1]:

- Uplink Capacity/Cell Throughput Enhancement for FR1-NTN [RAN1, RAN2, RAN4] - Specify Orthogonal Cover Codes (OCC) for DFT-s-OFDM PUSCH at least for multiplexing 2 or 4 UEs when PUSCH repetitions are used - Specify necessary signalling - Update RF requirements accordingly, if needed - Notes for this objective: - The enhancement is not targeting improvements/impacts of MU-MIMO capability - The enhancement is not targeted to PUSCH DMRS - No enhancement for initial access - Enhancements to PRACH are not in scope. - This feature may be applicable for UEs operating in terrestrial networks based on a common design

Work in RAN1 has started discussing on the OCC aspects [2].

This contribution discusses signalling impact for the support of uplink capacity/cell throughput enhacement.

# Discussion

RAN1 is currelty discussing how to signal OCC length and OCC sequence indications (see [2] subclause 3.1.5 and 3.1.6).

## OCC configuration

For DG PUSCH and CG PUSCH type 2, DCI is involved for scheduling and so we can assume that OCC length and index can be signalled by DCI. However, CG PUSCH type 1 is semi-statically configured by RRC and no DCI involvement and so from RAN2 point of view, it’s reasonable to assume that OCC length and index are configured by RRC signalling and so we propose;

Proposal 1:	RAN2 assumes;

- For DG PUSCH and CG PUSCH type 2, OCC length and OCC index won’t be configured by RRC; and

- For CG type 1, OCC length and OCC index are configured by RRC.

# Conclusion

In this contribution the following observations and proposals are made regarding UL Capacity/Cell throughput enhancement:

Proposal 1:	RAN2 assumes;

- For DG PUSCH and CG PUSCH type 2, OCC length and OCC index won’t be configured by RRC; and

- For CG type 1, OCC length and OCC index are configured by RRC.

# References

RP-243300 – Non-Terrestrial Networks (NTN) for NR Phase 3 – Thales, CATT

R1-2501402 "Feature lead summary #3 of AI 9.11.3 on NR-NTN uplink capacity and throughput", MediaTek