3GPP TSG-RAN WG2 Meeting #129bis		     R2-2502293

Wuhan, CN: 7th Apr – 11th Apr 2025

Source:	Qualcomm Incorporated

Title:	Discussion on Dataset and Parameter Sharing from the Network to the UE for Two-Sided Model Training

Agenda Item:	8.1.5 Model transfer/delivery

Document for:	Discussion and Decision

# Introduction & Background

This paper discusses the following

- Dataset (and/or parameters) transfer from the network to the UE for two-sided model training,

- Model delivery/transfer for inference, considering discussions during the SI phase and recent agreements in RAN1 as the baseline.

# Discussion

## 2.1   Dataset and Parameter Sharing from the Network to the UE for Two-Sided Model Training

The [POST129][029][AI Phy] Model transfer (Xiaomi Ericsson) report [1] summarizes different aspects associated with sharing the dataset and model parameters from the network side to the UE-side training entity (within or outside the MNO network) using over-the-air (OTA) and non-OTA signaling. In this paper, we further discuss the following

- Authentication and authorization procedures for sharing datasets and parameters from the NW side to the UE side training entity (within or outside the MNO network)

- Specification and implementation complexities/issues associated with OTA solution

### 2.1.1 Authentication and authorization procedures for sharing datasets and parameters from the NW side to the UE side training

TS 33.519 [2] and TS 33.501 [3] define the authentication and authorization of an application function that involves the following steps

- Application Request: An application (or API invoker) requests access to a specific network capability or resource.

- Authorization Check: The 3GPP network verifies the application's credentials and permissions through its authorization functions (e.g., CAPIF core function, NRF, or AUSF).

- Access Granting/Denial: The network grants or denies access to the requested resource based on the authorization check.

In the non-OTA approach, the UE side training entity within /outside the MNO network (e.g., Application function) requesting access to the dataset and parameters for UE side training is authenticated and authorized by the 3GPP system. Furthermore, the dataset and parameter are shared from the NW side to an authenticated and authorized UE side training entity within /outside the MNO network (e.g., Application function), therefore, concern associated with leakage of network topology, or any NW vendor proprietary information is minimized.

Observation 1: In the non-OTA approach, the UE side training entity within /outside the MNO network (e.g., Application function) requesting access to the dataset and parameters for UE side training is authenticated and authorized by the 3GPP system [2][3].

Observation 2: In the non-OTA approach, the dataset and parameter are shared from the NW side to an authenticated and authorized UE side training entity within /outside the MNO network (e.g., Application function), therefore, concern associated with leakage of network topology, or any NW vendor proprietary information is minimized.

### 2.1.2 Specification and implementation complexities/issues associated with Dataset and Parameters Sharing using Ove-the-Air Signaling

Two flavors of OTA solutions were proposed in the RAN2#129 meeting [4], as follows

- The network shares the dataset /parameter with a single UE, and

- The network shares the dataset/parameter with multiple UEs (by segmenting the dataset/parameter)

#### 2.1.2.1 The network shares the dataset /parameter with a single UE

In the below table, we discuss the specification and implementation challenges associated with the flavor of the OTA solution The network shares the dataset/parameter with a single UE over the air (by segmenting the dataset/parameter).

| Issues | Specification impact | Implementation complexity |
| --- | --- | --- |
| RRC buffer issue | Not feasible to have an RRC buffer >200Mbyte for UE in 5G [1] | Significant AS memory requirement at the UE |
| Representative UE selection |  | Significant challenges to provision UEs at different cells/gNBs - Inter-vendor collaboration may need to determine representative UE for a vendor |
| Security concern |  | A rogue UE can pretend to be a UE vendor device and get all the segments. Later, if a legitimate UE requests the dataset/parameters, the cell/gNB may reject the UE request. |

#### 2.1.2.2 The network shares the dataset/parameter with multiple UEs over the air (by segmenting the dataset/parameter)

Fig. 2.1.2.2-1 illustrates the dataset and parameters sharing from the network to the UE over the air. In Fig. 2.1.2.2-1, the gNB sends the dataset and parameters to the UE, thereafter, the UE sends the dataset and parameters to the UE side entity (data collection or training entity/server), as the training cannot be performed at the UE due to the sheer size of the dataset and the required computation for training.

Fig. 2.1.2.2-1: Dataset and parameter sharing using over-the-air signaling (with multiple UEs)

In the below table, we discuss the specification and implementation challenges associated with the flavor of the OTA solution The network shares the dataset/parameter with multiple UEs over the air (by segmenting the dataset/parameter).

| Issues | Specification impact | Implementation complexity |
| --- | --- | --- |
| Need for new SRB protocol stack | Segmentation beyond the RRC layer requires a new SRB protocol stack to perform segmentation, including functions such as handling segmentation, retransmission, etc [1] | A solution will be needed to micromanage dataset/parameter sharing at the segment level. |
| UE selection | Assuming the network performs UE selection for dataset/parameter sharing - NG-RAN identifies UE based on C-RNTI (temporary). It does not even know the UE (IMSI) or UE vendor information. - Signaling/procedure is needed between CNF/OAM and NG-RAN for UE selection | Assuming the UE side server is performing UE selection - The UE side server may need to continuously know which UEs are in the coverage of the cell/gNB transmitting the dataset/parameters - Coordination is required between UEs or UE and UE side server to ensure that UEs are requesting different segments |
| Handling due to mobility | Assuming the network performs UE selection for dataset/parameter sharing - The network needs to select new UEs for sharing the unsuccessful segment transmission - If UE moves to a non-supporting node, - If the target / new gNB transmits different datasets/parameters | Continuous UE selection will be required due to UE mobility for retrieving unsuccessful segment reception - The UE side server may be required to know the precise UE location for selecting new UEs |
| Security concern |  | The UE side server is required to know its UE positions in real time for UE selection, - This may pose a security concern where the UE side can discover network topology using UE location and received measurements |

Apart from individual challenges associated with different flavors of over-the-air signaling for the dataset and parameter sharing, the following challenges are common to both flavors

- Uu overhead for data collection from UE and dataset/model parameter transfer to UE

- Overloading CP with large datasets would disrupt core control message transmission (e.g. service degradation, reliability, etc)

- UE power issues, for example, UE should not be unnecessarily kept in RRC CONNECTED to share dataset/parameter

Proposal 1: Over-the-air signaling poses significant specification impacts and implementation challenges (both at the network side and UE side) for sharing the dataset and parameters to the UE side training entity (within/outside the MNO network). Thus, the OTA solution is not suitable and scalable.

## 2.2 Model Transfer/Delivery

### 2.2.1	Two-Sided Models

### RAN1 considered the following “directions” for two-sided model collaboration:

- Direction A: Sharing parameters/reference model/dataset that enables UE-side offline engineering

- Direction B: Sharing NW side encoder parameter to UE side for UE side inference directly with on-device operation

- Direction C: Fully standardized reference model(s) and parameters with specified CSI generation part and/or CSI reconstruction part

In RAN1 LS R1-2410922 [1], RAN1 mentioned that among the various Options and Sub-Options for Direction A, after down-selection, RAN1 is considering the following Sub-Options for further study with potential further down-selection.

- Option 4-1: sharing dataset containing N1 samples of {target CSI, CSI feedback} corresponding to the input and output of a nominal encoder, respectively

- Option 3a-1 without target CSI: sharing parameters corresponding to a nominal encoder,

- Option 3a-1 with target CSI: sharing parameters corresponding to a nominal encoder, along with dataset containing N2 samples of {target CSI}.

In RAN1 LS R1-2410922 [1], RAN1 did not ask RAN2 to study the feasibility/solution for direction B. Therefore, RAN2 should wait for RAN1 to conclude that the model transfer/delivery solution from NW to UE is needed for two-sided models. In direction A, the model is trained at the UE-side training entity and can be transferred/delivered to the UE for inference. Therefore, a standardized solution for one-sided model transfer/delivery (from 5GC or server in MNO) can be reused for the two-sided model (if introduced).

Proposal 2: RAN2 should wait for RAN1 to conclude if direction B is supported.

# Conclusion

Dataset and parameter sharing from the network to the UE-side training entity

Observation 1: In the non-OTA approach, the UE side training entity within /outside the MNO network (e.g., Application function) requesting access to the dataset and parameters for UE side training is authenticated and authorized by the 3GPP system [2][3].

Observation 2: In the non-OTA approach, the dataset and parameter are shared from the NW side to an authenticated and authorized UE side training entity within /outside the MNO network (e.g., Application function), therefore, concern associated with leakage of network topology, or any NW vendor proprietary information is minimized.

Proposal 1: Over-the-air signaling poses significant specification impacts and implementation challenges (both at the network side and UE side) for sharing the dataset and parameters to the UE side training entity (within/outside the MNO network). Thus, the OTA solution is not suitable and scalable.

Model transfer/delivery

Proposal 2: RAN2 should wait for RAN1 to conclude if direction B is supported.

# Reference

- [POST129][029][AI Phy] Model transfer (Xiaomi Ericsson), “Report of [POST129][029][AI Phy] Model transfer (Xiaomi/Ericsson)”

- TS 33.519, “5G Security Assurance Specification (SCAS)  for the Network Exposure Function (NEF)  network product class”

- TS 23.501, “Security architecture and procedures for 5G System”

- RAN2#129 meeting report