3GPP TSG-RAN2 Meeting #129bis	R2-2502297

# Wuhan, China, April 7 – April 11, 2025

Agenda Item:	7.0.2.16

Source: 	Xiaomi Communications

Title:  	Discussing on issues on DSR and proposed TP to PDCP

Document for:	Discussion and Decision

Introduction

In this contribution, we provide some views on an identified issue in Rel-18 XR for DSR triggering in PDCP.

Discussion

Currently, a PDCP SDU is considered to be associated with a DSR if it has not been transmitted in any MAC PDU and is a delay-critical PDCP SDU. And the definition of delay-critical PDCP SDU is captured in TS 38.323 as shown below [1].

Delay-critical PDCP SDU: if pdu-SetDiscard is not configured, a PDCP SDU for which the remaining time till discardTimer expiry is less than the remainingTimeThreshold. If pdu-SetDiscard is configured, a PDCP SDU belonging to a PDU Set of which at least one PDCP SDU has the remaining time till discardTimer expiry less than the remainingTimeThreshold.

The PDCP discard timer is maintained by PDCP layer per PDCP SDU. Obviously, PDCP SDUs’ discardTimer will be compared with a time threshold configured by the NW for delay-critical data.

Currently, the remaining time threshold is configured per LCG of MAC entity as shown below [2]:

#### – MAC-CellGroupConfig The IE MAC-CellGroupConfig is used to configure MAC parameters for a cell group, including DRX. MAC-CellGroupConfig information element -- ASN1START -- TAG-MAC-CELLGROUPCONFIG-START MAC-CellGroupConfig ::= SEQUENCE { … dsr-ConfigToAddModList-r18 SEQUENCE (SIZE (1..maxNrofLCGs-r18)) OF LCG-DSR-Config-r18 OPTIONAL, -- Need N … } LCG-DSR-Config-r18 ::= SEQUENCE { lcg-Id-r18 LCG-Id-r18, remainingTimeThreshold-r18 INTEGER (1..64), ... }

In cases there are multiple LCHs or multiple MAC entities associating to a PDCP entity, such as split bearer and PDCP duplication, we have agreed that DSR will be triggered in both MAC entities associated with the primary RLC entity and secondary RLC entity. However, which time threshold will be used by the PDCP is not clear in the current Spec as it only captures “the remaining time till discardTimer expiry is less than the remainingTimeThreshold”.

Figure 1: A split bearer from a UE perspective for DC case

In our understanding, NW should be allowed to configure separate time threshold for MN and SN node since both nodes have different load situation. Thus, DSR triggering and delay-critical PDCP data volume is indicated to a MAC entity based on the remainingTimeThreshold associated with the MAC entity. A Delay-critical PDCP SDU is defined as whose remaining time till discardTimer expiry is less than the remainingTimeThreshold associated with the indicated MAC entity.

Hence, we propose to clarify this:

Proposal 1.  A Delay-critical PDCP SDU is defined as whose remaining time till discardTimer expiry is less than the remainingTimeThreshold associated with the indicated MAC entity.

Proposal 2.   Consider adopting the TP in Annex in the PDCP CR for DSR.

Conclusions

Based on the discussion, our proposals are provided as follows:

Proposal 1.  A Delay-critical PDCP SDU is defined as whose remaining time till discardTimer expiry is less than the remainingTimeThreshold associated with the indicated MAC entity.

Proposal 2.   Consider adopting the TP in Annex in the PDCP CR for DSR.

References

TS 38.323-i40

TS 38.331-i30

Annex for DSR remaining issues:

Start of changes

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

AM DRB: a data radio bearer which utilizes RLC AM.

AM MRB: an MRB associated with at least one AM RLC bearer for PTP transmission.

Broadcast MRB: a radio bearer configured for MBS broadcast delivery.

DAPS bearer: a bearer whose radio protocols are located in both the source gNB and the target gNB during DAPS handover to use both source gNB and target gNB resources.

Delay-critical PDCP SDU: if pdu-SetDiscard is not configured, a PDCP SDU for which the remaining time till discardTimer expiry is less than the remainingTimeThreshold associated with the indicated MAC entity. If pdu-SetDiscard is configured, a PDCP SDU belonging to a PDU Set of which at least one PDCP SDU has the remaining time till discardTimer expiry less than the remainingTimeThreshold associated with the indicated MAC entity.

End of changes