3GPP TSG-RAN WG2 Meeting#129bis	R2-2502347

Wuhan, CN, April 7–11, 2025

Agenda item:	 8.10.2.1

Source:		Lenovo

Title:		Discussion on MRO enhancements for LTM

Document for:		Discussion and Decision

1	Introduction

RAN2#129 meeting agreed to send an LS to RAN3 indicating scenario of beam failure recovery shortly after successful LTM (“near failure”), also say that we will not specify a UE based solution for this, but that RAN3 can consider a NW based solution if they want. RAN3 can see if/how the solution (if specified) could also be used for the case of LTM failure.

In this paper, we further discuss MRO enhancements for LTM.

2	Discussion

In a LTM procedure, depending on the availability of a valid TA value, the UE performs either a RACH-less LTM or RACH-based LTM cell switch:

- the UE performs RACH-less LTM cell switch upon receiving the cell switch command:

  - if the TA value is provided in the cell switch command, the UE applies the TA value as instructed by the network. For this case, before cell switch command, early TA acquisition/UL synchronization procedure is triggered by PDCCH order to an LTM candidate cell, or the UE performs RACH for early UL synchronization or RACH for early TA acquisition;

  - if UE-based TA measurement is configured, but no TA value is provided in the cell switch command, the UE applies the TA value by itself if available.

- if no valid TA value is available, the UE performs RACH-based LTM cell switch upon receiving the cell switch command:

  - the UE performs CFRA to the target cell, if contention-free Random Access Resources are provided in LTM Cell Switch Command MAC CE;

  - or, the UE performs CFRA to the target cell, if no contention-free Random Access Resource is provided in LTM Cell Switch Command MAC CE, but contention-free Random Access Resources are provided in the RRC Reconfiguration message for the LTM procedure;

  - or, the UE performs CBRA to the target cell, if no contention-free Random Access Resource is provided.

To optimize RA resources for LTM procedure, the UE may report L1 measurement results (of source cell, or LTM candidate cell, or target cell) during LTM procedure in the RA report. Additionally, for optimization of early UL synchronization or early TA acquisition, a new RA purpose to indicate that the random access procedure was triggered by a PDCCH order for early UL synchronization or early TA acquisition may be included in the RA report.

Proposal 1: The UE can report L1 measurement results during LTM procedure, and a new RA purpose to indicate that the random access procedure was triggered by a PDCCH order for early TA acquisition, in the RA report.

3	Conclusion

In this paper, MRO enhancements for LTM are discussed. We have the following proposal:

Proposal 1: The UE can report L1 measurement results during LTM procedure, and a new RA purpose to indicate that the random access procedure was triggered by a PDCCH order for early TA acquisition, in the RA report.