3GPP TSG-RAN WG2 Meeting #129bis 	R2-2502375

Wuhan, China, April. 7th – 11th, 2025

Agenda Item:	8.2.2

Source:	Panasonic

Title:	Discussion on A-IoT paging

Document for:	Discussion

# Introduction

In the previous meeting, the following agreements have been made for A-IoT paging.

Agreements

- Parallel service requests by the same reader is not supported.

- The device is expected to only perform one procedure at a time.   FFS device behaviour if multiple requests are received in parallel (if needed).

- The “transaction ID” can be generated by reader based on CN corelation ID.  FFS how reader will generate “transaction ID”.  FFS the size of transaction ID

- 1 bit solution is excluded.   FFS the size.  Aim to have a reasonable size.

- RAN2 acknowledges that multi-reader scenario may exist but we will not specify something specific for this purpose.  We can rely on transaction ID and implementation to handle it.

Agreements on paging ID

- The “one identifier” in the paging message includes both the case of “one single device identifier” and “one group identifier”/”filtering criteria”, while the exact format of latter is supposed to be designed by SA2.

- The current assumption is that the paging identifier is transparent to the A-IoT MAC Layer and carried by upper layer.   FFS if there is really a need for visibility in the MAC layer

Agreements - The A-IoT paging message can include a number of msg1 resources - From RAN2 perspective, after initial paging message, the R2D transmission which determines the Msg1 resource(s), can be achieved by one of the below two ways, unless RAN1 concludes to use L1 signaling later: Way-1: introducing new R2D message other than the paging message, e.g., QueryRep-like; or Way-2: reusing the same paging message, using field(s) to indicate it is only to determine the Msg1 resource(s) and omitting the paging identifier (device ID/group ID) field 3. The service type of A-IoT (e.g., inventory only, inventory + command) is not included in paging message.

This paper is intended to discuss some remaining issues on paging, including device behavior if multiple requests are received in parallel, reader ID and skip response due to energy constraint.

# Discussion

## Device behaviour if multiple requests are received in parallel

As one of outcomes of the post RAN2#129 meeting email discussion [1], the following are proposed by rapporteur:

Proposal 1: Discuss and agree on the device behavior if it gets a new service request while one procedure is still ongoing (i.e. not completed or failed yet): (a) ignore all new requests, or (b) terminate the ongoing procedure and respond to the latest request. (a/b/ffs = 17/7/5) Proposal 2: Discuss and decide how a device determines the procedure is ‘ongoing’ or not.

Let us discuss proposal 2 first, because it will help us to decide on option (a) or option (b) for proposal 1.

Depending on service type (e.g. “inventory only” or “inventory + command”) and contention-based or contention-free random access, the definition of completion of the procedure would be different. See the following table 1 from reader perspective:

Table 1: Procedure completion from reader perspective

| Case number | Case description | Procedure completion from reader perspective |
| --- | --- | --- |
| 1 | Inventory with CBRA | Msg3 is successfully received |
| 2 | Inventory with CFRA | First D2R data transmission (i.e. Msg1) is successfully received |
| 3 | Inventory and command with CBRA | The scheduled D2R by R2D command is successfully received |
| 4 | Inventory and command with CFRA | The scheduled D2R by R2D command is successfully received |

However, from device perspective, it is less clear how the procedure can be seen as completed. Notice that we (RAN2) concluded that service type (e.g. “inventory only”, “inventory + command”) will not be included in paging message. Therefore, after being triggered device may not know the triggered procedure is for “inventory only” or “inventory +command”. Device needs to follow the R2D message in order to figure out whether it needs to transmit a next D2R. In the following, we separate the discussion between CBRA and CFRA.

Device determination of a successful/failed/pending procedure for CBRA

The following figure 1 illustrates possible steps for “inventory only” and “inventory + command” for CBRA and how device assumes the procedure status after each step.

Figure 1: Determination of a procedure status for CBRA

We would like to mention the following remarks with respect to figure 1.

-	Remark 1: A timer is assumed for device to expect Msg2 after sending Msg1. If no Msg2 is received within time window, device assumes the contention resolution has failed (e.g. random ID collision). Device should look for another access opportunity provided/triggered by the reader for re-access.

-	Remark 2: After sending Msg3, as Msg3 successful transmission case, if no R2D is received within a time window, device would assume the procedure has been successfully completed (see step 4d). It means that if reader receives Msg3 successfully and does not have follow-up command, reader need not send anything. This saves signalling overhead. Furthermore, it is aligned with the following agreement made in previous meeting in agenda 8.2.3 A-IoT random access:

NACK based mechanism is supported for D2R messages to determine re-access for at least msg3. FFS details including whether we need a timer or explicit message and when reader sends feedback

As Msg3 failure case, if the reader wants to request a retransmission of Msg3, reader must send the R2D within the time window. Reader can also use the time window to send follow-up command or simply inform procedure failure to device. We acknowledge that maintaining a timer might increase device complexity, however, this provides a clearly defined check point for device to close the pending status of ongoing procedure. The same mechanism is applied to the “inventory + command” case (see step 6d).

Device determination of a successful/failed/pending procedure for CFRA

For the CFRA case, the steps 1 and 2a/2b in figure 1 (CBRA) will be skipped, because they are for contention resolution. The first D2R transmission starts with step 3 in figure 1, or referred to as “Msg1” for CFRA. Basically, the rest of figure 1 can be applied to CFRA case. The result is figure 2 below (with re-numbering the msg and steps).

Figure 2: Determination of a procedure status for CFRA

To summarize, we propose the following:

Proposal 1. For CBRA, after sending Msg1 in response to an A-IoT paging, device assumes the procedure associated with this paging is ongoing (i.e. pending), until one of the following events occurs:

      - Msg2 is not received within a time window, in which case the procedure is concluded as failure

      - After sending Msg3,

        - Device receives NACK for re-access determination within a time window, in which case the procedure is concluded as failure

        - No R2D is received within a time window, in which case the procedure is concluded as successful

Proposal 2. For CFRA, after sending first D2R data (“Msg1”) in response to an A-IoT paging, device assumes the procedure associated with this paging is ongoing (i.e. pending), until one of the following events occurs:

      - Device receives NACK for re-access determination within a time window, in which case the procedure is concluded as failure

      - No R2D is received within a time window, in which case the procedure is concluded as successful

After discussing how device can determine a procedure is ongoing or not, we can discuss whether device should respond to new paging while these is an ongoing procedure. As summarized by rapporteur in the post email discussion, there are two options: (a) ignore all new requests, or (b) terminate the ongoing procedure and respond to the latest request. Although majortiy companies prefer option a, some companies mentioned that option b provides benefit that reader can interrupt the ongoing procedure by inserting some more urgent service. We would like to have further clarification on what are those urgent services and other use cases for option b, before down-selection.

Proposal 3. Further discuss device behavior if it gets a new service request while one procedure is still ongoing: (a) ignore all new requests, or (b) terminate the ongoing procedure and respond to the latest request.

## Reader ID

In the post email discussion, the necessity of reader ID information has been discussed. As summarized by rapporteur, 11 companies had the view that providing reader ID information may be needed or beneficial.

For example, in a multi-reader overlapping area, device may receive paging messages from different readers with the same or different service request from CN. Depending on the service requirements, it may be preferable for device to respond to one single reader only, or multiple readers (duplicated responses). For example, for inventory service, it may be sufficient for device to respond to one (“best”) reader and ignore the paging from other readers to save the energy and signaling overhead. However, for proximity determination, it is useful for the network to locate the device if the device responds to multiple readers. Therefore, network should be able to control whether a device should respond to single reader or multiple readers.

If the ”transaction ID” is generated by the reader without reader coordination), different readers would generate paging ID independently. It is likely that different transaction IDs will be used by different readers even if the paging is associated with the same CN request. If no other information to control the duplicated responses is included in paging except the transaction ID, it is effectively requested that the device needs to respond to multiple readers. On the other hand, if network would like to control the device to avoid duplicated responses to multiple readers, reader ID can be included in paging. Similar to the transaction ID, device now needs to maintain the reader ID after a successful access procedure (i.e. store it in the memory). If the new paging has a different reader ID, device should ignore the paging, regardless the same or different paging ID. In this way, the device is required to respond to one reader only. If the new paging has the same reader ID as the store one, device then compares the transaction ID to decide whether it is the subsequent paging or new paging.

Proposal 4. In additional to transaction ID, a reader ID can be optionally indicated in paging message. Device needs to maintain the reader ID (if indicated) and transaction ID in the memory after successful completion of the access procedure triggered by a paging.

Proposal 5. On deciding whether or not to respond to a paging, device compares the stored IDs with the IDs in the paging as follows:

| New transaction ID | New Reader ID | Device behavior | Intended scenario |
| --- | --- | --- | --- |
| Same as stored one | Not included | Not respond | Subsequent paging from same reader |
| Different than stored one | Not included | respond | device is requested to respond to multiple readers (assuming each reader generates transaction ID independently) |
| Same as stored one | Same as stored one | Not respond | Subsequent paging from same reader |
| Different as stored one | Same as stored one | respond | New paging from same reader |
| Same or different | Different than stored one | Not respond | Device is requested to respond to one reader (assuming reader-device association is fixed) |

## Skip response to paging due to energy constraint

In the study item phase, it has been discussed that if the device does not have sufficient energy to perform the remaining or follow-up procedure, the device can send energy status report to reader in a D2R message (e.g. MSG1, MSG3 and command response message). However, according to the WID, energy status reporting is not included in the scope.

If device can identify lack of energy for the follow-up communication, instead of sending energy status report, another option is that the device can simply skip responding to the paging in the first place, since sending a D2R message itself also consumes non-negligible amount of energy and negatively impacts the energy harvesting at the device. Therefore, a more important aspect is that the A-IoT device should be allowed to skip responding if the device does not have sufficient energy to complete the remaining access procedure triggered by the paging. Whether the energy is sufficient can be judged by device implementation. This procedure can be illustrated using Fig.3. below.

Fig.3. Device skips responding based on judgement of its energy

Proposal 6. Device is allowed to skip responding if device judges its remaining energy cannot sustain to complete whole access procedure triggered by this A-IoT paging.

# Conclusion

In this paper, we have discussed some remaining issues on paging, including device behavior if multiple requests are received in parallel, reader ID and skip response due to energy constraint. The proposals are as follows:

Proposal 1. For CBRA, after sending Msg1 in response to an A-IoT paging, device assumes the procedure associated with this paging is ongoing (i.e. pending), until one of the following events occurs:

      - Msg2 is not received within a time window, in which case the procedure is concluded as failure

      - After sending Msg3,

        - Device receives NACK for re-access determination within a time window, in which case the procedure is concluded as failure

        - No R2D is received within a time window, in which case the procedure is concluded as successful

Proposal 2. For CFRA, after sending first D2R data (“Msg1”) in response to an A-IoT paging, device assumes the procedure associated with this paging is ongoing (i.e. pending), until one of the following events occurs:

      - Device receives NACK for re-access determination within a time window, in which case the procedure is concluded as failure

      - No R2D is received within a time window, in which case the procedure is concluded as successful

Proposal 3. Further discuss device behavior if it gets a new service request while one procedure is still ongoing: (a) ignore all new requests, or (b) terminate the ongoing procedure and respond to the latest request.

Proposal 4. In additional to transaction ID, a reader ID can be optionally indicated in paging message. Device needs to maintain the reader ID (if indicated) and transaction ID in the memory after successful completion of the access procedure triggered by a paging.

Proposal 5. On deciding whether or not to respond to a paging, device compares the stored IDs with the IDs in the paging as follows:

| New transaction ID | New Reader ID | Device behavior | Intended scenario |
| --- | --- | --- | --- |
| Same as stored one | Not included | Not respond | Subsequent paging from same reader |
| Different than stored one | Not included | respond | device is requested to respond to multiple readers (assuming each reader generates transaction ID independently) |
| Same as stored one | Same as stored one | Not respond | Subsequent paging from same reader |
| Different as stored one | Same as stored one | respond | New paging from same reader |
| Same or different | Different than stored one | Not respond | Device is requested to respond to one reader (assuming reader-device association is fixed) |

Proposal 6. Device is allowed to skip responding if device judges its remaining energy cannot sustain to complete whole access procedure triggered by this A-IoT paging.

# Reference

[1][POST129][035][AIoT]Paging_v35_rapp-update.docx, ftp.3gpp.org/Email_Discussions/RAN2/[RAN2#129]/[POST129][035][AIoT] Paging (Qualcomm)