3GPP TSG-RAN WG2 Meeting #129bis	R2-2502387

Wuhan, China, Apr. 7th – 11th , 2025

Agenda Item:	8.11.2

Source: 	vivo

Title:         	Discussion on random access procedure in SBFD

Document for: 	Discussion and Decision

Introduction

In this contribution, we will further discuss the remaining issues on random access procedure in SBFD.

Discussion

RO selection

RAN2 had agreed at last meeting that [1]:

For initial RA transmission, the network can indicate the RO type (legacy RO or additional RO) to the SBFD-aware UE for the case of CBRA. Detailed signalling is FFS. If no RO type indication is provided by the NW, a UE selects RO type based on a SSB RSRP threshold. FFS whether NW can further indicate whether to select the additional RO type below or above this SSB RSRP threshold.

From our perspective, regardless whether the CBRA is UE-initiated or NW-initiated RACH procedure, the NW cannot know whether the channel state for a certain UE is good or not. The UE has better knowledge of the channel state by means of RSRP or CLI measurement. However, if the gNB has the preference that SBFD ROs are utilized as much as possible, it can indicate the preferred RO type in SIB1 for the case of CBRA.

Proposal 1: For initial RA transmission, the network can indicate the RO type (legacy RO or additional RO) to the SBFD-aware UE for the case of CBRA in SIB1.

For the FFS point on how the SSB RSRP threshold is used, there are two use cases the NW may consider whether to configure/schedule SBFD resources to SBFD-aware UE. One is configuring SBFD for coverage enhancement, and the other is to avoid severe UE-to-UE CLI.

Different configuration strategy determines how an RSRP threshold is used for RO selection in SBFD. For example, if the main concern of NW configuring SBFD is for coverage enhancement, the NW can configure the UE to select additional ROs when the RSRP threshold of the selected SSB/CSI-RS is below a configured RSRP threshold. Another example is that if the NW configures additional ROs only for larger RACH capacity and does not mean to have severe UE-to-UE CLI across different SBFD-capable cells, the NW can configure the UE to select additional ROs when the RSRP threshold of the selected SSB/CSI-RS is above a configured RSRP threshold. From the given examples, we can see how the UE prioritizes selection of additional ROs with the configured RSRP threshold depends on the target the NW wants to achieve.

Observation 1: How the UE selects SBFD RO with SSB RSRP condition should depend on the use case, e.g. for minimizing CLI across cells/UEs, or for coverage enhancement.

Therefore, we propose that the NW can configure two kinds of SSB RSRP thresholds for different usages, e.g. Threshold1 or Threshold2. When the NW configures additional RO for coverage enhancement, the NW configures Threshold1. The UE selects additional RO if the measured SSB RSRP is below Threshold1. When the NW configures additional RO for RACH capacity enhancement and aims to minimize inter-cell UE-to-UE CLI, the NW configures Threshold2. The UE selects additional RO if the measured SSB RSRP is above Threshold2. However, the NW should only configures either Threshold1 or Threshold2.

Proposal 2: For determining RO type with SSB RSRP, the NW can configure either Threshold1 or Threshold2, and the UE:

- Selects additional RO when the measured SSB RSRP is above Threshold1, if Threshold1 is configured;

- Selects additional RO when the measured SSB RSRP is below Threshold2, if Threshold2 is configured;

The RRC rapporteur suggests in the post email discussion that SSB threshold may not be configured by the network. We think this is against what we have agreed during last meeting, i.e. if the NW does not indicate RO type, the UE selects RO based on SSB RSRP threshold. Therefore, we propose RAN2 to confirm that, if the NW does not indicate RO type, the NW will configure an SSB RSRP threshold for RO type selection.

Proposal 3: If the NW does not provide RO type indication, the NW will configure an SSB RSRP threshold for RO type selection.

Order of RO type selection and RA type selection

RAN2 discussed about the order of RO type selection and RA type selection at last meeting [1]:

FFS whether RO type selection is performed before or after the RA type selection.

RAN1 has agreed that [2]:

Agreement For RACH configuration Option 2, support separate configuration of rsrp-ThresholdMsg1-RepetitionNum2/4/8 and msg1-RepetitionNum for PRACH transmission with preamble repetitions within additional-ROs and PRACH transmission with preamble repetitions within legacy-ROs. - msg1-RepetitionNum is configured in RACH-ConfigCommon in current specification, i.e., for RACH configuration Option 2, msg1-RepetitionNum can already be separately configured for legacy-ROs and additional-ROs. Agreement For RACH configuration Option 1, support separate configuration of rsrp-ThresholdMsg1-RepetitionNum2/4/8 for PRACH transmission with preamble repetitions within additional-ROs and PRACH transmission with preamble repetitions within legacy-ROs. - Same msg1-RepetitionNum is used for PRACH transmission with preamble repetitions within additional-ROs and PRACH transmission with preamble repetitions within legacy-ROs.

According to the current specification, the overall procedure is as below:

UL carrier selection(NUL/SUL)

-> BWP selection (DL/UL BWP对齐)

-> RACH resource (with features) set selection

-> 2/4 step RA type determination (based on triggering events & CFRA configuration)

-> initialization of variables specific to 2/4 step RA type

-> RACH resource selection according to determined 2/4 step RA type (SSB -> set preamble_index -> RO selection)

We can separately analyze the consequence if RO type selection is performed before or after the RA type selection:

- Way forward 1: RO type selection is performed before the RA type selection.

Since RAN1 has agreed on separate Msg1 repetition configuration for SBFD, it will affect how the UE determine the available resource set for Msg1 repetition. So it is reasonable that RO type selection is performed before RACH resource (with features) set selection and hence also before the RA type selection.

However, if RO type selection is before RA type selection, it is possible that the UE first determine to use SBFD RO and then determines to use 2-step RA type, but RAN1 has no conclusion whether to support 2-step RACH [3].

Conclusion: There is no consensus in RAN1 to support Type-2 random access procedure (2-step RACH) in SBFD symbols for SBFD-aware UEs.

We assume that for 2-step RACH, there is no need to configure SBFD RO for such RA type. One solution is to rely on the NW not to configure SBFD RO for 2-step RACH, to avoid the case that the UE first determines to use SBFD RO and then determines to set the RA_TYPE to 2-stepRA.

- Way forward 2: RO type selection is performed after the RA type selection.

If RO type selection is performed after the RA type selection, one straight forward question is how UE performs RACH resource set selection. One solution may be that the UE first use legacy Msg1 repetition configuration to determine available resource set, and then if the UE selects SBFD RO after the RA type selection, the UE use the SBFD-related Msg1 repetition configuration to re-determine the available resource sets for Msg1 repetition. We think this is complicated from the specification implementation perspective.

Taking the above analysis into consideration, we think it is more preferred that RO type selection is performed before the RA type selection.

Proposal 4: The RO type selection is performed before the RA type selection.

Observation 2: RAN1 has no consensus on whether to support 2-step RACH.

Proposal 5: Not support 2-step RACH for SBFD.

Proposal 6: Rely on the NW configuration not to configure SBFD RO for 2-step RACH, to avoid the case that the UE first determines to use SBFD RO and then determines to set the RA_TYPE to 2-stepRA.

PRACH transmission re-attempt

During R2#127bis meeting, RAN2 agreed as follows on PRACH transmission re-attempt [4]:

For the PRACH transmission re-attempt in one RACH procedure, after certain (configured) number of times of RACH attempt in SBFD RACH occasions, UE is allowed to switch to legacy RACH occasions. FFS about the case when UE select legacy ROs first.

RAN2 also discussed it during last meeting. Yet no consensus was reached [1]:

- FFS if switching from the PRACH resources in non-SBFD symbols to the PRACH resources in SBFD symbols is supported.

We think if the UE fails in accessing the network with legacy ROs, it may imply that the load on legacy ROs is congested, or the UE is at the cell edge. From this understanding, it is beneficial for the SBFD UE to switch to additional SBFD ROs for further re-attempt. Thus, we propose the UE switch to additional SBFD ROs after a configured number of times of RACH attempt in legacy RACH occasions, if the UE selects legacy ROs first.

Proposal 7a: If the UE selects legacy ROs first for a RACH procedure, it can switch to additional SBFD ROs after a configured number of times of RACH attempt in legacy ROs.

However, this leads to another question that whether the UE is allowed to switch the RO type back and forth repetitively during one RACH procedure. For instance, the NW set N1 = 2 for the UE to switch from legacy RO to SBFD RO, and set N2 = 3 for the UE to switch from SBFD RO to legacy RO, and the maximum PRACH transmission attempts number = 10. There are two options on how the UE performs RO type switching during one RACH procedure:

- Option 1: The UE is allowed to switch RO type for more than once during one RACH procedure.

If the UE first selects legacy RO, the UE may transmits 2 times on legacy RO -> and then 3 times on SBFD RO -> and then 2 times on legacy RO -> and then 3 times on SBFD RO until the total transmission number reach 10.

- Option 2: The UE is not allowed to switch RO type for more than once during one RACH procedure.

If the UE first selects legacy RO, the UE may transmits 2 times on legacy RO -> and then 8 times on SBFD RO until the total transmission number reach 10; If the UE first selects SBFD RO, the UE may transmits 3 times on SBFD RO -> and then 7 times on SBFD RO until the total transmission number reach 10.

We think repetitive RO type switching doesn’t make much sense and the gain is not clear. Besides, it brings complexity to power control and therefore we propose to follow option 2.

Proposal 7b: If P7a is agreed and the NW enables both direction of RO type switching, the UE is not allowed to switch RO type for more than once during one RACH procedure.

SSB/CSI-RS selection

SSB/CSI-RS selection is a step in random access procedure. If not indicated by the NW, the UE compares the RSRP of the SSB/CSI-RS with the threshold and select an SSB/CSI-RS with SS-RSRP/CSI-RSRP above rsrp-ThresholdSSB/rsrp-ThresholdCSI-RS. The UE then determines RO based on the selected SSB/CSI-RS [5].

In SBFD, legacy ROs and additional ROs can be both mapped to the same SSB/CSI-RS. Considering the NW can evaluate the CLI in SBFD resources based on the UE reports and adjust the SBFD configuration accordingly, the NW may configure different threshold for the UE to select additional ROs or legacy ROs to minimize the CLI in SBFD resources. However, we may just rely on the legacy SSB/CSI-RS selection method. Therefore, we propose RAN2 to confirm that SBFD operation has no impact on SSB/CSI-RS selection for random access procedure.

Proposal 8: Legacy SSB/CSI-RS selection procedure is reused for random access procedure with SBFD operation.

Conclusion

Based on the discussion, we have the following observation and proposals:

Proposal 1: For initial RA transmission, the network can indicate the RO type (legacy RO or additional RO) to the SBFD-aware UE for the case of CBRA in SIB1.

Observation 1: How the UE selects SBFD RO with SSB RSRP condition should depend on the use case, e.g. for minimizing CLI across cells/UEs, or for coverage enhancement.

Proposal 2: For determining RO type with SSB RSRP, the NW can configure either Threshold1 or Threshold2, and the UE:

- Selects additional RO when the measured SSB RSRP is above Threshold1, if Threshold1 is configured;

- Selects additional RO when the measured SSB RSRP is below Threshold2, if Threshold2 is configured;

Proposal 3: If the NW does not provide RO type indication, the NW will configure an SSB RSRP threshold for RO type selection.

Proposal 4: The RO type selection is performed before the RA type selection.

Observation 2: RAN1 has no consensus on whether to support 2-step RACH.

Proposal 5: Not support 2-step RACH for SBFD.

Proposal 6: Rely on the NW configuration not to configure SBFD RO for 2-step RACH, to avoid the case that the UE first determines to use SBFD RO and then determines to set the RA_TYPE to 2-stepRA.

Proposal 7a: If the UE selects legacy ROs first for a RACH procedure, it can switch to additional SBFD ROs after a configured number of times of RACH attempt in legacy ROs.

Proposal 7b: If P7a is agreed and the NW enables both direction of RO type switching, the UE is not allowed to switch RO type for more than once during one RACH procedure.

Proposal 8: Legacy SSB/CSI-RS selection procedure is reused for random access procedure with SBFD operation.

References

- R2#129 meeting minutes

- R1#119 meeting minutes

- R1#120 meeting minutes

- R2#127bis meeting minutes

- TS 38.321, V18.4.0