3GPP TSG-RAN WG2 Meeting #129bis	R2-2502388

Wuhan, China, Apr. 7th – 11th , 2025

Agenda Item:	8.11.3

Source: 	vivo

Title:         	SBFD other aspects

Document for: 	Discussion and Decision

Introduction

In this contribution, we will discuss about SBFD other aspects.

Discussion

CLI reporting and triggering configuration

RAN1 discussed about CLI reporting, down-selected from several existing CSI reporting IEs and agreed to reuse resourcesForChannelMeasurement [1].

In CSI-ReportConfig, reuse resourcesForChannelMeasurement providing the ID of a CSI-ResourceConfig that contains configuration of measurement resources for L1-SRS-RSRP and/or L1-CLI-RSSI.

As the issue is proposed in [2] and later confirmed during offline email discussion [3], we agree to confirm RAN1 agreement on CLI reporting.

Proposal 1: For L1 based UE-to-UE CLI reporting configuration, the resourcesForChannelMeasurement is reused for CLI measurement with the associated ID of CLI resource, i.e., SRS-RSRP resource and CLI-RSSI resource.

For CLI reporting triggering configuration, it is also proposed to confirm RAN1 agreement [2]. We share the same view and  do not observe the need for further discussion.

Proposal 2: Reuse CSI-AperiodicTriggerStateList to configure the trigger state associated with one or more aperiodic CSI-ReportConfig.

Conclusion

Based on the discussion, we have the following observation and proposals:

Proposal 1: For L1 based UE-to-UE CLI reporting configuration, the resourcesForChannelMeasurement is reused for CLI measurement with the associated ID of CLI resource, i.e., SRS-RSRP resource and CLI-RSSI resource.

Proposal 2: Reuse CSI-AperiodicTriggerStateList to configure the trigger state associated with one or more aperiodic CSI-ReportConfig.

References

- R1#118bis meeting minutes

- R2-2501130, Other aspects of SBFD, Qualcomm Incorporated

- [Post129][217][SBFD] List of open issues of RRC impact (Huawei)