GPP TSG-RAN WG2#129bis   									           R2-2502402

Wuhan, China, Apr. 7th – 11st, 2025

Agenda Item:	8.9.3

Source: 	NEC

Title:	        Discussion on UL Capacity Enhancement for IoT-NTN

Document for:	 Discussion and Decision

- Introduction

In this contribution, we follow the RAN2#129 agreements and rapporteur’s report on [Post129][307][R19 IoT NTN] CB-msg3/CB-msg4, and further discuss the FFS points and other points for CB-msg3 EDT on the following topics:

- CB-msg3 resource configuration: DSA window, Frequency domain configuration and OCC

- Contention resolution: single RNTI / multiple RNTIs, CB-msg4 monitoring window starting time

- CB-msg4 design

- CB-msg3 EDT re-attempt and fallback handling

- Discussion

RAN1 has replied to RAN2 LS on OCC for CB-msg3 NPUSCH and confirmed that the OCC solution that RAN1 is working on might be applied to CB-msg3 NPUSCH format 1 single tone at least for OCC length 2, for the subcarrier spacing (s) for which CB-msg3 is supported when the power imbalance is small [1].

Fig. 1 below illustrates the CB-msg3 with multiple resources in the time, frequency, and code domains (if considered in R-19).

Fig. 1 CB-msg3

  - CB-msg3 PUSCH resources configuration

For the legacy NB-IoT random access procedure, the number of RUs and TBS for EDT are defined in Table 16.3.3-2 and Table 16.3.3-3 in [2], and EDT-TBS-InfoList-NB-r15 in [3] as below:

Table 16.3.3-2: EDT TBS for Msg3 NPUSCH with edt-SmallTBS-Enabled set to 'true' [2]

| edt-TBS | edt-SmallTBS-Subset | Allowable TBS values |
| --- | --- | --- |
| 408 | not configured | 328, 408 |
| 504 | not configured | 328, 408, 504 |
| 504 | enabled | 408, 504 |
| 584 | not configured | 328, 408, 504, 584 |
| 584 | enabled | 408, 584 |
| 680 | not configured | 328, 456, 584, 680 |
| 680 | enabled | 456, 680 |
| 808 | not configured | 328, 504, 680, 808 |
| 808 | enabled | 504, 808 |
| 936 | not configured | 328, 504, 712, 936 |
| 936 | enabled | 504, 936 |
| 1000 | not configured | 328, 536, 776, 1000 |
| 1000 | enabled | 536, 1000 |

Table 16.3.3-3: MCS index for Msg3 NPUSCH and EDT [2]

| MCS Index | Number of RUs |  |  |
| --- | --- | --- | --- |
|  | edt-TBS = 328, 408, 504, or 584 | edt-TBS = 680 | edt-TBS = 808, 936, or 1000 |
| '011' | 3 | 3 | 4 |
| '100' | 4 | 4 | 5 |
| '101' | 5 | 5 | 6 |
| '110' | 6 | 8 | 8 |
| '111' | 8 | 10 | 10 |

EDT-TBS-InfoList-NB-r15 ::=	SEQUENCE (SIZE (1.. maxNPRACH-Resources-NB-r13)) OF EDT-TBS-NB-r15

EDT-TBS-NB-r15 ::=	SEQUENCE {

edt-SmallTBS-Enabled-r15		BOOLEAN,

edt-TBS-r15						ENUMERATED {b328, b408, b504, b584, b680, b808, b936, b1000}

}

Observation 1: The candidate RU numbers for EDT are 3, 4, 5, 6, 8, and 10, with each RU number covering various TBSs in the range of 328 to 1000.

The duration of a RU in different SCS and tone configurations for frame structure type1 and NPUSCH format 1 are defined in Table 10.1.2.3-1 of [4] as below:

Table 10.1.2.3-1: Supported combinations of , , and  for frame structure type 1[4]

| NPUSCH format |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | 3.75 kHz | 1 | 16 | 7 |
|  | 15 kHz | 1 | 16 |  |
|  |  | 3 | 8 |  |
|  |  | 6 | 4 |  |
|  |  | 12 | 2 |  |
| 2 | 3.75 kHz | 1 | 4 |  |
|  | 15 kHz | 1 | 4 |  |

Observation 2: The duration of one replica depends on the required number of RUs, the SCS, and the number of subcarriers.

Fig. 2 Different CE levels with different RU numbers and repetition numbers

Observation 3: For NB-IoT, the time duration for one CB-msg3 resource could be up to 40.96 s (i.e. 3.75 kHz SCS, 10 RUs, 160 slots, 128 repetitions), which is longer than the duration of one hyper-frame.

Observation 4: For NB-IoT, the time duration for one CB-msg3 resource might neither divides one hyper-frame duration nor is divisible by one hyper-frame duration.

Proposal 1: For the time domain CB-msg3 resource configuration parameters, both hyper-frame number (H-SFN) and frame number (SFN) are needed.

Proposal 2: For the DSA window starting time:

Option 1: DSA window starts with the CB-msg3 resource together.

Option 2: DSA window stars with an offset to the CB-msg3 resource.

Option 3: DSA window stars at a configured H-SFN and SFN.

A DSA window would allow UEs to select CB-msg3 resources across different time domains for replica transmissions when the number of replicas is larger than 1. The window size should be proportional to the CB-msg3 resource periodicity (i.e., scaled by the replica count). Thus, the DSA window size can be implicitly derived from replica numbers and CB-msg3 periodicity without explicit network signaling.

Proposal 3: The DSA window size may be:

Option 1: Implicitly indicated by the DSA replica number.

Option 2: Explicitly indicated by the network.

Since CB-msg3 resources are periodic, the DSA window will recur with a periodicity scaled by the CB-msg3 resource period. Consequently, the DSA window periodicity can also be implicitly derived.

Proposal 4: The DSA window periodicity may be:

Option 1: Implicitly indicated by the DSA replica number.

Option 2: Explicitly indicated by the network.

If both the DSA window size and DSA window periodicity are adopted, the network may configure the periodicity to be equal to or larger than the window size. This configuration would create temporal gaps between consecutive DSA windows, leading to inefficient utilization of time-domain resources.

Furthermore, according to RAN2's working assumption, the UE operation involves two sequential steps: first selecting the next DSA transmission window, then randomly choosing K replicas within the selected window. Configuring the periodicity to exceed the window size would not only result in resource fragmentation but also introduce unnecessary delays in DSA CB-msg3 transmissions.

Proposal 5: Eliminate the DSA window periodicity parameter by aligning it with the DSA window size (i.e., setting periodicity equal to window size).

For the CB-msg3 frequency domain resource configurations, the legacy NPRACH configures a set of contiguously allocated subcarriers as a pool for the UEs to choose one of the subcarrier to send the preambles by nprach-SubcarrierOffset and nprach-NumSubcarriers, and the legacy PUR configures a set of contiguously allocated subcarriers by the prb-AllocationInfo or npusch-SubCarrierSetIndex in PUR-Config.

Observation 5: For legacy PUR and NPRACH, the frequency domain resource is a set of contiguously allocated subcarriers.

Proposal 6: RAN2 further discusses the following frequency-domain configuration options:

Option 1: A single contiguous subcarrier set, i.e. one subcarrier starting point with one range

Option 2: Multiple contiguous subcarrier sets, i.e. several subcarrier starting points with corresponding ranges

Proposal 7: RAN2 to request RAN1 to validate the completeness of CB-msg3 parameter lists.

In the last meeting, we have the following working assumption:

Working assumption: The UE first selects the next DSA transmission window and then randomly select K replicas inside the window.

When CB-msg3 EDT procedure is triggered, the UE will determine the indexes of the replica occasions randomly. If the determined first replica is after the triggering timing, the UE will transmit CB-msg3 since the replica occasion. Taking Fig. 3 as an example, the UE determines the indexes of replica occasions as 3, 5, and 6, when the CB-msg3 EDT procedure is triggered. As the timing of the first occasion (occasion 3) is after the triggering time, the UE transmits CB-msg3 since the occasion 3. Taking Fig. 4 as an example, the UE determines the indexes of replica occasions as 1, 3, and 5, when the CB-msg3 EDT procedure is triggered. As the timing of the first occasion (occasion 1) is before the triggering time, the UE transmits CB-msg3 since the occasion 1 in the N+1 DSA window.

Using this procedure, the UE could transmit CB-msg3 timely without any unnecessary latency.

Fig. 3 Determined replica occasion instance

Fig. 4 Another determined replica occasion instance

Proposal 8: The next DSA window is the first window instance containing all the selected replica occasions after CB-msg3 is triggered.

Given RAN1’s emphasis on power imbalance for OCC-based CB-msg3 NPUSCH, narrower RSRP thresholds between CE levels could mitigate power imbalance among UEs.

Observation 6: The network could control the CB-msg3 EDT power imbalance between different UEs through the RSRP threshold configuration.

Proposal 9: The network should configure at least two RSRP thresholds with a narrow gap to enable CB-msg3 EDT with OCC.

Proposal 10: RAN2 to request RAN1 to assess the feasibility of enabling OCC-based CB-msg3 via narrow RSRP gaps.

  - Contention resolution

RAN2 has agreed that, for DSA, once the eNB successfully decodes one of the multiple replicas, it may respond without waiting for the remaining replica(s) and FFS when the response window(s) is/are started (or restarted) and stopped.

For NB-IoT, only type-B half-duplex FDD operation is supported [4]. In this type of operation, guard periods (GP, illustrated in Fig. 4), each referred to as a half-duplex guard subframe, are created by the UE by

-	not receiving a downlink subframe immediately preceding an uplink subframe from the same UE, and

-	not receiving a downlink subframe immediately following an uplink subframe from the same UE.

Fig. 4 Type-B half-duplex FDD

The legacy RA Response window starts at the subframe that contains the end of the last preamble repetition plus X subframes plus UE-eNB RTT and has length ra-ResponseWindowSize for the corresponding enhanced coverage level, where value X is determined based on the used preamble format and the number of NPRACH repetitions[5].

For DSA, if the Msg4 window starts at the subframe that contains the end of the last NPUSCH repetition plus X subframes plus UE-eNB RTT, it might 1) conflict with the terminal’s UL subframes with a small X value; or 2) introduce additional latency with a large X value as illustrated in Fig. 5.

Fig. 5 Legacy response window

Observation 7: Legacy Msg4 window design is incompatible with NB-IoT’s half-duplex constraints in DSA CB-msg3 EDT.

For DSA, one Msg4 monitoring window could start with:

- each replica transmission

- each DSA window

When the window starts with each replica transmission, the relation between the CB-msg3 occasion and the response window is illustrated in Fig. 6.

To prevent the misdetection of the Msg4 due to the type-B half-duplex FDD constraint, each CB-msg3 occasion should have at least periodicity larger than the UE-eNB RTT + response window size + guard period. Under the GEO scenario, the UE-eNB RTT would be above 480 ms. This means for the system the delay of one CB-msg3 EDT could be more than 2 s even without counting the duration of each CB-msg3 occasion in different CE levels. Starting the CB-msg4 monitoring window with each replica transmission is obviously against the design target of CB-msg3 EDT to cut off unnecessary delay in RTT level.

Fig. 6 The Msg4 monitoring window starts with each replica transmission

Observation 8: For NB-IoT, starting the CB-msg4 monitoring window with each replica transmission will result in a huge system delay, which is against the working target of CB-msg3 EDT to cut off the RTT level delays.

When the window starts with each DSA window, the relation between CB-msg3 occasion and the Msg4 monitoring window is illustrated in Fig. 7.

Fig. 7 Msg4 monitoring window starts with each DSA window

The window starts with each DSA window having no constraint to the CB-msg3 occasion periodicity within one DSA window. Each CB-msg4 occasion can be configured close to each other. For the system, the delay of one CB-msg3 EDT could be one UE-eNB RTT (480 ms), the duration of each CB-msg3 occasion in different CE levels, guard period, and potential processing time. As RAN2 agreed that the UE shall at most have one ongoing CB EDT procedure at any time, there are no constraints on the periodicity of the DSA window.

Observation 9: For NB-IoT, starting the Msg4 monitoring window with each DSA window is the only viable solution.

For eMTC, there is no type-B half-duplex FDD operation constraint. Multiple Msg4 monitoring windows are also feasible. To reduce impacts on specifications and UE implementation, a unified design between eMTC and NB-IoT is preferred.

Proposal 11: Adopt a unified DSA framework for NB-IoT and eMTC.

Proposal 12: To unify the NB-IoT and eMTC design, for each DSA CB-msg3 EDT, there is only one Msg4 monitoring window, and the window starts with the corresponding DSA window with accounting of the UE-eNB RTT and processing time.

FFS: The processing time and the window length.

For the DSA RNTI, it can be either a single RNTI or multiple RNTIs as illustrated in Fig. 8 below.

Fig. 8 Single RNTI or Multiple RNTIs for DSA

Regardless of the RNTI strategy, multiple UEs may select PUSCH resources associated with the same RNTI, requiring contention resolution via CB-Msg4 PDSCH decoding.

Proposal 13: Confirm the working assumption that one CB-msg4 can target multiple UEs simultaneously.

For multiple RNTIs for one DSA window, the RNTI can be determined based on the UE selection of the jth CB-msg3 Occasion, which is illustrated as option 1. The CB-msg4 PDCCH will first filter by the RNTI corresponding to the chosen CB-msg3 occasion. For any further contentions resolution, the UE has to decode and check the CB-msg4 PDSCH. RAN2 has agreed that the maximum replica number is 4 [6], thus the worst case is one UE has to detect four different RNTIs to find the right CB-msg4 PDCCH. If the network successfully decodes the CB-msg3 in the first replica transmission, the UE could stop monitoring CB-msg4 PDCCH with other RNTIs. As the RNTI already resolves partial or all of the contentions, the contention resolution in PDSCH is reduced. The decoding complicity of the PDSCH can be reduced. As the decoding reliability of a control channel is always higher than the data channel, option 1 ensures the miss detection rate and false alarm rate via the PDCCH detections.

For a single RNTI for one DSA window, the RNTI can be determined based on the UE selection of the ith DSA window, which is illustrated as option 2. The CB-msg4 PDCCH will firstly filter by the RNTI corresponding to the chosen CB-msg3 DSA window. For any further contentions resolution, the UE has to decode and check the Msg4 PDSCH. As all the CB-msg3 EDT in one DSA window share one RNTI, the UE has to decode the PDSCH to resolve the contentions in one CB-msg3 DSA window. Due to the multiplexing of different UEs from one DSA window in one CB-msg4 PDSCH message, the CB-msg4 PDSCH payload size of option 2 will be a multiple of the payload size of option 1. The PDSCH decoding complexity grows up as well as the Msg4 miss detection rate and false alarm rate. Plus, if multiple Msg4 monitoring windows are configured, a UE might need to detect the PDSCH several times, which results in higher decoding complexity, higher miss detection rate, and higher false alarm rate.

For a single RNTI for one DSA window, the RNTI can also be determined based on the UE selection of the kth CE Level or just related to CB-msg3 EDT, which is illustrated as option 3. Then all the contention resolutions have to be solved by the CB-msg4 PDSCH. Then the PDSCH decoding complexity is higher than option 2 as well as the miss detection rate and false alarm rate. However, option 3 simplifies the RNTI design and the PDCCH detection.

Observation 10: Single-RNTI solutions rely heavily on PDSCH for contention resolution, while multiple-RNTI approaches leverage PDCCH for partial resolution. PDCCH offers lower decoding complexity and higher reliability than PDSCH. Different options have different impacts on the UE power consumption and CB-msg3 contention resolution reliability.

Proposal 14: RAN2 to request RAN1 to evaluate the feasibility of the following RNTI strategies for DSA CB-msg3 EDT response:

Option 1: RNTI determined by the jth CB-msg3 occasion transmitting the CB-msg3 EDT

Option 2: RNTI determined by the ith DSA window transmitting the CB-msg3 EDT

Option 3: RNTI determined by the kth CE Level transmitting the CB-msg3 EDT or the CB-msg3 EDT behavior

  - CB-msg4 delivery

For CB-msg4, besides Contention Resolution ID, for CP-CIoT optimizations, if there is NAS signalling or DL data, RRCEarlyDataComplete message including NAS signalling or DL data can be transmitted to the UE. For UP-CIoT optimizations, RRCConnectionRelease message and possible DL data can be transmitted to the UE. If CB-msg4 from multiple UEs are multiplexed into one MAC PDU, considering there may be DL data besides RRC message for some UEs, CB-msg4 will be very large and the multiplexing is very complex. Besides, this multiplexing can also cause some decoding delay. Hence, if RRC message and/or DL data from multiple UEs exists, only one UE’s RRC message and/or DL data can be included in CB-msg4 similar to MSGB of 2-step RACH procedure.

- Proposal 15: There is only one UE’s RRC message and/or DL data that could be included in CB-msg4.

If RRC message and/or DL data for some UEs are not included in CB-msg4, once contention resolution is successful, the UE may further receive RRC message and/or DL data if exists. Moreover, NW may would like to move UE into RRC_CONNECTED mode. For both cases, C-RNTI should be included in CB-msg4. For CB-EDT procedure, once the contention resolution succeeds, the UE will monitor PDCCH scrambled by the indicated C-RNTI in the subsequent time similar to MSGB of 2-step RACH procedure.

- Proposal 16: C-RNTI could be included in CB-msg4.

Besides, during 4-step RACH and 2-step RACH procedures, when the contention resolution completes successfully, the UE can transmit a positive acknowledgement to the network. Similarly, for CB-EDT procedure, the UE also transmits acknowledgement to the network once contention resolution is successful. With this feedback information, the network may perform the subsequent scheduling for RRC message/DL data if exists, such that resource waste can be avoided in case contention resolution is not successful.

To perform HARQ feedback, the corresponding HARQ feedback resource should be indicated to the UE. In the current specification, the HARQ feedback resource for Msg4 can be indicated to the UE by DCI. However, for CB-EDT procedure, if CB-msg4 from multiple UE can be multiplexed into one MAC PDU, the HARQ feedback resource should be included in MAC PDU other than DCI.

- Proposal 17: HARQ feedback resource for each UE could be included in CB-msg4.

Based on the analysis above, the following information can be included in CB-msg4:

- Contention resolution ID

- C-RNTI

- HARQ feedback resource

- RRC message and/or DL data

For the first three contents above, similar to MSGB of 2-step RACH procedure, a unified MAC CE can include them and an example is shown in Fig. 9 below.

Fig. 9 One unified MAC CE design included in CB-msg4

- Proposal 18: For CB-Msg4, contention resolution ID, C-RNTI and HARQ feedback resource could be included one unified MAC CE.

  - Failure handling and fallback

The behavior of the UE needs to be defined if it fails to receive the corresponding contention resolution/response in the Msg4 monitoring window, similar to the 2 or 4-step RACH procedure. Firstly we could allow the UE to re-attempt CB-msg3 transmission for a configured number of times before declaring complete failure or moving to next step. Whether and how to implement power ramping for CB-msg3 re-attempt could be further investigated. Secondly, The network must control and distribute the load over the CB-PUSCH resources, especially for the re-attempts. For instance, when the load is temporarily too high and NW has detected high collision, the network could instruct the UE to apply a backoff for upcoming re-attempt over CB-msg3 resources to avoid that almost the same set of UEs repeatedly re-attempt at the same time and fail.

- Proposal 19: NW configures a maximum number of times a UE can re-attempt CB-msg3 transmission on the same CE.

- Proposal 20: Msg4 could include a backoff indication for UE to determine the backoff value for re-attempt.

If CB-msg3 transmission completely fails after maximum time of re-attempt, the UE can fall back to the normal RACH-based solution, similar to the 2-step RACH falling back to 4-step RACH. In addition, it might be possible for NW to ask UEs to fallback to legacy RACH procedure e.g., when the NW detects high load and would like to offload UEs from re-attempting over CB-POs.

- Proposal 21: UE falls back from CB-msg3 to 4-step RACH procedure after the maximum times of re-attempts or a fallback MAC CE is received.

- Conclusion

This contribution provides our observation and proposals for UL capacity enhancement as listed below:

Observation 1: The candidate RU numbers for EDT are 3, 4, 5, 6, 8, and 10, with each RU number covering various TBSs in the range of 328 to 1000.

Observation 2: The duration of one replica depends on the required number of RUs, the SCS, and the number of subcarriers.

Observation 3: For NB-IoT, the time duration for one CB-msg3 resource could be up to 40.96 s (i.e. 3.75 kHz SCS, 10 RUs, 160 slots, 128 repetitions), which is longer than the duration of one hyper-frame.

Observation 4: For NB-IoT, the time duration for one CB-msg3 resource neither divides one hyper-frame duration nor is divisible by one hyper-frame duration.

Observation 5: For legacy PUR and NPRACH, the frequency domain resource is a set of contiguously allocated subcarriers.

Observation 6: The network could control the CB-msg3 EDT power imbalance between different UEs through the RSRP threshold configuration.

Observation 7: Legacy Msg4 window design is incompatible with NB-IoT’s half-duplex constraints in DSA CB-msg3 EDT.

Observation 8: For NB-IoT, starting the CB-msg4 monitoring window with each replica transmission will result in a huge system delay, which is against the working target of CB-msg3 EDT to cut off the RTT level delays.

Observation 9: For NB-IoT, starting the Msg4 monitoring window with each DSA window is the only viable solution.

Observation 10: Single-RNTI solutions rely heavily on PDSCH for contention resolution, while multiple-RNTI approaches leverage PDCCH for partial resolution. PDCCH offers lower decoding complexity and higher reliability than PDSCH. Different options have different impacts on the UE power consumption and CB-msg3 contention resolution reliability.

Proposal 1: For the time domain CB-msg3 resource configuration parameters, both hyper-frame number (H-SFN) and frame number (SFN) are needed.

Proposal 2: For the DSA window starting time:

Option 1: DSA window starts with the CB-msg3 resource together.

Option 2: DSA window stars with an offset to the CB-msg3 resource.

Option 3: DSA window stars at a configured H-SFN and SFN.

Proposal 3: The DSA window size may be:

Option 1: Implicitly indicated by the DSA replica number.

Option 2: Explicitly indicated by the network.

Proposal 4: The DSA window periodicity may be:

Option 1: Implicitly indicated by the DSA replica number.

Option 2: Explicitly indicated by the network.

Proposal 5: Eliminate the DSA window periodicity parameter by aligning it with the DSA window size (i.e., setting periodicity equal to window size).

Proposal 6: RAN2 further discusses the following frequency-domain configuration options:

Option 1: A single contiguous subcarrier set, i.e. one subcarrier starting point with one range

Option 2: Multiple contiguous subcarrier sets, i.e. several subcarrier starting points with corresponding ranges

Proposal 7: RAN2 to request RAN1 to validate the completeness of CB-msg3 parameter lists.

Proposal 8: The next DSA window is the first window instance containing all the selected replica occasions after CB-msg3 is triggered.

Proposal 9: The network should configure at least two RSRP thresholds with a narrow gap to enable CB-msg3 EDT with OCC.

Proposal 10: RAN2 to request RAN1 to assess the feasibility of enabling OCC-based CB-msg3 via narrow RSRP gaps.

Proposal 11: Adopt a unified DSA framework for NB-IoT and eMTC.

Proposal 12: To unify the NB-IoT and eMTC design, for each DSA CB-msg3 EDT, there is only one Msg4 monitoring window, and the window starts with the corresponding DSA window with accounting of the UE-eNB RTT and processing time.

FFS: The processing time and the window length.

Proposal 13: Confirm the working assumption that one CB-msg4 can target multiple UEs simultaneously.

Proposal 14: RAN2 to request RAN1 to evaluate the feasibility of the following RNTI strategies for DSA CB-msg3 EDT response:

Option 1: RNTI determined by the jth CB-msg3 occasion transmitting the CB-msg3 EDT

Option 2: RNTI determined by the ith DSA window transmitting the CB-msg3 EDT

Option 3: RNTI determined by the kth CE Level transmitting the CB-msg3 EDT or the CB-msg3 EDT behavior

- Proposal 15: There is only one UE’s RRC message and/or DL data that could be included in CB-msg4.

- Proposal 16: C-RNTI could be included in CB-msg4.

- Proposal 17: HARQ feedback resource for each UE could be included in CB-msg4.

- Proposal 18: For CB-Msg4, contention resolution ID, C-RNTI and HARQ feedback resource could be included one unified MAC CE.

- Proposal 19: NW configures a maximum number of times a UE can re-attempt CB-msg3 transmission on the same CE.

- Proposal 20: Msg4 could include a backoff indication for UE to determine the backoff value for re-attempt.

- Proposal 21: UE falls back from CB-msg3 to 4-step RACH procedure after the maximum times of re-attempts or a fallback MAC CE is received.

- References

[1] R1-2410895

[2] TS 36.213

[3] TS 36.331

[4] TS 36.211

[5] TS 36.321

[6] RAN2#129 chairman notes

[7] Draft R2-250xxxx Post129_307_R19 IoT NTN CB-msg3_v19_Rapp