3GPP TSG-RAN WG2 Meeting #129bis                                                   R2-2502455

China, Wuhan, April 7th – 11th, 2025

Source:	Qualcomm Incorporated

Title:	Open issue for control plane

Agenda Item:	8.13.3

Document for:	Discussion and Decision

# Introduction & Background

In the WID [1], multi-hop U2N relay was approved as Rel-19 item, and this contributions discuss the contron plane issues.

- Specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay. The work starts with one additional hop relay (i.e., remote UE -> first relay UE -> last relay UE -> gNB) until RAN#107 and further check will be made in RAN#107 if it can be easily extended to two additional hops relays (i.e., remote UE -> first relay UE -> second relay UE -> last relay UE -> gNB). A necessary criterion for the specified mechanisms is easy extensibility to support two additional hop relays for the work done until RAN#107 and to be forward compatible for future extensions for additional relays.

  - Relay discovery and (re)selection [RAN2]

  - Signalling support for relay UEs and remote UE authorization if SA2 concludes it is needed [RAN3]

  - Impact on SRAP and QoS handling for multi-hop [RAN2]

  - Control plane procedures [RAN2, RAN3]

# Discussion

### 2.1 SI forwarding

In the last RAN2 meeting, there are some FFS issues left for SI forwarding.

The intermediate UE, when it receives SI from a parent relay, forwards the SI message only to the child UEs which requested the SI (i.e., the intermediate UE keeps track of the required SI for each child node).  FFS on the need for including a “remote UE ID” in the request.

The PC5-RRC message containing the required SI that is transmitted by the remote UE or by the intermediate relay UE to the parent node contains at least the requested SIB list.  FFS if the intermediate relay UE can respond directly instead of forwarding the request to the parent node if it has the requested SI.

The first FFS issue is whether a “remote UE ID” is needed in the SI request message. When an intermediate Relay UE receives a SI request from the child UE, the intermediate Relay UE should remember the child UE which requires the SI and determine whether the received SI to be forwarded to the child UE. Then there is no need include “Remote UE ID” in RemoteUEInformationSidelink and UuMessageTransferSidelink message for SI forwarding.

- There is no need to include “Remote UE ID” in RemoteUEInformationSidelink and UuMessageTransferSidelink message for SI forwarding, and each Relay UE should remember the child UE(s) requesting the SI according to L2 ID of the child UE, and forward the SI to the requesting child UE(s).

Another FFS issue is if the intermediate relay UE can respond directly instead of forwarding the request to the parent node if it has the requested SI. Each relay UE has the latest SI, then when the intermediate Relay UE receives SI requrest from the child UE and the intermediate relay UE already has the requested SI, the intermediate relay UE can respond the child UE with the requested SI insdead of forwording to the parent UE.

- If intermediate relay UE has already the requested SI, the intermediate relay can responds UuMessageTransferSidelink including the requested SI to the child UE insdeading of forwarding the request to the parent UE.

### 2.2 Paing forwarding

For paging forwording, RAN2 had agreements as following.

Agreements:

An intermediate relay UE in RRC_CONNECTED releases its paging-related information in the parent UE at least for itself.  FFS whether it also releases it for the child UEs and how the child UEs receive the paging when the intermediate relay UE is in RRC_CONNECTED.

When an intermediate relay UE is in RRC_IDLE/RRC_INACTIVE it can obtain paging from the parent relay UE in PC5-RRC (e.g., using RemoteUEInformationSidelink).  FFS if it can also receive it directly on Uu (when the intermediate relay UE is in coverage).

What triggers the intermediate relay UE to request paging monitoring by the parent relay (intermediate relay or last relay) in PC5-RRC (e.g., in RemoteUEInformationSidelink)?

•	FFS: when there is a change in the paging information of the intermediate UE or child UE

•	when the intermediate UE enters RRC_IDLE/RRC_INACTIVE

•	when the intermediate UE enters RRC_CONNECTED (to cancel paging monitoring request)

•	upon reception of paging monitoring request from a remote UE/child relay UE

•	FFS: upon change in the ability of the intermediate UE to monitor paging on Uu (e.g., moving in/out of coverage) to initiate/cancel paging monitoring by the parent relay).

The intermediate relay UE sends paging message (e.g., in UuMessageTransferSidelink) to a child node

•	Upon reception of paging message received from a parent node (intermediate relay or last relay) that is intended for a child node (intermediate relay or remote UE)

•	FFS upon acquisition (from the network) of paging message that is for a child node (intermediate relay or remote UE)

FFS upon acquisition (from the network) by intermediate relay UE of paging message that is for a child node (intermediate relay or remote UE)

There could a case that the intermediate relay UE also in coverage, then one issue is whether the intermediate relay UE can monitor paging for the Remote UE instead of forwarding the paging request to the parent UE. Considering the intermediate relay UE is usually at cell edge, when the intermediate relay UE moves in coverage and out-of coverage, the intermediate relay UE needs to initiate/cancel paging monitoring by parent UE if the intermediate relay monitors paging message for the child UE. Then it is preferred only the last relay UE monitors paging for the remote UE and the intermediate Relay UE always forwards paging request to the parent UE if receiving from the child UE.

- Only the last Relay UE monitors paging for the remote UE.

- The intermediate does not monitor paging for the child UE over Uu interface.

- The intermediate Relay UE always forwards paging request to the parent UE if receiving from the child UE.

FFS whether it also releases it for the child UEs and how the child UEs receive the paging when the intermediate relay UE is in RRC_CONNECTED.

Since the intermediate Relay UE does not monitor paging for the child UE, then the intermediate Relay UE does not need to release paging information for the child UEs in the parent relay UE when the intermediate relay UE is in RRC_CONNECTED state.

- The intermediate Relay UE does not release paging information for the child UEs in the parent relay UE when the intermediate relay UE is in RRC_CONNECTED

FFS if intermediate relay UE can also receive it directly on Uu (when the intermediate relay UE is in coverage).

The intermediate relay is acting as a Remote UE, then it is allowed to monitor paging for itself by implementation.

- When an intermediate relay UE is in RRC_IDLE/RRC_INACTIVE acting as Remote UE functionality, it can obtain paging for itself directly on Uu by implementation when the intermediate relay UE is in coverage, like today’s Remote UE.

FFS: upon change in the ability of the intermediate UE to monitor paging on Uu (e.g., moving in/out of coverage) to initiate/cancel paging monitoring by the parent relay).

Considering the intermediate relay UE is usually at cell edge, when the intermediate relay UE moves in coverage and out-of coverage, the intermediate relay UE may needs to initiate/cancel paging monitoring by parent UE if the intermediate relay monitors paging message freeuntly. Then as today, the intermediate relay UE does not initiate or cancel paging monitoring by the parent relay UE.

- When an intermediate relay UE moves in or out of coverage, it does not initiate or cancel paging monitoring by parent relay UE.

### 2.3 Scenario restriction

RAN2 agreed to support a baseline procedure for the case in which the intermediate relay UE should enter CONNECTED state when the Remote UE goes to CONNECTED state. In this case, the intermediate Relay UE is also acting as a Remote UE, so the intermediate Relay UE cannot support different indirect paths to the gNB.

Support a baseline procedure for the case in which the intermediate relay UEs for a remote UE all transition to RRC_CONNECTED (if not already there) when the remote UE goes to RRC_CONNECTED.

Support this case based on existing U2N framework, with all the UEs RRC_CONNECTED to the last relay UE’s serving cell.

If the intermediate Relay UE also is acting as a Remote UE, it cannot support different indirect paths to the gNB with same/different last/U2N/parent intermediate Relay UE(s) based on different PC5 unicast links.

Taking the following as example, if the intermediate Relay UE already established red relay path for Remote UE(s), then the green relay path should not be established. How to avoid green relay path is the issue to be addressed.

Figure 1

The intention to establish different PC5 connection is for different RSCs, there could be two options to avoid establish different PC5 connection:

Option 1: The intermediate Relay UE only broadcasts one RSC in discovery message(s)

If the intermediate Relay UE has established PC5 connection for one RSC associated with one last Relay UE and transmitted in discovery message, the intermediate Relay UE does not transmit another RSC in discovery message. With this option, when the Remote UEs select relay path, the Remote UEs only considers the RSC for which the PC5 link is already established on red relay path.

Option 2: The intermediate Relay UE rejects the new relay establishment request for a different RSC

Once the intermediate relay UE established the red relay path, the intermediate Relay reject the request from the Remote UE to establish a new relay path. This option will consume discovery message transmission and Remote UE relay (re)selection procedure.

- If the intermediate Relay UE has established PC5 connection for one RSC associated with one last Relay UE, the intermediate Relay UE does not transmit other RSC(s) in any discovery message.

- Sends LS to SA2.

Another scenario is if the intermediate Relay UE has Uu connection with the different cell with the last Relay UE cell, e.g. in the following figure, the intermediate Relay UE established RRC connection with on green path for its own service towards same or different cell of the last relay UE, but the intermediate Relay UE receives relay request from the Remote UE to establish relay path, what the intermediate Relay UE should do.

Figure 2

Considering the existing mechanisms, there could be the following options to be considered

Option 1: The intermediate Relay UE performs path switch from direct path to indirect path.

Since the intermediate Relay UE is acting a Remote UE, when the intermediate Relay UE receives relay establishment request from Remote UE, the intermediate Relay UE performs path switch from cell2 to cell 1 using existing direct to indirect path switching.

Option 2: The UE does not act as an intermediate Relay UE if already established direct Uu connection

If the UE already established direct Uu connection, the UE does not transmit discovery message which includes other relay UE information. Then the UE will not act as an intermediate Relay UE.

Option 2 restricts the multi-hop relay applied scenario. Then it is proposed to adopt option 1.

- If a UE in CONNECTED state via direct path receives relay establishment request from the Remote UE or downstream intermediate Relay UE, the UE can perform direct path to indirect path switch to act as an intermediate Relay UE for the relay traffic and Remote UE for its own traffic

- gNB can configures the UE whether to perform direct path to indirect path switch.

# Conclusion

SI forwarding,

- There is no need to include “Remote UE ID” in RemoteUEInformationSidelink and UuMessageTransferSidelink message for SI forwarding, and the intermediate Relay UE can member the child UE(s) requesting the SI according to L2 ID of the child UE, and forward the SI to the requesting child UE(s).

- If intermediate relay UE has already the requested SI, the intermediate relay can responds UuMessageTransferSidelink including the requested SI to the child UE insdeading of forwarding the request to the parent node.

Paing forwarding

- Only the last Relay UE monitors paging for the remote UE.

- The intermediate does not monitor paging for the child UE over Uu interface.

- The intermediate Relay UE always forwards paging request to the parent UE if receiving from the child UE.

- The intermediate Relay UE does not release paging information for the child UEs in the parent relay UE when the intermediate relay UE is in RRC_CONNECTED.

- When an intermediate relay UE is in RRC_IDLE/RRC_INACTIVE acting as Remote UE functionality, it can obtain paging for itself directly on Uu by implementation when the intermediate relay UE is in coverage, like today’s Remote UE.

- When an intermediate relay UE moves in or out of coverage, it does not initiate or cancel paging monitoring by parent relay UE.

Scenario restriction

- If the intermediate Relay UE has established PC5 connection for one RSC associated with one last Relay UE, the intermediate Relay UE does not transmit other RSC(s) in any discovery message.

- Sends LS to SA2.

- If a UE in CONNECTED state via direct path receives relay establishment request from the Remote UE or downstream intermediate Relay UE, the UE can perform direct path to indirect path switch to act as an intermediate Relay UE for the relay traffic and Remote UE for its own traffic

- gNB can configures the UE whether to perform direct path to indirect path switch.

# Reference

[1]. RP-241609    New WID on NR sidelink multi-hop relay    LG Electronics, InterDigital, FirstNet