3GPP TSG-RAN WG2 Meeting #129bis	R2-2502457

Wuhan, China, 7th – 11th April, 2025

Agenda item:	8.9.3

Source:	Samsung

Title:	On procedures for contention-based Msg3

WID/SID:	IoT_NTN_Ph3-Core

Document for:	Discussion and Decision

# Introduction

In RAN#107 the latest WID for IoT NTN Phase 3 was agreed [1], where Capacity enhancements for uplink:

- Support of Capacity enhancements for uplink - Study then specify, if beneficial, enhancements to enable multiplexing of multiple UEs (e.g. up to the min of 4 and the maximum allowed by the existing UL and DL signalling) in a single 3.75 kHz or 15 kHz subcarrier via orthogonal cover codes (OCC) for NPUSCH format 1 and NPRACH [RAN1, RAN2, RAN4] - Multi-tone support for 15 kHz SCS should also be considered - Specify necessary signalling, if needed - Update RF requirements accordingly, if needed Note: Impact of impairment shall be taken into account - Study and specify, if beneficial the following enhancements to reduce the necessary uplink and downlink signaling to complete an Early Data Transmission (EDT) transaction [RAN2]: - Msg3 transmission without msg1/ Random Access Response (RAR) - Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete - Study and specify RRM requirement, if identified [RAN4]

In this contribution, we provide some discussions on the objective of Support of Capacity enhancements for the uplink.

# Discussion

- Agreements from RAN2#129 meeting:

Agreements:

1.	It is FFS if separate CB-msg3 resources would be needed for CB-msg-3 using OCC or if the same CB-msg3 resources could be used

2.	RAN2 assumes that one possibility to take power imbalance under control is to define RSRP ranges that need to be respected to transmit CB-msg3 using OCC

3.	RAN2 assumes that at least the following will be part of the shared resources configuration for CB-msg3 (FFS on other aspects)

-	Time domain resources for (N)PUSCH occasions: periodicity and start time (e.g., start subframe, start SFN)

-	Frequency domain resources for (N)PUSCH occasions

-	repetition number

-	(N)PDCCH resource

-	MCS

4.	For CB-msg3 transmission, for eMTC NTN, up to three separate RSRP thresholds (on top of the minimum RSRP threshold and possibly different from the thresholds for PRACH) can be supported for achieving at most 4 CE levels; for NB-IoT NTN, up to two separate RSRP thresholds (on top of the minimum RSRP threshold possibly different from the thresholds for PRACH) can be supported for achieving at most 3 repetition levels.

5.	The UE shall at most have one ongoing CB EDT procedure at any time.

6.	The CB EDT Config has one minimum RSRP threshold (as agreed in RAN2#128) to use CB EDT.

7.	The CB EDT Config has two RSRP thresholds for NB-IoT for the three CE levels.

8.	CB EDT Config has three RSRP thresholds for eMTC for the four CE levels.

7.	As Signalling design Baseline RAN2 assumes the PUR config and the NPRACH config for shared (N)PUSCH config can be used and some of the parameters can be included in a new CB EDT config.

8.	RAN2 consider a new CBEDT-ConfigSIB-NB IE for configuring the CB EDT feature

Working assumption:

- One CB-MSG4 can target multiple UEs simultaneously (FFS on the details)

- For CB-MSG3, the Transmission window is configured by the network with a starting point (e.g. H-SFN offset), a window length, and a window periodicity (window length and periodicity could be the same). For k=1 the window length can be equal to 1: same behaviour as today

The UE first selects the next DSA transmission window and then randomly select K replicasinside the window.

RAN2 assumes that a pointer solution is not needed in Rel-19

## CB-Msg4 targeting multiple UEs

In the last meeting the following working assumption was taken:

Working assumption:

- One CB-MSG4 can target multiple UEs simultaneously (FFS on the details)

The current Work Item Description is clear that RAN2 shall study and specify efficient delivery, i.e reduced overhead of Msg4. In our view, so far in our understanding the only proposed manner in which Msg4 is made more efficient is to target multiple UEs in a Msg4. Targeting multiple UEs in a Msg4 is in particular useful for CB-Msg3, as multiple UEs may use the same Msg3 resources. Targeting multiple UEs is also important because of the low amount of PDCCH resources, and Msg4 may very well become a bottleneck for Contention-based Msg3. For 2-Step random access in NR, it was specified to target multiple UEs for the specific reason that multiple UEs may use the same resources.

Observation 1: The arguments for CB-Msg4 targeting multiple UEs include:

- WID sets out to study and specify Msg4 efficiency enhancements – no other type of technical proposal on the table targets CB-Msg4 enhancements.

- Without targeting multiple UEs in Msg4, the Msg4 may become the bottleneck for the CB-EDT procedures.

Therefore, we do not see any strong arguments against introducing CB-Msg4 that can target multiple UEs.

Proposal 1: Working assumption on CB-Msg4 targeting multiple UEs is confirmed.

## RNTI issues and Msg4 monitoring

In the e-mail discussion on CB-Msg3 and CB-Msg4 [2], Msg4 monitoring window and RNTI for DSA was discussed, with the following proposals:

Proposal 15: RAN2 to discuss the following options:

- Option 1: Single Msg4 monitoring window. It starts at the end of Msg3 transmission window plus UE-eNB RTT. FFS processing time is needed or not.

- Option 2: Multiple Msg4 monitoring windows. Each starts at the end of transmission of each replica plus UE-eNB RTT. FFS processing time is needed or not.

Proposal 16: RAN2 to discuss the following options:

- Option 1: Single RNTI for MSG4 monitoring

  - Option 1a V The RNTI is derived on the resource of DSA transmission window.

  - Option 1b V The RNTI is preconfigured/predefined.

- Option 2: Multiple RNTI for MSG4 monitoring. Each RNTI is derived on the resource of each replica.

Most of the arguments for the different options have been discussed. In our view, the only two options that technically make sense are:

- A: Single Msg4 monitoring window, which starts at the end of the Msg3 transmissions window plus UE-eNB RTT, where a single RNTI is used for Msg4 monitoring.

- B: Multiple Msg4 monitoring windows which starts at the end of transmission each replica plus UE-eNB RTT, where each monitoring window has a separate RNTI.

Arguments for one over the other has already been discussed, and we believe that Option A is the only option for efficient DSA operation, because it allows for multiple UEs in one Msg4, little RNTI usage and thus no issues with RNTI-collisions and does not have any duplexing issues.

Proposal 2: RAN2 introduces single Msg4 monitoring window with single RNTI, starting from the end of the Msg3 transmission window plus UE-eNB RTT.

The RNTI for Msg3 and Msg4 shall also be the same. We do not see any reason why the Msg3 RNTI would be different.

Proposal 3: Same RNTI is used for Msg3 and Msg4.

## Fallback procedures

In some circumstances, depending on the configuration, it may be expected that contention-based Msg3 transmission would be less reliable compared to legacy random access. Failure rate can be dependent on the synchronization of the network and the UE, the radio characteristics and the load on the contention-based Msg3 resources. Given that the legacy random access should be more reliable and able to take a higher load, a UE should be able to fall back towards legacy random access. There are two ways in which a UE could fallback to random access with RACH; 1) the UE detects the failure and initiates the fallback and 2) the network detects a failure and signals to a UE to fallback to random access with RACH. We think that it would make sense to have both options available.

Proposal 4: For fallback the following is introduced:

- UE-initiated fallback to random access with RACH if a UE fails a number of times is introduced. The threshold is configured by the network.

- Network initiated fallback to random access with RACH.

Another option would be to introduce fallback to Msg3, similar to UE receiving FallbackRAR in 2-step random access. In that case, the UE will receive an allocation in MsgB similar to the RAR received in Msg2, which schedules a Msg3. However, this is a more complex procedure that for instance requires Msg3 scheduling, which will take time to design and is probably not needed for IoT-purposes. So we do not think that it should be pursued.

Proposal 5: Network-initiated fallback to scheduled Msg3 (similar to FallbackRAR in 2-step RA) is not introduced for contention-based Msg3 procedures.

## Contention-based Msg3 transmit power

One issue that has not been discussed extensively is the transmit power that a UE performing contention-based Msg3 applies. For PRACH, the UE will ramp up the power. We also believe that such procedures would be beneficial for CB-Msg3. This is because it allows the chance of success to increase with each CB-Msg3 attempt, which ensures that certain UEs are not “starved”. Therefore we propose:

Proposal 6: Power ramping of CB-Msg3 is introduced. The initial power level and the power ramping step is configurable.

## CQI reporting in Msg3

CQI reporting in Msg3 during RRC resume and EDT is supported for both eMTC and NB-IoT. This is useful in order to schedule Msg4 and to assess the coverage situation if the UE is moved to RRC_CONNECTED. In NB-IoT it is supported by including the cqi-NPDCCH in RRCConnectionResumeRequest and in RRCEarlyDataRequest if cqi-Reporting is configured in SIB2. For eMTC it is supported by including the DCQR and AS RAI MAC CE which is configured by the mpdcch-CQI-Reporting in SIB2. For these cases, the CQI is measured on Msg2.

CQI reporting may also be useful during the contention-based Msg3, similar to how it is already considered useful during EDT. In particular it may be useful for scheduling Msg4 to multiple UEs, as it can indicate whether two UEs sending Msg4 during the same CB-Msg3 opportunity operate with different CQI. For NB-IoT it will not occupy further bits as the bits are already occupied in the RRCConnectionResumeRequest and RRCEarlyDataRequest, however for eMTC it may occupy further bits as it is included in a MAC CE. Therefore we propose the following:

Proposal 7: RAN2 to discuss how to handle reporting CQI in Msg3 for eMTC and NB-IoT.

## DSA mandatory or optional

During the e-mail discussions, one question that came up is whether DSA should be considered a mandatory or optional feature of the uplink capacity enhancements. We believe that from perspective of the network the benefits of DSA over SA is significant and that there is risk of market fracture. For features in configurable in RRC_CONNECTED, this market fracture can be handled. But for random access procedures such as SA and DSA, this market fracture may have the consequence where the network may be forced to deploy both solutions, reducing the resource efficiency.

Proposal 8: DSA is a mandatory feature of contention-based Msg3/Msg4.

# Conclusion

In this contribution we discussion further issues on contention-based Msg3:

Observation 1: The arguments for CB-Msg4 targeting multiple UEs include:

- WID sets out to study and specify Msg4 efficiency enhancements – no other type of technical proposal on the table targets CB-Msg4 enhancements.

- Without targeting multiple UEs in Msg4, the Msg4 may become the bottleneck for the CB-EDT procedures.

Proposal 1: Working assumption on CB-Msg4 targeting multiple UEs is confirmed.

Proposal 2: RAN2 introduces single Msg4 monitoring window with single RNTI, starting from the end of the Msg3 transmission window plus UE-eNB RTT.

Proposal 3: Same RNTI is used for Msg3 and Msg4.

Proposal 4: For fallback the following is introduced:

- UE-initiated fallback to random access with RACH if a UE fails a number of times is introduced. The threshold is configured by the network.

- Network initiated fallback to random access with RACH.

Proposal 5: Network-initiated fallback to scheduled Msg3 (similar to FallbackRAR in 2-step RA) is not introduced for contention-based Msg3 procedures.

Proposal 6: Power ramping of CB-Msg3 is introduced. The initial power level and the power ramping step is configurable.

Proposal 7: RAN2 to discuss how to handle reporting CQI in Msg3 for eMTC and NB-IoT.

Proposal 8: DSA is a mandatory feature of contention-based Msg3/Msg4.

# Reference

RP-250472, Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3, Mediatek, RAN#107, Incheon, ROK, March 2025.

R2-2502773, Report of [Post129][307][R19 IoT NTN] CB-msg3/CB-msg4 (Mediatek), Mediatek, RAN2#129bis, Wuhan, China, April 2025.