3GPP TSG-RAN WG2 Meeting #129bis  	                            R2-2502487

Wuhan, China, Apr. 7th – 11th, 2025							              (Revision of R2-2500742)

Agenda Item:	8.5.2

Source: 	Sony

Title:	On-demand SSB Scell operation discussion

Document for:	Discussion

# Introduction

In RAN2#129 meeting, on-demand SSB Scell operation was discussed, and the agreements are as follows.

Agreements on OD-SSB

- RAN2 leave it to RAN4 to conclude whether always-on SSB and/or OD-SSB are measured when both are transmitted in OD-SSB case 2.

In this contribution, we discuss some open issues regarding the OD-SSB and its measurements.

# Discussion

2.1 Cell on/off indication via backhaul

RAN1#116 agreed to support on-demand SSB SCell operation triggered by gNB and cell on/off indication via backhaul is one of the candidate solutions. With the existing CU-DU architecture, it is the CU who is responsible for the cell on/off decision. In gNB-CU configuration update message sent from gNB-CU to gNB-DU, it contains cells to be activated/deactivated list which gNB-DU shall activate/deactivate the corresponding cells as indicated by the cell group and physical cell identity.

With the existing signalling, it is not possible to support dynamic cell on/off use cases where the cell on/off decision is made based on the current traffic and energy consumption status of the cell. It is the gNB-DU who is aware of the up-to-date status of the cells but the cell on/off indication comes from gNB-CU.

Cell on/off indication via backhaul will be a promising solution if it can be enhanced to support:

- Assistance information from gNB-DU to gNB-CU to help gNB-CU making cell on/off decision. We think this information is needed because gNB-DU has more awareness and gNB-DU should be involved in cell on/off decision as explained below.

- Certain level of autonomy is given to gNB-DU to allow it make its own cell on/off decision according to the cell status. The gNB-DU has the up-to-date information e.g., number of active UEs, radio resource utilization ratio, traffic status of the cell. Therefore, if certain level of autonomy is given to gNB-DU, it can make proper cell on/off decision based on the above cell status. But gNB-CU should always control this procedure and being informed of any gNB-DU’s decision.

RAN3 should lead the work and it may have some impacts on the assistance information which can be potentially supported by the UEs in the cell.

Proposal 1: Enhancements to support dynamic cell on/off indication via backhaul should be supported and RAN3 should be informed about it.

2.2 OD-SSB measurement framework

In RAN2#127bis meeting, the L3 RRM framework on OD-SSB measurement has been discussed and two options are listed as follows.

Option1: Based on different measurement configuration when OD-SSB is transmitted.

Option2: Based on OD-SSB pattern ignoring SMTC when OD-SSB is transmitted.

Our understanding is that Option 1 is more aligned with the legacy measurement framework while Option 2 may introduce more specification work. Furthermore, the SMTC configurations in Option 1 will need more RAN1 input on the detailed OD-SSB pattern discussion. On the other hand, if the OD-SSB pattern is shared with UE, it will be beneficial to both options, e.g., the OD-SSB pattern will be helpful for UE to choose from the configured multiple SMTC configurations in Option 1 and OD-SSB pattern sharing is essential in Option 2.

Proposal 2: OD-SSB patterns are informed to UE from network.

In RAN2#129 meeting, it was agreed to revisit the proposals regarding the OD-SSB MAC-CE design, as listed in the following:

Proposal 1: The OD-SSB MAC-CE has a fixed size and include a bitmap to indicate whether OD-SSB is activated in each SCell (i.e., similar to legacy SCell A/D MAC-CE).

Proposal 2: The OD-SSB MAC-CE supports two formats: one format indicates up to 7 SCells and the other format indicates up to 31 SCells.

Proposal 3: Considering forward compatibility, configuration indices of candidate SSB periodicities in RRC (rather than actual values of SSB periodicities) are included in the OD-SSB MAC-CE.

It was in RAN1#120 meeting, a list of parameters of OD-SSB was agreed as below.

- For a cell supporting on-demand SSB SCell operation, introduce NEW higher layer parameters to configure each of the followings, for both Case #1 and Case #2. - Frequency of the on-demand SSB (i.e., od-ssb-absoluteFrequency) - FFS: Whether this parameter can be absent for Case #2 - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst (i.e., od-ssb-PositionsInBurst) - FFS: Whether this parameter can be absent for Case #2 - Periodicity of the on-demand SSB (i.e., od-ssb-Periodicity) - Sub-carrier spacing of the on-demand SSB (i.e., od-ssbSubcarrierSpacing) - For Case #2, this parameter is absent and sub-carrier spagcing of on-demand SSB is the same as that of always-on SSB. - Physical Cell ID of the on-demand SSB (i.e., od-ssb-physCellId) - For Case #2, this parameter is absent and physical cell ID of on-demand SSB is the same as that of always-on SSB. - Time location of on-demand SSB burst (i.e., od-ssb-sfn-Offset and od-ssb-halfFrameIndex) - Downlink transmit power of on-demand SSB (i.e., od-ss-PBCH-BlockPower) - For Case #2, this parameter is absent and downlink transmit power of on-demand SSB is the same as that of always-on SSB. - Number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated (i.e., od-ssb-nrofTx) - FFS: Which of the above parameters are a list

Besides periodicity that RAN2 is aware, there are other parameters e.g., frequency, SSB positions, time location of SSB burst that may be configured for OD-SSB SCell. It is impossible to include all the above parameters in the MAC CE activation command, otherwise the flexibility to support various OD-SSB configurations in one SCell will be greatly reduced. Considering the forward compatibility, which may need to add more parameters or support different combinations of parameters in the future, it is proposed that the RRC configuration + MAC-CE activation/deactivation is adopted.

Proposal 3: Configuration indices of candidate SSB configurations in RRC (rather than actual values of SSB parameters) are included in the OD-SSB MAC-CE.

# Conclusion

We propose RAN2 to discuss following proposals:

Proposal 1: Enhancements to support dynamic cell on/off indication via backhaul should be supported and RAN3 should be informed about it.

Proposal 2: OD-SSB patterns are informed to UE from network.

Proposal 3: Configuration indices of candidate SSB configurations in RRC (rather than actual values of SSB parameters) are included in the OD-SSB MAC-CE.