3GPP TSG-RAN WG2 Meeting #129bis  	                            R2-2502489

Wuhan, China, Apr. 7th – 11th, 2025				                            (Revision of R2-2500744)

Agenda Item:	8.5.4

Source: 	Sony

Title:	RAN2 impacts on SSB and RACH adaptations

Document for:	Discussion

# Introduction

The SSB adaptation and RACH adaptation was not discussed in RAN2 as it depends on the RAN1’s progress. And in RAN2#129 meeting, RAN2 reached the agreements on SSB and PRACH adaptations as following:

Agreements on SSB adaptation

- RAN2 confirms SSB adaptation in time domain is not supported for RRC idle/inactive UEs and Rel-19 NES-capable UE’s PCell.

- RAN2 preference is to keep SMTC based L3 RRM framework and to introduce additional SMTC configuration according to SSB adaptation for L3 RRM measurement on SCell with SSB adaptation.

- For L3 measurement, RAN2 assumes the adapted SSB on neighbor cell is measured based on legacy SMTC.

Agreements on RACH adaptation

- Legacy UEs and UEs non-capable of time domain PRACH adaptation are expected to use legacy PRACH resources per legacy configuration and procedures. No barring is needed.

2a.	RAN2 starts CBRA for RACH adaptation.

2b. 	RAN2 starts 4-step RACH adaptation.

3. 	From R2 perspective, not apply time-domain RACH adaptation to RACH resources for MSG1-based SI request.

- From R2 perspective, not apply time-domain RACH adaptation to IAB RACH resources.

5a. 	From R2 perspective, RACH adaptation is not modelled as RA feature(s).

5b.	From R2 perspective, RACH partitioning with all the features, i.e. RedCap, SDT, and Slicing, and feature combinations, are supported for PRACH adaption in time domain.

- Will follow legacy mechanism regarding how to select RACH resource.

In this contribution, we discuss some RAN2 impact on SSB adaptation and RACH adaptation based on latest RAN1’s progress.

# Discussion

2.1 SSB adaptation

The framework on how to deal with SSB adaptation has not been discussed in RAN2. RAN1 agreed that at least SSB burst periodicity is adapted for SSB adaptation, and the periodicity of on-demand SSB is one of 5ms, 10ms, 20ms, 40ms, 80ms or 160ms. It has been concluded in RAN1#118b meeting that there is no consensus on the support of adaptation of SSB for idle UEs in Rel-19 and further in RAN1#119 meeting, it was agreed that there is no RAN1 consensus to support SSB adaptation in time domain for Rel-19 NES-capable UE’s PCell.

As can be observed that both on-demand SSB and SSB adaptation is for SCell connected UEs in Rel-19 NES. For on-demand SSB, RAN2 has agreed that a new MAC CE will be specified on on-demand SSB transmission activation/deactivation. It is beneficial that at least the same design principle can be re-used for SSB adaptation as well.

In RAN1#120 meeting, it’s been decided to down-selection between two alternatives for switching the periodicity:

- (MAC-CE) - MAC-CE details for SSB burst periodicity adaptation is up to RAN2. - (DCI) - Alt 1-3: DCI based signalling is used

And we think the same principle can be applied on OD-SSB MAC CE design, although it is still FFS on how to include the periodicity in the MAC CE.

Proposal 1: RAN2 to consider a joint design of the MAC CE for both on-demand SSB and SSB adaptation.

2.2 RACH adaptation

It was agreed in RAN1#116b that the time domain RACH adaptation is based on additional PRACH resources for NES -capable UEs in addition to PRACH resources for legacy UEs. RAN1 has further agreed to use DCI-based adaptation to trigger the additional PRACH resource. Then it is up to RAN2 to decide how the network will configure the additional PRACH resource and it is easy to conclude that the additional PRACH resources should be included in a SIB and further details e.g., to define a new SIB or to be included in SIB1 or any other existing SIB can be decided later.

Proposal 2: The additional PRACH resources for RACH adaptation should be included in a SIB.

# Conclusion

We propose RAN2 to discuss following proposals:

Proposal 1: RAN2 to consider a joint design of the MAC CE for both on-demand SSB and SSB adaptation.

Proposal 2: The additional PRACH resources for RACH adaptation should be included in a SIB.