3GPP TSG-RAN WG2 Meeting #129		R2-2502536

Hubei, China, 7th – 11st Feb. 2025

Source:	China Telecom

Title:	Discussion on asymmetric DL sTRP/UL mTRP scenarios

Agenda Item:	8.12.2

Document for:	Discussion and Decision

- Introduction

In last meeting, RAN2 has reached the following agreements.

RAN2 understands that if a joint/UL TCI state is configured with a PL offset, PHR trigger is based on the PL change of the PL-RS associated to the joint/UL TCI, where the PL change takes into account the PL offset. FFS whether/how to capture this.

In this contribution, we will discuss potential RAN2 specific impacts of asymmetric DL sTRP/UL mTRP scenarios.

- Discussion

In last meeting, RAN2 understands that if a joint/UL TCI state is configured with a PL offset, PHR trigger is based on the PL change of the PL-RS associated to the joint/UL TCI, where the PL change takes into account the PL offset. Due to the differences in antenna position and transmission power between UL-only TRP and DL_UL TRP, when the transmission power of UL-only TRP is limited (i.e. the coverage capability of UL-only TRP is weaker than that of DL_UL TRP), the path loss of UL-only TRP will be higher, and vice versa, the path loss will be lower. However, in which case to use positive PL offset is determined by the design of PL calculation formula, we can send an LS to RAN1 to ask if they will discuss about this.

Proposal 1: It is recommended to send an LS to RAN1 to ask if they will discuss about PL calculation formula.

If proposal 1 is not approved, RAN2 may discuss PL calculation formula for UL-only TRP. The basic form of the PL calculation formula for a UL-only TRP can be expressed as: RS transmit power – RSRPPL-RS ± PL offset. From the perspective of path loss interpretation, we recommend to use a negative sign (i.e., PLUL-only TRP =RS transmit power – RSRPPL-RS - PL offset), which applies a positive offset when the transmit power of the UL-only TRP is limited.

Proposal 2: If proposal 1 is not approved, PL calculation formula for a UL-only TRP can be captured as PLUL-only TRP =RS transmit power – RSRPPL-RS - PL offset.

A Power Headroom Report (PHR) shall be triggered if any of the following events occur: - phr-ProhibitTimer expires or has expired and the path loss has changed more than phr-Tx-PowerFactorChange dB for at least one RS used as pathloss reference for one activated Serving Cell of any MAC entity of which the active DL BWP is not dormant BWP since the last transmission of a PHR in this MAC entity when the MAC entity has UL resources for new transmission; NOTE 1: The path loss variation for one cell assessed above is between the pathloss measured at present time on the current pathloss reference and the pathloss measured at the transmission time of the last transmission of PHR on the pathloss reference in use at that time, irrespective of whether the pathloss reference has changed in between. The current pathloss reference for this purpose does not include any pathloss reference configured using pathlossReferenceRS-Pos in TS 38.331 [5].

When the PL calculation formula for UL-only TRP is updated, according to the current standard definition, there seems to be no impact on the standard in terms of triggering PHR reporting when the path loss variation exceeds phr-Tx-PowerFactorChange. However, considering that the coverage capability of some UL-only TRP PL might be relatively weaker, it is worth discussing whether a separate threshold should be set for UL-only TRP to achieve more accurate reporting.

Observation 1: There seems to be no impact on the standard in terms of triggering PHR reporting when the path loss variation exceeds phr-Tx-PowerFactorChange.

Proposal 3: RAN2 to discuss whether a separate threshold is need for UL-only TRP case to achieve more accurate reporting.

- Conclusion

Based on the discussion, we have the following observations and proposals.

Proposal 1: It is recommended to send an LS to RAN1 to ask if they will discuss about PL calculation formula.

Proposal 2: If proposal 1 is not approved, PL calculation formula for a UL-only TRP can be captured as PLUL-only TRP =RS transmit power – RSRPPL-RS - PL offset.

Observation 1: There seems to be no impact on the standard in terms of triggering PHR reporting when the path loss variation exceeds phr-Tx-PowerFactorChange.

Proposal 3: RAN2 to discuss whether a separate threshold is need for UL-only TRP case to achieve more accurate reporting.