3GPP TSG-RAN WG2 Meeting #129bis	R2-2502551

Wuhan, China, 7th – 11th of April 2025

Agenda item:	8.8.4

Source:	Nokia, Nokia Shanghai Bell

Title:	Remaining Aspects of MBS in Rel-19 NR NTN

WID/SID:	NR_NTN_Ph3 - Release 19

Document for:	Discussion and Decision

# 1	Introduction

In December 2023, RAN approved a Rel-19 work item (WI) aimed at defining Phase 3 enhancements to Non-Terrestrial Networks (NTN). There was a minor revision of the WID at RAN#103. Among the objectives captured in the WID [1] the following can be found:

- Specify signaling of the intended service area of a broadcast service (e.g. MBS broadcast) via NR NTN [RAN2, RAN3] - Specify SIB signaling to indicate the intended service area in case the satellite footprint covers a larger area. [RAN2] - Specify the necessary signaling between CN and NG-RAN. [RAN3]

In RAN2#127bis meeting, the following decision has been made [2]:

Agreements: 1. For each MBS service we include one or more intended service area IDs into MCCH. FFS whether the list of the intended service areas (and related IDs) is also included in MCCH or if it is provided in a new or existing SIB. We will consider possible enhancements (including enhancements left up to UE implementation) to allow UE skipping MCCH re-acquisition when UE is not within intended service area of any interested broadcast service.

Few other topics have been marked as “Come back in the next meeting”. At RAN2#128 meeting (November 2024) the following agreements have been made:

Agreements: 1. The encoding of TN coverage introduced in Rel-18 in TS38.331, including tn-ReferenceLocation-r18 and tn-DistanceRadius-r18, is reused for the geographical area of the circle. 2. The encoding of Polygon in TS37.355 is reused for the geographical area of the Polygon. 3. The IntendedServiceArea is considered as the IE name of the geographical area (we can still update the name in the CR implementation if needed) 4. A signalled intended service area for a MBS BC service may include geographic areas across the current serving cell and overlapping neighbor cell(s). 5. RAN2 understands that the geographic area information for the intended service areas can be semi-static and not cause frequent updates. 6. Introduce a new SIB to include a list of intended service areas and related pointer (FFS if we point to the intended services areas via the index in the list or with an ID or another way) 7. The legacy SIB modification procedure is applied to update the intended service area information in the new SIB.

In the meantime, SA1 has responded to our LS on ETWS primary notification. In their response LS [4] the following is stated:

SA1 thanks RAN2 for the LS on requirements for ETWS primary notification (R2-2407823). SA1 would like to inform RAN2 of the resulting alignment CR which is attached.

This topic has been addressed at RAN2#129 meeting (February 2025) and the following decisions have been made [4]:

Agreements: 1. In the new SIB, explicit network-indicated area ID is used to label an intended service area in the list 2. It shall be possible to signal multiple service area IDs to one MBS service (we Insert a list of service area IDs in MCCH) 3. Introduce “warning area coordinates” in ETWS Primary Notification (SIB6) and in ETWS Secondary Notification (SIB7). FFS on the signalling details for “warning area coordinates” (SIB6 is not segmented)

In addition to the aforementioned agreements, RAN2 has also sent the LS to SA2 and SA4 [14], wherein it has been asked if Intended Service Area (ISA) can be conveyed in Service Announcement:

# 2. Actions: # To SA WG 4: # RAN2 kindly request SA4 to confirm whether the inclusion of the ISA in the Service Announcement can be supported within the Rel-19 timeframe.

This paper presents our further view on what remains to be addressed for MBS Rel-19 objective in RAN2.

# 2	Discussion

## 2.1	Intended Service Area (ISA) signalling

The following has been agreed at RAN2#129 [4]:

1. In the new SIB, explicit network-indicated area ID is used to label an intended service area in the list 2. It shall be possible to signal multiple service area IDs to one MBS service (we Insert a list of service area IDs in MCCH)

The high-level structure and relationship as shown in Figure 1 has been agreed.

Figure 1. ISA and MBS service high-level structure and relationship.

As can be seen, the MBS service definition is given in MCCH (legacy behaviour) and ISA ID points to a new SIB where ISA geographical details are provided. Multiple services can be present within a single service area and one service can be present in multiple ISAs. (i.e. many-to-many relationship, as agreed in RAN2#129).

Observation 1: Multiple services can be present within a single intended service area (ISA) and one service can be present in multiple ISAs. (i.e. many-to-many relationship, as agreed in RAN2#129).

When inspecting the NR RRC [7], it can be found that MCCH comprises MBSMulticastConfiguration which can have a list of MBS sessions (i.e. MBS-SessionInfoListMulticast IE). Thus, under each of such sessions (i.e. MBS-SessionInfo) there needs to be an identifier pointing to a specific service area or areas defined in the new SIB.

Proposal 1: Insert a list of service area IDs under each MBS-SessionInfo provided in MCCH.

## 2.2	ISA in Service Announcement

Intended Service Area in Service Announcement has been discussed in RAN2#129 based on [15]. The main motivation used by the authors of [15] was to allow the UE to skip MCCH reading if the UE determined (based on Service Announcement) that it is outside of the ISA. An LS to SA4 has been sent [14] asking about the feasibility of such scheme in Release 19 timeframe.

While this approach could work, we do not think it is essential to involve other working groups (SA WGs in particular) in addressing this objective of RAN-centric work item.

Observation 2: Including ISA in Service Announcement is one potential option, but not the primary solution RAN2 considers in Release 19.

Moreover, such approach would introduce new requirements to the application function (AF) and also to Multicast/Broadcast Service Function (MBSF). Furthermore, it might require a coordination between the operator and the application function if the application shall know about ISA. Eventually, AF does not need to know that certain service is transmitted via NTN RAN and not the “ordinary” TN.

Observation 3: ISA in Service Announcement will results in new requirements to the application function (AF) and also to Multicast/Broadcast Service Function (MBSF).

Observation 4: ISA in Service Announcement might require a coordination between the operator and the application function if the application shall know about ISA.

Thus, in our view the gains of ISA in Service Announcement are too small compared to the pain of introducing this.

Proposal 2: RAN2 does not consider including ISA in Service Announcement, irrespective of the SA4 response to R2-2501583.

## 2.3	MBS Interest Indication (MII)

At RAN2#127bis the following has been discussed and not resolved:

Proposal 4: For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature may initiate the MBS Interest Indication procedure when UE is entering or leaving the intended area. - QC thinks this is up to UE and there is nothing to reflect in the spec. Xiaomi agrees - HW thinks that when entering the area this makes sense, but wonder about the case when the UE leaves the area. - Apple thinks the UE could initiate the MII procedure in any case and then it’s up to NW implementation how to behave. LG agrees with Apple. Oppo also agrees. CMCC thinks in this case the NW has no idea of the UE location in this case. Come back in the next meeting

In section 5.9.4.2 of [7] the following text can be found:

An MBS capable UE in RRC_CONNECTED may initiate the procedure in several cases including upon successful connection establishment/resume, upon entering or leaving the broadcast service area, upon MBS broadcast session start or stop, upon change of interest, upon change of priority between MBS broadcast reception and unicast/multicast reception, upon change to a PCell providing SIB21 (i.e. where the SIB1 scheduling information contains SIB21), upon receiving SIB20 of an SCell via dedicated signalling, upon handover, and upon RRC connection re-establishment, upon change to a PCell providing nonServingCellMII in SIB1, upon starting or stopping reception of MBS broadcast service on a non-serving cell, upon change of CFR information or subcarrier spacing for MBS broadcast reception on a non-serving cell.

As can be seen, the MII might be sent when UE enters or leaves the broadcast service area and this is already supported by NR RRC [7].

Observation 5: As per legacy NR RRC the MBS capable UE may send the MBS Interest Indication upon entering or leaving the broadcast service area.

Thus, it that nothing new has to be agreed for MBS II in NTN. Furthermore, the behaviour described in [7] is not binding (the term used in the procedure is “UE may initiate”), so in our view this issue is not worth spending additional time. Note that the MII feature can be used by the UE in case UE cannot find the MBS broadcast service that it is expecting in a cell. That would make, e.g., a gNB decides whether to handover the UE to a possible neighbor gNB/cell providing MBS broadcast service UE has interest in. Thus, as has been indicated at RAN2#127bis, it is a UE implementation issue, and a UE can decide when to send the MII (which could be somehow related to the intended service area).

Proposal 3: RAN2 does not specify any new behaviour with respect to MBS Interest Indication when Rel-19 NTN capable UE enters or leaves the MBS broadcast service area.

## 2.4 	Configuration aspects of MBSBroadcastConfiguration and service continuity in SIB21

At RAN2#127bis the following has been discussed and not decided [2]:

In MBSBroadcastConfiguration, the intended service area within each neighbor cell is provided for each MBS broadcast service session. In SIB21, the intended service area is provided for each MBS broadcast service session, or for each FSAI (depending on common understanding selected). - Ericsson thinks we should focus on the scenario where the quasi-Earth-fixed cell is replaced due to satellite movement - ZTE thinks we currently support both and then supports p4 - QC also thinks both scenarios should be supported. - HW thinks this covers the case of UE movement. For the satellite movement there would be no difference wrt previous releases. HW is ok to reflect the second part of p4 - Ericsson disagrees with the proposal as there is no need for service continuity for this. Apple thinks this is needed for cell reselection. - CATT wonders whether we really need to provide the service area for service continuity, in a per neighbour cell way. - Ericsson thinks we need to clarify the UE behavior when receiving this information Come back in the next meeting

As can be seen in [7], MBS-SessionInfo (a part of MBSBroadcastConfiguration) comprises mtch-neighbourCell. This IE indicates neighbour cells which provide this service on MTCH. Thus, achieving the first goal stated above seems to be possible using the existing means (i.e. it is feasible to tell the UE in which neighbouring cells the same service is provided). There is no exact indication about the intended service area, but that is perhaps not critically needed (the UE will know the service area details from its current serving cell).

Proposal 4: RAN2 does not work on providing the intended service area within each neighbor cell for each MBS broadcast service session, as this can be known thanks to mtch-neighbourCell.

Regarding the second aspect highlighted above, whether intended service area is provided in SIB21 for each session or per FSAI, this would somewhat change/extend the purpose of SIB21. SIB21 is defined as follows [7]:

SIB21 contains the mapping between the current and/or neighbouring carrier frequencies and MBS Frequency Selection Area Identities (FSAI). SIB21 information element -- ASN1START -- TAG-SIB21-START SIB21-r17 ::= SEQUENCE { mbs-FSAI-IntraFreq-r17 MBS-FSAI-List-r17 OPTIONAL, -- Need R mbs-FSAI-InterFreqList-r17 MBS-FSAI-InterFreqList-r17 OPTIONAL, -- Need R lateNonCriticalExtension OCTET STRING OPTIONAL, ... } MBS-FSAI-List-r17 ::= SEQUENCE (SIZE (1..maxFSAI-MBS-r17)) OF MBS-FSAI-r17 MBS-FSAI-InterFreqList-r17 ::= SEQUENCE (SIZE (1..maxFreq)) OF MBS-FSAI-InterFreq-r17 MBS-FSAI-InterFreq-r17 ::= SEQUENCE { dl-CarrierFreq-r17 ARFCN-ValueNR, mbs-FSAI-List-r17 MBS-FSAI-List-r17 } MBS-FSAI-r17 ::= OCTET STRING (SIZE (3)) -- TAG-SIB21-STOP -- ASN1STOP

Thus, introducing the service area information to SIB21 does not match its main purpose (i.e. frequency mapping to FSAI). FSAIs are configured per gNB or per cell. If these are to be configured per intended service area, this requires lots of changes to core and service announcements. Overall, the benefits are limited, if any, while it might have a large specification impact, not only in RAN2.

Proposal 5: As FSAIs are configured per gNB or per cell, RAN2 does not pursue FSAI to intended service area mapping, which might have impact on core and service announcements (not limited to RAN2) for no major benefits.

# 3	Conclusion

This document has made the following observations:

Observation 1: Multiple services can be present within a single intended service area (ISA) and one service can be present in multiple ISAs. (i.e. many-to-many relationship, as agreed in RAN2#129).

Observation 2: Including ISA in Service Announcement is one potential option, but not the primary solution RAN2 considers in Release 19.

Observation 3: ISA in Service Announcement will results in new requirements to the application function (AF) and also to Multicast/Broadcast Service Function (MBSF).

Observation 4: ISA in Service Announcement might require a coordination between the operator and the application function if the application shall know about ISA.

Observation 5: As per legacy NR RRC the MBS capable UE may send the MBS Interest Indication upon entering or leaving the broadcast service area.

and proposed the following:

Proposal 1: Insert a list of service area IDs under each MBS-SessionInfo provided in MCCH.

Proposal 2: RAN2 does not consider including ISA in Service Announcement, irrespective of the SA4 response to R2-2501583.

Proposal 3: RAN2 does not specify any new behaviour with respect to MBS Interest Indication when Rel-19 NTN capable UE enters or leaves the MBS broadcast service area.

Proposal 4: RAN2 does not work on providing the intended service area within each neighbor cell for each MBS broadcast service session, as this can be known thanks to mtch-neighbourCell.

Proposal 5: As FSAIs are configured per gNB or per cell, RAN2 does not pursue FSAI to intended service area mapping, which might have impact on core and service announcements (not limited to RAN2) for no major benefits.

# References

- RP-240775 Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3 3GPP TSG RAN Meeting #103 Maastricht, The Netherlands, March 18-22, 2024

- R2-2409213 Report from Break-out session on NR-NTN and IoT-NTN 3GPP TSG-RAN WG2 Meeting #127bis Changsha, China, Apr 15th – 19th, 2024

- R2-2410913	Report from Break-out session on NR-NTN and IoT-NTN 3GPP TSG-RAN WG2 Meeting #128 Orlando, USA, Nov 18th – 22nd, 2024

- R2-2501333	Report from Break-out session on NR-NTN and IoT-NTN 3GPP TSG-RAN WG2 Meeting #129 Athens, Greece, Feb 17th -21st, 2025

- S1-244857 Reply LS on requirements for ETWS primary notification 3GPP TSG SA WG WG1 Meeting #108 Orlando, FL, USA, 18th – 22nd November 2024

- S1-244753 Warning notification with geofencing for ETWS over satellite access (CR to TS 22.268) 3GPP TSG-SA1 Meeting #108	 Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

- 3GPP TS 38.331 NR RRC, v18.4.0, December 2024

- R2-2407823 LS on requirements for ETWS primary notification 3GPP TSG-RAN WG2 Meeting #127 Maastricht, Netherlands, August 19th – 23rd, 2024

- R2-2405703 Report from Break-out session on NR-NTN and IoT-NTN 3GPP TSG-RAN WG2 Meeting #126 Fukuoka, Japan, May 20th – 24th, 2024

- R2-2407573 Report from Break-out session on NR-NTN and IoT-NTN 3GPP TSG-RAN WG2 Meeting #127 Maastricht, The Netherlands, Aug 19th – 23rd, 2024

- R2-2407619 [Offline 307] [R19 NR NTN] service area information (Apple) 3GPP TSG-RAN WG2 Meeting #127 Maastricht, The Netherlands, Aug 19th – 23rd, 2024

- R2-2402355, Discussion on support of broadcast service via NR NTN, CATT, China Broadnet, 3GPP TSG RAN2#125bis meeting, Rel-19, April 2024.

- R3-245844 LS on Supporting MBS broadcast service for NR NTN 3GPP TSG RAN WG3 125-bis Hefei, China, 14 - 18 October, 2024

- R2-2501583 LS on Inclusion of NTN intended service area in the Service Announcement 3GPP TSG-RAN WG2 Meeting #129 Athens, Greece, Feb 17th -21st, 2025

- R2-2501306  Support for broadcast services in NR NTN 3GPP TSG-RAN WG2 Meeting #129 Athens, Greece, Feb 17th -21st, 2025