3GPP TSG-RAN WG2 Meeting #129bis	R2-2502582
Wuhan, China, April 7 – 11, 2025

Agenda Item:	8.5.2

Source:	InterDigital

Title:	On demand SSB transmission for SCell

Document for:	Discussion, Decision

# Introduction

The Rel-19 WID on Network Energy Savings for NR has the following objective on on-demand SSB SCell [1]:

The WID includes the following objective accordingly:

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

In RAN2#127 and 127bis, the objective was discussed and the following was agreed:

Agreements on OD-SSB

Scenarios on OD-SSB:

- RAN2 start the discussion from Scenario 2/2A and wait for RAN1 conclusion on Scenario 3A/3B.

OD-SSB transmission indication:

- RRC based OD-SSB transmission indication is used to indicate at least the initial activation/deactivation state of OD-SSB configuration. FFS on reconfiguration.

- New MAC-CE for OD-SSB transmission indication is introduced. We will not change legacy SCell activation/deactivation MAC CE. FFS if we need further optimization for scenario 2A.

- No need to restrict the OD-SSB activation/deactivation state indication in RRC to initial configuration. No special specification effort is required.

- Don’t introduce further new MAC CE that combines SCell activation/deactivation and OD-SSB indication for scenario 2A.

- NW should be able to send OD-SSB indication for multiple SCells simultaneously by a MAC CE.

Measurement on OD-SSB:

- Measurement based on OD-SSB in both case 1 and case 2 will be considered. (case 1 and case 2 defined in RAN1).

- For Case #1, the UE does not expect to measure SSB when on-demand SSB transmission is deactivated. In other words, the UE expects to measure SSB when on-demand SSB transmission is activated.

- RAN2 does not handle the issue raised in R2-2407414 in OD-SSB based measurements.

- RAN2 study how the UE to perform L3 measurement according to OD-SSB L3 RRM configuration.

The following was agreed in RAN1#120:

Agreement For adaptation of SSB in time-domain, for adapting SSB burst periodicity for an SCell - Support group common DCI signalling for switching the SSB burst periodicity using DCI format 2_9 with [cellDTRX-RNTI] - FFS: which scenario(s) is this applicable for (e.g. as defined in 9.5.1) Note: Above does not prevent RAN2 from designing a MAC CE based on OD-SSB feature and also used for SSB burst adaptation

Full agreements made in RAN1#116 meeting are provided in the appendix. In this contribution, we discuss the enhancements for enabling on-demand SSB SCell along with the MAC CE design to indicate it.

# Discussion

For CA deployments, SCells with SSB-less Scell has been supported since Rel-15 for both FR1 and FR2. In Rel-18 the support for SSB-less Scell is extended for collocated cells in FR1 that are in inter-band CA (e.g. within max receive time, frequency and power difference). In the inter-base CA case, the NW signals to the UE which cell is the reference cell (e.g. a Pcell or another Scell), where the reference cell acts as the timing reference and AGC source of the SSB-less Scell.

Supporting inter-band CA for FR2 comes with challenges, e.g. achieving synchronization, L1/L3 measurements and beam management, especially when the cells are non-collocated and only the Scells operate in FR2 while other cells in FR1. For energy saving purposes in CA, it is important for the Scells to operate in a NES mode as long as possible without or with limited SSB transmission.

## Signalling for OD-SSB reception

MAC CE signalling

During Rel-18 SI, it was discussed that the Scells which are used as capacity cells and operating in NES mode are not expected to transmit SSBs or only transmit sparse SSBs. gNB initiated triggering of OD-SSB in the context of Scell activation procedure has been agreed by R1.

Per R2 agreement, reception of on-demand SSB on an SCell is indicated by a new MAC CE signalling separate from the legacy MAC CE for SCell (de)activation. Further, since the activation of the SCell itself is based on the network’s decision, i.e. based on the reception of UE measurement and L1 CSI, the activation decision may lag the indication for the reception of an OD-SSB. For scenario 2A where the OD-SSB and SCell are indicated and activated together, R2 agreed to not introduce further new MAC CE that combines SCell activation/deactivation and OD-SSB indication.

Per RAN1 agreements, MAC CE based signalling is used to indicate the (de)-activation of on-demand SSB transmission on an SCell supporting on-demand SSB SCell operation. RAN1 further agreed that for a cell supporting on-demand SSB SCell operation, multiple candidate values can be configured by RRC for the periodicity of the on-demand SSB, and the applicable periodicity value can be indicated by MAC CE. It is FFS whether there are any other relevant parameters. However, looking at the list of parameters from RAN1, it appears like there are no other parameters that need to be indicated dynamically.

Proposal 1:	The OD-SSB indication MAC CE signalling indicates OD-SSB (de)-activation per SCell, along with the applicable OD-SSB periodicity per activated SCell. No other parameters need to be included in the MAC CE.

For indication of SSB periodicity adaptation (WI objective 3), RAN1 has already agreed to use DCI to indicate it. Therefore, there is no need to use MAC CE in addition for the purpose of adapting SSB burst periodicity. Adaptation of SSB periodicity is anyway anticipated to be applicable for the whole cell, rather than a UE specific adaptation. DCI signaling is therefore sufficient and is better suited for such indication.

Proposal 2:	The OD-SSB indication MAC CE signalling is not used for indication of SSB periodicity adaptation (WI objective 3).

Given R1 agreed to indicate a value associated with the OD-SSB periodicity, R2 can assume that there will be an additional field to indicate the periodicity index from the values configured in RRC. RAN1 agreed that no new periodicity configuration values will be introduced in Rel-19 for the OD-SSB, other than legacy values (5 ms, 10 ms, 20 ms, 40 ms, 80 ms, or 160 ms).

Observation 1:	RAN1 agreed to reuse legacy values for the configuration of the OD-SSB periodicity, including (5 ms, 10 ms, 20 ms, 40 ms, 80 ms, or 160 ms).

Therefore, 3 bits is sufficient to indicate which periodicity is applied for the OD-SSB.

Proposal 3:	3 bits is sufficient to indicate which periodicity is applied for the OD-SSB.

From a MAC CE design perspective, R2 agreed that NW should be able to send OD-SSB indication for multiple SCells simultaneously by a MAC CE. Therefore, a bit map can be used to indicate the SCells for which the OD-SSB is indicated. For each indicated SCell, an additional (optional) field is added to indicate the applicable periodicity index. The OD-SSB MAC CE therefore has a variable size. A sample MAC CE is illustrated in Figure 2 for such assumption, at least for the case of up to 7 SCells.

Figure 1: illustration of a variable-size OD-SSB MAC CE short format

Provided that the MAC CE does not provide (de)-activation of SCells, it is beneficial to keep the MAC CE simple and fixed in size.

Proposal 4:	A bit map in a variable size MAC CE is used to indicate the SCells for which the OD-SSB is indicated. For each indicated SCell, an additional (optional) field is added to indicate the applicable OD-SSB periodicity.

Similar to the SCell Activation MAC CE, there can be two formats: one short variable size format, which indicates up to 7 SCells. Another long variable size format, which indicates up to 31 SCells. The short variable size format is illustrated in Figure 1, while the long variable size format is illustrated in Figure 2, where 3 bits are used to indicate the OD-SSB periodicity.

Figure 2: illustration of a variable-size OD-SSB MAC CE long format

Proposal 5:	Similar to SCell Activation MAC CE, the OD-SSB MAC-CE supports two formats: one format indicates up to 7 SCells and another format for up to 31 SCells.

In terms of deactivation of the OD-SSB transmission, the following mechanisms were agreed:

- Deactivation by RRC. RAN2 agreed that “RRC based OD-SSB transmission indication is used to indicate at least the initial activation/deactivation state of OD-SSB configuration. No need to restrict the OD-SSB activation/deactivation state indication in RRC to initial configuration.”

- Deactivation by MAC CE. RAN1 agreed to support explicit indication of deactivation via MAC-CE. The MAC CE design above is in line with this agreement, as the C bit can be used to activate/deactivate the OD-SSB transmission.

- Deactivation after N transmission occasions. RAN1 also agreed to support “Option 2: Configuration/indication of the number N of OD-SSB bursts”. While the agreement says “configuration/indication”, we think configuring the value N is sufficient and there is no need to also support the dynamic indication of N, given the gNB already has an option to dynamically deactivate the OD-SSB transmission by MAC CE and the value of N is not envisioned to change dynamically. This reduces the size of the MAC CE and complexity, especially since the MAC CE can indicate up to 32 cells.

Proposal 6:	For the agreed RAN1 “Option 2: Configuration/indication of the number N of OD-SSB bursts”, the value of N is configured in RRC and is not indicated in the OD-SSB MAC CE.

# Conclusion

The following proposals are provided for on demand SSB reception on SCells:

Proposal 1:	The OD-SSB indication MAC CE signalling indicates OD-SSB (de)-activation per SCell, along with the applicable OD-SSB periodicity per activated SCell. No other parameters need to be included in the MAC CE.

Proposal 2:	The OD-SSB indication MAC CE signalling is not used for indication of SSB periodicity adaptation (WI objective 3).

Observation 1:	RAN1 agreed to reuse legacy values for the configuration of the OD-SSB periodicity, including (5 ms, 10 ms, 20 ms, 40 ms, 80 ms, or 160 ms).

Proposal 3:	3 bits is sufficient to indicate which periodicity is applied for the OD-SSB.

Proposal 4:	A bit map in a variable size MAC CE is used to indicate the SCells for which the OD-SSB is indicated. For each indicated SCell, an additional (optional) field is added to indicate the applicable OD-SSB periodicity.

Proposal 5:	Similar to SCell Activation MAC CE, the OD-SSB MAC-CE supports two formats: one format indicates up to 7 SCells and another format for up to 31 SCells.

Proposal 6:	For the agreed RAN1 “Option 2: Configuration/indication of the number N of OD-SSB bursts”, the value of N is configured in RRC and is not indicated in the OD-SSB MAC CE.

# References

RP-234065, “New WID: Enhancements of network energy savings for NR”, Ericsson

3GPP TR 38.864, Study on network energy savings for NR (Release 18), V18.1.0, Mar 2023

R4-2416911, “LS on SSB adaptation”, RAN4