3GPP TSG-RAN2 Meeting #129bis	R2-2502588

Wuhan, China, Apr. 7th – 11th, 2025

Agenda item:	8.11.2

Source:	Qualcomm Incorporated

Title:	Views on random access for SBFD

WID/SID:	NR_duplex_evo-Core – Release 19

Document for:	Discussion and Decision

# Introduction

RAN plenary #102 approved the work item on NR duplex operation, Sub-Band Full Duplex (SBFD) and the latest WID is in [1]. The objective of the work item is to specify SBFD operation at the gNB side within a TDD carrier. One of the objectives is to specify SBFD operation to support random access in SBFD symbols for UE in RRC-connected UE while studying the benefit of extending the random-access operation in SBFD symbols to UE(s) in RRC idle and inactive state.

In RAN2 #127, the following agreements were made regarding random access for SBFD operation.

Working assumption: Random access procedure in SBFD symbols is supported for all the existing RACH trigger events. RAN2 assume RACH configuration for SBFD via SIB and/or dedicated RRC signalling is supported. Detailed signalling FFS. RAN2 to strive for a common SBFD CBRA framework independent of RRC state. FFS whether/how early indication is used during a SBFD RA procedure. RAN2 focus on 4-step RACH for SBFD RA, FFS on 2-step if needed. Cell-specific SBFD time/frequency configuration is provided by SIB1 (or via dedicated signalling to covey cell specific configuration). FFS on UE specific dedicated RRC configuration if needed, pending on RAN1 progress.

In RAN2 #127bis, the following agreements were made regarding random access for SBFD operation.

RAN2 understand that if additional RO is selected by SBFD-aware UE, early identification via Msg1 is possible from NW point of view for this UE without specification impact. From R2 point of view, there is no need to introduce SBFD as a new feature combination in the current PRACH preamble partitioning framework. Upon initiation of CBRA RACH procedure for a SBFD-aware UE, UE selects one type of ROs between legacy-ROs and additional-ROs based on certain specified/configured conditions/prioritizations, if no additional indication (FFS if there needs to be any) is from network. For the PRACH transmission re-attempt in one RACH procedure, after certain (configured) number of times of RACH attempt in SBFD RACH occasions, UE is allowed to switch to legacy RACH occasions. FFS about the case when UE select legacy ROs first. The following two RACH configuration options are considered for SBFD based random access: - Option 1: Use one single RACH configuration based on the existing parameters of the single RACH configuration. Can extend the existing parameters if needed. - Option 2: Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration For RACH configuration Option 2, RAN2 needs to specify RRC signalling for the new SBFD based RACH configuration with a new set of parameters. The RACH configuration for SBFD is transmitted via SIB1. FFS dedicated RRC signalling detail. FFS whether NW can provide both configurations.

RAN2 #128 made agreements related to random access for SBFD operation.

CFRA The RO type is indicated by NW for CFRA. FFS on signaling (can FFS for the SI request case if needed). CBRA FFS on the following options Option 1 - Upon initiation of RACH procedure for a SBFD-aware UE, network provides the indication on the prioritization of the additional ROs over legacy RO. If there is no such indication from the NW, FFS on the following mechanism - UE select legacy RO or SBFD RO based on SSB RSRP, or - UE select the legacy RO, or - UE select the SBFD RO, or - Other metrics than SSB RSRP. Option 2 UE select legacy RO or SBFD RO based on SSB RSRP if such condition is configured, and if not configured, then UE can prioritize one type of the ROs, FFS which one. RACH configuration Only one RACH configuration option (i.e., either RACH configuration Option 1 with Alt 1-1 or RACH configuration Option 2) is supported in a cell.

RAN2 #129 made greements related to random access for SBFD operation.

When a SBFD aware UE supporting one or both SBFD RACH configuration options accesses a cell, the UE can apply the supported SBFD RACH configuration option in the cell. When a SBFD aware UE supporting a SBFD RACH configuration option accesses a cell configured with a different SBFD RACH configuration option, the UE applies the legacy RA operation, and does not apply the SBFD RACH configuration. For initial RA transmission, the network can indicate the RO type (legacy RO or additional RO) to the SBFD-aware UE for the case of CBRA. Detailed signalling is FFS. If no RO type indication is provided by the NW, a UE selects RO type based on a SSB RSRP threshold. FFS whether NW can further indicate whether to select the additional RO type below or above this SSB RSRP threshold. FFS whether RO type selection is performed before or after the RA type selection. FFS if switching from the PRACH resources in non-SBFD symbols to the PRACH resources in SBFD symbols is supported.

In this contribution, we would like to discuss the configurations of random access resource in SBFD symbols for UE in RRC_CONNECTED and RRC_IDLE/INACTIVE.

# Discussion

There were already several basic agreements regarding supporting SBFD random access operation in RAN1.

Agreement Confirm the working assumption: Working assumption: For SBFD aware UEs in RRC CONNECTED state, support CBRA and CFRA in SBFD symbols. Agreement For SBFD aware UEs in RRC CONNECTED state, support Type-1 random access procedure (4-step RACH) in SBFD symbols. - FFS Type-2 random access procedure (2-step RACH)

The working assumption to support both CBRA and CFRA in SBFD symbols were confirmed in RAN1. RAN1 also agreed to support Type-1 random access (4-step RACH) in SBFD symbols and will further discuss Type-2 random access (2-step RACH) after finalizing the design of 4-step RACH. RAN2 #127 further agreed to focus on the 4-step RACH for SBFD RACH, FFS on 2-step if needed.

Observation 1: For the RRC_CONNECTED UE, RAN1 has agreed to support both CBRA and CFRA in SBFD symbols.

Observation 2: Both RAN2 and RAN1 has agreed to at least support 4-step RACH and FFS on 2-step RACH.

Several benefits can be observed if RACH procedure can use SBFD resource, such as shorter access latency, PRACH collision reduction and PRACH coverage improvement. In our view, all the existing RACH triggering events can be supported by random access in SBFD symbols. One may think that SI request may be the exception. However, we don’t see a reason to exclude it, since it has the same procedure if network configures the SBFD resource for SI request. It is up to network to configure or indicate which SI message can be requested via SBFD resource.

Proposal 1: RAN2 confirms the working assumption that random access procedure in SBFD symbols is supported for all the existing RACH trigger events.

For RACH configuration option 2, RAN2 has agreed to introduce additional RACH configuration for SBFD RACH resources and corresponding RACH parameters. RAN2 further agreed that there is no need to introduce SBFD as a new feature in the current PRACH partitioning framework. In that sense, a new separate RACH configure common RRC IE should be introduced for SBFD RACH resource in the RACH configuration option 2.

Proposal 2: For the RACH configuration option 2, a new separate SBFD RACH configuration IE is defined.

RAN2 #129 has agreed that network can indicate the RO type (legacy-ROs or additional-ROs) to the UE for the initial CBRA transmission. The network indication on the prioritization of the legacy-ROs and additional-ROs is optionally configured by network via RRC signalling.

Observation 3: For initial CBRA transmission, network can indicate the RO type (legacy-ROs or additional-ROs) to the SBFD-aware UE for the CBRA.

Proposal 3: The network indication on the prioritization of the legacy-ROs and additional-ROs is optional for network to configure via RRC signaling.

For RACH configuration Option 1, the main purpose of introducing additional-ROs is to improve RACH capacity and reduce PRACH collision probability in legacy-ROs or reduce the random-access latency.

If SBFD-aware UE is capable to support PRACH transmission in the additional-ROs, it would be beneficial if UE selects the additional-ROs in the SBFD symbols with the high priority. Using additional-ROs is also beneficial for early indication of the UE SBFD-awareness. Thus, for RACH configuration Option 1, it is good that UE has priority to select additional-ROs in the SBFD symbols.

For RACH configuration Option 2, the additional PRACH resource like additional-ROs or different type of preambles supporting SBFD operation are separately configured. The motivation for RACH configuration Option 2 is to improve the PRACH cell coverage, e.g. by introducing a long preamble sequence. Then, the design criteria for the selection of using the additional PRACH configuration should enable to extend the cell coverage for cell edges UEs. This can be achieved by comparing the RSRP of the SSB to a threshold (e.g. the criteria can be RSRP < threshold to select the additional RO). However, this will prohibit cell-centre SBFD-aware UE from early indication of the SBFD-aware based on msg1 preamble transmission in the additional-ROs. It may also create inter-UE CLI, due to higher transmission power by the cell edge UEs.

Observation 4: For PRACH configuration Option 2, prioritization of the additional-RO for cell-edge SBFD-aware UE, UE will improve the PRACH cell-coverage. However, it may create inter-UE CLI and will restrict cell centre SBFD-aware UEs from early indication based on msg1 PRACH transmission in additional-RO.

RAN2 #129 has agreed that if no RO type indication is provided by the NW, UE selects RO type based on a SSB RSRP threshold, and how to use this RSRP threshold such as above or below the SSB RSRP threshold for additional-ROs selection is FFS.

In our view, for the cell center SBFD-aware UEs whose RSRP are above the configured SSB RSRP threshold, those UE should always prioritize the additional-ROs no matter network configures RACH configuration option 1 or 2. There is no reason to not allow the cell center SBFD-aware UE to utilize the additional-ROs for the benefit of RACH capacity improvement, PRACH collision reduction, RACH latency reduction and RACH coverage extension. This selection criteria are applied for both RACH configuration option 1 and 2.

Proposal 4: Upon initiation of RACH procedure for a SBFD-aware UE, if no indication is received from network, the cell center SBFD-aware UE whose RSRP is higher than the configured SSB RSRP threshold should always prioritize the additional-RO.

For the cell edge SBFD-aware UEs whose RSRP are below the configured SSB RSRP threshold, the RO type selection can consider to be based on different RACH configuration options. Since the motivation of RACH configuration Option 2 mainly for the coverage extension, cell edge SBFD-aware UE should be allowed to use the additional-ROs to enjoy the benefits provided by RACH configuration option 2. For RACH configuration option 1, the cell-edge SBFD-aware UE selects the legacy-ROs to reduce the potential interference cause by inter-UE CLI.

Proposal 5: Upon initiation of RACH procedure for a SBFD-aware UE, if no indication is received from network, the cell edge SBFD-aware UE whose RSRP is lower than the configured SSB RSRP threshold selects the type of RO based on the RACH configuration options configured by network, i.e., cell edge SBFD-aware UE selects the additional-ROs if network configures RACH configuration option 2, and selects the legacy-ROs if network configures RACH configuration option 1.

Since the SSB RSRP threshold could be optionally configured, if neither network indication nor the RSRP threshold is configured by network, the initial RACH attempt should be determined based on the prioritization of the additional-ROs over legacy-ROs.

Proposal 6: Upon initiation of RACH procedure for a SBFD-aware UE, if neither network indication nor the RSRP threshold is configured by network, the initial RACH attempt should be determined based on the prioritization of the additional-ROs over legacy-ROs.

Regarding PRACH transmission repetition, RAN1 #118 made the following agreements regarding the PRACH transmission with preamble repetitions.

Agreement For SBFD aware UEs, at least support the following: - PRACH transmission with preamble repetitions within additional-ROs (not across additional-Ros and legacy ROs) Note: SBFD aware UEs support PRACH transmission with preamble repetitions only within legacy-ROs (not across additional-ROs and legacy ROs) as in legacy releases.

RAN1 #118bis has conclusion.

Conclusion PRACH transmission with preamble repetitions across additional-ROs and legacy-ROs is not supported.

RAN1 #119 made follow agreements regarding the PRACH transmission repetition.

Agreement Confirm the following working assumption Working Assumption For a PRACH transmission with Npreamblerep preamble repetition within additional-ROs, support reusing current specification rules for the determination of the set of Npreamblerep valid additional-ROs and the time period of the set(s) of Npreamblerep valid additional-ROs. - the time period of the set(s) of Npreamblerep valid additional-ROs is determined separately from the time period for legacy-ROs

Agreement For RACH configuration Option 2, support separate configuration of rsrp-ThresholdMsg1-RepetitionNum2/4/8 and msg1-RepetitionNum for PRACH transmission with preamble repetitions within additional-ROs and PRACH transmission with preamble repetitions within legacy-ROs. - msg1-RepetitionNum is configured in RACH-ConfigCommon in current specification, i.e., for RACH configuration Option 2, msg1-RepetitionNum can already be separately configured for legacy-ROs and additional-ROs. Agreement For RACH configuration Option 1, support separate configuration of rsrp-ThresholdMsg1-RepetitionNum2/4/8 for PRACH transmission with preamble repetitions within additional-ROs and PRACH transmission with preamble repetitions within legacy-ROs. - Same msg1-RepetitionNum is used for PRACH transmission with preamble repetitions within additional-ROs and PRACH transmission with preamble repetitions within legacy-ROs.

Based on RAN1 agreements, it is supported that PRACH transmission with preamble repetitions can be configured within the additional-ROs. In the current RACH resource partitioning framework, besides the Msg1 repetition, network may configure different sets of RACH resource associated with different features or feature combinations. RAN2 #129 has discussed whether the SBFD-aware UE selecting the additional-ROs or legacy-ROs should before or after the RACH resource sets selection for the feature or feature combinations.

According to RAN1 agreements, for RACH configuration option 2, the RSRP thresholds of Msg1 repetition and the Msg1 repetition numbers for PRACH repetitions are separately configured within additional-ROs and legacy-ROs. The parameter settings and value may be different in the two types of ROs. For RACH configuration option 1, the RSRP thresholds for Msg1 repetition is also separately configured. Thus, the conditions for performing Msg1 repetition and the corresponding Msg1 repetition number is different for additional-ROs and legacy-ROs.

Since the evaluation on whether performing Msg1 repetition is in the RACH partitioning resource selection step, it is simpler to select RO type (additional-ROs and legacy-ROs) before the RACH resource set selection than after. If RO type selection happens after the set of RACH resource selection for the feature or feature combinations, UE has to evaluate and determine the RACH resource of feature combinations on both types of ROs. This is not aligned with the current RO type selection criteria agreed in RAN2 (e.g., network indication, RSRP threshold, configured priority, and etc..) and it may cause unnecessary complexity of RACH resource selection procedure. Considering the simplicity of RACH procedure and less specification impact, the RO type selection should be performed before the set of RACH resource selection for the feature combinations.

Proposal 7: The RO type selection between the additional-ROs and legacy-ROs should be performed before the selection on the set of RACH resources associated with the feature combinations.

For RACH configuration option 2, the RACH resource partitioning can be separately configured within additional-ROs and legacy-ROs. It is possible that network may configure the set of RACH resources for certain feature combinations on legacy-ROs and may not configure RACH resources for those feature combinations in additional-ROs (or vice versa).

If RAN2 can agree the proposal 7 that RO type selection should be performed before the RACH resource set selection, the availability checking and RACH resources set selection should be limited to the selected RO type. In that sense, UE may lose the opportunity to select the set of RACH resources configured on the non-selected RO type.

In our understanding, how to configure the RACH resource partitioning in a given RACH resource type (SBFD or non-SBFD) is up to network implementation. Network can decide to configure the RACH resource associated with the feature combinations on one type of RO or both types.  Cross-checking the RACH resource partitioning resource in both type of ROs may have too much complexity for the RACH procedure and it is not preferred.

Proposal 8: If UE has met the conditions of selection on one type of RO, UE does not need to check and evaluate the set of RACH resource of feature combinations configured in another type of ROs.

In case of PRACH attempt failures, UE may perform PRACH re-attempt. In RAN2 #127bis, it has been agreed that ‘after certain (configured) number of times of RACH attempt in SBFD RACH occasions, UE is allowed to switch to legacy RACH occasions. FFS about the case when UE select legacy ROs first.’

If network indicates that the SBFD-aware UE selects the legacy-ROs first, and after UE keeps attempting on the legacy-RO with a number of times and if it is still failed, it implies the legacy-ROs resource may be congested and overload. UE should be allowed to switch to the SBFD RACH resource for re-attempt.

Proposal 9: For the PRACH transmission re-attempt in one RACH procedure, after certain (configured) number of times of RACH attempt in legacy RACH occasions, SBFD-aware UE is allowed to switch to SBFD RACH occasions for RACH re-attempt.

For the RACH fallback procedure between two types of RACH resource (additional-ROs under SBFD resource or legacy-ROs under legacy RACH resource), some companies raised the concern in RAN2 #129 that the ping-pong switch between the two types of RACH resource should be avoid. To resolve this issue, it can be defined that only one time fallback between two types of RACH resource is allowed.

Proposal 10: The RACH fallback (after certain number of RACH attempts in one type of RACH occasions) from one type of RACH resource to the other type of RACH resource is only allowed once.

When UE performs RACH fallback procedure from type of RACH resource to the other, the feature combinations on the two types of RACH resources should be considered. In legacy RACH fallback procedure (e.g., Msg1 repetition), the fallback is only allowed to the same feature combinations with the increased Msg1 repetition number in order to reduce the complexity of RACH fallback under RACH partitioning framework.

In the context of RACH fallback between SBFD RACH and legacy RACH resource, the similar design principle can be applied. The RACH fallback can only be allowed when the additional-ROs and legacy-ROs have the same he same feature combinations. UE is allowed to switch to the other type of ROs with the same feature combinations. If there is no same feature combination on another type of ROs when performing fallback, it can be for further study.

Proposal 11: For RACH fallback procedure from one type of RACH resource to another type of RACH resource, UE is allowed to switch the type of RACH occasions configured within the same feature combinations. FFS on the case of no same feature combination configured on another type of RACH occasions when performing fallback.

It is important for gNB to know the UE SBFD-aware capability in early stage of the random-access procedure to leverage the SBFD operation. The SBFD-aware indication is useful to inform the UE SBFD capability at the earliest time during RACH, e.g., network may schedule the resource for Msg3 in the SBFD resource in Msg2 if network can be aware the SBFD capable UE performing the RACH.

If network has configured the RACH occasions on SBFD symbols and UE also supports it, the SBFD-aware UE sends the Msg1 in the SBFD RACH occasions as the SBFD-aware indication to network. So, by using the SBFD RACH occasions can be already an implicit way as the SBFD-aware indication.

If the SBFD-aware UE selects the legacy RACH occasion, the additional SBFD-aware indication is needed. If Msg1 based SBFD-aware indication is not supported, Msg3 based SBFD-aware indication should be supported from RAN2 perspective.

Proposal 12: If Msg1 based SBFD-aware indication is not supported, Msg3 based SBFD-aware indication should be supported from RAN2 perspective.

In RACH configuration Option 2, when there are legacy-RO in legacy PRACH configuration and additional-RO in the SBFD PRACH configuration that are time overlapping and have same start symbol index in the uplink slot. In such case, if legacy UE selects the legacy RO and SBFD-aware UE selects the additional-RO, the computation from the existing RA-RNTI formulation will be the same. There is ambiguity on the reception of RA response. To resolve this issue, RAN2 to study either update the RA-RNTI formulation or rely on gNB implementation to avoid overlapping of legacy-RO and additional-ROs in the configuration

Proposal 13: RAN2 to study the RA-RNTI collision issue for RACH configuration Option 2.

# Conclusion

We have the following observations, and we’d recommend RAN2 to discuss and adopt the following proposals:

Observation 1: For the RRC_CONNECTED UE, RAN1 has agreed to support both CBRA and CFRA in SBFD symbols.

Observation 2: Both RAN2 and RAN1 has agreed to at least support 4-step RACH and FFS on 2-step RACH.

Observation 3: For initial CBRA transmission, network can indicate the RO type (legacy-ROs or additional-ROs) to the SBFD-aware UE for the CBRA.

Observation 4: For PRACH configuration Option 2, prioritization of the additional-RO for cell-edge SBFD-aware UE, UE will improve the PRACH cell-coverage. However, it may create inter-UE CLI and will restrict cell centre SBFD-aware UEs from early indication based on msg1 PRACH transmission in additional-RO.

Proposal 1: RAN2 confirms the working assumption that random access procedure in SBFD symbols is supported for all the existing RACH trigger events.

Proposal 2: For the RACH configuration option 2, a new separate SBFD RACH configuration IE is defined.

Proposal 3: The network indication on the prioritization of the legacy-ROs and additional-ROs is optional for network to configure via RRC signaling.

Proposal 4: Upon initiation of RACH procedure for a SBFD-aware UE, if no indication is received from network, the cell center SBFD-aware UE whose RSRP is higher than the configured SSB RSRP threshold should always prioritize the additional-RO.

Proposal 5: Upon initiation of RACH procedure for a SBFD-aware UE, if no indication is received from network, the cell edge SBFD-aware UE whose RSRP is lower than the configured SSB RSRP threshold selects the type of RO based on the RACH configuration options configured by network, i.e., cell edge SBFD-aware UE selects the additional-ROs if network configures RACH configuration option 2, and selects the legacy-ROs if network configures RACH configuration option 1.

Proposal 6: Upon initiation of RACH procedure for a SBFD-aware UE, if neither network indication nor the RSRP threshold is configured by network, the initial RACH attempt should be determined based on the prioritization of the additional-ROs over legacy-ROs.

Proposal 7: The RO type selection between the additional-ROs and legacy-ROs should be performed before the selection on the set of RACH resources associated with the feature combinations.

Proposal 8: If UE has met the conditions of selection on one type of RO, UE does not need to check and evaluate the set of RACH resource of feature combinations configured in another type of ROs.

Proposal 9: For the PRACH transmission re-attempt in one RACH procedure, after certain (configured) number of times of RACH attempt in legacy RACH occasions, SBFD-aware UE is allowed to switch to SBFD RACH occasions for RACH re-attempt.

Proposal 10: The RACH fallback (after certain number of RACH attempts in one type of RACH occasions) from one type of RACH resource to the other type of RACH resource is only allowed once.

Proposal 11: For RACH fallback procedure from one type of RACH resource to another type of RACH resource, UE is allowed to switch the type of RACH occasions configured within the same feature combinations. FFS on the case of no same feature combination configured on another type of RACH occasions when performing fallback.

Proposal 12: If Msg1 based SBFD-aware indication is not supported, Msg3 based SBFD-aware indication should be supported from RAN2 perspective.

Proposal 13: RAN2 to study the RA-RNTI collision issue for RACH configuration Option 2.

# References

- RP‑241614, Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei