3GPP TSG-RAN WG2 Meeting #129-bis						R2-2502599

Wuhan, China, 7th – 11th April, 2025

Title: 		Discussion on LCM for UE-sided model for Beam Management use case

Source: 	Huawei, HiSilicon

Agenda item:	8.1.2.2

Document for:	Discussion/Decision

# Introduction

In RAN2 #129 meeting, RAN2 made the following agreements for the UE-side model for BM case, including the functionality applicability report, status exchange of functionality applicability report and agreements for UE-side data collection.

Agreements 1. Inference configuration/parameters can be signaled in step 3 and/or Inference configuration can be signaled in step 5 (i.e., option a and option b from RAN1). 2. The full inference configuration is sent in CSI-ReportConfig. 3. Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability. 4. FFS signaling details for option B (e.g., whether it is signalling in CSI-ReportConfig or otherconfig) 5. Support the explicit reporting of applicability/inapplicability in initial report and subsequent reporting it reports only applicability it changed. FFS if we report explicit cause 6. If option A is configured in Step 3, for periodic CSI reporting, the UE autonomously activate the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete in step 4 (i.e. without need to wait RRCReconfiguration in Step 5). 7. The provided periodic CSI configuration should be consistent with reported UE capabilities 8. FFS option B 9. Semi-persistent and aperiodic CSI reporting of applicable functionality is activated following legacy CSI framework: - Semi-persistent reporting, activated by MAC CE/DCI - Aperiodic CSI reporting, activated by DCI

- Agreements - Data collection related configuration(s) and associated ID(s)(if needed) can be included in training data collection configuration. - For data collection configuration UE-side model training, the UE can send a request for data collection (e.g. start/stop). FFS whether a suggested data collection configuration/associated IDs (if specified)/parameters can be provided to the network. - The network can provide or release the data collection configuration (at any point in time), with or without UE request. - The following methods for network control of the initiation and configuration for data collection: - The network can decide when to start/stop the data collection and send configuration. - The network can configure whether UE is allowed to initiate request for data collection (e.g. start/stop indication).

In this paper, we discuss the FFSs highlighted in the above box considering the LS from RAN1.

# Discussion

## Direction A and B for applicable functionality determination

According to the LS[2], RAN1 discussed two Directions A) and B) for the functionality configuration in Step 3 as highlighted in yellow and green respectively:

- In Step 3, following configurations are provided from NW to UE: - UE is allowed to do UAI reporting via OtherConfig, - The applicability report is based on A) and/or B) - It is up to RAN 2 to design the container - A) one or more of CSI-ReportConfig for inference configuration (wherein the associated ID may be configured in CSI framework as working assumption applied) - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3 - B) One set or multiple sets of inference related parameters for applicability report only (not for inference) - It is up to RAN2 to design the container. - The set of inference related parameters selected from the IEs in/or the IEs referred by CSI-ReportConfig as a starting point, e.g., - the associated ID - Note: this doesn’t imply the associated ID is mandatory - Set A related information - Set B related information - Report content related information - For BM-Case 2, - Time instances related information for measurements - Time instances related information for prediction

For Direction A, the gNB provides one or more of CSI-ReportConfig for inference operation to the UE in Step 3, while for Direction B, the gNB provides only the parameter settings for Set A and B for a functionality and is not a full CSI report configuration for inference as indicated in the RAN1 LS where it is mentioned that it is for applicable reporting only and not for inference.

Although the benefit for Direction A is a simple procedure for NW, the full inference configurations in Step 3 have high probability of being non-applicable because NW does not check inference related parameters in advance, which may not be a resource friendly signalling overhead point of view. In contrast, Direction B can avoid the NW configuring full inference configuration without knowing the applicability of the configuration. Besides, based on procedure of Direction B, another motivation is to reduce the signalling overhead (more efficient) in checking for the applicability due to using some parameter settings of Set A and B instead of the full CSI-RS inference configurations including the reporting type configurations (i.e. periodic, aperiodic etc.) that may not get activated as in Direction A.

Observations 1: The motivation of Direction B is as below:

- Avoid the NW having to configure full inference configuration without knowing the applicability of the configuration

- Reduce the signalling overhead (more efficient) in checking for the applicability due to using some parameter settings of Set A and B instead of the full inference CSI-RS configurations including the reporting type configurations (i.e. periodic, aperiodic etc.) that may not get activated as in Direction A.

According to the LS[2], RAN1#119 meeting makes agreement that in Step 3, the task of RAN2 is to design the container and RAN2 briefly discussed the design of the container of Direction A and B in R2-2501517[3] with the following 4 options:

Option 1: The full inference configuration is sent in CSI-ReportConfig and the set(s) of inference related parameters is/are sent in OtherConfig.

Option 2: The full inference configuration and the set(s) of inference related parameters are sent in CSI-ReportConfig. A flag is introduced in CSI-ReportConfig to indicate whether a full inference configuration or a set of inference related parameters is contained in that CSI-ReportConfig.

Option 3: The full inference configuration is sent in CSI-ReportConfig and the set(s) of inference related parameters is/are sent in a new IE in a list in CSI-MeasConfig, e.g., CSI-ReportConfigPartial.

Option 4: The full inference configuration is sent in CSI-ReportConfig and the set(s) of inference related parameters is/are sent in OtherConfig. Additionally, the IDs of the full inference configurations are sent in OtherConfig.

For Direction B, the gNB provides only the parameter settings for Set A and B for a functionality and they are used only for applicability reporting and not for inference, as indicated in the RAN1 LS. Even if the UE reports the configuration corresponding to a functionality as applicable, the network will have to send another RRC reconfiguration (i.e. in Step 5) to provide the CSI-ReportConfig configurations for inference. So, it is more logical to configure the parameter settings for Set A and B in OtherConfig together with the configuration for the applicability reporting via UAI.  Hence, Option 1 makes more sense to us than Option 2 where the configuration of applicable reporting is included into the CSI-ReportConfig.

Observation 2: For Direction B, only the parameter settings for Set A and B for a functionality are used only for applicability reporting and not for inference as indicated in the RAN1 LS and hence including applicability reporting configuration of Direction B in OtherConfig as in Option 1 makes more sense than in CSI-ReportConfig as in Option 2.

For Direction B in Option 3, a new IE is introduced in a list in CSI-MeasConfig. The description of CSI-MeasConfig in 38.331 is:

The IE CSI-MeasConfig is used to configure CSI-RS (reference signals) belonging to the serving cell in which CSI-MeasConfig is included, channel state information reports to be transmitted on PUCCH on the serving cell in which CSI-MeasConfig is included and channel state information reports on PUSCH triggered by DCI received on the serving cell in which CSI-MeasConfig is included.

The IEs under CSI-MeasConfig are to configure UE to report the channel state information, but in our purpose, we just want UE to use these set(s) of inference related parameters to confirm and report whether they are applicable, but not use them to do CSI report. Hence, we do not think the set(s) of inference related parameters should be included in a new IE in list in CSI-MeasConfig.

Observation 3: For Option 3, in the current definition of the IEs under CSI-MeasConfig, it is to configure UE to report the channel state information in the CSI report, and not to use these set(s) of inference related parameters to confirm and report whether they are applicable. Hence Option 3 is not suitable.

In Option 4, the motivation is to allow the same ID space to be used to provide the association between the full inference configuration in the CSI-ReportConfig and the parameter settings of Set A and B in the OtherConfig.  Since it is our understanding that the signaling details are dependent on the L1 parameter list from RAN 1, whether such common ID is needed can wait for further details from RAN 1.

Based on the above observations, it is proposed that:

Proposal 1: For Direction B, one set or multiple sets of inference related parameters for applicability report is included in OtherConfig together with the configuration of UAI for applicability reporting.

Furthermore, in RAN1 LS, the following information on Direction B is provided:

  - The set of inference related parameters selected from the IEs in/or the IEs referred by CSI-ReportConfig as a starting point, e.g.,

    - the associated ID

      - Note: this doesn’t imply the associated ID is mandatory

    - Set A related information

    - Set B related information

    - Report content related information

    - For BM-Case 2,

      - Time instances related information for measurements

      - Time instances related information for prediction

Based on the above, we can further provide the following signalling details:

Proposal 2: As per RAN1 LS, the following information should be included as part of Direction B configuration:

    - Optionally one or more associated ID;

    - Set A related information

    - Set B related information

    - Report content related information

    - For BM-Case 2,

      - Time instances related information for measurements

      - Time instances related information for prediction

Further details will be based on RAN1 L1 parameters list.

## Applicability reporting procedure

As in the RAN1 LS, the report of applicability depends on the configuration defining the functionality in Step 3 based on Direction A) and B) as follow:

- In Step 4, UE reports applicability for all the above A) one or more CSI-ReportConfig and/or B) set(s) of inference related parameters - FFS on whether/what other information along with the applicability is needed

In RAN2#127bis, RAN2 agreed that the applicable reporting can be in UAI or in the RRC Reconfiguration Complete message and the aim is to align the report of applicability for both messages.

UAI is supported and RRCReconfigurationComplete message can be used to report applicable functionality.   We should aim to align the design on how the applicable functionality are signaled.   FFS on the applicability reporting content.

The above agreement is assumed to be applied to both Direction A) and B).

In RAN2#129 meeting agreement, RAN2 agrees the following procedure based on Direction A.

Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability.

Regardless of whether Direction A) and/or B) is applied in Step 3, we think the purpose of Step4 is the same, which is to report the applicability of the full inference configuration in Direction A or of the parameter settings of Set A and B for Direction B. The agreement based on Direction A in RAN2#129 should be extended to Direction B.

Proposal 3: Upon receiving a full inference configuration in CSI-ReportConfig or a set of inference parameters in OtherConfig, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability.

However, the design of the signaling on reporting applicability for Direction A) or B) may not be the same. For Direction A) in Step 3, the UE may report a bitmap or CSI-ReportConfig IDs corresponding to the inference configuration to indicate the applicability of the functionality. If gNB provides Direction B) where one set or multiple sets of inference related parameters are signaled for applicability report, the UE may indicate the set of the inference related parameters of the applicable functionalities (e.g. a Set ID).  To align the design on how the applicable functionality are signaled in both UAI and RRC Reconfiguration Complete message, the UAI message containing the applicable reporting can be carried as container in RRC Reconfiguration Complete.

Observation 4: Regardless of whether Direction A) and/or B) is applied in Step 3. UAI and RRCReconfigurationComplete messages can be used to report applicable functionality and RAN2 aim to align the design on how the applicable functionality are signaled in both messages.

Proposal 4: To align the design on how the applicable functionality are signaled in both UAI and RRC Reconfiguration Complete message, the UAI message containing the applicable reporting can be carried as container in RRC Reconfiguration Complete.

Proposal 5: The applicability reporting in the UAI message may indicate either a certain CSI config (for Direction A) and/or some set of parameters (for Direction B)) as applicable (e.g. using a bitmap, or set of indexes etc. which can be further discussed in a stage-3 design).

For Direction A) where one or more CSI-ReportConfig is provided in Step 3 for inference, RAN1 agrees that:

- Applicable aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after the applicability reported. - Applicable periodic CSI Report is considered as activated only if the applicability of the corresponding CSI-ReportConfig is reported in RRCReconfigurationComplete.

Also, RAN1 has conclusion below, if CSI-ReportConfig for inference configuration is provided in Step 5:

- Note: UE is not expected to be configured with a CSI-ReportConfig for inference configuration for a non-applicable set of inference parameters or a non-applicable CSI-ReportConfig

For Direction A,  after Step 3, UE may have non-applicable CSI-ReportConfig. Based on the note above, UE is not expected to be configured for inference configuration with a non-applicable CSI-ReportConfig. For such non-applicable functionalities/configurations, UE can autonomously release those CSI report configurations that are not applicable after Step 4, or wait until Step 5 for the NW to release those configurations.  Note that this may occur not just during initial configuration case in Step 3. An applicable CSI Report may become non-applicable some time in future after initial configuration. RAN2 agree that UE doesn’t autonomously deactivate or de-configure an active functionality as follow:

When a functionality becomes non-applicable the UE doesn’t autonomously deactivate. NW is expected to deactivate active functionality when it receives report from UE that it is non-applicable.

Sticking to the same principle as this agreement, UE should not autonomously release non-applicable CSI report configurations after Step 4.

Proposal 6: if Direction A (i.e., CSI-ReportConfig for inference configuration is provided) in Step 3, UE doesn’t autonomously release those CSI report configurations that are non-applicable after Step 4 but leave them to gNB to de-configure the non-applicable CSI-ReportConfig in subsequent.

## Applicability reporting contents

Associated ID is agreed as a working assumption by RAN1 to be used to ensure the consistency of NW-side additional condition across training and inference for UE-sided model for BM-Case 1 and BM Case 2, when needed. The network side additional condition alignment can be done in the following ways.

- First, the UE trains the model based on the dataset and associates it with the related associated ID, if provided by the network.

- The UE reports the applicable functionality to the NW, which may be associated with the NW-side additional condition identified by the associated ID.

- NW selects the applicable functionality and determines the inference configuration based on the NW-side additional condition.

In Step 3, following configurations are provided from NW to UE: - UE is allowed to do UAI reporting via OtherConfig, - The applicability report is based on A) and/or B) - It is up to RAN 2 to design the container - A) one or more of CSI-ReportConfig for inference configuration (wherein the associated ID may be configured in CSI framework as working assumption applied) - B) One set or multiple sets of inference related parameters for applicability report only (not for inference) - the associated ID Note: this doesn’t imply the associated ID is mandatory

From RAN1 #119 agreement highlight above, the NW-side additional condition can be indicated in step 3 optionally on the Direction A and B for applicability report.

For Option 2 (for Direction B), the associated ID can also be optionally indicated by the UE to the NW in Step 4. In our view, associated ID is optional for the whole LCM process, i.e., even for training phase the NW might not provide it and the UE should still be able to determine the applicability of the functionality based on implementation, but this needs to be concluded by RAN1 based on the above reply from RAN1.

Observation 5: Based on the RAN1’s agreement for Direction A and B, the NW-side additional condition can be indicated in Step 3 optionally. For Direction B, the associated ID can also be optionally indicated by the UE to the NW in Step 4.

Based on Observation#2, the UE reporting behavior can be as follows depending on whether the network indicates or not the NW-side additional condition in Step 3 in all options or whether the UE indicates back or not in Step 4 for Options 2:

- Case 1: If the network indicates the associated ID, the network can indicate the network-side additional condition it is interested in. In this case, the UE reports only the applicable functionalities corresponding to the indicated associated ID(s).

- Case 2: If the network does not indicate the associated ID, the UE can determine which of the functionalities are applicable from UE perspective considering model availability and UE side additional conditions and report those functionalities together with the corresponding associated IDs, if available. Based on this, the network can select and configure the selected applicable functionality for inference according to the current network-side addition condition.

For Case 1, NW may indicate the interested associated IDs when requesting the applicable functionality reporting, and then the UE only needs to report the applicable functionalities associated with the indicated associated IDs. In this case, the UE can do the filtering based on the associated ID information, so that it does not need to report all the applicable functionality information stored. However, if the NW-side additional condition changes to the one not requested earlier from the UE, the network will have to ask the UE to report the applicable functionalities again, with the new associated ID.

For Case 2, when the NW side additional condition changes, the NW can do an update based on the associated ID reported earlier. However, in some situations, it may cause some signaling overhead in case the network is able to currently support only a very few NW-side additional conditions/associated IDs. For, example, if the UE has applicable functionality for associated ID #1, #2, and #3, but the current NW supports only ID#4, then the UE reporting of other IDs is meaningless.

Based on the above analysis, both Case 1 and Case 2 are useful and the choice between them will depend on the NW implementation and current NW-side additional conditions. We think it should be left to NW implementation which option to use at a certain time. If the NW indicates associated ID(s) to UE, the UE checks the indicated associated ID for determination of applicable functionalities; otherwise, the UE only checks UE-side additional conditions and model availability and reports applicable functionalities together with the supported associated IDs, if available.

Proposal 7: The NW-side additional condition is optional in Step 3. The NW can decide whether to provide the NW-side additional condition by implementation, and the applicable functionalities determination can be supported in the following ways:

- In case the network does not provide the UE with associated IDs, the UE performs applicable functionality filtering without considering NW-side additional conditions and reports the (UE-considered) applicable functionalities together with their associated IDs, if available.

- In case the network provides the UE with associated IDs, the UE performs applicable functionality filtering considering them and only reports the functionalities meeting the NW-side additional condition.

There are some FFSes in the running RRC CR.  One is on the indication of which serving cell a functionality is for in a CA scenario in the applicability reporting:

FFS if applicability reporting is supported for multiple serving cells and applicabilityCellId is needed to unambiguously identify CSI report configurations for prediction.

It makes sense that applicability report is supported for multiple serving cells, which depends on UE capability (e.g. how many serving cells can the UE support for inference etc.). Combined with the RAN1 #119 agreement, the applicability report is based on A) and/or B).

- In Step 3, following configurations are provided from NW to UE: - UE is allowed to do UAI reporting via OtherConfig, - The applicability report is based on A) and/or B) - It is up to RAN 2 to design the container - A) one or more of CSI-ReportConfig for inference configuration (wherein the associated ID may be configured in CSI framework as working assumption applied) - B) One set or multiple sets of inference related parameters for applicability report only (not for inference)

On Direction A, NW will send CSI-Reconfiguration for inference configuration, which is specific to a serving cell. If applicability report is based on direction A), naturally, we can include applicabilityCellId in applicability report.

On the other hand, for Direction B, since the OtherConfig is UE specific, if UE support applicability report for multiple serving cell, the NW need to indicate which serving cell the set of inference related parameters are for.  E.g., indicating for each set of Inference related parameters can be used for which serving cells.  The details of the signaling can be further discussed (e.g. whether the NW can indicate a list of serving cells related to a set of inference parameters). Then, UE can use applicabilityCellId to distinguish applicability report for different serving cells.

Proposal 8: For Direction B, the gNB needs to be able to indicate the serving cell(s) in which a set of inference parameters are related to.

Proposal 9: For both Direction A and B, the UE needs to indicate for the applicable reporting in the RRCReconfigurationComplete and UAI which serving cells a full inference configuration or a set of inference parameters are applicable for.

Another FFS is other than the applicability status of the functionality, whether there is other information on the reporting.

FFS other content in ApplicabilityReport, e.g., for option B

Initial KPI of applicable functionality can be provided as part of applicable functionality reporting. For example, prediction accuracy can be calculated during training by using data for the ground truth as reference against the trained model. This prediction accuracy can be defined as initial KPI. It would be beneficial for the network to know how well the applicable functionality has been trained (by the OTT server) before it configures the UE to perform prediction on the functionality or trigger the data collection for training to improve on the KPI or perform performance monitoring before activating the applicable functionality. Without such initial KPI information of a reported applicable functionality by UE, the network may activate an applicable functionality where it may not meet its expectation and requiring starting the performance monitoring immediately after activation.

Observation 6: Without such initial KPI information of a reported applicable functionality by UE, the network may activate an applicable functionality where it may not meet its expectation and requiring starting the performance monitoring immediately after activation.

Proposal 10: Initial KPI of trained functionality can be as part of applicable functionality reporting.

## Applicability status changes

In RAN2#129, the reporting of the changes of the applicability of a functionality is discussed with the following agreement that the UE reports explicitly applicability/inapplicability in initial report (i.e. in the response of the RRCReconfigurationComplete where the functionality is configured for Direction A or B), and any subsequent changes of the configured functionalities, UE only reports the functionalities with applicability status change.

Support the explicit reporting of applicability/inapplicability in initial report and subsequent reporting it reports only applicability it changed. FFS if we report explicit cause

However, this agreement does not capture whether the UE ‘shall’ or ‘may’ report and the running CR currently leave it as ‘may’:

Running CR: A UE capable of providing assistance information related to the applicability of configurations subject to the applicability determination procedure may initiate the procedure in several cases, including upon being configured to report assistance information about the applicability of configurations related radio measurement predictions and upon change of the applicability of the configurations related to radio measurement predictions.

We think this should be a ‘shall’ as UE not reporting this may result in performance degradation. For example, if an activated functionality becomes non-applicable, UE has to report it so that the NW can deactivate/de-configure the functionality to avoid any system performance degradation).

Proposal 11: When the status of the functionality which has been previously requested by the network changes from applicable to non-applicable or vice-versa, UE shall report the status change information to network.

On the other hand, it should be noted that as opposed to the changes of NW side conditions, UE internal conditions are unseen and unpredicted from NW perspective. Even if UE could have the flexibility to timely indicate the status changes of the applicable functionality due to UE internal conditions, signaling overhead is introduced, especially for the cases where the changes among different functionality statuses are frequent. On the other hand, if status updates frequency is limited by the network (e.g. using a prohibit timer), then there may be a delay in UE reporting a functionality as non-applicable. In this case, the NW may need to configure more resources for performance monitoring, but this will introduce additional resource overhead.

Observation 7: Changes among different functionality statuses have an impact on functionality management. Frequent changes may result in frequent activation/deactivation or may require more resources for performance monitoring, which will introduce signaling/resource overhead.

From network perspective, it is preferred to choose such functionalities for inference which are likely to be used for a longer time and which are not prone to frequent status changes, e.g. due to dynamic changes of UE-side conditions. To help the network to choose proper configurations, it would be useful if on top of bare applicability information, the UE also provided also some kind of “stability of applicability” indication. For example, if the UE knows a certain inference operation is likely to exceed its processing power or that the model is only applicable for the current UE speed, the network should be aware of this. Such indication does not have to mention an explicit reason why the functionality status may change, but instead the UE could for example indicate the time during which the UE commits to keep the applicability of the functionality. Such information would allow the network to achieve a better tradeoff between signaling/resource overhead and system performance guarantee and would be helpful for performing appropriate functionality management. For example, if the applicable status can be maintained for a relatively long time, less resources for functionality monitoring are needed. If the applicable status cannot be maintained for a relative long time or cannot be determined reliably, the NW may decide not to activate the functionality, even though it is applicable for the time being, or the NW may still choose to activate the functionality, but configure more resources for performance monitoring.

Observation 8: Frequent functionality applicability changes/updates may have a detrimental impact on the signaling overhead, system performance and complicate AIML functionality management by the network.

Proposal 12: RAN2 should investigate the means to avoid the need of frequent applicability status updates and ensure the stability of inference operation once it is activated (e.g., UE can provide some kind of “stability of applicability” indication for each applicable functionality etc.).

As a commonly used method to avoid the frequent reporting, a prohibit timer can be configured. However, with the prohibit timer, the NW may not get the applicability information timely and cause some problem, as shown in the following cases.

- Case 1: if a functionality becomes “non-applicable” but cannot be updated because the prohibit timer is still running, meanwhile the NW decides to configure this functionality for inference, the inference configuration can fail. In this case, the UE needs to indicate the inference configuration failure to the NW after receiving the inference configuration.

- Case 2: a functionality becomes “applicable” but cannot be updated timely, the NW cannot configure the functionality for inference immediately if the NW is interested in this functionality.

From the two cases, it can cause different results if the NW cannot know the applicability update from “applicable to non-applicable” and from “non-applicable to applicable”. Thus, we need to configure different prohibit timer for these different cases. For example, if the NW wants to avoid the first cases and can endure the second case, the NW can configure the value of the prohibit timer for “applicable to non-applicable” update as 0 or a small value, and configure a proper value (which is larger than the prohibit timer for “applicable to non-applicable”) for the prohibit timer for “non-applicable to applicable” update.

Observation 9: The prohibit timer can be applied to avoid the frequent report for the applicability update of functionalities. However, if the UE cannot report the “applicable to non-applicable” update timely, the NW may configure a non-applicable functionality for inference.  If the UE cannot report the “non-applicable to applicable” update timely, the NW cannot configure the functionality for inference immediately when the NW is interested in this functionality.

Proposal 13: The NW can configure different values of the prohibit timer for “applicable to non-applicable” update and “non-applicable to applicable” update to be applied for functionalities that are not currently configured for inference operation. If a functionality that is currently configured for inference operation becomes non-applicable, the UE should immediately report the status change.

## UE-side data collection for training

### Discussion on data collection configuration and UE request

In UE-side Data Collection part, we have agreement as following:

Agreement For data collection configuration UE-side model training, the UE can send a request for data collection (e.g. start/stop).

We need to consider a scenario where the UE initiates a data collection request but does not receive a response from the network (i.e., the network fails to provide the necessary configuration for UE-side data collection), the UE may repeatedly send request messages, leading to excessive signaling overhead. To mitigate this issue, it is essential to implement mechanisms to limit such abnormal behavior and prevent the associated signaling overhead.

As a commonly used method to avoid the frequent request by UE, a prohibit timer can be configured, after UE send request for data collection configuration, prohibit timer start, during prohibit timer window, if NW does not send data collection configuration, UE will not send request again until prohibit timer end.

Proposal 14:  If NW doesn’t respond UE request for data collection, prohibit timer can be used by UE to avoid excessive signaling overhead .

At RAN2#129 meeting, there was a FFS:

FFS whether a suggested data collection configuration/associated IDs (if specified)/parameters can be provided to the network.

In RAN1#120 meeting, some agreements have been made regarding data collection for training:

Agreement For UE-sided model, for configuring the resource for data collection purpose, support - CSI-ReportConfig can used for configuring the resources for data collection purpose without CSI report. - One CSI-ResourceConfigId is configured for Set A. - One CSI-ResourceConfigId is configured for Set B. - Note: UE performs measurement on all resources - One or two associated IDs can be configured in CSI-ReportConfig - When Set B is equal or a subset of set A (i.e., NZP-CSI-RS-ResourceId/SSB-Index in the resource set for Set B is within the NZP-CSI-RS-ResourceId/SSB-Index in the resource set for Set A), one associated ID is configured, - Otherwise, one associated ID is configured for Set A and another one associated ID is configured for Set B - FFS: whether/how to support 'aperiodic' CSI RS Note: This is not related to whether/how to support delivery/transmission of the collected data for training for UE-sided model.

RAN1 is also discussing UE feature list for R19 AI for PHY which will include UE capabilities on data collection. In this case, we believe UE capability reporting can provide detailed information on UE’s data collection capabilities.  Based on this detailed information the UE can just indicate a simple request to NW when needed. As a result, the NW can be able to make a decision based on the UE’s data collection capabilities and provides feedbacks to UE. If we allow the UE to indicate (suggested) data collection configuration(s) to NW, there may be some issues:

(1) signalling overhead in Uu interface

(2) perhaps lots of specification impacts, and cross-WGs efforts may be needed

(3) it may lead to ping-pong between UE and NW, e.g. the UE may send quite a lot of data collection configurations to NW, but NW may reject all or most of them, and later the UE may request again

Proposal 15:  RAN2 to agree that the UE just indicates a simple request to NW if it needs data collection configurations, and UE capability reporting is left to RAN1.

In the previous RAN2 meetings, RAN2 agreed that the network can configure whether UE is allowed to initiate requests for data collection. The gNB may select the use cases that the UE can request. For example, gNB can select the UE to request data collection configuration for BM-Case 1 but not BM-Case 2. We think that such selection is useful as the network can use it to control the UE request (FFS on whether such selection is sent in dedicated UE signaling or in SIB).

Observation 10: It is useful for the network to select certain use cases where the UE can request for data collection.

Proposal 16: Network can configure whether the UE is allowed to initiate requests for data collection for certain use cases (e.g. BM-Case 1 and/or BM-Case 2 etc.)

### Discussion on the scalability aspects of CP options for data transfer

In [5], we analyze Option 2 (via CN) and Option 3 (via OAM) for data transfer over CP and UP.

For CP options, there are Option 2-CP (via NAS signalling) and Option 3-CP (via RRC signalling).

Take Option 3-CP for example, firstly, we think all data contents should be standardized and RAN1 should decide on the contents. Secondly, the main concern is about how to transfer large training data from UE to NW via CP options. Our views are as below:

- As RAN1 mentioned, the transmission latency is relaxed

- Option 3-CP is similar to NW-sided data collection. For NW-sided data collection, RAN2 has some progress, and generally the UE can collect data based on network configuration, and then NW can use UEInformationRequest/Response to request the UE to transfer the data. In addition, the UE may send some assistance information to NW to help NW decision. In general, we think NW-sided data collection method is flexible enough, and it balances the impact on UE side and data transmission efficiency

For Option 2-CP, there is similar analysis as Option 3-CP, and thus Option 2-CP can balance the impact on UE side and data transmission efficiency.

Proposal 17: No scalability issues are observed for Option 2-CP and Option 3-CP.

### Discussion on the SA5 LS S5-250828 and draft reply LS

In [4], SA5 would like RAN2 to confirm if the following SA5 assumptions are correct and provide feedbacks if any on the following for data transfer.

SA5 has made the following analysis about the termination points of the UP tunnel for this solution: 1. If the UP tunnel is between UE and Server, SA5 does not see any difference between solution 3 and solution 1a); 2. If the UP tunnel is between UE and 5GC, SA5 does not see any difference between solution 3 and solution 2. SA5 assumes that the UP tunnel associated with data transfer is between UE and gNB for the solution 3. SA5 would like to get confirmation of this assumption from TSG RAN and RAN2, as well as RAN’s perspective on the UP tunnel termination points.

For the bullet 1 and 2, we see no problems. For the UP tunnel between UE and gNB, we observe that there is similar discussion for two-sided model in RAN2 [5], i.e. solution 1b (i.e. gNB -> UE via UP). For solution 1b for two-sided model, the summary is as below [5]:

It is observed that whether PDU session can be terminated at gNB needs to be further checked with SA2, and also this solution might require new protocol stack. Considering the significant specification impact to support establishing UP tunnel between gNB and UE, a huge effort is needed across different WGs to support solution 1b in 5GA, which is not practical.

And then the email rapporteur made the following proposal:

OTA solution

Proposal 4 [easy agreeable, phase 1 14/14]: From RAN2 point of view, when gNB is the NW dataset/model parameter collection entity, solution 1b (i.e. gNB -> UE via UP) is not considered as a candidate solution in 5GA.

In summary, we think the technical analysis for solution 1b can be applicable for "UP tunnel between UE and gNB" for Option 3-UP for UE-side data collection.

Proposal 18:  From RAN2 point of view, the UP tunnel associated with data transfer is between UE and gNB for the solution 3 is not considered.

For proposal 18, we also prepare a draft reply LS to SA5.

Proposal 19:  It is proposed RAN2 to discuss reply LS to SA5 (details can be found in section 5 Annex).

## Performance monitoring

In RAN1 #118bis meeting, RAN1 has agreed Type 1 Option 2 where UE-assisted performance metric is reported to NW for performance monitoring. In RAN1 #119, RAN1 further agreed that the monitoring Type 1 Option 2 support the reusing of CSI framework for the configuration for monitoring result report in L1 signaling:

Agreement At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), support to reuse CSI framework for the configuration for monitoring result report in L1 signaling: - Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring - The ID of an inference report configuration is configured in the configuration for monitoring to link the inference report configuration and monitoring report configuration - FFS how to identify the connection between RSs in the resource set(s) for monitoring and Set A beams - FFS on whether to support all the combination on time domain behavior of the reportConfigType for infernece report and the reportConfigType for monitoring report - FFS on the timing related issues UE measures the dedicated resource set(s) for monitoring.

Hence, for the time being, there seems to be no RAN2 impact other than ASN.1 impact.

Proposal 20: For monitoring Type 1 Option 2 of UE-side model monitoring, there is no RAN2 impact currently based on RAN1 progress so far other than possible ASN.1 impact.

# Conclusion

In this paper, our observations and proposals are summarized as follows:

Applicable functionality determination

Observations 1: The motivation of Direction B is as below:

- Avoid the NW having to configure full inference configuration without knowing the applicability of the configuration

- Reduce the signalling overhead (more efficient) in checking for the applicability due to using some parameter settings of Set A and B instead of the full inference CSI-RS configurations including the reporting type configurations (i.e. periodic, aperiodic etc.) that may not get activated as in Direction A.

Observation 2: For Direction B, only the parameter settings for Set A and B for a functionality are used only for applicability reporting and not for inference as indicated in the RAN1 LS and hence including applicability reporting configuration of Direction B in OtherConfig as in Option 1 makes more sense than in CSI-ReportConfig as in Option 2.

Observation 3: For Option 3, in the current definition of the IEs under CSI-MeasConfig, it is to configure UE to report the channel state information in the CSI report, and not to use these set(s) of inference related parameters to confirm and report whether they are applicable. Hence Option 3 is not suitable.

Proposal 1: For Direction B, one set or multiple sets of inference related parameters for applicability report is included in OtherConfig together with the configuration of UAI for applicability reporting.

Proposal 2: As per RAN1 LS, the following information should be included as part of Direction B configuration:

    - Optionally one or more associated ID;

    - Set A related information

    - Set B related information

    - Report content related information

    - For BM-Case 2,

      - Time instances related information for measurements

      - Time instances related information for prediction

Further details will be based on RAN1 L1 parameters list.

Applicable functionality reporting procedure

Observation 4: Regardless of whether Direction A) and/or B) is applied in Step 3. UAI and RRCReconfigurationComplete messages can be used to report applicable functionality and RAN2 aim to align the design on how the applicable functionality are signaled in both messages.

Proposal 3: Upon receiving a full inference configuration in CSI-ReportConfig or a set of inference parameters in OtherConfig, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability.

Proposal 4: To align the design on how the applicable functionality are signaled in both UAI and RRC Reconfiguration Complete message, the UAI message containing the applicable reporting can be carried as container in RRC Reconfiguration Complete.

Proposal 5: The applicability reporting in the UAI message may indicate either a certain CSI config (for Direction A) and/or some set of parameters (for Direction B)) as applicable (e.g. using a bitmap, or set of indexes etc. which can be further discussed in a stage-3 design).

Proposal 6: if Direction A (i.e., CSI-ReportConfig for inference configuration is provided) in Step 3, UE doesn’t autonomously release those CSI report configurations that are non-applicable after Step 4 but leave them to gNB to de-configure the non-applicable CSI-ReportConfig in subsequent.

Applicability reporting contents

Observation 5: Based on the RAN1’s agreement for Direction A and B, the NW-side additional condition can be indicated in Step 3 optionally. For Direction B, the associated ID can also be optionally indicated by the UE to the NW in Step 4.

Observation 6: Without such initial KPI information of an reported applicable functionality by UE, the network may activate an applicable functionality where it may not meet its expectation and requiring starting the performance monitoring immediately after activation.

Proposal 7: The NW-side additional condition is optional in Step 3. The NW can decide whether to provide the NW-side additional condition by implementation, and the applicable functionalities determination can be supported in the following ways:

- In case the network does not provide the UE with associated IDs, the UE performs applicable functionality filtering without considering NW-side additional conditions and reports the (UE-considered) applicable functionalities together with their associated IDs, if available.

- In case the network provides the UE with associated IDs, the UE performs applicable functionality filtering considering them and only reports the functionalities meeting the NW-side additional condition.

Proposal 8: For Direction B, the gNB needs to be able to indicate the serving cell(s) in which a set of inference parameters are related to.

Proposal 9: For both Direction A and B, the UE needs to indicate for the applicable reporting in the RRCReconfigurationComplete and UAI which serving cells a full inference configuration or a set of inference parameters are applicable for.

Proposal 10: Initial KPI of trained functionality can be as part of applicable functionality reporting.

Applicability status changes

Observation 7: Changes among different functionality statuses have an impact on functionality management. Frequent changes may result in frequent activation/deactivation or may require more resources for performance monitoring, which will introduce signaling/resource overhead.

Observation 8: Frequent functionality applicability changes/updates may have a detrimental impact on the signaling overhead, system performance and complicate AIML functionality management by the network.

Observation 9: The prohibit timer can be applied to avoid the frequent report for the applicability update of functionalities. However, if the UE cannot report the “applicable to non-applicable” update timely, the NW may configure a non-applicable functionality for inference.  If the UE cannot report the “non-applicable to applicable” update timely, the NW cannot configure the functionality for inference immediately when the NW is interested in this functionality.

Proposal 11: When the status of the functionality which has been previously requested by the network changes from applicable to non-applicable or vice-versa, UE shall report the status change information to network.

Proposal 12: RAN2 should investigate the means to avoid the need of frequent applicability status updates and ensure the stability of inference operation once it is activated (e.g., UE can provide some kind of “stability of applicability” indication for each applicable functionality etc.).

Proposal 13: The NW can configure different values of the prohibit timer for “applicable to non-applicable” update and “non-applicable to applicable” update to be applied for functionalities that are not currently configured for inference operation. If a functionality that is currently configured for inference operation becomes non-applicable, the UE should immediately report the status change.

UE-side data collection for training

Observation 10: It is useful for the network to select certain use cases where the UE can request for data collection.

Proposal 14:  If NW doesn’t respond UE request for data collection, prohibit timer can be used by UE to avoid excessive signaling overhead .

Proposal 15:  RAN2 to agree that the UE just indicates a simple request to NW if it needs data collection configurations, and UE capability reporting is left to RAN1.

Proposal 16: Network can configure whether the UE is allowed to initiate requests for data collection for certain use cases (e.g. BM-Case 1 and/or BM-Case 2 etc.)

Proposal 17: No scalability issues are observed for Option 2-CP and Option 3-CP.

Proposal 18:  From RAN2 point of view, the UP tunnel associated with data transfer is between UE and gNB for the solution 3 is not considered.

Proposal 19:  It is proposed RAN2 to discuss reply LS to SA5 (details can be found in section 5 Annex).

Performance monitoring

Proposal 20: For monitoring Type 1 Option 2 of UE-side model monitoring, there is no RAN2 impact currently based on RAN1 progress so far other than possible ASN.1 impact.

# References

- R2-2407848, LS on applicable functionality reporting for beam management UE-sided model

- R1-2410898, Reply LS on applicable functionality reporting for beam management UE-sided model

- R2-2501517, Summary of offline discussion [AT129][026][AI Phy] Signaling for BM

- S5-250828, Reply LS on AI/ML UE sided data collection, Source: SA5, To: RAN, RAN2, Cc: SA, SA2, SA3. Contact companies: Intel, NEC, Huawei

- R2-250xxxx_[POST129][029][AI Phy] Model transfer (XiaomiEricsson)_P2_v18_Rapp, Xiaomi, Ericsson

- R2-2501110, Discussion on UE-sided data collection for training, Huawei, HiSilicon

- Draft_Minutes_report_RAN1#120_v030

# Annex - draft reply LS to SA5 (S5-250828)

3GPP TSG-SA2 Meeting #129-bis	R2-25xxxxx

Wuhan, China, 7 - 11 April, 2025

Title:	Reply LS on AI/ML UE sided data collection

Response to:	S5-250828

Release:	Rel-19

Work Item:	AIML_MGT_Ph2

Source:	RAN2

To:	SA5

Cc:	RAN, SA, SA2, SA3

Contact person:

Jun Chen

jun.chen@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

RAN2 would like to thank SA5 for their LS in S5-250828 on AI/ML UE sided data collection. RAN2 confirm the feasibility of SA5's assumptions except for the following one:

SA5 assumes that the UP tunnel associated with data transfer is between UE and gNB for the solution 3.

From RAN2 point of view, for UP tunnel terminated at gNB, it needs SA2 involvement and it may lead to the significant specification impact across different WGs, so it is not considered by RAN2.

# 2	Actions

To: SA5

ACTION: 	RAN2 kindly asks SA5 to take the RAN2’s reply above into consideration.

# 3	Dates of next TSG RAN WG2 meetings

RAN2#130	2025-05-19 – 2025-05-23		Malta, Malta

RAN2#131	2025-08-25 – 2025-08-29		Bengaluru, India