3GPP TSG-RAN WG2 Meeting #129-bis	R2-2502609

Wuhan, China, 7th – 11th April, 2025

Agenda Item:	8.13.2

Source: 	Huawei, HiSilicon

Title: 	Relay discovery and (re)selection for multi-hop Relay

Document for:	Discussion and Decision

# Introduction

In RAN #105 meeting, the new work item on NR sidelink multi-hop relay [1] was approved.

The objective of this work item is to specify solutions that are needed to support multi-hop Layer-2 UE-to-Network relay for a single indirect path via SL relay UEs based on Rel-17/18 SL relay functionalities [RAN2, RAN3] 1. Specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay. The work starts with one additional hop relay (i.e., remote UE -> first relay UE -> last relay UE -> gNB) until RAN#107 and further check will be made in RAN#107 if it can be easily extended to two additional hops relays (i.e., remote UE -> first relay UE -> second relay UE -> last relay UE -> gNB). A necessary criterion for the specified mechanisms is easy extensibility to support two additional hop relays for the work done until RAN#107 and to be forward compatible for future extensions for additional relays. A. Relay discovery and (re)selection [RAN2] B. Signalling support for relay UEs and remote UE authorization if SA2 concludes it is needed [RAN3] C. Impact on SRAP and QoS handling for multi-hop [RAN2] D. Control plane procedures [RAN2, RAN3] 2. Specify the following intra-gNB service continuity scenarios for multi-hop U2N relay based on Rel-17/18 procedures (for remote UE): First Priority: A. Intra-gNB multi-hop indirect to direct path switching using existing framework B. Intra-gNB multi-hop indirect to single-hop indirect path switching using existing framework Second Priority in order of importance: C. Intra-gNB direct to multi-hop indirect path switching D. Intra-gNB single-hop indirect to multi-hop indirect path switching The scenarios C and D are limited to path switching to a target indirect path consisting of the last relay UE in “direct” RRC Connected mode and all the other intermediate relay(s) in “indirect” RRC Connected mode to the same cell. NOTE: The current existing measurement framework and existing data forwarding mechanism should be reused.

In RAN 2# 129 meeting, some agreements were made for relay discovery and (re)selection which are listed as below.

Agreements for relay discovery and (re)selection: - From RAN2 perspective, when an intermediate/last relay UE serves two different downstream UEs, there is no assumption on whether it presents the same or different L2IDs for connection establishment. - Reuse the same Uu RSRP thresholds for discovery transmission as in the legacy (i.e., single hop L2 U2N relay) for L2 Remote UE and L2 U2N last Relay UE in multi-hop U2N relay. - Reuse the same discovery resource pool(s) and configurations as in the legacy (i.e., single hop L2 U2N relay) in multi-hop U2N relay. - For model A discovery, there are no additional AS criteria on discovery message forwarding (the existence of the PC5 link is sufficient). - For model B discovery, if the PC5 link on which the discovery message is received has already been established, no additional AS criteria on discovery message forwarding are applied. FFS if the PC5 link is not already established. - For multi-hop U2N Relay reselection trigger at the remote UE, reuse the following R17 single-hop U2N Relay reselection trigger condition: - PC5 signal strength of current first relay UE is below a (pre)configured signal strength threshold. - When U2N Remote UE receives a PC5-S link release message from current first relay UE. - When U2N Remote UE detects PC5 RLF with the current first relay UE. - Indicated by upper layer. - L2 multi-hop U2N Remote UE in idle/inactive triggers relay reselection upon PC5-RRC signaling from the first relay UE indicating (FFS on the detailed PC5-RRC signaling design): - cell reselection, handover, Uu RLF, or Uu RRC connection establishment/resume failure between the last relay UE and network. - release of RRC connection between an intermediate relay UE and the network. - PC5 RLF or PC5-S connection release between the multi-hop U2N Relay UE(s). - FFS: link quality degradation of the upstream links resulting in a notification. - FFS if any of these conditions can occur without notifying the remote UE (e.g., allowing recovery by the intermediate relay UE). - FFS on the signalling contents of the PC5-RRC indication from the first relay UE. - FFS applicability to L3. - The intermediate relay UE can adopt the same AS triggering condition of relay (re)selection of the R17 remote UE. FFS upper layer conditions and whether the spec impact will be modelled as a separate relay UE behaviour or through functioning as a remote UE. - Upon intermediate relay UE performing relay (re)selection, it can notify its child node. If the cause of (re)selection already causes a notification, there is no double-trigger of the notification. - FFS if (re)selection as such is captured as a cause of notification (intention to maintain no double-trigger) - FFS if the notification is required in all cases - FFS the content in the notification - FFS the child node behaviour after receiving this notification - FFS other condition/timing that the intermediate relay UE notifies its child node

In this paper, some discussion on the above FFS for the relay discovery and relay (re)selection aspects are provided.

# Discussion

Rel-19 Multi-hop relay can further extend the coverage by supporting the sidelink based multiple hop relay on top of Rel-17 single hop U2N relay. Figure 1 shows the scenario where the remote UE connects with the network through two relay UEs, i.e., remote UE -> first relay UE -> last relay UE -> gNB.

Figure 1. Multi-hop relay with two relaying hops

In this paper, we use the terminology agreed in RAN2#127 meeting for the convenience of discussion.

- last relay UE: the U2N relay UE directly connected to the gNB

- intermediate relay UE: the relay UEs except for the last relay UE

- first relay UE: the intermediate relay UE directly connected to the remote UE

## 2.1	Conditions for performing Relay discovery

In RAN2#129 meeting, the following agreements were reached for discovery, and a remaining issue is whether the AS criteria on discovery message forwarding should be applied if the PC5 link is not already established for model B discovery.

For model A discovery, there are no additional AS criteria on discovery message forwarding (the existence of the PC5 link is sufficient). For model B discovery, if the PC5 link on which the discovery message is received has already been established, no additional AS criteria on discovery message forwarding are applied. FFS if the PC5 link is not already established.

Figure 2-1 shows the scenario of multi-hop U2N relay discovery for model A. And specifically, Figure 2-2 shows the signaling procedures of model A discovery and model B discovery.

Figure 2-1. Multi-hop U2N relay discovery(model A)

(a) model A                                             (b) model B

Figure 2-2. Model A discovery and model B discovery

For model B, whether the intermediate relay UE needs to establish a PC5 connection to the parent relay UE before executing the discovery procedure, there may be two possible cases.

Case 1: The intermediate relay UE has already established the PC5 connection with its parent relay UE, acting as a remote UE. (e.g., for its own data communication before serving as the relay UE), the same as in Figure 2-1.

Case 2: The intermediate relay UE has not yet established the PC5 connection with a multi-hop relay UE.

In case 1, the intermediate relay UE can perform the discovery if it is capable of providing relaying service for multiple remote UEs. No AS condition is required since there is an existing valid connection with the network. Furthermore, according to the legacy Rel-17 relay mechanism, if a UE connects through an indirect link, it will suspend the Uu RLM, meaning no quality measurements will be taken for the Uu link. Therefore, no Uu threshold is needed for the intermediate relay UE to determine whether it can perform as an intermediate relay UE.

Proposal 1. Model B, for an intermediate relay UE that has established the PC5 connection with the parent relay UE, no AS condition (e.g., the Uu threshold condition or others ) is required for performing the discovery.

Case 2 occurs when the UE is not connected to network through the multi-hop relay but is capable to acting as an intermediate relay UE. For model B, since the remote UE is selecting a candidate intermediate relay UE, we believe a UE which can function as the intermediate relay UE should respond to the solicitation message form the remote UE. The following scenario may occur:

Scenario 1: The UE is performing the multi-hop relay (re)selection, as shown in Figure 3-1. In this scenario, we believe the UE can first perform the discovery as a remote UE. checking the Uu RSRP (to determine whether it is below a threshold) to decide if it can function as a remote UE. Once the intermediate relay UE connects to a parent relay UE, it will then perform the discovery as a relay UE. In this case, the Uu RSRP is only used when the intermediate relay UE is functioning as a remote UE.

Figure 3-1. The UE is performing the multi-hop relay (re)selection.

Proposal 2: For model B, upon receiving the solicitation message form a multihop remote UE, the intermediate relay UE  should be allowed to forward the solicitation message to assist the remote UE in finding a candidate multi-hop path.

## 2.2	Relay selection and reselection

In RAN2#129 meeting, some agreements were reached for relay selection and reselection, and some remaining issues are left as below.

L2 multi-hop U2N Remote UE in idle/inactive triggers relay reselection upon PC5-RRC signaling from the first relay UE indicating (FFS on the detailed PC5-RRC signaling design): - cell reselection, handover, Uu RLF, or Uu RRC connection establishment/resume failure between the last relay UE and network. - release of RRC connection between an intermediate relay UE and the network. - PC5 RLF or PC5-S connection release between the multi-hop U2N Relay UE(s). - FFS: link quality degradation of the upstream links resulting in a notification. - FFS if any of these conditions can occur without notifying the remote UE (e.g., allowing recovery by the intermediate relay UE). - FFS on the signalling contents of the PC5-RRC indication from the first relay UE. - FFS applicability to L3. The intermediate relay UE can adopt the same AS triggering condition of relay (re)selection of the R17 remote UE. FFS upper layer conditions and whether the spec impact will be modelled as a separate relay UE behaviour or through functioning as a remote UE. • Upon intermediate relay UE performing relay (re)selection, it can notify its child node. If the cause of (re)selection already causes a notification, there is no double-trigger of the notification. - FFS if (re)selection as such is captured as a cause of notification (intention to maintain no double-trigger) - FFS if the notification is required in all cases - FFS the content in the notification - FFS the child node behaviour after receiving this notification - FFS other condition/timing that the intermediate relay UE notifies its child node

Discussions for FFSs.

-	FFS: link quality degradation of the upstream links resulting in a notification.

We believe that if the upstream PC5 links in the multi-hop chain degrade, either the intermediate or the first relay UE will trigger relay reselection or cell selection when they are in IDLE/INACTIVE state. Upon relay (re)selection or cell selection, the upstream relay UE in the multi-hop chain will send a notification message to the child relay UE, which will then forward it to the remote UE.

Similarly, if the upstream PC5 links degrade in the multi-hop chain, either the intermediate or the first relay UE will trigger a path switch when they are in RRC_CONNECTED state. Upon the path switch, the upstream relay UE in the multi-hop chain will send a notification message to the child relay UE, which will be forwarded to the remote UE

Thus, there is no need for prior notification of upstream PC5 link quality degradation to the remote UE. Therefore, we propose

Proposal 3: Link quality degradation of the upstream links will lead to path switching (for the CONNECTED state) or relay (re)selection or cell selection (for the IDLE/INACTIVE state), rather than triggering an immediate notification. The relay UE will send a notification once path switching or relay (re)selection or cell selection occurs.

-	FFS if any of these conditions can occur without notifying the remote UE (e.g., allowing recovery by the intermediate relay UE).

Considering that group handover/mobility is not supported, it is simpler to adhere to the legacy mechanism where the multi-hop U2N Remote UE triggers relay reselection when cell reselection, handover, Uu RLF, or Uu RRC connection establishment/resume failure is indicated by the multi-hop U2N Relay UE(s), as in the R17 single-hop case.

As indicated in the previous FFS for the multi-hop relay scenario, it is beneficial for the remote UE to be informed by the serving Intermediate Relay UE/First Relay UE when it receives a notification message indicating cell reselection, handover, Uu RLF, or Uu RRC connection establishment/resume failure of the U2N Last Relay UE. This allows the remote UE to take corrective action and perform reselection. Therefore, we propose that the notification message received from the U2N Last Relay UE should be forwarded downstream by the Intermediate/First Relay UE to the Remote UE. To achieve this, we can reuse the PC5 RRC Notification message.. FFS if any enhancement is needed.

Proposal 4: The notification message received from the U2N Last Relay UE indicating a failure on the Uu interface should be forwarded downstream by the Intermediate/First Relay UE to the Remote UE. To achieve this, we can reuse the PC5 RRC Notification message.

The above Proposal 4 would also imply that there is no need to support any recovery mechanism by the Intermediate Relay UE in R19. Therefore, we propose the following:.

Proposal 5: Failure Scenarios in which the remote UE is not notified and recovery is handled by the intermediate relay UE not considered to be supported in R19.

-	FFS on the signalling contents of the PC5-RRC indication from the first relay UE.

-	FFS the content in the notification

-	FFS the child node behaviour after receiving this notification

If the Multihop Remote UE is directly connected to the Last Relay UE and the Last Relay UE encounters a failure on the Uu link, it will reuse the existing notification message to inform the Multihop Remote UE and the content in the PC5-RRC indication can be the same as legacy.

However, if there are other relay UEs in the multi-hop chain, there is a possibility of encountering a multi-hop failure on the PC5 link. In this case, it would be more efficient for the child relay UE to forward the notification message received from the parent relay UE until the Notification Message is delivered to the Remote UE so that Remote UE can take appropriate action.

Proposal 6. if there are multiple relay UEs in the multi-hop chain, there is a possibility of encountering a multi-hop failure on the PC5 links. In this case, it would be more beneficial for the child relay UE to forward the notification message received from the parent relay UE until the Notification Message is delivered to the Remote UE.

For the content in the PC5-RRC indication when an intermediate relay UE forward the notification message received from the parent relay UE, there can be two candidate solutions.

1. the intermediate relay UE forward the PC5-RRC indication from the parent relay UE without the content update. For example, if the handover happens at the last relay UE, the last relay UE will indicate the intermediate relay UE with “relayUE-HO” type in the notification message. The intermediate relay UE can trigger the notification message to the child node, also with the “relayUE-HO” type.

2. the intermediate relay UE updates the content in the notification message based on the reaction of the intermediate relay UE when it receives the notification message. For example, upon receiving the notification message which indicate the handover of the last relay UE, the intermediate relay UE will trigger relay reselection and re-establishment, thus the intermediate relay UE can indicate the child node with the notification message indicating that the relay reselection happened.

From our perspective, the second solution is clearer. The first solution could introduce some ambiguity, as the intermediate relay UE can never perform handover, Uu RLF, or cell reselection.

Observation 1.  If the intermediate relay UE forward the notification message form the parent relay UE without the content update, there will be some ambiguity since some conditions of the last relay UE will not happen at the intermediate relay UE.

Proposal 7. Upon receiving the notification message form the parent relay UE, the intermediate relay UE/first relay UE would update the indication in the notification message to the remote UE to indicate the condition happened on itself.

-	FFS applicability to L3.

Since the RAN 2 part of the scope is limited to the L2 Mulit hop Relay design RAN 2 will not look into L3 aspects. Hence we can possibly close this FFS unless any issue is identified.

Proposal 8: RAN2 will focus solely on the L2 Multi-hop Relay design. Therefore, we do not need to discuss the applicability of L3 Multi-hop relays.

Detailed content of the discovery message for relay selection and reselection

There may be several candidate U2N Relay UE(s) or candidate Intermediate Relay UE(s) near the U2N Remote UE during selection and reselection. According to the current spec, the only information available to the Remote UE regarding  these candidate relay UE(s) is the PC5 link quality between the remote UE and candidate relay UE. For multi-hop relay, this information may not be sufficient to select a good relay UE.

Considering the case where  the Remote UE discovers the following candidate relay UE(s).

- Relay UE 1: The SD-RSRP is value 1, and the relay UE 1 is connected to the gNB through a U2N Relay UE. Value 1 satisfies the relay selection criterion, as it is above the configured threshold.

- Relay UE 2:  The SD-RSRP is value 2, and the relay UE 2 is connected to the gNB through a U2N Relay UE and an Intermediate Relay UE. Value 2 satisfies the relay selection criterion, and is larger than value 1.

Based on the PC5 link quality, the Remote UE is likely to select the relay UE 2, as the PC5 link quality between the Remote UE and the relay UE2 is better. However, from the QoS perspective, the Relay UE 1 may be better since the hop number between the relay UE 1 and the network is smaller. Therefore, we propose that the hop number of each candidate Intermediate Relay UE be indicated to the U2N Remote UE during the discovery procedure.  This additional information could help the Remote UE to select a better relay UE.

Proposal 9: RAN2 should discuss whether/how to support indicating the hop number of the candidate Intermediate Relay UE to the U2N Remote UE during the discovery procedure.

# Conclusion

In this paper, we discuss the relay discovery and relay (re)selection aspects and provide the following observations and proposals.

Conditions for performing Relay discovery

Proposal 1. Model B, for an intermediate relay UE that has established the PC5 connection with the parent relay UE, no AS condition (e.g., the Uu threshold condition or others ) is required for performing the discovery.

Proposal 2: For model B, upon receiving the solicitation message form a multihop remote UE, the intermediate relay UE  should be allowed to forward the solicitation message to assist the remote UE in finding a candidate multi-hop path.

Discussions for FFSs.

-	FFS: link quality degradation of the upstream links resulting in a notification.

Proposal 3: Link quality degradation of the upstream links will lead to path switching (for the CONNECTED state) or relay (re)selection or cell selection (for the IDLE/INACTIVE state), rather than triggering an immediate notification. The relay UE will send a notification once path switching or relay (re)selection or cell selection occurs.

-	FFS if any of these conditions can occur without notifying the remote UE (e.g., allowing recovery by the intermediate relay UE).

Proposal 4: The notification message received from the U2N Last Relay UE indicating a failure on the Uu interface should be forwarded downstream by the Intermediate/First Relay UE to the Remote UE. To achieve this, we can reuse the PC5 RRC Notification message.

Proposal 5: Failure Scenarios in which the remote UE is not notified and recovery is handled by the intermediate relay UE not considered to be supported in R19.

-	FFS on the signalling contents of the PC5-RRC indication from the first relay UE.

-	FFS the content in the notification

-	FFS the child node behaviour after receiving this notification

Proposal 6. if there are multiple relay UEs in the multi-hop chain, there is a possibility of encountering a multi-hop failure on the PC5 links. In this case, it would be more beneficial for the child relay UE to forward the notification message received from the parent relay UE until the Notification Message is delivered to the Remote UE.

Observation 1.  If the intermediate relay UE forward the notification message form the parent relay UE without the content update, there will be some ambiguity since some conditions of the last relay UE will not happen at the intermediate relay UE.

Proposal 7. Upon receiving the notification message form the parent relay UE, the intermediate relay UE/first relay UE would update the indication in the notification message to the remote UE to indicate the condition happened on itself.

-	FFS applicability to L3.

Proposal 8: RAN2 will focus solely on the L2 Multi-hop Relay design. Therefore, we do not need to discuss the applicability of L3 Multi-hop relays.

Detailed content of the discovery message for relay selection and reselection

Proposal 9: RAN2 should discuss whether/how to support indicating the hop number of the candidate Intermediate Relay UE to the U2N Remote UE during the discovery procedure.

# References

[1] RP-242349 Revised WID SL_relay_multihop

[2] 3GPP TS 38.300, V18.3.0, NR; NR; NR and NG-RAN Overall Description

[3] R2-2500700	Relay discovery and (re)selection for multi-hop Relay

[4] R2-2410006, Report of [POST127][402][Relay] Multi-hop relay control plane