3GPP TSG-RAN WG2 #129bis	R2-2502678

# Wuhan, China, 7th – 11th April 2025

Agenda Item:	8.8.2

Source:	Ericsson

Title:	DL coverage enhancements

Document for:	Discussion/Decision

# Introduction

During RAN2#129, the two scenarios depicted in Figure 1 [1] were agreed to be the guidelines for further discussion on the potential enhancements of SMTCs and measurement gaps.

- Scenario A. All neighbour cells (within a cluster) are active simultaneously. They share the same SMTC offset and SMTC periodicity.

- Scenario B. Cells sharing the same SMTC offset and periodicity are spread across the satellite’s footprint, where these cells typically are not directly adjacent neighbours with each other.

Figure 1. Beam-hopping scenarios under RAN2 consideration after RAN2#129.

Figure 2. Corresponding SSB transmission patterns for the scenarios under RAN2 consideration after RAN2#129.

This contribution provides an analysis of the proposed scenarios and suggests potential enhancements to address the needs introduced by DL CE Rel-19 NTN.

# 2	Discussion

## 2.1	Extension of SMTCs

Following RAN1's agreement to support a 160 ms additional default SSB periodicity, RAN2 is examining the implications for SMTC configurations. Figure 3 represents a potential deployment scenario where a satellite employs a power-sharing mechanism, with each beam corresponding to a cell, and not all neighbouring cells active simultaneously (scenario B in Figure 1). Considering the limited spectrum available for NTN, this example assumes a frequency reuse factor of 3, alongside a beam-hopping pattern designed to minimize interference within the same satellite.

Figure 3. Potential beam hopping pattern where each colour represents a frequency.

A comprehensive analysis of potential enhancements to SMTCs and measurement gaps must account for inter- and intra-frequency neighbours, RRC_CONNECTED and RRC_IDLE/INACTIVE states, and various combinations of SSB transmission offsets and periodicities. The scenarios agreed upon by RAN2 and depicted in Figure 1 and Figure 2 do not encompass all possible cases. Scenario A assumes SSBs are transmitted with the same periodicity and offset among cells in a cluster of neighbour cells, while Scenario B considers SSBs with the same periodicity but different offsets. Given that NTN beam-hopping aims to provide coverage and balance load efficiently, we propose discussing the scenario where a neighbouring cell has both different SSB periodicity and offset. It is crucial to clarify whether RAN2's scope includes configuring different SSB periodicities for cells operating on the same frequency layer and served by the same satellite. Specifically, we must determine if the network can configure two neighbouring cells on the same frequency, e.g., one with an SSB periodicity of 20 ms and another of 160 ms, utilizing the beam-hopping feature.

RAN2 to discuss and clarify whether different SSB periodicities and SSB offsets are expected in the same frequency layer.

In Scenario A, illustrated in Figure 1, it is evident that when beam-hopping patterns are sensibly combined, and all beams share the same periodicity and offset, the current provision of 4 parallel SMTCs suffices for a UE to ensure efficient operation.

- When the SSB transmission of neighbor cells has the same periodicity and offset, the current number of 4 parallel SMTCs is sufficient.

Figure 4. Combination of different SSB periodicity but same SSB offset.

Similarly, in the scenario depicted in Figure 4, SSB transmissions share the same offset but have different periodicities. For simplicity, only 2 cells are shown. In this case, the current provision of 4 parallel SMTCs may suffice for efficient operation. If a single SMTC can cover the SSB transmissions for both cells, there may be instances where the UE can measure both cells 1 and 2 within the SMTC, while in other cases, only one cell can be measured due to the extended periodicity of the other. We consider that a sensible UE implementation should be able to handle the difference in periodicity.

- When the SSB transmission of neighbor cells has different periodicity but the same offset, the current number of 4 parallel SMTCs is sufficient.

In scenario B illustrated in Figure 1, SSB transmissions share the same periodicity but have different offsets. This presents a challenge due to the limited number of SMTCs and measurement gaps compared to number of potential neighbours, a situation that may also occur in terrestrial networks. If all neighbour cells have different offsets, with ideal hexagon cells, the UE would need 6 SMTCs. However, NTN beam-hopping may enable a frequency reuse factor of 1 without significant crossbeam interference. Therefore, the current provision of 4 parallel SMTCs may not suffice for efficient operation, and enhancements could be considered for RRC_IDLE/INACTIVE and RRC_CONNECTED states.

- When the SSB transmission of neighbor cells has the same periodicity and different offset, the current number of 4 parallel SMTCs is insufficient.

Figure 5. Combination of different SSB periodicity and different SSB offset.

In the scenario illustrated in Figure 5, SSB transmissions have different periodicities and offsets. For simplicity, only 3 cells are depicted. Depending on the deployment specifics, the current provision of 4 parallel SMTCs may suffice for efficient operation. However, the UE must be aware of the different periodicities and offsets, which is currently not feasible with the present ASN.1 structure in system information and measurement objects. A straightforward solution would be creating a new RRC parameter, e.g., SMTC5, specifying SSB transmission periodicity and offset for a specific list of PCIs when they differ from the main SMTC. However, a new parameter would not be backwards compatible. Therefore, RAN2 should decide whether to introduce this change in a backward-compatible manner as an extension of SMTC4.

- When the SSB transmission of neighbor cells has the different periodicity and offset, the current number of 4 parallel SMTCs is sufficient, but needs to account for the changed periodicities.

Introduce a way to define 4 SMTCs with different periodicity and offset for a frequency layer.

Discuss whether the new definition should be backwards compatible with pre-Rel-19 NTN UEs.

Finally, for inter-frequency neighbours, the specification allows configuring a maximum of 8 frequency layers with relevant cell reselection information in SIB4. From Release 17, each frequency layer can accommodate 4 SMTCs to account for different offsets caused by satellite movement and varying propagation delays. Given the number of potentially relevant neighbour cells in typical scenarios, these provisions are sufficient, and no further enhancements are required for the inter-frequency case.

Inter-frequency SMTC enhancements are not needed to support 160 ms default SSB periodicity.

## 2.2	Location-based STMCs and measurement gaps adjustments

In a typical NTN deployment, the neighbour cells that a UE can physically measure depend upon the position the UE occupies within the serving cell. The relatively large cell size makes it unlikely that a UE needs to measure all possible NTN neighbour cells, let alone its potential TN neighbour cells.

Figure 6. Relative position of 2 UEs w.r.t. to NTN neighbour cells.

Figure 6 presents two UEs in opposite areas of a serving cell. While one UE can potentially only measure cells 1, 2, and 3, the second UE can potentially only measure cells 4, 5, and 6. The introduction of beam hopping, and the extended SSB default periodicity makes evident an issue which is well-known since Release 17: neighbour cell measurement configurations do not take UE location into consideration.

- In a typical NTN deployment, a UE does not need to be configured to measure all potential surrounding neighbour cells.

As a result, the network could potentially configure the UE  to monitor and try to measure on time-frequency resources where it is not possible for that specific UE to receive anything. This creates unnecessary signalling overhead and energy consumption and reduced capacity (due to unnecessary measurement gaps). To facilitate network operations and optimize UE measurements, we suggest that RAN2 studies how the network could configure each UE with the most appropriate measurement gaps and SMTCs for its surrounding conditions with a common solution for all RRC states (RRC_IDLE/INACTIVE and RRC_CONNECTED).

RAN2 to explore common solutions for RRC_IDLE/INACTIVE and RRC_CONNECTED so that the UE can be configured with the most appropriate measurement gaps and SMTCs for its current location.

## 2.3	Cell level DTX

- Notes for this objective:

  - SSB channel enhancement other than SSB periodicity extension is not considered

    - RAN1 should consider issues such as UE’s cell search complexity and impact to initial cell selection, latency and success rate, for the above extension

  - The SSB periodicity enhancements potentially defined in this WID only apply to NTN operation

  - Antenna gain of UE shall be assumed to be -5.5dBi in case of smartphone in FR1-NTN, the UE is assumed to be a full duplex UE, and at least 2Rx are considered at the UE

  - NGSO to be considered in priority: LEO Set-1 @ 600 km

In RAN2#128, potential enhancements to cell level DTX/DRX based on the Release 18 NES solutions were discussed to support power sharing and beam hopping. As indicated in the excerpt from [2], the downlink coverage enhancements note related to Release 18 NES was removed after RAN#106. Therefore, we consider that cell DTX enhancements based on Release 18 NES are beyond the scope of Release 19, and RAN2 should not continue discussions on this feature.

Cell DTX enhancements based on Rel-18 NES are not within the scope of Rel-19 downlink coverage enhancements.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	When the SSB transmission of neighbor cells has the same periodicity and offset, the current number of 4 parallel SMTCs is sufficient.

Observation 2	When the SSB transmission of neighbor cells has different periodicity but the same offset, the current number of 4 parallel SMTCs is sufficient.

Observation 3	When the SSB transmission of neighbor cells has the same periodicity and different offset, the current number of 4 parallel SMTCs is insufficient.

Observation 4	When the SSB transmission of neighbor cells has the different periodicity and offset, the current number of 4 parallel SMTCs is sufficient, but needs to account for the changed periodicities.

Observation 5	In a typical NTN deployment, a UE does not need to be configured to measure all potential surrounding neighbour cells.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 to discuss and clarify whether different SSB periodicities and SSB offsets are expected in the same frequency layer.

Proposal 2	Introduce a way to define 4 SMTCs with different periodicity and offset for a frequency layer.

Proposal 3	Discuss whether the new definition should be backwards compatible with pre-Rel-19 NTN UEs.

Proposal 4	Inter-frequency SMTC enhancements are not needed to support 160 ms default SSB periodicity.

Proposal 5	RAN2 to explore common solutions for RRC_IDLE/INACTIVE and RRC_CONNECTED so that the UE can be configured with the most appropriate measurement gaps and SMTCs for its current location.

Proposal 6	Cell DTX enhancements based on Rel-18 NES are not within the scope of Rel-19 downlink coverage enhancements.

# References

R2-2501415, Report of [AT129][304][R19 NR NTN] LS to RAN1 on DL coverage enhancements, Huawei, RAN2#129, February 2025.

RP-243300, “Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3”, 3GPP TSG RAN#106, Dec. 2024.