3GPP TSG-RAN WG2 Meeting #129bis	R2-2502689

Wuhan, China, 7th – 11th April, 2025

Agenda item:	8.18

Source:	Samsung

Title:	Redirection to IoT NTN and NR NTN

WID/SID:	TEI19

Document for:	Discussion and Decision

# Introduction

In this TEI19 contribution, we provide some discussions redirecting UEs to NTN frequencies.

# Discussion

## Background on mobility in NTN

- NR NTN and IoT NTN were introduced in Release 17. As part of both Release 17 and Release 18 work on NR NTN and IoT NTN, mobility has been one of the key topics. The following list highlights all of the mobility enhancements introduced:

- Idle Mode

  - T-Service

  - Distance-based measurement initiation

  - NTN to TN measurement relaxation

  - NTN assistance information in TN

- Connected mode

  - Time based measurement event (conditional handover)

  - Distance based measurement event (for normal RRM and conditional handover)

  - Satellite switch with resync

- As seen above, a lot of effort has been put in to mobility enhancements for NTN over multiple releases. However, TN to NTN mobility is still not fully supported. For both NR NTN and IoT NTN, mobility from a terrestrial network to a non-terrestrial network has some support. In release 18 it is supported to broadcast neighbour NTN cell ephemeris for idle mode mobility purposes. Since Release 17, handovers are supported from TN to NTN, but connected mode measurements are not considered supported as there are no RAN4 requirements. This leaves the network and an operator in an awkward position where mobility from TN to NTN is somewhat controlled by the UE, as the RAN4 requirements for RRC idle and RRC inactive only apply if the GNSS is activated. This makes it possible for a UE to never measure NTN cells if the UE implementation does not want to turn GNSS.

Observation 1: A lot of effort have been put in mobility enhancements for NTN, but TN to NTN mobility is still not fully supported.

- In RAN2#129, the following was agreed in the LTE TN to NR NTN idle mode work item [1]:

Agreements:

- RAN2 confirms that redirection from LTE TN to NR NTN is not supported before Rel-19.

- Redirection from LTE TN to NR NTN is supported in Rel-19 introducing a new capability signaling for LTE TN to NR NTN redirection purpose and also a satellite ID(s) in RedirectedCarrierInfo.

- This means that for LTE TN to NR NTN, redirection is supported by sending an indication with a satellite ID.

Observation 2: Redirection from LTE TN to NR NTN is supported in Release 19.

## Release with re-direction to an NTN cell

- There is however one type of mobility that was not discussed in RAN2 in Release 17 or Release 18: Connection release with re-direction. Release with re-direction is an important feature for network controlled mobility for inter-RAT and inter-frequency mobility. Release with re-direction to an NTN cell may have the benefit over connected mode measurements in that GNSS does not need to be turned on until the re-direction command is received, thereby not impacting UE power consumption more than needed in a TN-NTN network. It is also a more simple method of performing network.

- Owing to the two different issues in redirecting to an NTN cell for NR NTN and IoT NTN, in this paper we treat them separately.

### Redirection to NR NTN

- For redirection to NR NTN, the following was agreed in RAN2#129 TEI19 [1]:

Agreements:

- RAN2 intends to support re-direction from NR TN to NR NTN in Rel-19. Come back in the next meeting to discuss what needs to be added in Rel-19 to existing specs to ensure this is fully supported. In any case we will not introduce any changes to pre-rel-19 specs for this.

- It was mentioned in the last meeting that NR TN to NR NTN redirection may already be considered to be supported or that the pieces to support it are already in place. The pieces that were argued to allow for redirection is that 1) SIB19 may be broadcasted in a TN cell containing neighbouring NTN cell information, including carrier frequency and NTN-Config, and 2) UE indicates the capability of acquiring SIB19 in connected mode. This means that when a UE is redirected, the UE may use the frequency information broadcasted in SIB19, which can then be tied with an NTN-Config element.

- However, there are some issues with this procedure. For instance, it is not clear whether a UE supports such a procedure or not, so there is at least a need for a capability. Secondly, that the UE shall infer things in this matter is not in the specifications, and since it is not really that straightforward – it needs to be specified in some manner. Therefore, we do not view redirection to NR NTN to be supported.

Observation 3: Redirection from NR TN to NR NTN cannot really be considered supported without procedural clarification and a capability in Release 19.

- One option could thus be to introduce a new capability and clarify how the UE derives whether the target cell is an NTN cell and how the UE may determine which ephemeris to use during the cell selection procedures.

- Option A would be an implicit solution where when the UE receives and redirection message, the UE checks if the frequency corresponds to any frequency in SIB19, and if there is a match then the UE assumes that the band is an NR NTN band. The UE then uses the NTN assistance information associated with the frequency in order to search and detect cells in the cell selection procedure during redirection. A capability is introduced to indicate whether the UE supports this type of redirection to the NTN bands.

- Option B would be a more clean-sleet solution where redirection includes an indication whether the band is NTN and optionally an NTN assistance information element. A capability is introduced to indicate whether the UE supports this type of redirection to the NTN bands.

- In our view, both Option A and Option B can potentially work. However, as we believe that NR TN and NR NTN cannot be considered supported without capabilities before Release 19, we think it would be better to go for a cleaner solution that does not rely on the network using an implicit solution.

Proposal 1: RAN2 to discuss options to support redirection to NR NTN:

- Option A: UE uses the frequency received in redirectedCarrierInfo and matches it to frequencies in SIB19 to determine whether the frequency is an NTN frequency and its associated NTN assistance information. A new capability is introduced to support this for the NTN bands UE indicates support of.

- Option B: The UE is configured with an indicator whether the band is NTN and/or with NTN assistance information. A new capability is introduced to support this for the NTN bands the UE indicates support of.

- For this purpose we have outlined the two different options in Appendix A1 and B1 as a way to explain how they can be implemented. We believe that they can be taken as baselines.

Proposal 2: RAN2 to consider the text proposal in Appendix A1 and B1 as baseline, depending on Option A or B in Proposal 1.

### Redirection to IoT NTN

- As stated above, redirection from LTE TN to NR NTN has already been agreed to be supported and there are already stable drafts of the procedures available [2]. As discussed during the last meeting, the main issue with any type of network-initiated mobility from TN to IoT NTN has to do with capabilities. In Release 17, it was decided that the capabilities of a UE is reported according to which network type that the UE is currently connected to:

- ---------------------------------- 36.331 V18.5.0 ----------------------------------

5.6.3	UE capability transfer

5.6.3.1		General

Figure 5.6.3.1-1: UE capability transfer

The purpose of this procedure is to transfer UE radio access capability information from the UE to E-UTRAN.

If the UE is NTN capable, the UE reports its E-UTRAN radio access capabilities for the network type (TN or NTN) to which it is connected.

If the UE has changed its E-UTRAN radio access capabilities, the UE shall request higher layers to initiate the necessary NAS procedures (see TS 23.401 [41]) that would result in the update of UE radio access capabilities using a new RRC connection.

NOTE:	Change of the UE's GERAN UE radio capabilities in RRC_IDLE is supported by use of Tracking Area Update.

- ---------------------------------- 36.331 V18.5.0 ----------------------------------

- This obviously works well when a UE is only connected to a terrestrial network or a non-terrestrial network. However for mobility between the two networks it is more challenging. For handovers as an example, the UE will not indicate its NTN-capabilities to the terrestrial network, making handovers using standardized procedures difficult. Similarly, for redirection, a terrestrial network will not know whether the UE is capable of NTN or the bands that the UE support. Furthermore, according to current discussions on redirection from LTE TN to NR NTN and NR TN to NR NTN, regardless of how capabilities would be handled, it is unlikely the redirection to IoT NTN is considered supported. It is also important to note that this is not an issue for LTE TN to NR NTN, as the UE in a E-UTRAN reports its NR capabilities to the network, which include the NR NTN capabilities.

Observation 4: In the current IoT NTN capability framework, the UE reports capabilities according to network type, making redirection to NTN difficult.

Observation 5: Even without the issue of reporting NTN capabilities to a TN, redirection from TN to IoT NTN would not be considered supported.

- However, for the issue of reporting NTN capabilities to a terrestrial network, we believe that there is a solution to this issue. One solution is that any capabilities of supporting TN to NTN redirection is considered to be TN-capabilities. In other words, the UE reports its support of redirection to NTN to the terrestrial network. In addition to this, the UE may report the supported NTN bands. We believe that unlike a handover, which requires virtually all of the capabilities to be indicated, the support of redirection to NTN and which bands that the UE support would be sufficient for a redirection. Therefore, we propose:

Proposal 3: To support TN to IoT NTN redirection, the capabilities related to redirection from TN to IoT NTN are considered TN-capabilities, and are thus signalled to the terrestrial network.

- As for the redirection procedure and the needed enhancements, we believe that the solution introduced for LTE TN to NR NTN can be used for TN to IoT NTN redirection.

Proposal 4: To support TN to IoT NTN redirection, the LTE TN to NR NTN solution may be reused. Release message contains satellite ID, which refers to satellite elements in SIB33.

# Conclusion

In this contribution, we discuss proposals related to Release with re-direction to an NTN frequency:

Observation 1: A lot of effort have been put in mobility enhancements for NTN, but TN to NTN mobility is still not fully supported.

Observation 2: Redirection from LTE TN to NR NTN is supported in Release 19.

Observation 3: Redirection from NR TN to NR NTN cannot really be considered supported without procedural clarification and a capability in Release 19.

Observation 4: In the current IoT NTN capability framework, the UE reports capabilities according to network type, making redirection to NTN difficult.

Observation 5: Even without the issue of reporting NTN capabilities to a TN, redirection from TN to IoT NTN would not be considered supported.

Proposal 1: RAN2 to discuss options to support redirection to NR NTN:

- Option A: UE uses the frequency received in redirectedCarrierInfo and matches it to frequencies in SIB19 to determine whether the frequency is an NTN frequency and its associated NTN assistance information. A new capability is introduced to support this for the NTN bands UE indicates support of.

- Option B: The UE is configured with an indicator whether the band is NTN and/or with NTN assistance information. A new capability is introduced to support this for the NTN bands the UE indicates support of.

Proposal 2: RAN2 to consider the text proposal in Appendix A1 and B1 as baseline, depending on Option A or B in Proposal 1.

Proposal 3: To support TN to IoT NTN redirection, the capabilities related to redirection from TN to IoT NTN are considered TN-capabilities, and are thus signalled to the terrestrial network.

Proposal 4: To support TN to IoT NTN redirection, the LTE TN to NR NTN solution may be reused. Release message contains satellite ID, which refers to satellite elements in SIB33.

# Reference

R2-2501333, Report from session on NR NTN and IoT NTN, Session chair (ZTE), RAN2#129, Athens, Greece, February 2025.

R2-2501418, Introduction of LTE TN to NR NTN idle mode mobility, CATT, RAN2#129, Athens, Greece, February 2025.

# Appendix

## RRC TP A1

------------------------------- Example based on 38.331 V18.5.1 -------------------------------

RRCRelease message

-- ASN1START

-- TAG-RRCRELEASE-START

RRCRelease ::=                      SEQUENCE {

rrc-TransactionIdentifier           RRC-TransactionIdentifier,

criticalExtensions                  CHOICE {

rrcRelease                          RRCRelease-IEs,

criticalExtensionsFuture            SEQUENCE {}

}

}

RRCRelease-IEs ::=                  SEQUENCE {

redirectedCarrierInfo               RedirectedCarrierInfo           OPTIONAL,   -- Need N

cellReselectionPriorities           CellReselectionPriorities       OPTIONAL,   -- Need R

suspendConfig                       SuspendConfig                   OPTIONAL,   -- Need R

deprioritisationReq                 SEQUENCE {

deprioritisationType                ENUMERATED {frequency, nr},

deprioritisationTimer               ENUMERATED {min5, min10, min15, min30}

}                                                                   OPTIONAL,   -- Need N

lateNonCriticalExtension                OCTET STRING                OPTIONAL,

nonCriticalExtension                    RRCRelease-v1540-IEs        OPTIONAL

}

RRCRelease-v1540-IEs ::=            SEQUENCE {

waitTime                           RejectWaitTime                OPTIONAL, -- Need N

nonCriticalExtension               RRCRelease-v1610-IEs          OPTIONAL

}

RRCRelease-v1610-IEs ::=            SEQUENCE {

voiceFallbackIndication-r16        ENUMERATED {true}              OPTIONAL, -- Need N

measIdleConfig-r16                 SetupRelease {MeasIdleConfigDedicated-r16}    OPTIONAL, -- Need M

nonCriticalExtension               RRCRelease-v1650-IEs           OPTIONAL

}

RRCRelease-v1650-IEs ::=            SEQUENCE {

mpsPriorityIndication-r16          ENUMERATED {true}              OPTIONAL, -- Cond Redirection2

nonCriticalExtension               RRCRelease-v1710-IEs           OPTIONAL

}

RRCRelease-v1710-IEs ::=            SEQUENCE {

noLastCellUpdate-r17                ENUMERATED {true}             OPTIONAL,   -- Need S

nonCriticalExtension                SEQUENCE {}                   OPTIONAL

}

RedirectedCarrierInfo ::=           CHOICE {

nr                                  CarrierInfoNR,

eutra                               RedirectedCarrierInfo-EUTRA,

...

}

RedirectedCarrierInfo-EUTRA ::=     SEQUENCE {

eutraFrequency                      ARFCN-ValueEUTRA,

cnType                              ENUMERATED {epc,fiveGC}        OPTIONAL    -- Need N

}

CarrierInfoNR ::=                   SEQUENCE {

carrierFreq                         ARFCN-ValueNR,

ssbSubcarrierSpacing                SubcarrierSpacing,

smtc                                SSB-MTC                        OPTIONAL,      -- Need S

...

}

. . .

-- TAG-RRCRELEASE-STOP

-- ASN1STOP

. . .

CarrierInfoNR field descriptions

carrierFreq Indicates the redirected NR frequency. If the indicated carrierFreq has the same value as carrierFreq of an element in ntn-NeighCellConfigList, the UE considers the redirection to be to an NTN frequency and applies the associated ntn-Config.

ssbSubcarrierSpacing Subcarrier spacing of SSB in the redirected SSB frequency. Only the following values are applicable depending on the used frequency: FR1: 15 or 30 kHz FR2-1/FR2-NTN: 120 or 240 kHz FR2-2: 120, 480, or 960 kHz

smtc The SSB periodicity/offset/duration configuration for the redirected SSB frequency. It is based on timing reference of PCell. If the field is absent, the UE uses the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing.

------------------------------- Example based on 38.331 V18.5.1 -------------------------------

## RRC TP B1

------------------------------- Example based on 38.331 V18.5.1 -------------------------------

RRCRelease message

-- ASN1START

-- TAG-RRCRELEASE-START

RRCRelease ::=                      SEQUENCE {

rrc-TransactionIdentifier           RRC-TransactionIdentifier,

criticalExtensions                  CHOICE {

rrcRelease                          RRCRelease-IEs,

criticalExtensionsFuture            SEQUENCE {}

}

}

RRCRelease-IEs ::=                  SEQUENCE {

redirectedCarrierInfo               RedirectedCarrierInfo           OPTIONAL,   -- Need N

cellReselectionPriorities           CellReselectionPriorities       OPTIONAL,   -- Need R

suspendConfig                       SuspendConfig                   OPTIONAL,   -- Need R

deprioritisationReq                 SEQUENCE {

deprioritisationType                ENUMERATED {frequency, nr},

deprioritisationTimer               ENUMERATED {min5, min10, min15, min30}

}                                                                   OPTIONAL,   -- Need N

lateNonCriticalExtension                OCTET STRING                OPTIONAL,

nonCriticalExtension                    RRCRelease-v1540-IEs        OPTIONAL

}

RRCRelease-v1540-IEs ::=            SEQUENCE {

waitTime                           RejectWaitTime                OPTIONAL, -- Need N

nonCriticalExtension               RRCRelease-v1610-IEs          OPTIONAL

}

RRCRelease-v1610-IEs ::=            SEQUENCE {

voiceFallbackIndication-r16        ENUMERATED {true}              OPTIONAL, -- Need N

measIdleConfig-r16                 SetupRelease {MeasIdleConfigDedicated-r16}    OPTIONAL, -- Need M

nonCriticalExtension               RRCRelease-v1650-IEs           OPTIONAL

}

RRCRelease-v1650-IEs ::=            SEQUENCE {

mpsPriorityIndication-r16          ENUMERATED {true}              OPTIONAL, -- Cond Redirection2

nonCriticalExtension               RRCRelease-v1710-IEs           OPTIONAL

}

RRCRelease-v1710-IEs ::=            SEQUENCE {

noLastCellUpdate-r17                ENUMERATED {true}             OPTIONAL,   -- Need S

nonCriticalExtension                SEQUENCE {}                   OPTIONAL

}

RedirectedCarrierInfo ::=           CHOICE {

nr                                  CarrierInfoNR,

eutra                               RedirectedCarrierInfo-EUTRA,

...

}

RedirectedCarrierInfo-EUTRA ::=     SEQUENCE {

eutraFrequency                      ARFCN-ValueEUTRA,

cnType                              ENUMERATED {epc,fiveGC}        OPTIONAL    -- Need N

}

CarrierInfoNR ::=                   SEQUENCE {

carrierFreq                         ARFCN-ValueNR,

ssbSubcarrierSpacing                SubcarrierSpacing,

smtc                                SSB-MTC                        OPTIONAL,      -- Need S

...,

[[

ntn-AssistanceInformation-r19	CHOICE {

ntn-BandIndication-r19			NULL,

ntn-ConfigList-r19				SEQUENCE (SIZE(1..FFS))  OF NTN-Config-r17

}

]]

}

. . .

-- TAG-RRCRELEASE-STOP

-- ASN1STOP

. . .

CarrierInfoNR field descriptions

carrierFreq Indicates the redirected NR frequency.

ntn-AssistanceInformation This indicates whether a UE is being redirected to an NTN frequency. The network may either only indicate that the band is an NTN band, or the network may configure the UE the with NTN assistance information.

ssbSubcarrierSpacing Subcarrier spacing of SSB in the redirected SSB frequency. Only the following values are applicable depending on the used frequency: FR1: 15 or 30 kHz FR2-1/FR2-NTN: 120 or 240 kHz FR2-2: 120, 480, or 960 kHz

smtc The SSB periodicity/offset/duration configuration for the redirected SSB frequency. It is based on timing reference of PCell. If the field is absent, the UE uses the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing.

------------------------------- Example based on 38.331 V18.5.1 -------------------------------