# 3GPP TSG-WG2 Meeting #129bis                                            R2-2502696

Wuhan, China, Apr. 7th – 11th, 2025

Source: 			CMCC

Title:  		    Discussion on data transmission for A-IoT

Agenda item:	8.2.4

Document for: 	Discussion

Introduction

In RAN2#129 meeting, we discussed the data transmission for A-IoT and achieved the following agreements.

- For CBRA, it is up to Reader to decide whether to reuse the random ID as the AS ID or to assign a new AS ID. FFS how this is signalled, which message is used and size of AS ID. - From device perspective, it is only required to use one AS ID. - CFRA is not supported for group ID - RAN2 assumes, AS ID is needed for CFRA at least for inventory + command procedure - For CFRA, if a valid AS ID is not already assigned, continue the discussion on AS-ID assignment based on the following options: - Option 2: the device includes a random ID in “Msg 1”. And same as CBRA, it is up to Reader to decide whether to reuse the random ID as the AS ID or to assign a new AS ID. - Option 3: New “Msg 2” for AS ID assignment, complementary option or independent from option 2 - Option 4: “Msg 2” (including the “Command”) for AS ID assignment, complementary option or independent from option 2 - To support segmentation, a 1 bit indication is introduced to indicate whether there is more data or not, if SA2 indicates that CN can provide an estimated expected D2R message size. If not possible, FFS if the 1 bit is sufficient. - Segment retransmission is supported. - For segment retransmission, reader explicitly indicates an offset in the MAC layer– e.g. number of bits successfully received so far (from the start). FFS This implies that unsegmented packet can also be retransmitted. FFS if this applies to msg3 - R2D segmentation is not supported for R19 A-IoT. - From RAN2 perspective only the following types of procedures will be considered in the normative phase: “Inventory only” and “Inventory and command”. - Aim to design simple MAC PDU format design - Support multiplexing of information for multiple devices in R2D message for msg2. FFS others for multicast messages - At least the following field are required for at least for R2D in the MAC header– message type, length for SDU and variable part(s). - FFS whether for D2R we need message type field, any length and need for padding - Specify message types and contents. As starting point consider the following MAC message types. - R2D MAC PDU (Paging/R2D trigger (depending on agreement on WF)) - D2R MAC PDU (MSG1) (FFS if this requires a MAC header or not) - R2D MAC PDU (MSG2) - D2R MAC PDU (MSG3 and data) - R2D MAC PDU (R2D data) - Other message types are FFS. The message types may evolve based on functionality agreements.

In this contribution, we will discuss the data transmission functionalities and MAC PDU format of A-IoT.

Discussion

Retransmission of Segment/ unsegmented packet

In the last meeting, some basic guidelines for segmentation have been agreed. However, some detail issues are still open.

For segment retransmission, reader explicitly indicates an offset in the MAC layer. An example of offset from chairman notes is that the number of bits successfully received currently. From our perspective, it is simple solution for device implementation, which may avoid store the last successfully transmit point at the device side. However, considering the maximum data size of D2R, the biggest offset indicator can be 10 bits. The overhead of R2D message is a little bigger. Another possible solution is where the offset is used to indicate the difference compared to the last successful reception. If the size of the middle segment is large, such offset indictor can also be about 10 bits. So, we propose that for segmentation retransmission, the reader explicitly indicates that number of bits successfully received currently in R2D message.

Observation 1：The Maximum size of D2R data is about 1000bit, 10-bits offset indication is enough for R19.

Proposal 1: For segmentation retransmission, the reader explicitly indicates that number of bits successfully received currently with a 10-bits offset in R2D message.

The device acquires the position of pending data with the memory address information provided in the A-IoT command from the CN. After receiving the D2R data size indicated in the R2D scheduling message, the devices decide to segment the pending data if the data size is smaller than the pending data size. For subsequent segment transmission, the devices should also follow the reader control for the segment size. Of course, if the pending data size is smaller than the indicated data size in R2D message, the device will transmit the data without segmentation. However, something needs to be clarified is that the data size provided in R2D message can be changed by reader implementation. That to say, the segmentation may be needed for retransmission even if perform unsegment for initial transmission.

Proposal 2: The size of D2R segment is indicated by the reader with R2D message.

For unsegmented packets, the device also follows R2D scheduling for retransmission. The reader indicates that new data size with considering transmission reliability, by implementation. Different from the segmentation case, the retransmission of unsegmented packet is always start from the beginning of data. If we reuse the same offset based solution, the offset should be set to “0”. From device’ perspective, the offset indicator is not necessary. If the reader re-assign data size and D2R resource, that may implicitly mean that it is used for scheduling retransmission. So, no indicator is needed for call for retransmission. Similar as unsegmented packet retransmission handling, for the first segment case, the successfully received data size is “0”. During the following retransmission, the reader does not need to indicate the offset.

Proposal 3: Unsegmented packet retransmission is supported.

For the subsequent segment retransmission, it is obviously that the device has successfully receive the command message and store the start address in the VM. The following D2R scheduling message does not need to contain the command message. However, for first segment and the unsegmented packet, the reader can not identify the D2R transmission failure is caused by R2D receiving failure or D2R transmission failure. The command message should be included in the R2D message to indicate device the star address.

Observation 2: Reader cannot identify the D2R transmission failure is caused by R2D receiving failure or D2R transmission failure for the first segment and the unsegmented packet.

Proposal 4: The start address in command message needs to be stored in the VM of device.

Proposal 5: For the first segment retransmission and unsegmented packet, the “offset” indicator is not needed.

Proposal 6-1: In case subsequent segment retransmission, the command message does not need to be re-contained in the R2D message.

Proposal 6-2: In case first segment and unsegmented packet retransmission, the command message should be re-contained in the R2D message.

Retransmission of Msg3

As to Msg3 retransmission, it has not been concluded whether the segmentation is applied. As we discussed in RA paper [2], the segmentation should be introduced to Msg3 transmission. Retransmission mechanism of Msg3 is similar as data retransmission rule, e.g., offset indication for segmentation and no indicator for unsegmented packet in R2D message. However, the R2D message for scheduling Msg3 segmentation retransmission are somewhat complexity. If the segment of Msg3 is successfully received, the reader may use AS ID addressed R2D for subsequent scheduling; if the segment of Msg3 is not successfully received, the reader may retransmit Msg2. So, the device performing Msg3 segmentation may monitor Msg2 and AS ID addressed R2D for D2R scheduling simultaneously. For unsegmented Msg3, device may monitor the Msg2 for retransmission and Msg4 for NACK indication.

Proposal 7: Msg3 segmentation should be supported.

Proposal 8: Msg3 retransmission can follow the data retransmission rule.

Proposal 9-1: If the segment of Msg3 is successfully received, the reader uses AS ID addressed R2D for subsequent scheduling; if the segment of Msg3 is not successfully received, the reader retransmits Msg2 for scheduling.

Proposal 9-2: Device performing Msg3 segmentation may monitor Msg2 and AS ID addressed R2D simultaneously.

Proposal 9-3: Device performing unsegmented Msg3 transmission may monitor Msg2 and Msg4 simultaneously.

Data/Msg3 transmission failure

As we discussed and concluded during data segmentation issue, the devices perform segment transmission with reader indication in R2D message. As to Msg3 transmission, the reader can retransmit Msg2 to allocate resource for Msg3 retransmission if it fails to receive Msg3. For Msg3 transmission, NACK based feedback is supported. So, if no feedback is received or receive R2D message contain the assigned AS ID and command, device can consider the Msg3 transmission successfully. Otherwise, transmission failure. For data transmission, NACK based feedback can be reused, e.g., if no feedback is received for unsegmented data or the last segment (remaining data indicator is 0) , device can consider the data transmission successfully.

Proposal 10: If no Msg4 feedback is received or received AS ID addressed R2D message with command, device can consider the Msg3 transmission successfully. Otherwise, transmission failure.

Proposal 11: For data transmission, NACK based feedback can be reused, e.g., if no feedback is received for unsegmented packet or the last segment (remaining data indicator is 0), device can consider the data transmission successfully. Otherwise, data transmission failure.

Data and Msg3 transmission failure can be handled by reader (A-IoT enable gNB) implementation, for example, re-send the data with R2D scheduling as indicated D2R resource, or access in subsequent access round or paging round if no D2R resource allocated in R2D message. Other timer based or counter based solution should not be considered at device side, to lower the device implementation complexity.

Proposal 12: RAN2 does not introduce timer-based or counter-based solution at device side to determine whether data/Msg3 transmission or retransmission failure.

Proposal 13: Data transmission failure can be handled by reader (A-IoT enable gNB) implementation, e.g., re-send the data with R2D scheduling, or re-access in subsequent access round or paging round if no D2R resource allocated in R2D message.

Message size reporting

The message size information is optionally provided by the AIoTF implementation. In last meeting, RAN2 had agreed that if SA2 indicates that CN can provide an estimated expected D2R message size, a 1 bit indication is introduced to indicate whether there is more data or not for segmentation. For the case that message size information is not provided by the CN, it is still not sure whether

1 bit indication is sufficient. Considering R2D scheduling efficiency, 1 bit indication bring less information of D2R data size. Instead, we suggest rough or exact data size data size reporting. Because the CN may provide data size to gNB, the data size reporting from device can be handled under network control. For example, if the gNB receives size information from the CN, it can inform the device to avoid data size reporting. Considering that the possible data size could be around 1000 bits, the overhead associated with such reporting cannot be ignored. In addition, the data size can be reported only once for decreasing the D2R transmissions overhead. If RAN2 support rough data size reporting, the allocated data size can not fulfil the D2R transmissions requirement and segmentation can be applied at device side, the 1-bit indication can be used alone with subsequent transmission. In our understanding, it is a beneficial to report 1 bit indication instead of always report rough remaining data size.

Proposal 14: For forward compatible for large data size, the data size reporting from device can be handled under network control. For example, if the gNB receives size information from the CN, it can inform the device to avoid data size reporting.

Proposal 15: The data size reporting can be reported only once.

proposal 16: If RAN2 support rough data size reporting, 1-bit indication can be used alone with subsequent transmission.

AS ID

"AS ID" issue has been discussed in Post email discussion. How to assign AS ID for CBRA and CFRA has been discussed fully, and the views of most companies are basically the same, achieving convergence of opinions. we do not repeat the comments in this contribution. However, discussions on the AS ID validation are more scattered. Consensus has been reached only on the solution based on power-off release of AS ID. Moreover, these solutions discussed in the email discussion are not mutually exclusive or conflicting. We believe further discussion is necessary to introduce a solution that meets the system performance requirements without over-designing or increasing the complexity of the A-IoT device and reader.

Companies generally agree that the device should release the AS ID upon power off and receiving the new transition ID in paging message. During the email discussion, we noticed that some companies raised the question of whether capturing the same transition ID in the paging message would cause the device to release the AS ID. The underlying scenario is whether to allow commands across paging rounds. If this use case is supported, the reader can use the same AS ID to perform command operations to dedicated device who has been inventoried within the subsequent paging round. It avoids the need to first go through the inventory process before issuing a command, thereby reducing latency and improving efficiency. However, in this case, the reader needs to avoid using the current AS ID in the subsequent paging round until a new service type replaces the transition ID. Additionally, this also poses a requirement for the minimum time the device needs to store the AS ID in VM. The minimum time to store the AS ID can also somehow solve the so-called timer-based solution. For timer-based solution, it requires the device to maintain accurate timing, which increases the complexity to device.

Proposal 17: RAN2 is asked to introduce the minimum time to store AS ID in VM as a requirement for devices and captured in the specification.

PDU format

General

RAN2 has support multiplexing of information for multiple devices in R2D message for msg2. For the subsequent R2D message, e.g., Msg4 containing NACK message, read/write command, there are not conclusion on whether multiplex can be introduced. In our understanding, the NACK message as indication of Msg3 transmission failure only include the AS ID or random ID. The length of a concatenated Msg4 is not too large to bring complexity and latency to the device compared to multiplexing Msg2. So, NACK message for multiple devices can be multiple in one Msg4.

Proposal 18: NACK message for multiple devices can be multiple in one Msg4.

However, the command message, especially the write command, may contain large amounts of data. Multiplexing commands from multiple devices into a single R2D data packet can impose significant energy overhead on the devices. Meanwhile, if R2D messages include different types of command messages, it may require a certain header field to indicate the type of command. This brings greater challenges to the design of the R2D format. For that reason, we suggest RAN2 RAN2 not to consider multiplex command message towards multiple devices within one R2D message.

Proposal 19: RAN2 is asked not to consider multiplex command message towards multiple devices within one R2D message.

D2R format

Considering the conclusion of last meeting, only Msg1 and Msg3/data are considered as D2R message type. For Msg1, it contains Random value RN16 for CBRA and acknowledges of paging message for CFRA. The MAC header is not required for reader to identify such D2R message. For Msg3 and data, they are scheduled by the response to different R2D messages. The data transmission is used to acknowledge the command and will not start before Msg3 transmission. So, the message type is not needed for D2R.

The following is an example of A-IoT Msg1:

Fig.1: An example of A-IoT Msg1

Fig.2: An example of A-IoT Msg3 without segmentation

Fig.3: An example of D2R data with segmentation

Proposal 20: The MAC header is not required for Msg1.

Proposal 21: Message type is not needed for Msg3 and D2R data.

R2D format

As we analysis on RA contribution [2], we suggest RAN2 to consider variable length of message type indication.

Proposal 22: RAN2 is suggested to consider variable length of message type indication.

With the discussion on AS ID allocation, the Msg2 can be used to indicate the new assigned AS ID or confirm the random Id is reused. In addition, the D2R resource for Msg3 transmission. Considering the multiplex, we give an example of Msg2

Fig.4: An example of A-IoT Msg2

As we discussed and suggested in 2.6.1, the Msg4 can be multiplex NACK of multiple devices.

Fig.5: An example of A-IoT Msg4

An example of R2D with command is as follows,

Fig.6: An example of A-IoT R2D with command

Conclusion

According to the above discussion, the following observations and proposals are given:

Retransmision of Segmentation and unsegmented packet

Observation 1：The Maximum size of D2R data is about 1000bit, 10-bits offset indication is enough for R19.

Observation 2: Reader cannot identify the D2R transmission failure is caused by R2D receiving failure or D2R transmission failure for the first segment and the unsegmented packet.

Proposal 1: For segmentation retransmission, the reader explicitly indicates that number of bits successfully received currently with a 10-bits offset in R2D message.

Proposal 2: The size of D2R segment is indicated by the reader with R2D message.

Proposal 3: Unsegmented packet retransmission is supported.

Proposal 4: The start address in command message needs to be stored in the VM of device.

Proposal 5: For the first segment retransmission and unsegmented packet, the “offset” indicator is not needed.

Proposal 6-1: In case subsequent segment retransmission, the command message does not need to be re-contained in the R2D message.

Proposal 6-2: In case first segment and unsegmented packet retransmission, the command message should be re-contained in the R2D message.

Retransmision of Msg3

Proposal 7: Msg3 segmentation should be supported.

Proposal 8: Msg3 retransmission can follow the data retransmission rule.

Proposal 9-1: If the segment of Msg3 is successfully received, the reader uses AS ID addressed R2D for subsequent scheduling; if the segment of Msg3 is not successfully received, the reader retransmits Msg2 for scheduling.

Proposal 9-2: Device performing Msg3 segmentation may monitor Msg2 and AS ID addressed R2D simultaneously.

Proposal 9-3: Device performing unsegmented Msg3 transmission may monitor Msg2 and Msg4 simultaneously.

Data/ Msg3 failure

Proposal 10: If no Msg4 feedback is received or received AS ID addressed R2D message with command, device can consider the Msg3 transmission successfully. Otherwise, transmission failure.

Proposal 11: For data transmission, NACK based feedback can be reused, e.g., if no feedback is received for unsegmented packet or the last segment (remaining data indicator is 0), device can consider the data transmission successfully. Otherwise, data transmission failure.

Proposal 12: RAN2 does not introduce timer-based or counter-based solution at device side to determine whether data/Msg3 transmission or retransmission failure.

Proposal 13: Data transmission failure can be handled by reader (A-IoT enable gNB) implementation, e.g., re-send the data with R2D scheduling, or re-access in subsequent access round or paging round if no D2R resource allocated in R2D message.

Message size reporting

Proposal 14: For forward compatible for large data size, the data size reporting from device can be handled under network control. For example, if the gNB receives size information from the CN, it can inform the device to avoid data size reporting.

Proposal 15: The data size reporting can be reported only once.

proposal 16: If RAN2 support rough data size reporting, 1-bit indication can be used alone with subsequent transmission.

AS ID

Proposal 17: RAN2 is asked to introduce the minimum time to store AS ID in VM as a requirement for devices and captured in the specification.

PDU format

General

Proposal 18: NACK message for multiple devices can be multiple in one Msg4.

Proposal 19: RAN2 is asked not to consider multiplex command message towards multiple devices within one R2D message.

D2R format

Proposal 20: The MAC header is not required for Msg1.

Proposal 21: Message type is not needed for Msg3 and D2R data.

R2D format

Proposal 22: RAN2 is suggested to consider variable length of message type indication.

Reference

[1] RP-243326, New work item: Solutions for Ambient IoT (Internet of Things) in NR

[2] R2-2502717, Further consideration on A-IoT Random access, CMCC