3GPP TSG-RAN WG2 Meeting #129bis       	                      R2-2502733

Wuhan, China, 7th-11th April 2025

Agenda item:	8.3.5

Source:	CMCC

Title:	Discussion on LCM for measurement event prediction

WID/SID:	FS_NR_AIML_Mob

Document for:	Discussion

- Introduction

The objectives for Rel-19 AI/ML for Mobility were approved in [1]. This contribution will provide our initial consideration on LCM framework for measurement event prediction.

- Discussion

For LCM functional framework and studied aspects, as mentioned in our contribution on RRM measurement prediction [2], the progress on AI/ML for Air Interface can be reused as much as possible. This contribution will analysis the aspects related to measurement event prediction.

- 2.1 Definition of AI/ML-enabled Feature and functionality

Based on TR 38.843 [3], we could observe that AI/ML-enabled Feature refers to a Feature where AI/ML may be used and there may be either one or more than one Functionalities defined within an AI/ML-enabled feature. That means functionality could have finer granularity than AI/ML-enabled feature.

During the study phase, direct prediction and indirect prediction are studied and evaluated for measurement event prediction. And for indirect prediction, RAN2 focus on sub-use case 2 (to directly predict cell level results based on cell level results) of RRM measurement prediction, and agreed the following definitions considering the options of RRM measurement prediction.

RAN2#128 Agreements on event prediction 1 Agree to listed 4 definitions of indirect measurement event prediction Indirect measurement event prediction for temporal domain case A: In indirect measurement event prediction, future measurement result(s) is predicted by a RRM measurement prediction model for intra-frequency temporal domain case A at first. Afterwards, predicted and optionally actual historical measurement result(s) are used to derive whether a measurement event at one future time instance occurs, without further involvement of an AI/ML model. Indirect measurement event prediction for temporal domain case B: In indirect measurement event prediction, measurement result(s) is predicted by a RRM measurement prediction model for intra-frequency temporal domain case B at first. Afterwards, predicted and optionally actual historical measurement result(s) are used to derive whether a measurement event at one time instance occurs, without further involvement of an AI/ML model. Indirect measurement event prediction for frequency domain: In indirect measurement event prediction, measurement result(s) is predicted by a RRM measurement prediction model in frequency domain at first. Afterwards, predicted and optional actual historical measurement result(s) of serving cell are used to derive whether a measurement event at one time instance occurs, without further involvement of an AI/ML model. Indirect measurement event prediction for spatial domain: In indirect measurement event prediction, measurement result(s) is predicted by a RRM measurement prediction model in spatial domain at first. Afterwards, predicted measurement result(s) and actual measurement results are used to derive whether a measurement event at one time instance occurs, without further involvement of an AI/ML model

Based on the above progress, we understand that AI/ML-enabled measurement event prediction can be a feature, and direct or indirect prediction can be functionality. For indirect prediction, the functionality can be further refined into indirect measurement event prediction for temporal domain case A, temporal domain case B, frequency domain and spatial domain. RAN2 can discuss which granularity will be applied for functionality of measurement event prediction.

Proposal 1: RAN2 to discuss the granularity of functionality for measurement event prediction:

- Direct prediction and indirect prediction

- For indirect prediction, the functionality can be further refined into indirect measurement event prediction for temporal domain case A, temporal domain case B, frequency domain and spatial domain

- 2.2 UE capability and Applicable functionality reporting

Since only UE-sided model is considered for measurement event prediction, the UE capability and applicable functionality reporting can reuse the procedure defined for AI/ML based beam management with UE-sided model.

Fig 1. Basic procedure for applicable functionality reporting

Proposal 2: The UE capability and applicable functionality reporting procedure defined for AI/ML based beam management can be reused for AI/ML based measurement event prediction with UE-sided model.

For the content of full inference configuration and/or inference related information, the following aspects can be further discussed:

- For direct prediction:

- observation window, prediction window, model input and output related info (e.g., L3 cell level RSRP), event related info (e.g. event offset, TTT)

- associated ID, which is introduced in AI/ML for Air Interface, and indicates the network-side additional condition

- For indirect prediction:

- similar to RRM measurement prediction, e.g. observation window, prediction window, model input and output related info, MRRT, frequency info, sample period

- event related info (e.g. event offset, TTT)

- associated ID

Proposal 3: RAN2 to discuss the content of full inference configuration and/or inference related information for direct and indirect prediction respectively, and take the above content as starting point.

- 2.3 Functionality management

During the discussion on AI/ML for Air Interface WI, RAN2 discussed the following options for functionality/model management for UE-sided model, and agreed that option 1a (Network decision, network-initiated AI/ML management) is as baseline, and option 2c is not considered in R19.

- Option 1: Decision by the network

1a) Network-initiated

1b) UE-initiated and requested to the network

- Option 2: Decision by the UE

2a) Event-triggered as configured by the network, UE’s decision is reported to the network

2b) UE-autonomous, UE’s decision is reported to the network

2c) UE-autonomous, UE’s decision is not reported to the network

For AI/ML for mobility, the similar mechanism can be reused for measurement event prediction with UE-sided model.

Proposal 4: Option 1a (Network decision, network-initiated AI/ML management) can be used as baseline for AI/ML based measurement event prediction with UE-sided model.

Furthermore, RAN2 should focus on the discussion on information reported by UE to assist the network to make decision on management, for example,

- the monitoring related info, e.g. KPIs for indirect prediction (prediction accuracy of RRM measurement), or for direct prediction (F1-score)

- the real measurement results

Proposal 5: RAN2 to discuss the information reported by the UE to assist the network to make decision on management, e.g. monitoring related info, real measurement results.

- 2.4 Data collection

Data collection for training

For the collected data transfer in AI/ML for Air Interface, RAN2 discussed the following potential solutions and has no consensus for now.

- Option 1: UE collects and directly transfers training data to the Over-The-Top (OTT) server

1a) OTT (3GPP transparent)

1b) OTT (non-3GPP transparent)

- Option 2: UE collects training data and transfers it to Core Network. Core Network transfers the training data to the OTT server

- Option 3: UE collects training data and transfers it to OAM. OAM transfers the needed data to the OTT server

For AI/ML for mobility, RAN2 can wait for the progress in AI/ML for NR air interface, and focus on the content for data collection.

Regarding the data content for model training, the different information may be needed for different prediction options, which should be discussed separately. Specifically,

- For direct prediction: e.g. L3 cell level measurement results, event configuration info, event occurrence info (e.g. occurrence time)

- For indirect prediction: e.g. L1/L3 measurement results, MRRT, frequency info, event configuration info, event occurrence info (e.g. occurrence time)

Proposal 6: RAN2 to discuss the data content to be collected for model training for indirect prediction and direct prediction, e.g. L3 cell level measurement results and event info for direct prediction, L1/L3 measurement results, MRRT and event info for indirect prediction.

Data collection for inference

Based on the previous agreements, for indirect prediction, the intermediate output (i.e., the output of RRM prediction model) is RSRP of serving/neighboring cells. And the final output is the expected occurrence time of a certain measurement event (ex. event A3). Therefore, the UE can report the final output to the network for handover decision, and the intermediate output can be reported optionally.

Proposal 7: For indirect measurement event prediction, the UE can report the expected occurrence time of a certain measurement event and optionally the predicted RSRP of serving/neighboring cells to the network for handover decision.

For direct prediction, the model output is the probability of event occurrence within a time window. Therefore, the UE can report the probability of event occurrence to assist the network to determine whether to perform handover procedure.

Proposal 8: For direct measurement event prediction, the UE can report the probability of event occurrence within a time window to the network for handover decision.

# 3	Conclusion

Here are the proposals on LCM for measurement event prediction.

Definition of AI/ML-enabled Feature and functionality

Proposal 1: RAN2 to discuss the granularity of functionality for measurement event prediction:

- Direct prediction and indirect prediction

- For indirect prediction, the functionality can be further refined into indirect measurement event prediction for temporal domain case A, temporal domain case B, frequency domain and spatial domain

UE capability and Applicable functionality reporting

Proposal 2: The UE capability and applicable functionality reporting procedure defined for AI/ML based beam management can be reused for AI/ML based measurement event prediction with UE-sided model.

Proposal 3: RAN2 to discuss the content of full inference configuration and/or inference related information for direct and indirect prediction respectively, and take the above content as starting point.

Functionality management

Proposal 4: Option 1a (Network decision, network-initiated AI/ML management) can be used as baseline for AI/ML based measurement event prediction with UE-sided model.

Proposal 5: RAN2 to discuss the information reported by the UE to assist the network to make decision on management, e.g. monitoring related info, real measurement results.

Data collection

Proposal 6: RAN2 to discuss the data content to be collected for model training for indirect prediction and direct prediction, e.g. L3 cell level measurement results and event info for direct prediction, L1/L3 measurement results, MRRT and event info for indirect prediction.

Proposal 7: For indirect measurement event prediction, the UE can report the expected occurrence time of a certain measurement event and optionally the predicted RSRP of serving/neighboring cells to the network for handover decision.

Proposal 8: For direct measurement event prediction, the UE can report the probability of event occurrence within a time window to the network for handover decision.

# 4	Reference

- RP-242393, Revised SID on AIML for mobility in NR

- R2-2502711, Discussion on LCM for RRM measurement prediction with UE sided model, CMCC

- TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface