3GPP TSG-RAN WG3 Meeting #129bis       	                      R2-2502738

Wuhan, China, 7th-11th April 2025

Agenda item:	8.10.3

Source:	CMCC

Title:	Discussion on SON/MDT for network slicing

WID/SID:	NR_ENDC_SON_MDT_Ph4-Core

Document for:	Discussion

- Introduction

In last meeting, RAN3 sent a LS [1] to RAN2 on slice-based cell reselection.

RAN3 has discussed how to optimize slice based cell reselection via e.g. logged MDT and identified the following scenario: - A UE supporting slice-based cell reselection may not find any suitable cell in the frequencies corresponding to the highest ranked NSAG, thereby UE have to camp on cells belonging to the lower ranked NSAG or eventually use legacy cell reselection procedure to find a suitable cell. RAN3 would like to ask RAN2 to take the above scenario into account, and to discuss whether and how it can be addressed and whether any standard changes are needed.

This contribution will discuss the potential SON/MDT enhancements for network slicing.

- Discussion

For slice-based cell re-selection, the UE shall derive reselection priorities based on the rules specified in TS 38.304[2], considering the NSAG priority received by NAS and the slice information received in SIB16 or RRCRelease, e.g. nsag-CellReselectionPriority, sliceAllowedCellListNR, sliceExcludedCellListNR.

The UE shall derive reselection priorities for slice-based cell reselection according to the following rules: - Frequencies that support at least one prioritized NSAG received from NAS have higher reselection priority than frequencies that support none of the NSAG(s) received from NAS. - Frequencies that support at least one NSAG provided by NAS are prioritised in the order of the NAS-provided priority for the NSAG with highest priority supported on the frequency. - Among the frequencies (one or multiple) that support the highest prioritised NSAG(s) with the same NAS-provided priorities, the frequencies are prioritized in the order of their highest nsag-CellReselectionPriority given for these NSAG(s). If no nsag-CellReselectionPriority is given for a NSAG at a frequency, the lowest priority value is used (i.e, lower than any of the network configured values for these frequencies). - Frequencies that support none of the NSAG(s) provided by NAS are prioritized in the order of their cellReselectionPriority.

Furthermore, if the cell is either listed in the sliceAllowedCellListNR or is not listed in the sliceExcludedCellListNR, or neither sliceAllowedCellListNR nor sliceExcludedCellListNR is configured, the UE will consider the cell to support the NSAG.

The UE considers a cell on an NR frequency to support all slices of an NSAG if - the nsag-ID and TA of the NSAG indicated for the NR frequency in dedicated signalling but not in SIB16 (see TS 38.331 [3]) are included in the NSAG information provided by NAS; or - the nsag-ID and TA of the NSAG indicated for the NR frequency in SIB16 (see TS 38.331 [3]) are included in the NSAG information provided by NAS; and - the cell is either listed in the sliceAllowedCellListNR (if provided in the sliceInfoList) or the cell is not listed in the sliceExcludedCellListNR (if provided in the sliceInfoList); or - Neither sliceAllowedCellListNR nor sliceExcludedCellListNR is configured in the sliceInfoList.

On the other hand, it is also specified that if the highest ranked cell or best cell does not support the NSAG, the UE will re-derive the re-selection priority based the NSAG supported by this cell or legacy cell priority. Then the UE continues to evaluate based on the re-derived re-selection priority, and will select a cell which supports the next priority NSAG within UE.

For a UE performing slice-based cell reselection, if the highest ranked cell or best cell in a frequency fulfils the inter- freqeuency cell reselection criteria (see clause 5.2.4.5) based on reselection priority for the frequency and NSAG derived according to this clause or fulfils intra-frequency and equal priority inter-frequency cell reselection criteria (see clause 5.2.4.6), but this cell does not support the NSAG according to this clause: - if this cell supports any other NSAG(s) according to this clause, the UE shall re-derive a reselection priority for the frequency by considering the NSAG(s) supported by this cell (rather than those of the corresponding NR frequency); - Otherwise, the UE shall re-derive a reselection priority for the frequency as if none of the NSAG(s) provided by NAS is supported.

On the other hand, if the sliceAllowedCellListNR or sliceExcludedCellListNR is not configured, the slice based cell re-selection may be impacted. For example, the following slice information is broadcast in SIB16, including frequency info, supported NSAG ID, NSAG based cell re-selection priority, slice allowed cell list and slice excluded cell list. It is assumed that the priority of NSAG#1 is higher than NSAG#2 within UE, and the larger number for cell re-selection priority means the higher priority.

| Frequency | NSAG ID | NSAG based cell re-selection priority | Legacy cell re-selection priority | Slice allowed cell list | Slice excluded cell list |
| --- | --- | --- | --- | --- | --- |
| F1 | NSAG#1 | 4 | 3 | Cell#1, Cell#2 | - |
|  | NSAG#2 | 5 |  | - | Cell#4 |
| F2 | NSAG#1 | 6 | 5 | - | - |
| F3 | NSAG#2 | 4 | - | - | - |
| F4 | N/A | - | 4 | - | - |

Based on the slice based cell re-selection mechanism specified in TS 38.304 [2], the UE derives that the reselection priority for the frequency is F2>F1>F3>F4. And the procedure is as follows:

- Firstly, considering the highest NSAG (i.e. NSAG#1), both F1 and F2 support NSAG#1, and the NSAG based cell re-selection priority for F1 and F2 are 4 and 6 respectively, so the priority of F2 is higher than F1, i.e. F2>F1;

- Secondly, considering the next highest priority NSAG (i.e. NSAG#2), both F1 and F3 support NSAG#1, since F1 has been considered in the first step, so the priority of F3 is lower than F1, i.e. F2>F1>F3;

- Lastly, all NSAGs supported by UE have been considered, then the frequency doesn’t support any NSAG will be considered. Since there is only one frequency supports none NSAG (i.e. F4), the priority is lowest, i.e. F2>F1>F3>F4.

Then the UE will evaluate from the highest priority frequency, i.e. F2. For F2, neither slice allowed cell list nor slice excluded cell list is configured, which means all cells on F2 support NSAG#1. The UE will camp on the best cell on F2 (e.g. cell#3).

However, it may happen that after the UE camps on the cell (e.g. cell#3), the UE is aware of the cell#3 does not support the NSAG#1 via SIB16, especially when the UE resides at the boundary of different TAs. That is the scenario RAN3 indicated that a UE supporting slice-based cell reselection may not find any suitable cell in the frequencies corresponding to the highest ranked NSAG (i.e. NSAG#1), thereby UE have to camp on cells belonging to the lower ranked NSAG (i.e. NSAG#2) or eventually use legacy cell reselection procedure to find a suitable cell.

In this case, when the UE would like to initiate the highest priority NSAG (i.e. NSAG#1), but the camped cell#3 cannot satisfy the requirements on NSAG#1, a handover procedure may be needed to let the UE access a target cell supporting the NSAG#1, which increase the latency and complexity.

Observation 1: If neither slice allowed cell list nor slice excluded cell list is configured which means all cells on the frequency support the NSAG, but the UE finds the best cell or highest ranked cell does not support the NSAG after the UE camps on, a handover procedure may be needed to let the UE access a target cell supporting the NSAG, which increase the latency and complexity.

If the UE can log and report this case to the network, the network can optimize the configuration of sliceAllowedCellListNR or sliceExcludedCellListNR, for example, configure the sliceExcludedCellListNR as cell#3 which means the cell#3 does not support the NSAG#1, and other cells on F2 support NSAG#1. with the information, the UE can re-derive the cell re-selection priority and can camp on F1 which also supports the highest NSAG#1.

Observation 2: If the UE can log and report the highest ranked cell or best cell to the network, the network can optimize the configuration of sliceAllowedCellListNR or sliceExcludedCellListNR, which will assist the UE camp on the more suitable cell supporting the highest priority NSAG.

On the other hand, if only sliceExcludedCellListNR is configured and the highest ranked cell or best cell is not within the sliceExcludedCellListNR, the UE will consider the cell to support the NSAG. However, after the UE camps on the cell, it may find the cell does not support the NSAG via SIB16, which is similar to the above case.

Based on the above analysis, if the UE can report the NSAG(s) supported by the highest ranked cell or best cell, it can be helpful for NW to adjust the sliceAllowedCellListNR and/or sliceExcludedCellListNR. Then the UE will find a more suitable cell for slice-based cell re-selection.

Proposal 1: The NSAG(s) supported by the highest ranked cell or best cell, the NSAG(s) supported by the UE and the priority information can be reported to the network, which can be helpful for NW to optimize the sliceAllowedCellListNR and/or sliceExcludedCellListNR, or slice deployment, and can assist the UE to re-select a suitable cell for slice-based cell re-selection.

# 3	Conclusion

Here are the observations and proposals for SON/MDT enhancements for network slicing.

Observation 1: If neither slice allowed cell list nor slice excluded cell list is configured which means all cells on the frequency support the NSAG, but the UE finds the best cell or highest ranked cell does not support the NSAG after the UE camps on, a handover procedure may be needed to let the UE access a target cell supporting the NSAG, which increase the latency and complexity.

Observation 2: If the UE can log and report the highest ranked cell or best cell to the network, the network can optimize the configuration of sliceAllowedCellListNR or sliceExcludedCellListNR, which will assist the UE camp on the more suitable cell supporting the highest priority NSAG.

Proposal 1: The NSAG(s) supported by the highest ranked cell or best cell, the NSAG(s) supported by the UE and the priority information can be reported to the network, which can be helpful for NW to optimize the sliceAllowedCellListNR and/or sliceExcludedCellListNR, or slice deployment, and can assist the UE to re-select a suitable cell for slice-based cell re-selection.

# 4	Reference

R3-250914 LS on SON for Network Slicing

TS 38.304 NR; User Equipment (UE) procedures in Idle mode and RRC Inactive state