3GPP TSG-WG2 Meeting #129bis                                                                     R2-2502739

Wuhan, China, 7th – 11th April 2025

Agenda item:	8.8.2

Title:	Analysis on DL coverage enhancements due to extended SSB periodicity

Source:	CMCC

Document for:	Discussion

- Introduction

In previous RAN2 meetings, the following statements on DL coverage enhancements have been agreed with some remaining issues:

For RAN2 129 meeting:

From RAN2 perspective, we support option a (clustered cells illuminated) and option b (scattered cells illuminated) for further discussion on SMTC. No LS is sent to RAN1.

RAN2 observes that if the cells active simultaneously are in clusters, existing SMTC mechanism (a maximum of 4 SMTCs per frequency) may be sufficient (FFS if any solution identified for option b applies to option a as well).

RAN2 observes that if the cells active simultaneously are scattered, (for the case of intra-frequency neighbouring cells), the SSBs of surrounding neighbour cells may be transmitted at different times and existing SMTC mechanism may not be enough.

For RAN2 128 meeting:

If we need to bar UEs not supporting DL-CE (via legacy mechanism), we introduce a new barring in SIB1 to be able to selectively bar “UEs supporting DL-CE” (FFS if we will finally refer to “UEs supporting extended SSB periodicity” instead). FFS on the details (e.g. how many bits, etc.)

In this contribution, we will further provide our views on the remaining issues of DL coverage enhancements.

- Discussion

2.1 SMTC enhancements

In previous RAN2#128 meetings, SMTC impacts for downlink coverage enhancement in NR NTN had been discussed,  the following agreements have been achieved:

Agreements:

- As part of the work on DL-CE, RAN2 investigates related UE power consumption impact (including legacy UEs)

- RAN2 will consider whether to introduce separate signalling (e.g. new SMTC5 list) for DL CE cells SMTCs, e.g. if different periodicities need to be signalled or to prevent reselection to specific cells

In previous RAN2#129 meetings, related agreements are as follow:

Agreements:

- RAN2 assumes it will be possible to have different SSB periodicity among neighbour cells in the same frequency layer

- RAN2 assumes that in a NR NTN cell, SSB beam sweeping in different spatial directions is possible as in a NR TN cell: the whole cell is covered by the different SSB beams in half-frame

- RAN2 also assumes that, with the current status of RAN1 discussion, if one cell is defined by multiple “satellite beams”, the satellite beams are all simultaneously active or inactive (“beam hopping” applies equally to all the satellite beams of a given cell)

- The number of SMTC/gaps a UE needs to consider at any time will not be increased further

- For SMTC configuration

Observation 1: Current restriction on MO configuration chooses at most one measurement object with the same ssbFrequency which is not sufficient for multiple SSB periodicity among neighbour cells in the same frequency layer. Hence, the SMTC configuration needs to be modified. For example, adding the SSB periodicity parameter to the existing item SSB-MTC4-r17.

- Proposal 1: For supporting different SSB periodicity among neighbour cells in the same frequency layer, the SMTC configuration needs to be modified to allow multiple periodicity configuration, e.g. adding SSB periodicity to the existing item SSB-MTC4-r17.

- For SMTC enhancement solutions

There are several possible solutions for SMTC enhancements:

Solution 1: Configure multiple SMTCs (more than 4) and SMTC selection (e.g. location-based SMTC selection, UE autonomous selection)

RAN2 observes that if the cells active simultaneously are scattered, (for the case of intra-frequency neighbouring cells), the SSBs of surrounding neighbour cells may be transmitted at different times and existing SMTC mechanism may not be enough. Solution 1 can be a option to enhance the existing SMTC mechanism to accommodate the differential arriving time of neighbour cell SSBs due to beam hopping.

The SMTC configuration for UE  needs to be synchronized between the network side and UE. Under this premise, the key issue of this solution is how to determine which SMTC configuration the UE is using among multiple SMTCs (more than 4). And there are following details need to be discussed.

Figure 1 Two scenarios of the satellite target coverage area in NTN system

As shown in Figure1, there are mainly 2 options which can be considered for the mapping of NR beams and NR cells to one satellite beams:

- Scenario 1: Different PCIs for mutiple beam footprints in one satellite cell (1-to-1 mapping);

- Scenario 2: Single PCI for Multi-beam footprints in one satellite cell (multiple-to-1 mapping)

Regarding Scenario 1,  there might be a baseline that one-to-one mapping one beam (similar to one Terrestrial-network cell) to one Satellite cell. There are two sub options in this scenario:

In Option a, current SMTC configurations may be sufficient.

In  Option b, the number of neighbour cells for given cell will be increased depending on scattered level of active cells. Hence, more than 4 SMTC configurations need to be configured to UE, along with corresponding assistance information, for UE to perform SMTC selection.

Regarding Scenario 2, there might be a baseline that multiple-to-one mapping one beam (similar to one Terrestrial-network cell) to one Satellite cell. Hence, more than 4 SMTC configurations need to be configured to UE, along with corresponding assistance information, for UE to perform SMTC selection.

For UE in RRC idle/active, UE can only make decision based on the information received in SIB, therefore, it is essential to expand the information carried in SIB. It is prefer to add  assitance information corresponding to measurement, e.g., area informationt in SMTC configuration, e.g, item SSB-MTC4-r17, for SMTC configuration selection.

For UE in RRC connected, the network side can dynamically configure SMTC for UE based on its assitance information.

Solution 2: SSB-specific  SMTC selection

For Solution 2, we consider introducing additional SSB specific information, such as SSB index, in SMTC configuration. Specifically, each SMTC configuration corresponds to one or more SSB index. Furthermore, these configurations can be provided through dedicated signaling or system information.

In this solution, it is still FFS on how to select the SSB to be measured by UE and whether the current SMTC can meet the requirements when selecting SMTC based on SSB as a dimension.

Solution 3: UE assistance information reporting

This solution 3 require UE in RRC connected to report UE assistance information , e.g., beam/footprint/location/area information, which is beneficial for synchronizing information between NW and UE, so that NW can configure more accurate information for UE to identity the specific SSB reception location/window to avoid unnecessary power consumption.

Solution 4: Configure one SMTC with small periodicity, in each period (SMTC occasion) UE is indicated which PCI to use

In this Solution,  the SMTC window is  is divided into smaller units (SMTC occasion) with small periodicity, and network side indicate UE that which PCI will use the SMTC occasion. This solution involves significant changes to the existing SMTC configuration information and mechanism, and there are compatibility issues with other related processes.

Observation2: It will be facilitate the accurate SMTC configuration by gNB for  a given UE, based on the  assistance information reported from the UE, to avoid unnecessary measurement performed by the UE.

Proposal 2: RAN2 could consider requiring a UE in connected mode to report assistance information, e.g., beam/footprint/location/area information,  to gNB through UAI procedure.

- Proposal 3: It is proposed to add  assistance information corresponding to measurement, e.g., area information in SMTC configuration, e.g, item SSB-MTC4-r17, for SMTC configuration selection.

2.2 Cell barring

Agreements of RAN2#127bis meetings are as follow:

- If it turns out that there is a need to bar UEs not supporting DL-CE, Rel-19 UEs not supporting DL-CE can be barred from accessing a cell operating with DL-CE using the existing NTN bar bit, in the same way as pre-Rel19 NTN UEs (this is an extension of the previous agreement to include also Rel-19 UE not supporting DL-CE)

2. If it turns out that there is a need to bar UEs not supporting DL-CE, then we need to introduce a barring mechanism to control access of UEs supporting Rel19 NTN DL-CE. FFS on the details. (This also implies that UEs supporting Rel19 NTN DL-CE will not consider the existing NTN barring bit)

In previous RAN2#128 meetings, impact with extended SSB periodicity for downlink coverage enhancement in NR NTN had been discussed,  the following agreements in cell barring aspect has achieved:

- If we need to bar UEs not supporting DL-CE (via legacy mechanism), we introduce a new barring in SIB1 to be able to selectively bar “UEs supporting DL-CE” (FFS if we will finally refer to “UEs supporting extended SSB periodicity” instead). FFS on the details (e.g. how many bits, etc.)

Observation 3:  The current SSB periodicity configuration supports 160ms, and the network side in the same cell can switch SSB periodicity configuration between 5ms and 160ms. In order to simplify the signaling process, the same cell barring mechanism is used for all kinds of UEs that support different SSB periodicity, including R19 UE supporting DL CE, R19 UE not supporting DL CE and pre-R19 UE, wherein a new cell barring field is introduced for R19 UE supporting DL CE, and both  R19 UE and pre-R19 UE use the existing NTN bar bit.

Proposal 4: A new barring field cellBarredNTN-DLCE in SIB1 is assigned one bit and enumerated by {barred, notBarred}.

Observation 4: The scope of DL-CE is greater than that of extending SSB periodicity. In order to clarify the enhancement aspect, it is recommended to focus on DL-CE as "UEs supporting extending SSB periodicity".

Proposal 5: RAN2 refers DL-CE to “UEs supporting extended SSB periodicity”.

2.3 DTX/DRX

In previous RAN plenary meeting, the study on NES DTX/DRX pattern applied to DL-CE has been excluded.

Observation 5: since the Rel-18 NES DTX/DRX pattern had been excluded in last RAN plenary, there is no way to combine the multiple NES DTX/DRX pattern (for example, in beam/footprint level) and current SMTC/SSB pattern in cell/carrier level.

Proposal 6: RAN2 confirms not to discuss DTX/DRX enhancement.

- Conclusion

Observation 1: Current restriction on MO configuration chooses at most one measurement object with the same ssbFrequency which is not sufficient for multiple SSB periodicity among neighbour cells in the same frequency layer. Hence, the SMTC configuration needs to be modified. For example, adding the SSB periodicity parameter to the existing item SSB-MTC4-r17.

- Proposal 1: For supporting different SSB periodicity among neighbour cells in the same frequency layer, the SMTC configuration needs to be modified to allow multiple periodicity configuration, e.g. adding SSB periodicity to the existing item SSB-MTC4-r17.

Observation2: It will be facilitate the accurate SMTC configuration by gNB for  a given UE, based on the  assistance information reported from the UE, to avoid unnecessary measurement performed by the UE.

Proposal 2: RAN2 could consider requiring a UE in connected mode to report assistance information, e.g., beam/footprint/location/area information,  to gNB through UAI procedure.

- Proposal 3: It is proposed to add  assistance information corresponding to measurement, e.g., area information in SMTC configuration, e.g, item SSB-MTC4-r17, for SMTC configuration selection.

Observation 3:  The current SSB periodicity configuration supports 160ms, and the network side in the same cell can switch SSB periodicity configuration between 5ms and 160ms. In order to simplify the signaling process, the same cell barring mechanism is used for all kinds of UEs that support different SSB periodicity, including R19 UE supporting DL CE, R19 UE not supporting DL CE and pre-R19 UE, wherein a new cell barring field is introduced for R19 UE supporting DL CE, and both  R19 UE and pre-R19 UE use the existing NTN bar bit.

Proposal 4: A new barring field cellBarredNTN-DLCE in SIB1 is assigned one bit and enumerated by {barred, notBarred}.

Observation 4: The scope of DL-CE is greater than that of extending SSB periodicity. In order to clarify the enhancement aspect, it is recommended to focus on DL-CE as "UEs supporting extending SSB periodicity".

Proposal 5: RAN2 refers DL-CE to “UEs supporting extended SSB periodicity”.

Observation 5: since the Rel-18 NES DTX/DRX pattern had been excluded in last RAN plenary, there is no way to combine the multiple NES DTX/DRX pattern (for example, in beam/footprint level) and current SMTC/SSB pattern in cell/carrier level.

Proposal 6: RAN2 confirms not to discuss DTX/DRX enhancement.

- References

- Chairman's Notes RAN2#129

- Chairman's Notes RAN2#128

- Chairman's Notes RAN2#127bis