3GPP TSG-RAN WG2 Meeting #129bis	R2-2502741

Wuhan, China, 7th April – 11th April, 2025

Agenda Item		:	8.8.4

Source		:	LG Electronics Inc.

Title		:	Discussion on support for broadcast service in NTN

Document for		:	Discussion and Decision

1	Introduction

In last meetings, RAN2 agreed following statement:

RAN2#127 Agreements: - For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature should not establish MRB(s) for the MBS session associated to the intended area when it is outside the intended area (capture this in Stage 2) - For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature may initiate the broadcast MRB establishment procedure when UE is inside the intended area; the UE may initiate the broadcast MRB release procedure when UE leaves the intended area (capture this in stage 3) RAN2#128 Agreements: - A signalled intended service area for a MBS BC service may include geographic areas across the current serving cell and overlapping neighbor cell(s). - RAN2 understands that the geographic area information for the intended service areas can be semi-static and not cause frequent updates. RAN2#129 Agreements: - In the new SIB, explicit network-indicated area ID is used to label an intended service area in the list. - It shall be possible to signal multiple service area IDs to one MBS service (we insert a list of service area IDs in MCCH). - Introduce “warning area coordinates” in ETWS Primary Notification (SIB6) and in ETWS Secondary Notification (SIB7). FFS on the signaling details for “warning area coordinates” (SIB6 is not segmented)

In this contribution, we provide our views on support for broadcast service in NTN.

2	Discussion

2.1	MCCH monitoring relaxation

It was agreed that the new SIB provides intended service area (ISA) information and its ID to UE, and the association between ISA ID and MBS session ID (TMGI) is present in MCCH. Some companies believe that it would be a power efficient to allow the UE not to monitor MCCH based on ISA information in the new SIB. However, there is a problem of skipping MCCH acquisition when the UE is not located within any ISA. Because the UE could discover the interested MBS service and its service area only through MCCH, if the UE receives an MCCH change notification, the UE needs to monitor MCCH even if it is not located within any ISA. Therefore, the UE needs to monitor MCCH regardless of whether it is within the new SIB based ISA.

Observation 1	The UE could discover the interested MBS service and its service area only through MCCH.

Proposal 1	The UE needs to monitor MCCH regardless of whether it is within the new SIB based ISA.

2.2	MBS service continuity in NTN

Combining the information in the Service Announcement and SIB21, the UE can know the frequencies for each MBS session. Furthermore, it was agreed that it is possible to signal ISA ID to MBS session. Therefore, the UE can determine whether it may prioritize the frequencies associated with its ISA. That is, the legacy mechanism and the ISA in new SIB are already sufficient for MBS service continuity in NTN.

Proposal 2	There is no need to introduce additional enhancements related to MBS service continuity to support operations and benefits in the legacy specification.

2.3	MII procedure in NTN

RAN2 did not conclude whether the enhancement is needed or not to support MII procedure initiation upon entering/leaving the intended service area. One company thinks that the UE in RRC_CONNECTED does not need to initiate the MII procedure upon leaving the intended service area. However, it is worth that the UE indicates absence of interest in any of MBS broadcast services provided in serving cell. The network may release the MBS broadcast service reception capability of the UE by using this information. As a result, an MBS-capable UE in RRC_CONNECTED may need to initiate the MII procedure initiation upon entering or leaving the intended service area.

On the other hand, most of the companies commented in RAN2#127bis thinks that the MII procedure may be initiated by RRC_CONNECTED UE in any time. However, in our understanding, it is a misunderstanding of the procedural text. According to the TS 38.331 section 5.9.4.2, several cases are specified when UE may initiate the MII procedure. Although UE behaviour is specified as “may”, it does not mean that the MBS-capable UE in RRC_CONNECTED could initiate the MII procedure whenever it wants. Therefore, modification of specification is needed to support the MII procedure initiation upon entering or leaving the intended service area.

Observation 2	When UE in RRC_CONNECTED is not interested in any of MBS broadcast services provided in serving cell, it is redundant to maintain the MBS broadcast service reception capability.

Proposal 3	An MBS-capable UE in RRC_CONNECTED may initiate the MII procedure upon entering or leaving the intended service area.

It can be easily implemented in the current spec by adding a simple clarification as a note. There is a condition initiating the MII procedure upon entering or leaving the broadcast service area. In our understanding, the intended service area is the broadcast service area.

Proposal 4	Add a note to section 5.9.4.2 of TS 38.331 as the intended service area can be considered as the broadcast service area.

3. Conclusion

In this contribution, following statements are proposed:

Observation 1	The UE could discover the interested MBS service and its service area only through MCCH.

Observation 2	When UE is not interested in any of MBS broadcast services provided in serving cell, it is redundant to maintain the MBS broadcast service reception capability.

Proposal 1	The UE needs to monitor MCCH regardless of whether it is within the new SIB based ISA.

Proposal 2	There is no need to introduce additional enhancements related to MBS service continuity to support operations and benefits in the legacy specification.

Proposal 3	An MBS-capable UE in RRC_CONNECTED may initiate the MII procedure upon entering or leaving the intended service area.

Proposal 4	Add a note to section 5.9.4.2 of TS 38.331 as the intended service area can be considered as the broadcast service area.

4. Reference

[1] Report of 3GPP TSG RAN WG2 meeting #129, Athens, Greece.