3GPP TSG-RAN WG2 Meeting #129bis	R2-2502748

Wuhan, China, Apr. 7th – 11th, 2025

# Agenda item:	8.6.4

# Source:	ETRI

# Title:	Remaining issues on conditional LTM

# Document for:	Discussion, Decision

- Introduction

In RAN#129 meeting, agreements on conditional LTM design have been made as the followings [1]:

Agreements on Conditional LTM - From a given source cell, network configures either L1 execution condition or L3 execution condition for a CLTM candidate cell. - The UE starts execution condition evaluation once it receives CLTM configuration including C-LTM execution condition. - For conditional LTM based on L1 measurements (LTM3/5-like event), the evaluation is per candidate beam. For conditional LTM based on L3 measurements (condA3/condA5 events), the evaluation is per candidate cell (i.e., with consolidation of multiple beams of the candidate cell). - When the C-LTM execution condition is satisfied with a candidate beam (any beam from the candidate RS set) / cell, UE performs RACH-less C-LTM (with that triggered beam for L1 based C-LTM case) if UE has a valid TA for the associated candidate cell. Otherwise, UE performs RACH-based C-LTM. - For L1 based C-LTM (assuming single beam fulfils C-LTM execution condition), beam selection for RACH-less LTM with CG is based on the beam meets C-LTM execution condition. - For L1 based C-LTM events, the C-LTM execution is triggered when at least one beam fulfills the C-LTM event condition. When multiple candidate beams satisfy the C-LTM condition, it is up to UE implementation to select a beam and perform C-LTM. - Do not support DG-based first UL transmission for RACH-less conditional LTM. - For RACH-based conditional LTM procedure, CFRA can be supported when CFRA resource is included in candidate cell configurations, otherwise CBRA is performed. - For intra-CU CLTM, it is up to NW implementation to ensure that only the candidate cells belonging to the same CU as the current serving cell are provided with the execution conditions. - Support the CLTM fast failure recovery, which reuses R18 LTM failure recovery mechanism (i.e. based on CBRA).

In this paper, we discuss remaining issues regarding conditional intra-CU LTM.

- Discussion

  - Evaluation of CLTM execution conditions

According to the running RRC CR [2], once CLTM switch conditions are fulfilled, the UE shall stop evaluating the CLTM switch conditions. It shall then evaluate the LTM cell switch conditions based on the cltm-ExecutionConditions included in the LTM-Candidate IE for the selected LTM candidate configuration.

1> if this procedure is triggered due to fulfilment of CLTM cell switch conditions: 2> if more than one LTM candidate configuration has triggered this procedure: 3> select one of the LTM candidate configurations as the selected cell for the LTM cell switch execution; Editor’s Note: FFS how to handle the case where more than one LTM candidate cell fulfil the LTM cell switch execution. 1> stop the LTM conditions evaluation (based on L1 and/or L3 measurements), if any, for all the LTM candidate configurations associated with the cell group for which the LTM cell switch procedure is triggered; 1> if LTM conditions evaluation was done based on L1 measurements: 2> inform lower layers to stop the LTM cell switch conditions evaluation or all LTM candidate configurations; 1> if the LTM-Candidate IE for the selected LTM candidate configuration includes the field cltm-ExecutionConditions: 2> if the field l3-Conditions is included within cltm-ExecutionConditions: 3> perform the LTM cell switch conditions evaluation based on L3 measurements as specified in 5.3.5.18.x according to the received cltm-ExecutionConditions; 2> else if the field l1-Conditions is included within cltm-ExecutionConditions: 3> inform lower layers to initiate the LTM cell switch conditions evaluation based on L1 measurements according to the received field cltm-ExecutionConditions. 1> if the LTM cell switch is triggered on the MCG; or

In this context, the cltm-ExecutionConditions in the LTM-Candidate IE specify the execution conditions to be evaluated after the selected LTM candidate cell becomes the new serving cell. However, since the selected LTM candidate cell remains a target cell—not a serving cell—until the current CLTM cell switch is successfully completed, the UE should not initiate the evaluation of LTM cell switch conditions based on the received cltm-ExecutionConditions before the CLTM cell switch is successfully completed.

Additionally, while executing subsequent CPAC procedures [3], the UE is not required to continue evaluating the execution conditions of other candidate PSCells or PCells. To maintain consistency with SCPAC operation, it is preferable to disallow the UE from evaluating the execution conditions of other CLTM candidates until the CLTM cell switch is completed.

10.20 Subsequent Conditional PSCell Addition or Change in TS 37.340 - The UE autonomously releases the subsequent CPAC configuration upon RRC re-establishment and upon RRC release. - While executing subsequent CPAC, the UE is not required to continue evaluating the execution condition of other candidate PSCell(s) or PCell(s). - The UE is not required to continue evaluating the execution conditions of other subsequent CPAC candidate PSCell(s) when PSCell change/addition or PCell change is triggered.

Proposal 1: while executing the CLTM cell switch, the UE is not required to initiate or continue the evaluation of execution condition of other candidate beam(s)/cell(s).

  - CLTM TAT expiration

At the last meeting, RAN2 made significant progress regarding the operation of CLTM TAT (i.e., ltm-Candidate-TimeAlignmentTimer) as follow:

- CLTM TAT is started/re-started when UE receives TA value for the candidate cell sent by the MAC CE from current serving cell. - If the CLTM TAT associated with one candidate cell is running, UE considers the TA for the candidate cell is valid; if CLTM TAT associated with one candidate cell expires, UE considers the TA for the candidate cell is invalid. - If UE has valid TA value at the time CLTM execution, UE performs RACH-less CLTM, otherwise UE performs RACH base CLTM. - The CLTM TAT is per candidate configuration.

However, the expiration of the CLTM TAT was not discussed. Based on the current agreements, the PDCCH-ordered early TA acquisition procedure is initiated by the source cell based on the measurement reports received from the UE. The UE then performs a RACH procedure with the candidate cell indicated in the PDCCH and receives a TA for that candidate cell from the source cell. Upon receiving the TA, the UE starts or restarts CLTM TAT to maintain the validity of the candidate cell's TA. If the CLTM TAT is still running when the CLTM cell switch is triggered, the UE considers the TA valid and performs a RACH-less CLTM to the candidate cell. Conversely, if the CLTM TAT expires before the CLTM cell switch is triggered, the UE considers the TA invalid. In this case, if the CLTM cell switch is triggered, the UE performs a RACH-based CLTM instead.

A question arises when the CLTM TAT expires but the CLTM cells switch is not triggered: What action should the UE do in this situation? Instead of abandoning the RACH-less CLTM approach, it may be preferable for the UE to acquire a new TA in preparation of a potential future CLTM switch. However, the method for re-triggering the PDCCH-ordered early TA acquisition procedure has not yet been discussed. As previously noted, this procedure is initiated by the source cell. In this scenario, however, the source cell cannot trigger it because it is unaware of whether the CTLM TAT for a certain candidate cell has expired.

Observation 1: Since TA validity is maintained by the UE, the source cell is unaware of whether the CLTM TAT has expired.

One potential solution is for the UE to notify the source cell when the CLTM TAT expires. This would allow the source cell to decide whether to re-initiate the PDCCH-ordered early TA acquisition procedure, based on both measurement reports and TA validity. While this notification could be sent via an RRC message, defining a new MAC CE would be more suitable, as the CLTM TAT is managed at the MAC layer.

Such a notification mechanism could also prevent the unnecessary allocation of radio resources for configured-grant (CG)-based initial uplink transmission in RACH-less CLTM procedures. Due to the unpredictable timing of CLTM cell switches—for both the serving cell and multiple candidate cells—radio resources must be reserved in advance for potential RACH-less CLTM. However, most of these resources are likely to remain unused. A RACH-less CLTM can only proceed if the UE has a valid TA for the candidate cell. If not, the candidate cell does not need to allocate resources for CG-based RACH-less CLTM. Thus, information about the UE's TA validity status is helpful for determining whether each candidate cell should reserve or release radio resources.

Observation 2: the Information about the UE’s TA validity is helpful for determining which candidate cell should prepare or release radio resources for CG-based RACH-less CLTM.

Proposal 2: if the CLTM TAT expires, the UE shall notify the source cell of the expiration via a new MAC CE.

Proposal 3: if the early TA for a candidate cell becomes invalid, the source cell may re-trigger the PDCCH-ordered early RACH procedure based on the measurement reports from the UE.

- Conclusions

In this paper, we discuss CLTM related issue. Following are the observations and proposals made in this paper:

Proposal 1: while executing the CLTM cell switch, the UE is not required to initiate or continue the evaluation of execution condition of other candidate beam(s)/cell(s).

Observation 1: Since TA validity is maintained by the UE, the source cell is unaware of whether the CLTM TAT has expired.

Observation 2: the Information about the UE’s TA validity is helpful in determining which candidate cell should prepare or release radio resources for CG-based RACH-less CLTM.

Proposal 2: if the CLTM TAT expires, the UE shall notify the source cell of the expiration via a new MAC CE.

Proposal 3: if the early TA for a candidate cell becomes invalid, the source cell may re-trigger the PDCCH-ordered early RACH procedure based on the measurement reports from the UE.

- References

- Chair notes RAN2#129

- Running RRC CR for inter-CU and conditional LTM

- TS 47.340, Multi-connectivity Stage 2