3GPP TSG-RAN WG2 Meeting #129bis	R2-2502752

Wuhan, China, 07 – 11 April 2025

Agenda item:	8.1.4

Source:	T-Mobile USA, Nokia

Title:	On UE request of data collection configuration

WID/SID:	FS_NR_AIML_Air-Ph2 - Release 19

Document for:	Discussion and Decision

# 1	Introduction

This contribution discusses the FFS issue related to RAN2#129 agreement on data collection for UE-side model only for the beam management use case:

“For data collection configuration UE-side model training, the UE can send a request for data collection (e.g. start/stop).  FFS whether a suggested data collection configuration/associated IDs (if specified)/parameters can be provided to the network”.

# 2	Discussion

In this paper we would like to discuss whether or not it should be possible to provide suggested data collection parameters such as associated IDs in the request for data collection configuration.

During RAN2#129, the main reason provided to support the suggested data collection parameters within the request for data collection configuration was that the UE knows which measurements are needed to train a UE-side model. Therefore, it would be reasonable to assume that the UE only requests data collection to train a UE-side model when its current radio conditions and other parameters are appropriate, e.g., location in the cell, available or visible beams, bandwidth part, etc, to collect the missing data. The network is also aware of these parameters and knows well which types of beam predictions, possibly associated with specific associated ID(s) are supported in the serving cell. Therefore, the network can know without additional information what are the reasonable measurements that can be configured for a particular UE at the moment when the UE sends the request.

Observation 1: The network can know without suggested information from UE, which measurements are reasonable to configure for the UE based on the radio and other parameters of the UE.

UEs would send their requests for data collection independently, which would most probably result in multiple UEs sending requests to collect measurements based on the same or similar information. These requests from multiple UEs could also help the network to identify the measurements that are important to be configured in the UEs. On the other hand, there can be data collection needed for training NW-side model(s). It is quite reasonable to assume that the gNB accumulates the UE-side requests and considers its own data collection needs and may create measurement configurations that are needed or useful for both types of data collection, UE-side and NW-side, based on the accumulated information. In this way the network can enhance the efficiency of the data collection both for UE and NW-side models by reducing the number of measurements to be performed and data to be collected.

Observation 2: The network is in the best position to decide which measurement configurations are to be used, based on the gathered historic data from different/many UEs.

In a single cell there can be multiple UEs with similar parameters that can be configured for data collection beyond the UEs that sent request for data collection. When the NW selects the UE(s) to be configured for some measurements for data collection, the network can consider a number of aspects, such as UE radio conditions, UE capabilities, UE load, etc., beyond whether the UE sent a request for data collection.

Observation 3: The network is in the best position to select the UEs to be configured for data collection.

Based on the observations above we think that sending the request for data collection itself provides enough information for the network to configure the appropriate measurements for UE-side data collection for beam management, and the benefits of the UE providing additional information about the requested data collection parameters in the request sent for data collection are limited.

Proposal 1: No additional information or suggestions from UE is needed in the UE request for UE-side data collection for beam management.

# 3	Conclusion

This document has made the following observations and proposal:

Observation 1: The network can know without suggested information from UE, which measurements are reasonable to configure for the UE based on the radio and other parameters of the UE.

Observation 2: The network is in the best position to decide which measurement configurations are to be used, based on the gathered historic data from different/many UEs.

Observation 3: The network is the best position to select the UEs to be configured for data collection.

Proposal 1: No additional information is needed in the UE request for UE-side data collection for beam management.