3GPP TSG-RAN WG2 Meeting #129bis      											                     R2-2502771

Wuhan, China, April 7th – 11th, 2025                                                        Revision of R2-2501164

Agenda item:	8.9.3

Source:	MediaTek Inc.

Title:	Discussion on CB-Msg3 procedure

Document for:	Discussion and Decision

# Introduction

Rel-19 IoT NTN work item was revised in RAN#105 [1] and one of the objectives is to enhance EDT procedure:

- Support of Capacity enhancements for uplink

  - Study and specify, if beneficial the following enhancements to reduce the necessary uplink and downlink signaling to complete an Early Data Transmission (EDT) transaction [RAN2]:

    - Msg3 transmission without msg1/ Random Access Response (RAR)

    - Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete

In this contribution, we continue discussing the CB-Msg3 on details of DSA mechanism, Msg4 monitor window, RNTI, details of Msg4 and fallback.

# Discussion

TA validation

When the CB-Msg3 configuration was discussed in the posting meeting discussion [Post129][307][R19 IoT NTN] CB-msg3/CB-msg4, it is assumed that the NB-IoT 15kHz SCS and eMTC CE mode B is not applicable. We would like to confirm the assumption in RAN2.

As indicated in the R4 LS[2] to RAN2, UE can meet UE UL transmit timing requirements. But for NB-IoT with 15kHz SCS, the timing error may be larger than half CP.

RAN4’s feedback: For Msg3 transmission without Msg1/Msg2, UE can meet UE UL transmit timing requirements defined for IoT NTN in TS 36.133[attached in Annex] from UE RRM requirement definition point of view. Note 1: RAN4 assumed that NTA_Ref is not configured by the network for MSG3 transmission without Msg1/Msg2. RAN4 will follow the conclusion of the other working groups in the future, if any. Note 2: For NB-IoT with 15kHz SCS, the timing error may be larger than half CP.

However, the RAN1 LS[3] to RAN2 regarding the same question says a different thing. Only NB-IoT with 3.75kHz SCS and eMTC CE mode A UE can satisfy requirement. RAN1 may further discuss the cases of NB-IoT with 15kHz SCS and eMTC CE mode B.

RAN1’s response: From RAN1 perspective, assuming Note 1 and Note 2 are satisfied, an RRC Idle UE with a pre-compensated TA (i.e., the one used for Msg1 transmission during random access for IoT NTN) can satisfy requirement for Msg3 transmission/reception without Msg1/Msg2 at least in the following cases: - NB-IoT with 3.75kHz SCS. - eMTC CE mode A. Note 1: RAN1 assumes NTA=0 for Msg3 transmission without Msg1/Msg2 Note 2: RAN1 assumes that the RAN4 UL synchronization requirements specified in Table 7.20A.2-1 and Table 7.24A.2-1 of TS 36.133 apply to the Msg3 transmission without Msg1/Msg2 in NB-IoT over NTN and eMTC over NTN, respectively. Note 3: RAN1 may further discuss the cases of NB-IoT with 15kHz SCS and eMTC CE mode B

Since the RAN4 and RAN1 have different views on it, RAN2 can follow RAN1’s view or RAN4’s view, or RAN2 can send LS to RAN1 asking for progress.

Proposal 1: RAN2 to discuss the following options regarding the TA validation.

Option 1: NB-IoT with 15kHz SCS and eMTC CE mode B are precluded from the scope of CB-Msg3.

Option 2: All type of NB-IoT and eMTC UE are supported in CB-Msg3

Option 3: RAN2 send LS to RAN1 asking for progress.

Details of DSA mechanism

If DSA is configured, UE will transmit multiple replicas to reduce the possibility of collision. It can be extra burden if the IoT device is sensitive to power consumption. It might be not so beneficial comparing the legacy 4-step RA. As a result, CB-Msg3 may not be used. Therefore, we believe the DSA mechanism should be an optional capability for UE. If DSA is not supported by UE, SA is used. Since UE can initiate the CB-Msg3 transmission based the configurations in SIB, the network does not need to know the UE’s capability on DSA, A UE capability parameter for this is not needed.

The DSA configuration contains multiple NPUSCH occasions.  A SA only UE can simply select the next NPUSCH occasion among the configurations. The Msg4 monitor window and RNTI design should also take this into account if optional DSA to UE is agreed.

Proposal 2: DSA transmission is an optional feature without UE capability signaling for UE supporting CB-Msg3.

Proposal 3: If UE only supports SA, the DSA configuration can be reused.

The CB-msg3 for both SA and DSA has been agreed to be CE level specific.

The configured maximum TBS for CB-Msg3 has been agreed during the RAN2#128. Whether the maximum TBS is CE level specific is yet to be determined. The maximum TBS is highly related to the radio resource. Moreover, the radio resource is highly related to the repetition number which varies by the CE level. Clearly, the maximum TBS shall be specific to CE level.

RAN2#128 Agreement: - The UE triggers CB-Msg3 only if the size of pending UL data is less than the configured maximum TBS (FFS if the maximum TBS is same or different for different CE levels)

Proposal 4: The maximum TBS is specific to CE level.

Regarding the DSA mechanism for CB-Msg3, it has been agreed that a configurable time window for selecting DSA occasion will be introduced. Within this time window, the UE will randomly select PUSCH resource in both time and frequency domains.

RAN2#128 Agreement: - There will be a configurable time window for DSA CB-Msg3 occasion selection. FFS on the details (e.g. when the time window starts) - Within the configured time window, the UE shall select randomly different time domain occasions for transmitting different replicas. And for each time domain occasion, the UE shall select randomly a frequency domain resource.

First, the UE can select the nearest PUSCH occasion to transmit the first replica after the CB-Msg3 has been initiated. Once the UE has completed the transmission and there are more replicas to transmit as configured, the UE can start the time window.

The DSA transmission window length will be affected by the length of PUSCH resources. Hence, this configuration parameter should also be specific to CE levels.

Proposal 5: The DSA transmission window length for CB-Msg3 is specific to CE levels.

Msg4 monitor window

The Msg4 monitor window was discussed in the post meeting discussion [Post129][307][R19 IoT NTN] CB-msg3/CB-msg4. The DL Msg4s scheduled by the network can have various repetition numbers based on radio condition of each UE. It should basically follow the CE level. Therefore, the length of Msg4 monitor window in configuration should also be specific to CE level.

Proposal 6: The Msg4 response window configuration is specific to CE level.

Multiple Msg4 monitor window vs single Msg4 monitor window was discussed. We prefer multiple Msg4 monitor window, which starts after the transmission of each replica. The biggest pain point is that the single window starts after the DSA transmission window, which brings a transmission delay. It also rules out the possibility that UE receives the Msg4 within the transmission window and thus the rest of the replicas can be skipped. Window length in multiple window design is mostly likely shorter than single window, it is more power efficient. Additionally, if optional DSA transmission as UE capability is agreed, the DSA configuration will be used as SA configuration. In this case, the monitor window starts after transmission window cause too much latency.

For the concern about the conflict of network response and UE transmission, network can address it through implementation (e.g. not sending response in PUSCH occasions).

On how to determine the starting point of Msg4 monitoring window, we think that each window could start at the end of transmission of each replica plus UE-eNB RTT (FFS whether additional processing time is needed).

Proposal 7: Multiple Msg4 monitoring windows is supported. Each window starts at the end of transmission of each replica plus UE-eNB RTT (FFS whether additional processing time is needed).

CB-Msg3 RNTI

A new RNTI is needed for monitoring PDCCH for Msg4. RAN2 has agreed that the RNTI is derived based on the resources associated with the PUSCH occasion used for the contention based Msg3 EDT transmission.

RAN2#127bis Agreements:

The RNTI used at least to schedule Msg4 transmission is derived based on the resource associated to the PUSCH occasion used for contention based Msg3 EDT transmission (FFS on the details. FFS how this is impacted by DSA)

For the network, the CB-Msg3 RNTI is used to scramble the PDCCH for the Msg4 transmission. In the legacy RA, the TC-RNTI from RAR is not only used to monitor the Msg4, but also to scramble the Msg3. Scrambling increases coding gain while reducing inter-cell interference, it was widely used in LTE. For CB-Msg3, the CB-Msg3 RNTI should also be used to scramble the contention-based Msg3. But this impact the RAN1 Spec. We should send LS to RAN1 for confirmation.

Proposal 8: Send LS to RAN1 to confirm that CB-Msg3 RNTI is used to scramble the contention-based Msg3.

Single RNTI vs multiple RNTI was discussed. If single RNTI is adopted, the Msg4s corresponding to the DSA transmission window will all be addressed with that single RNTI. Then the UE will have to decode every Msg4 corresponding to the DSA transmission window. The false alarm rate is too high. The multiple RNTIs will reduce the false alarm. For the concern that the UE will need to monitor multiple RNTIs simultaneously, we think the additional complexity is worth reducing the false alarm.

Proposal 9: Multiple Msg4 monitoring windows. Each RNTI is derived on the resource of each replica.

RAN2#129 working assumption 1. One CB-MSG4 can target multiple UEs simultaneously (FFS on the details)

As the working assumption, one CB-MSG4 can target multiple UEs simultaneously.

If there is no DL response to the UL EDT transmission, there will be only a contention resolution MAC CE in the Msg4. So that there will be enough room left in a MAC PDU for other information. The Msg4s responding to the Msg3s from the same time domains but the different frequency domains can be included in the same MAC PDU. The signaling overhead efficiency will be improved accordingly.

Proposal 10: RAN2 confirm the working assumption that one CB-Msg4 can target multiple UEs simultaneously.

There can be 3 options for how the UEs in time domain and/or frequency domain were multiplexed:

Option 1: The UEs using the different time but the same frequency can be multiplexed.

Option 2: The UEs using the same time but the different frequency can be multiplexed.

Option 3: The UEs using the different time and different frequency can be multiplexed.

Firstly, option 3 can be ruled out, because the number of multiplexed UE can be very large. The UE will have to decode every Msg4 to find the responses for itself. For option 1, the UE that transmitted the data early will have to wait for other UEs to finish the data transmission to complete the multiplexing, which is not desirable. Therefore, option 2 is the best choice.

The CB-Msg3 RNTI calculation should reflect this design.

Therefore, the time domain information (e.g., subframe index, SFN) will be part of formulation of CB-Msg3 RNTI. The frequency domain information (e.g., subcarrier index) will not.

Proposal 11a: The Msg4s responding to the Msg3s from the same time domains, but the different frequency domains can be multiplexed in the same MAC PDU.

Proposal 11b: The time domain information of CB-Msg3 UL resource (e.g., subframe index, SFN) is taken into account for CB-Msg3 RNTI calculation. The frequency domain information (e.g., subcarrier index) is not.

Multi-carrier is supported in NB-IoT since Release 13. This feature becomes even more critical in NTN scenario where capacity may be a significant challenge due to extensive cell coverage. For CB-Msg3, multi-carrier support should also be implemented.

Proposal 12a: For NB-IoT, CB-Msg3 transmission on non-anchor carrier shall be supported.

Additionally, since the carrier-level UL/DL resource can be provided in NB-IoT, UEs can send UL transmissions on different UL carriers and then monitor DL responses on the same DL carrier. To differentiate Msg4s responding to different UL carriers, the carrier index is needed in the formulation of CB-Msg3 RNTI.

Proposal 12b: For NB-IoT, the carrier index of CB-Msg3 UL resource is taken into account for CB-Msg3 RNTI calculation.

Details of Msg4

Like legacy EDT, for some CB-Msg3 transmission, especially CP solution, the RRC message in Msg4 is not mandatory. In this case, a Msg4 with only MAC CE(s) should be allowed. It is referred to as L2 ACK.

Proposal 13: A Msg4 for CB-Msg3 with only MAC CE(s) is allowed when RRC message is not mandatory.

Since the contention resolution is essential and cannot be done via DCI due to the size limitation, using the L1 ACK as the Msg4 for CB-Msg3 is not appropriate.

Proposal 14: Do not support the use of L1 ACK as the Msg4 for the CB-Msg3.

In the legacy 4-step RA, the HARQ ACK/NACK is needed for the DL Msg4.The resource for the HARQ ACK/NACK is assigned in the DCI which scheduled the Msg4. RAN2 needs to discuss whether the HARQ ACK/NACK is also applicable to the CB-Msg4. Especially, for NB-IoT, when the multiple UEs is multiplexed in one Msg4, how a number of UEs send the HARQ ACK/NACK by a single resource in the DCI needs to be addressed. If only the UEs in frequency domain are multiplexed, the same subcarrier index used to transmit the Msg3 can be used to transmit the HARQ ACK/NACK.

Proposal 15: RAN2 to discuss whether and how the HARQ ACK/NACK is applied to CB-Msg4.

Proposal 16: For NB-IoT, if the Msg4 multiplex is used, the same subcarrier index used to transmit the Msg3 can be used to transmit the HARQ ACK/NACK of Msg4.

Fallback to RRC Connected mode

Same as legacy EDT, in CB-Msg3, network should be able to steer UE to RRC connected mode by a RRC signalling in Msg4 when there will be sequential DL transmission. This is referred as fallback procedure.

Proposal 17: CB-Msg3 fallback to RRC connected mode is supported.

Unlike EDT, the RAR that contains the Timing Advance Command is skipped in CB-Msg3. To maintain better UL timing advance in RRC connected mode, a Timing Advance Command MAC CE can be included in the fallback Msg4.

There is no RAR to start the time alignment timer, it is acceptable if the UE does not move to RRC connected mode. The UE must start the time alignment timer when receiving Msg4 which indicates fallback.

Proposal 18: A TAC MAC CE can be included in the Msg4 when fallback to RRC Connected mode.

Proposal 19: UE starts the time alignment timer when receiving Msg4 which indicates fallback to RRC Connected mode.

A new RNTI based on PUSCH resource is used to monitor Msg4. This RNTI cannot be used as the C-RNTI in RRC connected mode as this RNTI can be reused by another UE which selects the same PUSCH resource. Therefore, a new C-RNTI should be assigned by network when fallback. Normally it can be carried by RRC message. But if the contention resolution MAC CE and RRC message can be sent in different MAC PDU (i.e., early contention resolution is supported for CB-Msg3), the C-RNTI should be carried by MAC CE together with contention resolution MAC CE.

Proposal 20: New C-RNTI should be included in the Msg4 for the CB-Msg3 when fallback to RRC Connected mode.

Fallback to 4-step RA

There is another kind of fallback may be supported. If the CB-Msg3 cannot succeed, UE can try the legacy 4-step RA. Or when the network find that the Msg3 cannot be decoded due to the bad timing alignment, network can indicate the correspond UEs to stop current procedure, and initiate legacy 4-step RA instead. A new MAC CE in Msg4 can be introduced for this.

Proposal 21: UE can initiate the legacy 4-step RA when the CB-Msg3 procedure fails.

Proposal 22: New fallback MAC CE in Msg4 is introduced to indicate the corresponding UEs to stop current CB-Msg3 and initiate legacy 4-step RA.

In this case, a backoff time can also be indicated to UE to avoid collision with other UE in the following 4-step RA. A new backoff MAC CE in Msg4 can be introduced for this.

Proposal 23: A new backoff MAC CE in Msg4 can be introduced when UE fallback to 4-step RA as indicated by the network.

When the UE start the legacy 4-step RA, it can be discussed whether the current CE level in CB-Msg3 should be used. The current CE level used in CB-Msg3 may be increased after a failed attempt of CB-Msg3. When UE fallback and start the legacy 4-step RA, does the UE need to start from the beginning? RAN2 can further discuss.

Proposal 24: RAN2 to discuss whether the CE level for 4-step RA starts from the beginning or from the CE level used in CB-Msg3.

Re-attempt

It is always possible that the network cannot decode the Msg3 due to the collision with or without the DSA mechanism. Increasing CE level does not help to reduce the collision. Therefore, the re-attempt mechanism should be supported. The UE can re-attempt a configured times in the current CE level. If the maximum re-attempt number of current CE level has been reached, UE can be in the next CE level.

To support the re-attempt mechanism, a max re-attempt number per CE level can be introduced in the configuration. UE can re-attempt in the same CE level until the max re-attempt number has been reached. A backoff MAC CE can be included in the Msg4 for the next re-attempt.

Proposal 25: Re-attempt in the same CE level due to contention resolution failure is supported.

Proposal 26: Parameter for maximum re-attempt number per CE level is introduced. UE can re-attempt in the same CE level until the max re-attempt number has been reached.

Proposal 27: Backoff MAC CE in Msg4 is supported for re-attempt.

Power ramping is used in the legacy 4-step RA. If the re-attempt is supported in CB-Msg3, the power ramping can also be considered. For the NTN GEO scenario, the UEs are actually always using the maximum transmission power to meet the target power. There is no room for the power ramping. But it is not same case for the NTN LEO scenario. And the high power UE for NTN with power class 2 and even power class 1 is now being discussed in RAN4. There will be more room to increase the transmission power. Therefore, the power ramping can be optionally supported for CB-Msg3.

Proposal 28: Power ramping parameters are optionally configured by the network.

# Conclusion

TA validation

Proposal 1: RAN discusses the following options regarding the TA validation.

Option 1: NB-IoT with 15kHz SCS and eMTC CE mode B are precluded from the scope of CB-Msg3.

Option 2: All type of NB-IoT and eMTC UE are supported in CB-Msg3

Option 3: RAN2 send LS to RAN1 asking for progress.

Details of DSA mechanism

Proposal 2: DSA transmission is an optional feature without UE capability signaling for UE supporting CB-Msg3.

Proposal 3: If UE only supports SA, the DSA configuration can be reused.

Proposal 4: The maximum TBS is specific to CE level.

Proposal 5: The DSA transmission window length is specific to CE levels.

Msg4 monitor window

Proposal 6: The Msg4 response window configuration is specific to CE level.

Proposal 7: Multiple Msg4 monitoring windows is supported. Each window starts at the end of transmission of each replica plus UE-eNB RTT (FFS whether additional processing time is needed).

CB-Msg3 RNTI

Proposal 8: Send LS to RAN1 to confirm that CB-Msg3 RNTI is used to scramble the contention-based Msg3.

Proposal 9: Multiple Msg4 monitoring windows. Each RNTI is derived on the resource of each replica.

Proposal 10: RAN2 confirm the working assumption that one CB-Msg4 can target multiple UEs simultaneously.

Proposal 11a: The Msg4s responding to the Msg3s from the same time domains, but the different frequency domains can be multiplexed in the same MAC PDU.

Proposal 11b: The time domain information of CB-Msg3 UL resource (e.g., subframe index, SFN) is taken into account for CB-Msg3 RNTI calculation. The frequency domain information (e.g., subcarrier index) is not.

Proposal 12a: For NB-IoT, CB-Msg3 transmission on non-anchor carrier shall be supported.Proposal 12b: For NB-IoT, the carrier index of CB-Msg3 UL resource is taken into account for CB-MSg3 RNTI calculation.

Details of Msg4

Proposal 13: A Msg4 for CB-Msg3 with only MAC CE(s) is allowed when RRC message is not mandatory.

Proposal 14: Do not support the use of L1 ACK as the Msg4 for the CB-Msg3.

Proposal 15: RAN2 to discuss whether and how the HARQ ACK/NACK is applied to CB-Msg4.

Proposal 16: For NB-IoT, if the Msg4 multiplex is used, the same subcarrier index used to transmit the Msg3 can be used to transmit the HARQ ACK/NACK of Msg4.

Fallback to RRC Connected mode

Proposal 17: CB-Msg3 fallback to RRC connected mode is supported.

Proposal 18: A TAC MAC CE can be included in the Msg4 when fallback to RRC Connected mode.

Proposal 19: UE starts the time alignment timer when receiving Msg4 which indicates fallback to RRC Connected mode.

Proposal 20: New C-RNTI should be included in the Msg4 for the CB-Msg3 when fallback to RRC Connected mode.

Fallback to 4-Step RA

Proposal 21: UE can initiate the legacy 4-step RA when the  CB-Msg3 procedure fails.

Proposal 22: New MAC CE in Msg4 is introduced to indicate the corresponding UEs to stop current CB-Msg3 and initiate legacy 4-step RA.

Proposal 23: A new backoff MAC CE in Msg4 can be introduced when UE fallback to 4-step RA as indicated by the network.

Proposal 24: RAN2 to discuss whether the CE level for 4-step RA starts from the beginning or from the CE level used in CB-Msg3.

Re-attempt

Proposal 25: Re-attempt in the same CE level due to contention resolution failure is supported.

Proposal 26: Parameter for maximum re-attempt number per CE level is introduced. UE can re-attempt in the same CE level until the max re-attempt number has been reached.

Proposal 27: Backoff MAC CE in Msg4 is supported for re-attempt.

Proposal 28: Power ramping parameters are optionally configured by the network.

# References

[1] RP-242397, “Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3”, Rapporteur (MediaTek)

[2] R2-2407931	Reply LS to RAN2 on UL synchronization for contention based Msg3 transmission without Msg1/Msg2 (R4-2414114; contact: ZTE)	RAN4	LS in	Rel-19	IoT_NTN_Ph3-Core	To:RAN2	Cc:RAN1

[3] R2-2407920	Reply LS to RAN2 on UL synchronization for contention based Msg3 transmission without Msg1/Msg2 (R1- 2407548; contact: ZTE)	RAN1	LS in	Rel-19	IoT_NTN_Ph3-Core	To:RAN2, RAN4