3GPP TSG RAN WG2 #129bis			R2-2502782

Wuhan, China, April 7th – 11st, 2025

Source:	NTT DOCOMO, INC.

Title:	Discussion on adaptation of common signal and channel

Agenda Item:	8.5.4

Document for: 	Discussion and Decision

- Introduction

At the RAN#105 meeting, the new work item of “Enhancement of network energy savings for NR” was agreed [1]. In this contribution, we provide our views on adaptation of common signal and channel.

Agreements on OD-SSB@RAN2#129

On Paging adaptation

1.	For N, values smaller than T/32 are not supported.

2.	The maximum possible value for Ns is 8.

3.	Introduce a separate PEI configuration.

4.	Paging adaptations are configured semi-statically and updated via system information update notification.

5.	A new UE capability is added for R19 NES paging enhancement, and the new capability is included in UE-RadioPagingInfo. FFS on whether we have a common capability for all NES features.

on SSB adaptation

1.	RAN2 confirms SSB adaptation in time domain is not supported for RRC idle/inactive UEs and Rel-19 NES-capable UE’s PCell.

2.	RAN2 preference is to keep SMTC based L3 RRM framework and to introduce additional SMTC configuration according to SSB adaptation for L3 RRM measurement on SCell with SSB adaptation.

3.	For L3 measurement, RAN2 assumes the adapted SSB on neighbor cell is measured based on legacy SMTC.

on RACH adaptation

- Legacy UEs and UEs non-capable of time domain PRACH adaptation are expected to use legacy PRACH resources per legacy configuration and procedures. No barring is needed.

2a.	RAN2 starts CBRA for RACH adaptation.

2b. 	RAN2 starts 4-step RACH adaptation.

3. 	From R2 perspective, not apply time-domain RACH adaptation to RACH resources for MSG1-based SI request.

- From R2 perspective, not apply time-domain RACH adaptation to IAB RACH resources.

- 5a. 	From R2 perspective, RACH adaptation is not modelled as RA feature(s).

- 5b.	From R2 perspective, RACH partitioning with all the features, i.e. RedCap, SDT, and Slicing, and feature combinations, are supported for PRACH adaption in time domain.

- Will follow legacy mechanism regarding how to select RACH resource.

- Discussion

For NES purpose, we discuss the candidates for adaptations and applicable scenarios for each common signal/channel. First, we’d like to clarify how to obtain NES gain by adaptation over the legacy operation. One approach is to introduce new properties/parameters to reduce occasion/duration in time domain at gNB. The other approach is to introduce more efficient/dynamic mechanism to change property of common signal/channel following a change in condition/situation (e.g., when configured PRACH capacity is lacking, NW decides to provide additional PRACH resources more dynamically by less overhead signaling to satisfy the situation). Any techniques in this objective should be evaluated from these perspectives for NES operation.

Observation 1:

- For adaptation of common signal/channel in NES operation, the following two approaches should be considered.

  - Introduce new properties/parameters to reduce the gNB TX/RX occasion/duration in time domain.

  - Introduce more efficient/dynamic mechanism to change properties of common signals/channels.

2.1	SSB adaptation

Indication mechanism

On the applicable scenario for SSB adaptation, RAN1 could reach conclusion that SSB adaptation to be supported only on SCell as shown following agreements [2][3][4].

Agreement@RAN1#118 For adaptation mechanism(s) of SSB in time-domain, - For Rel-19 NES-capable UE’s PCell (Connected mode), adaptation of CD-SSB on sync raster is not supported - FFS: Adaptation for SSB that is not CD-SSB is supported (A2) - FFS: Adaptation for SSB not on sync raster is supported (A3) - For Rel-19 NES-capable UE’s SCell - Adaptation of SSB configured for the SCell is supported for the following cases - FFS: Adaptation for CD-SSB (B1) including UE impact compared to legacy operation where the SSB is configured with periodicity>20msec for SCell - Adaptation for SSB that is not CD-SSB on sync raster (B2’) - Adaptation for SSB that is not CD-SSB not on sync raster (B3’) Conclusion@RAN1#118bis There is no consensus on the support of adaptation of SSB for idle mode UEs in Rel-19 Conclusion@RAN1#119 There is no RAN1 consensus to support SSB adaptation in time domain for Rel-19 NES-capable UE’s PCell (connected mode)

While RAN1 has agreed that DCI based indication is supported for SSB adaptation, it is still up to RAN2 discussion whether to support additional MAC CE signaling to do so. We do not see any motivation to support duplicated and more resource inefficient way of MAC CE in addition to support of DCI.

Proposal 1:

- Not support MAC CE based signalling to indicate SSB adaptation in addition to DCI agreed in RAN1.

Impact on L3 measurement

There was the discussion on whether to extend the current L3 measurement framework by introducing SSB adaptation. RAN2 agreed that SMTC based L3 RRM framework was kept and additional SMTC configuration according to SSB adaptation would be introduced.

The legacy L3 measurement configuration can configure the periodicity from the range between 5ms and 160ms and the SSB indexes to be measured as measurement object, which may or may not be aligned with the actual SSB burst periodicity and SSB index completely. This means which SSB occasions to be measured are determined by a semi-static manner.

If we consider when NW determines to adapt SSB (and what property of SSB), one criterion should be that UE’s speed and channel condition in a given cell. For example, if a UE is in relatively high speed, the UE should be provided with adaptive SSB whose periodicity is relatively shorter to follow the channel fluctuation so that NW can obtain accurate measurement result from the UE in a timely manner.

In this situation, it is preferable that all indicated adaptive SSB should be L3-measured for accurate and timely evaluation of the SCell strength. If the update of measurement object requires the legacy scheme, e.g., RRC Reconfiguration signaling, there seems to be less benefit by introducing dynamic adaptation of SSB, because anyway gNB has to send RRC signaling for applying adaptive SSB TX and corresponding measurement.

On the details of how to enhance the current L3 framework according to SSB adaptation, the following two directions can be considered.

(1) Measurement configurations to be used are indicated along with SSB adaptation (via DCI)

(2) Measurement configurations to be used are associated with adapted SSB, and only after SSB adaptation, the configurations are applicable.

As discussed above, since it seems appropriate operation that measurement occasions for adapted SSB TX are aligned with the actual the SSB TX patterns, we think direction (2) would be enough. The direction (1) might cause unnecessary flexibility which causes burden on UEs to prepare for possible combinations of measurement configurations and adaptive SSB TX patterns.

On which parameters to be enhanced in measurement configurations, we think, at least, SMTC configurations to set periodicity and duration, should be enhanced because RAN1 agreed that multiple SSB burst periodicities was configured for each adaptive SSB.

Proposal 2:

- Support the unified mechanism for L3 measurement of adaptive SSB and OD-SSB, e.g.,

  - Multiple SMTC configurations can be configured within a measurement object

  - Each SMTC configuration is associated with each adaptive SSB

  - A SMTC configuration is applicable only when the associated adaptive SSB is activated.

2.2	PRACH adaptation

Applicable scenario

We believe that Rel-19 additional PRACH is to improve RACH capacity and/or RACH latency for better communication performance during NES operation (while gNB is infrequent wake-up to monitor legacy PRACH resources by configuring less ROs or longer periodicity).

The applicable case would be that NW may configure smaller number of ROs and/or longer RACH periodicity as a default for NES operation by reducing RO monitoring. Generally, NW may be able to expect the rough number of IDLE UEs, while cannot expect the probability of traffic occurrence for each of IDLE UEs. Configuring many ROs for those IDLE UEs will cause inefficient NW energy usage due to unused ROs, but if only a limited number of ROs is configured while traffic occurs for many idle UEs, a lot of PRACH transmissions will occur within a cell, that exceeds with the RO capacity. In order to address this, additional PRACH resources can be indicated to new UEs which try to access a NES cell.

Based on the above analysis, we consider applicable use-case and scenario for adaptation PRACH. The legacy specification supports several cases where additional PRACH configuration can be configured as follows:

- sPcell or scell

- Contention based RACH or contention free based RACH

- 4-step RACH or 2-step RACH

- Feature combination, e.g.,

  - Redcap / eRedCap

  - Small data transmission

  - NSAG

  - msg3 / msg1 repetition

- Other use-cases, e.g., IAB

From our understanding, at least CFRA should be out of scope of the PRACH adaptation. PRACH resources for CFRA are generally provided for RRC-connected UEs by dedicated configuration so that each UE has its own resources. We do not see any motivation to change more dynamically these resources.

Proposal 3:

- Not support PRACH adaptation in Rel-19 for CFRA.

Configuration/indication of additional PRACH

RAN1 agreed that additional PRACH resources are semi-statically provided, and DCI is used to indicate its availability. Around discussion of signaling, some companies mention that additional PRACH resources should be available only via semi-static signaling, i.e., without DCI. However, we rather believe DCI-based provision should be supported. This is because, there seems no introduction of new parameters applied to additional PRACH to generate more confined patterns, which means additional PRACH finally turn out to be almost same as legacy PRACH resources. In this sense, if additional PRACH resources are provided by semi-static signaling, we can say the legacy SIB1 can provide almost all PRACH patterns that is combination of legacy and additional. We cannot see any motivation to support what the legacy framework already supports, so additional PRACH resources should be activated only via DCI.

Proposal 4:

- For PRACH adaptation mechanism, prefer not to support additional PRACH resources available only via semi-static signalling (i.e., SIB1), i.e., additional PRACH resources are only available when DCI is indicated.

2.3	Paging adaptation

Firstly, we consider the legacy paging configurations and necessary extension for further NES gain.

UE acquires paging configuration from SIB1 and determines paging frames (PF), paging occasions (PO) and paging monitoring occasions (PMO) as follows.

- SFN for the PF is determined by:

  - (SFN + PF_offset) mod T = (T div N)*(UE_ID mod N)

- Index (i_s), indicating the index of the PO is determined by:

  - i_s = floor (UE_ID/N) mod Ns

- Configurations

  - Default or UE specific DRX cycle (T={32, 64, 128, 256 [radio frame]} or extended ={256, 512, 2014 [radio frame]} )

  - N = {T, T/2, T/4, T/8, T/16}

  - Ns: number of paging occasions for a PF ({4,2,1})

  - UE_ID: 5G-S-TMSI mod 1024 or 5G-S-TMSI mod 4096 for eDRX

For example, in case of T = 128 rf and N = T/16 (for confining PF/PO within a paging cycle as possible), there would be multiple PFs (at most 8) every 160 [ms] within 1280 [ms] for different UEs. Within a PF, the number of PO and time resource position of paging PDCCH (paging monitoring occasion) comprising each PO can be configured by NW. Although reducing the paging cycle (or UE’s DRX cycle) can result in fewer paging frames within a paging cycle, it is not the preferable method for UE energy saving. Additionally, NW may still need to wake up frequently due to the short paging cycle after all.

Agreements on adaptation@RAN2#127bis

Agreements on paging adaptation

1.	Select option-b as baseline for R19 NES paging enhancement.

2.	R2 should aim at signaling overhead minimization (e.g., Ns=8 and FFS for other larger values)

3.	Allowing legacy and R19 UEs to co-ex in the same PF/PO is possible, based on NW configuration.

4.	Legacy UE is not barred.

5.	Rel-19 UEs only monitor the PO(s) according to Rel-19 paging configuration.

There are some remaining issues which can be discussed in RAN2, we show our preference as below:

Whether the legacy paging occasion and new ones can be overlapped or not, which was raided by some companies, we do not see any necessity of specification restrictions. Although new parameters of “PDCCH monitoring occasions of PO” for rel-19 confined POs will be introduced, it can be still possible that same PDCCH occasions are pointed by both new ones and legacy ones, for example, for the case when NW decides that frequency resources for the PDCCH occasions can accommodate both new and legacy. The same logic can be applicable to PEI PDCCH MO for rel-19 and legacy.

Proposal 5:

- Clarify that PDCCH occasions of PO and PEI for Rel-19 and legacy can be same or different, which is up to NW implementation.

  - No specification impact is expected.

With respect to the increase of the number of Ns, it can be discussed whether to increase number of POs associated with one PEI (keep the legacy value of po8 or increase to po16). In our understanding, the number of POs in PEI which equals to 8 is intended to cover 2 paging frames, where each PF contains up to 4 POs. Here, for rel-19 more confined POs/PFs, if NW sets one PEI for one PF, same capacity of PO# indicated by one PEI can be achieved. We are relatively open to discuss this aspect and slightly prefer not to extend the value.

Proposal 6:

- Discuss whether or not to extend the number of POs in one PEI up to 16 for Rel-19 paging adaptation

  - Prefer not to extend the legacy value of “po-numPerPEI”

- Conclusion

In this contribution, we provided the following observations and proposals of adaptation of common signal and channel for network energy saving.

Observation 1:

- For adaptation of common signal/channel in NES operation, the following two approaches should be considered.

  - Introduce new properties/parameters to reduce the gNB TX/RX occasion/duration in time domain.

  - Introduce more efficient/dynamic mechanism to change properties of common signals/channels.

Proposal 1:

- Not support MAC CE based signalling to indicate SSB adaptation in addition to DCI agreed in RAN1.

Proposal 2:

- Support the unified mechanism for L3 measurement of adaptive SSB and OD-SSB, e.g.,

  - Multiple SMTC configurations can be configured within a measurement object

  - Each SMTC configuration is associated with each adaptive SSB

  - A SMTC configuration is applicable only when the associated adaptive SSB is activated.

Proposal 3:

- Not support PRACH adaptation in Rel-19 for CFRA.

Proposal 4:

- For PRACH adaptation mechanism, prefer not to support additional PRACH resources available only via semi-static signalling (i.e., SIB1), i.e., additional PRACH resources are only available when DCI is indicated.

Proposal 5:

- Clarify that PDCCH occasions of PO and PEI for Rel-19 and legacy can be same or different, which is up to NW implementation.

  - No specification impact is expected.

Proposal 6:

- Discuss whether or not to extend the number of POs in one PEI up to 16 for Rel-19 paging adaptation

  - Prefer not to extend the legacy value of “po-numPerPEI”

References

- RP-242354, Revised WID: Enhancements of network energy savings for NR, Ericsson

- 3GPP, RAN1#118 meeting, Chairman’s notes

- 3GPP, RAN1#118bis meeting, Chairman’s notes

- 3GPP, RAN1#119 meeting, Chairman’s notes

- 3GPP, RAN2#127bis meeting, Chairman’s notes

- 3GPP, RAN1#120 meeting, Chairman’s notes

- 3GPP, RAN2#129 meeting, Chairman’s notes