3GPP TSG-RAN WG2 Meeting #129bis	R2-2502789

Wuhan, China, 7 - 11 April, 2025

Title: 	Discussion on MRO for LTM

Source: 	Huawei, HiSilicon

Agenda item:	8.10.2.1

Document for:	Discussion and Decision

- Introduction

In this paper, we mainly analyze possible RAN2 impacts regarding MRO for LTM.

- Discussion

RAN2#129 made the following agreements:

Send an LS to RAN3 indicating scenario of beam failure recovery shortly after successful LTM (“near failure”), also say that we will not specify a UE based solution for this, but that RAN3 can consider a NW based solution if they want. RAN3 can see if/how the solution (if specified) could also be used for the case of LTM failure.

To facilitate the further discussion, we illustrate the steps during the LTM procedure to have a retro on the design of Rel-18 LTM feature, compared to those during legacy L3 handover.

Figure 1: Handover procedures for Legacy L3 HO and LTM

The division of failure cases for LTM is similar as those for handover, since the LTM is built and augmented on top of L3 handover. For handovers, the SON features including the radio link failure (RLF) report and successful handover report (SHR) provides the UE feedback information. The RLF and SHR could be augmented to also provide useful feedback information to address the LTM mobility issues.

We will discuss the possible MRO enhancement for those SON reports.

Early sync info in RLF report

Early sync is the key to achieve the goal of reducing the interruption time. If it is RACH-less, RAN2 discuss whether to include early sync attempts towards multiple LTM candidate Cell in one RA report to identify the issues of network-based TA procedure, e.g., how frequent and which is the proper triggering timing the source DU should trigger the PDCCH order to maintain the TA validity, and which of candidate cells should be involved to acquire the TA so as to have a target cell with a valid TA.

As shown in Figure 1, for possible LTM procedure-1 (RACH-less access based on network TA), the time period between the reception of the PDCCH order and the reception of HO command MAC CE can be logged at UE and reported to NW, and then NW can optimize early UL sync procedure. For example, if the time period is large, the estimated TA may not be good enough when performing LTM cell switch, and NW can take it into account.

In summary, we suggest that RAN2 consider to enhance the early sync info to RLF report.

Proposal 1: For RACH-less access based on network TA, include the following information in RLF report:

- Time period between the reception of the PDCCH order and the reception of HO command MAC CE related to the failed LTM cell switch

TCI state/Beam Info

We notice that only the existence of L1 measurement is not sufficient. L1 measurement is the result of the UE movement at the time of failure occurrence.  The beam indicated with a TCI state included in the CSC is also a determining factor impacting whether the RA-less access succeeds or not, i.e., the first UL transmission can be received on the indicated beam before T304 expires or not. It is suggested that UE logs the beam info in the cell switch command, for the network to know how the UE movement affects the beam selection of the target cell. Combining this beam info with the L1 measurement result, and the time gap since the CSC till the failure, the network could perform analysis to optimize the choice of target beam.

Proposal 2: It is proposed RAN2 to agree to enhance the RLF report with the beam information contained in the LTM cell switch command.

- Conclusion

Based on the discussions above, we have the following proposals:

Proposal 1: For RACH-less access based on network TA, include the following information in RLF report:

- Time period between the reception of the PDCCH order and the reception of HO command MAC CE related to the failed LTM cell switch

Proposal 2: It is proposed RAN2 to agree to enhance the RLF report with the beam information contained in the LTM cell switch command.