3GPP TSG-RAN WG2 Meeting #129bis	 R2-2502864

Wuhan, China, April 7-11, 2025

Source:	KDDI Corporation

Title:	Discussion on On-demand SSB

Tdoc Type:	Discussion

Agenda Item:	8.5.2

Document for:  Discussion

Release:       Rel-19

1	Introduction

In RAN2#129, the following discussion points were left for this meeting. In this contribution, we will provide our considerations on the related points.

R2-2500256 Discussion on on-demand SSB for Scell Apple discussion Rel-19 Netw_Energy_NR_enh-Core Proposal 1: The OD-SSB MAC-CE has a fixed size and includes a bitmap to indicate whether OD-SSB is activated in each SCell (i.e., similar to legacy SCell A/D MAC-CE). Proposal 2: The OD-SSB MAC-CE supports two formats: one format indicates up to 7 SCells and the other format indicates up to 31 SCells. Proposal 3: Considering forward compatibility, configuration indices of candidate SSB periodicities in RRC (rather than actual values of SSB periodicities) are included in the OD-SSB MAC-CE. [Xiaomi]: Understand RAN1 already agreed both implicit deactivation and explicit deactivation. If we signal both of them with the single bit-map, it may bring the additional complexities. [Ericsson]: In addition to periodicity, RAN1 also agreed additional ones (something like reference point). It may not be easy to handle them with a fixed MAC CE. [Samsung]: Understand it is almost impossible to have a fixed MAC CE since some SCell activates OD-SSB and some other SCell deactivates OD-SSB, and different information is applied to activation and deactivation. Revisit the issues that raised in P1-P3 next meeting.

2	Discussion

2.1 Parameters to be included in the OD-SSB MAC CE

Agreement @RAN1#118 For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s), multiple candidate values can be configured by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell. - Periodicity of the on-demand SSB - FFS: Any other relevant parameters Agreement@RAN1#120 - For a cell supporting on-demand SSB SCell operation, to provide time domain location of on-demand SSB by RRC, - For Case #2, adopt Alt A in the previous RAN1 agreement, i.e., - Based on two parameters, where one is to indicate SFN offset from a reference point and the other is to indicate half frame index - The reference point is SFN which satisfies (SFN index *10) modulo (OD-SSB periodicity) = 0 - If SFN offset parameter is NOT configured, UE assumes SFN offset set to 0. - If half frame index parameter is NOT configured, UE assumes half frame index set to 0. - The value range of SFN offset is 0 to 15 unless longer periodicity for on-demand SSB than 160 ms is introduced. - The value range of half frame index is 0 or 1. - FFS: Whether/how to indicate time offset between always-on SSB and on-demand SSB (e.g., similar to ssb-TimeOffset) - By defining reference point as SFN where AO-SSB burst exists Agreement - For a cell supporting on-demand SSB SCell operation, introduce NEW higher layer parameter (i.e., od-ssb-config) for on-demand SSB configuration, for both Case #1 and Case #2. - For a cell supporting on-demand SSB SCell operation, introduce NEW higher layer parameters to configure each of the followings, for both Case #1 and Case #2. - Frequency of the on-demand SSB (i.e., od-ssb-absoluteFrequency) - FFS: Whether this parameter can be absent for Case #2 - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst (i.e., od-ssb-PositionsInBurst) - FFS: Whether this parameter can be absent for Case #2 - Periodicity of the on-demand SSB (i.e., od-ssb-Periodicity) - Sub-carrier spacing of the on-demand SSB (i.e., od-ssbSubcarrierSpacing) - For Case #2, this parameter is absent and sub-carrier spagcing of on-demand SSB is the same as that of always-on SSB. - Physical Cell ID of the on-demand SSB (i.e., od-ssb-physCellId) - For Case #2, this parameter is absent and physical cell ID of on-demand SSB is the same as that of always-on SSB. - Time location of on-demand SSB burst (i.e., od-ssb-sfn-Offset and od-ssb-halfFrameIndex) - Downlink transmit power of on-demand SSB (i.e., od-ss-PBCH-BlockPower) - For Case #2, this parameter is absent and downlink transmit power of on-demand SSB is the same as that of always-on SSB. - Number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated (i.e., od-ssb-nrofTx) - FFS: Which of the above parameters are a list

Since OD-SSB is transmitted with different timing and periodicity compared to always-on SSB, the UE requires additional information to correctly process OD-SSB. We think the MAC CE to indicate the OD-SSB transmission should include at least,

- Scell index: to specify the scell where OD-SSB is being transmitted

- the periodicity of OD-SSB: to inform the UE of the expected OD-SSB transmission interval

- the time offset between always-on SSB and on-demand SSB: to allow the UE to correctly align OD-SSB reception and measurement with respect to the always-on SSB timing.

- the SSB index of the OD-SSB: to specify which SSB index corresponds to the OD-SSB transmission.

Proposal 1: The OD-SSB MAC CE is designed to include at least Scell index, periodicity of OD-SSB, time offset between AO- SSB and OD-SSB, and the SSB index of the OD-SSB.

2.2 L3 RRM measurements for OD-SSB

We share the same view as mentioned in [1], that the legacy SMTC L3 measurement is configured per frequency, not per cell, making it difficult for the network to configure a single SMTC window that covers both AO-SSB and OD-SSB. Using the legacy SMTC L3 measurement approach for OD-SSB can lead to unnecessary power consumption, as the UE may continue measuring when no OD-SSB is transmitted or when the number of SSBs is significantly reduced. To address this inefficiency, we propose that the OD-SSB SMTC should be associated with the adaptive OD-SSB periodicity.

Proposal2: OD-SSB SMTC should be associate with the adaptive OD-SSB periodicity.

In the legacy measurement configuration, the measurement and report configuration are linked through a measId, and only one type of SMTC can be configured per frequency. This approach is inefficient for OD-SSB because the network would need to configure multiple OD-SSB periodicities explicitly within the measurement configuration, leading to unnecessary complexity and signaling overhead. We propose that the OD-SSB SMTC should be dynamically activated only when OD-SSB is active, and the UE should apply the OD-SSB-specific SMTC only when OD-SSB is activated.

Proposal3: The UE apply the OD-SSB specific SMTC when the OD-SSB is activated.

3	Conclusion

We make the following observations and proposals:

Proposal 1: The OD-SSB MAC CE is designed at least include Scell index, periodicity of OD-SSB, time offset between AO- SSB and OD-SSB, and the SSB index of the OD-SSB. Proposal 2:

Proposal2: OD-SSB SMTC should be associate with the adaptive OD-SSB periodicity.

Proposal3: The UE apply the OD-SSB specific SMTC when the OD-SSB is activated.

4	Reference

[1] R2-2500245 Discussion on on-demand SSB for Scell Apple