3GPP TSG-RAN WG2 Meeting #129bis	                                 R2-2502869

Wuhan, China, Apr. 7th – 11th , 2025

Source:        ZTE Corporation

Title:           Consideration on Supporting 7M Channel Bandwidth

Agenda item:   8.19

Document for:  Discussion and Decision

- Introduction

In the last meeting, a LS was sent on the 7M channel bandwidth [1], in this paper, we share our views on this issue.

- Discussion

In this chapter, we first discuss how to introduce 7M bandwidth in the RAN2 spec, then we discuss some other issues.

2.1 Introduce 7 M

As in the LS, the main question is whether the 7M can be implemented as release independent from Rel 15.

ACTION: RAN4 respectfully requests that RAN2 consider the signalling aspects related to adding the 7 MHz channel bandwidth, and to inform RAN4 if RAN2 sees any reason from a signalling perspective why the 7 MHz channel bandwidth cannot be release independent from Release 15.

From RAN2 aspect, there are 2 options to implement a feature from R15:

- Option 1: Implement the feature in the latest release version and add a magic sentence to allow early implementation

- Option 2: Implement the feature from the R15.

The option 1 is quite easy, while the option 2 would introduce additional complexity especially for adding new maximum supported bandwidth. Take the 35M/45M feature [2][3] as an example, this feature was implemented without introducing 35M/45M as the maximum supported bandwidth in the R15/16, instead, a clarification was added as below according to the [4][5].

supportedBandwidthDL The UE may report a supportedBandwidthDL wider than the channelBWs-DL, this supportedBandwidthDL may not be included in the Table 5.3.5-1 of TS 38.101-1[2]/TS 38.101-2[3] for the case that the UE is unable to report the actual supported bandwidth according to the Table 5.3.5-1 of TS 38.101-1[2]/TS 38.101-2[3].

With the CR[4][5], it means for the band supports 35M/45M as the maximum bandwidth, the UE would report 40M/50M in the R15/R16 to avoid introduce new maximum bandwidth. The main reason is that it’s complex to add new supported bandwidth for each old release version with the current FeatureSets structure.

FeatureSets ::= SEQUENCE { … [[ featureSetsDownlink-v15a0 SEQUENCE (SIZE (1..maxDownlinkFeatureSets)) OF FeatureSetDownlink-v15a0 OPTIONAL ]], … [[ featureSetsUplink-v1640 SEQUENCE (SIZE (1..maxUplinkFeatureSets)) OF FeatureSetUplink-v1640 OPTIONAL ]], … [[ featureSetsDownlinkPerCC-v1780 SEQUENCE (SIZE (1..maxPerCC-FeatureSets)) OF FeatureSetDownlinkPerCC-v1780 OPTIONAL, featureSetsUplinkPerCC-v1780 SEQUENCE (SIZE (1..maxPerCC-FeatureSets)) OF FeatureSetUplinkPerCC-v1780 OPTIONAL ]], … [[ featureSetsDownlink-v1830 SEQUENCE (SIZE (1..maxDownlinkFeatureSets)) OF FeatureSetDownlink-v1830 OPTIONAL ]] }

Thus if the RAN4 intends to also include 7M as the maximum bandwidth, it’s better to go to the option 1.

Proposal 1: Send an LS to inform RAN4 that RAN2 prefer to implement the feature in the R19 and allow early implementation from R15.

2.2 Lessons from 3M Feature Implementation

Another issue is about the implementation complexity, from the LS, the 7M would share the similar features as 3M.

| Support3MHz-ChannelBW-Asymmetric-r18 Indicates whether the UE supports 3 MHz channel bandwidth in uplink with larger than 3 MHz channel BW in DL, including short RACH preamble formats with 15kHz SCS, and long PRACH formats with 1.25kHz SCS. This feature is supported for 15kHz SCS only. It is applicable only to single-carrier operation and applies to bands where the UE indicates support for asymmetricBandwidthCombinationSet with 3 MHz UL according to clause 5.3.6 of TS 38.101-1 [2]. This feature is not applicable to Ues indicating supportOfRedCap-r17 or supportOfERedCap-r18. NOTE 1: The UE supporting this feature supports configuration of 15 PRB UL BWP operation. NOTE 2: If the UE indicates support in asymmetricBandwidthCombinationSet for a 3MHz UL in a band according to clause 5.3.6 of 38.101-1 [2], this feature shall be indicated for the band. | Band | No | FDD only | FR1 only |
| --- | --- | --- | --- | --- |
| support3MHz-ChannelBW-Symmetric-r18 Indicates whether the UE supports 3 MHz symmetric channel bandwidth in DL and UL, including the following functional components: - Reception of 12 PRB PBCH based on RB-level puncturing; - Short RACH preamble formats with 15kHz SCS, and long PRACH formats with 1.25kHz SCS; - Reception of 15 PRB CORESET0. This feature is supported for 15kHz SCS only. It is applicable only to single-carrier operation and when an associated SS/PBCH block is located according to Table 5.4.3.3-2 in TS 38.101-1 [2]. This feature is not applicable to Ues indicating supportOfRedCap-r17 or supportOfERedCap-r18. NOTE: The UE supporting this capability supports configuration of 15 PRB BWP operation in DL and UL. | Band | No | FDD only | FR1 only |

For the 3M feature discussion, RAN1 includes the Support3MHz-ChannelBW-Asymmetric-r18/ support3MHz-ChannelBW-Symmetric-r18 in their feature list table, meanwhile RAN4 define Asymmetric bandwidth combination with 3M in RAN4 spec and send a LS to RAN2. In RAN2, normally these features would be implemented by following the RAN1/4’s definition and thus the above 2 capabilities are newly introduced even though it can be indicated by the existing channelBWs-DL/UL.

After several rounds discussion, 2 options were provided as in the [6]:

- Option 1: Indicate the 3M in the channelBWs-DL/UL (with the reserving bit)

- Option 2: Do not indicate the 3M in the channelBWs-DL/UL but define support3MHz-ChannelBW-Asymmetric-r18/ support3MHz-ChannelBW-Symmetric-r18;

The option 1 was a quite simple way but was not chosen at last for that the field description of RAN1 feature

“ support3MHz-ChannelBW-Asymmetric-r18/ support3MHz-ChannelBW-Symmetric-r18” is quite lengthy, if we add this description to the legacy channelBWs-DL/UL field, it would make the field description of the channelBWs-DL/UL becomes more and more unreadable.

To avoid the similar things happen to 7M, it’s better to capture something that similar to the current 3M feature field description in the RAN1/4 spec and then RAN2 can implement the 7M indication with the reserving bits in the channelBWs-DL/UL.

Proposal 2: Send an LS to inform RAN4/1 to capture something that similar to the current 3M feature field description (i.e.Support3MHz-ChannelBW-Asymmetric-r18/ support3MHz-ChannelBW-Symmetric-r18) for the 7M feature (if 7M supports the similar features) and then RAN2 can implement the 7M bandwidth indication with the reserving bits in the channelBWs-DL/UL without any lengthy field description.

- Conclusion and proposals

With the above analysis, we have the following proposals:

Proposal 1: Send an LS to inform RAN4 that RAN2 prefer to implement the feature in the R19 and allow early implementation from R15.

Proposal 2: Send an LS to inform RAN4/1 to capture something that similar to the current 3M feature field description (i.e.Support3MHz-ChannelBW-Asymmetric-r18/ support3MHz-ChannelBW-Symmetric-r18) for the 7M feature (if 7M supports the similar feature) and then RAN2 can implement the 7M bandwidth indication with the reserving bits in the channelBWs-DL/UL without any lengthy field description.

- Reference

- R4-2503017  LS to RAN2 on RRC and MAC impacts for Rel-19 NR MIMO Ph5  Source: RAN4 To:RAN2 Cc:	RAN3

- R2-2102393	Support of 35 MHz and 45 MHz channel bandwidth for FR1	Apple Inc, T-Mobile	CR	Rel-15	38.306	15.12.0	0511	1	F	NR_FR1_35MHz_45MHz_BW-Core

- R2-2102394	Support of 35 MHz and 45 MHz channel bandwidth for FR1	Apple Inc, T-Mobile	CR	Rel-16	38.306	16.3.0	0512	1	A	NR_FR1_35MHz_45MHz_BW-Core

- R2-2106691	CR on the 35M45M supporting-R15	B	15.13.0	ZTE Corporation, Sanechips

- R2-2106692	CR on the 35M45M supporting-R16	A	16.4.0	ZTE Corporation, Sanechips

- R2-2410768	Report of [Post127bis][011][less5MHz] 331 CR (ZTE)	ZTE Corporation	discussion	Rel-18	NR_FR1_lessthan_5MHz_BW-Core

- R2-2411163	Correction on the Less than 5M Bandwidth	ZTE Corporation, vivo	CR	Rel-18	38.331	18.3.0	5187	1	F	NR_FR1_lessthan_5MHz_BW-Core

- R2-2411164	Correction on the Less than 5M Bandwidth	ZTE Corporation	CR	Rel-18	38.306	18.3.0	1220	-	F	NR_FR1_lessthan_5MHz_BW-Core