3GPP TSG-RAN WG2 Meeting #129-bis           	R2-2502889
Wuhan, China, 7 – 11 April, 2025

Agenda Item:	8.7.6

Source: 	Meta

Title:  	Discussion on RAN Awareness and UL Rate Control for XR

Document for:	Discussion and Decision

# Introduction

In RAN #107 plenary meeting, the WI objective on RAN awareness and UL rate control was [1]:

- Specify uplink congestion signalling [RAN2, RAN3]: - Specify in MAC layer XR rate control signalling over downlink per QoS flow/per DRB and the associated F1 signalling, if any, to enable faster source rate adaption to uplink congestion.

IIn this paper, we discuss our views on XR UL rate control.

# Discussion

During the RAN2#128 meeting, the following two options are considered for per QoS flow lever indication via MAC CE: 1) flows are indicated by MAC CE or 2) by RRC while MAC CE is per DRB.

During the RAN3#127 meeting, the F1AP interface impact for CU-DU split case was discussed and it was concluded that both options have F1AP impact [14].

With Option 1, the QoS to DRB mapping information needs to be sent from CU to DU. DU then derives the recommended bit rate on a per-QoS flow level based on the network congestion conditions.

With Option 2, the QoS flow to DRB mapping information is configured via RRC by the CU. In order to derive the recommended bit rate on a per-QoS flow level, CU also needs to send the QoS flow to DRB mapping information to DU.

Since both options have F1AP impact and that Option 1 is a simpler and cleaner solution, it is proposed to downselect Option 1.

Proposal 1: Down-select Option1, e.g., support per-QoS flow level rate indication via MAC CE.

# Conclusion

In this paper, we discuss our views on XR UL rate control.

Proposal 1: Down-select Option1, e.g., support per-QoS flow level rate indication via MAC CE.

# References

[1] RP-240107, Revised WID on XR (eXtended Reality) for NR Phase 3

[2] RP-234013, Moderator's summary on new WI XR (eXtended Reality) for NR Phase 3, MediaTek (Moderator)

[3] RP-233317, Meta’s Views on Rel-19 XR Evolution, Meta

[4] R2-2213226, Reply LS to SA2 on XR, vivo, Huawei

[5] TR 23.700-70, Study on architecture enhancement for Extended Reality and Media service (XRM); Phase 2

[6] SP-231198, New SID on 5GS XRM Ph2

[7] RP-240647, Discussion on Rel-19 XR WID, Meta, CATT, CMCC, Google, vivo, ZTE Corporation, Sanechips

[8] RP-241540, Discussion on Rel-19 XR WID, Meta, CATT, CMCC, Google, vivo, ZTE Corporation, Sanechips, Huawei, HiSilicon, Spreadtrum, Samsung, Apple, MediaTek

[9] RP-241651, Outcome from the XR discussion for NR Phase 3 WI scope, Nokia

[10] RP-242224, “On RAN Awareness and UL Rate Control for XR”, Meta, CATT, CMCC, Google, vivo, ZTE Corporation, Sanechips, Huawei, HiSilicon, Spreadtrum, Samsung, Apple, MediaTek Inc., Futurewei

[11] TS 38.321, “Medium Access Control (MAC) protocol specification”

[12] RP-243305, “Release 19 XR enhancements -Way forward on WID scope revision”

[13] S4-242182 “Reply to LS on appropriate range and granularity of bit rate adaptation for XR applications.”

[14] R3-250805, Reply LS on XR UL Rate Control