3GPP TSG-RAN WG2 Meeting #129bis	R2-2502966

Wuhan, China, Apr. 7th – 11th, 2025

Agenda item:		7.2

Source:				Samsung

Title:						Discussion on eEMR/IMR and LTM L2 Reset/UE-based TA

measurement

Document for:		Discussion

1 Introduction

This contribution discusses the following issues:

- reselectionMeasAvailable in RRCSetupComplete

- reselectionMeasAvailable and validityStatus in RRCResumeComplete

- update of measurement configuration for eEMR (enhanced early measurement report) and IMR (Idle/inactive cell reselection measurement report)

- removal of measIdleValidityDuration

- clarification for ServingCellNoResetID and ServingCellUE-MeasuredTA-ID

2 Discussion

2.1 reselectionMeasAvailable in RRCSetupComplete

According to the clause 5.3.3.4 in TS 38.331 [1], the UE includes the reselectionMeasAvailable in RRCSetupComplete if it has valid NR reselection measurements available for any frequency in measReselectionCarrierListNR in VarMeasReselectionConfig and SIB1 contains reselectionMeasurementsNR (relevant parts are copied and pasted below).

1> set the content of RRCSetupComplete message as follows: (omitted) 2> if the SIB1 contains reselectionMeasurementsNR and the UE has valid NR reselection measurements available for any frequency listed in measReselectionCarrierListNR in VarMeasReselectionConfig: 3> include the reselectionMeasAvailable;

Since the validity check is determined at the time of reporting NR reselection measurement results, there is no case for the UE to include the reselectionMeasAvailable in RRCSetupComplete.

Observation 1: The UE does not include the reselectionMeasAvailable in RRCSetupComplete since whether available NR reselection measurement results are valid or not is determined at the timing of reporting them.

Note that how to include the reselectionMeasAvailable in RRCResumeComplete is specified in clause 5.3.13.4 (only relevant parts are copied and pasted below).

1> set the content of the of RRCResumeComplete message as follows: (omitted) 4> if the SIB1 contains reselectionMeasurementsNR: 5> if measReselectionCarrierListNR is present in VarMeasReselectionConfig and the UE has NR reselection measurements available for any frequency listed in measReselectionCarrierListNR in VarMeasReselectionConfig; or 5> if measReselectionCarrierListNR is not present in VarMeasReselectionConfig and if the UE has NR reselection measurements available: 6> include the reselectionMeasAvailable;

We think that the above procedure text should be applied to clause 5.3.3.5 as well.

Proposal 1: RAN2 to adopt the TP provided in Annex 1.

2.2 reselectionMeasAvailable and validityStatus in RRCResumeComplete

According to the clause 5.3.13.4 in TS 38.331 [1], the UE does not include validityStatus in RRCResumeComplete if measReselectionCarrierListNR is not present in VarMeasReselectionConfig (relevant parts are copied and pasted below, see the yellow highlighted part).

1> set the content of the of RRCResumeComplete message as follows: (omitted) 2> if the reselectionMeasurementReq is included in the RRCResume message: 3> if validatedMeasurementsReq is included in the RRCResume and measReselectionValidityDuration is included in VarMeasReselectionConfig: 4> if measReselectionCarrierListNR is present in VarMeasReselectionConfig: 5> if the UE has valid cell reselection measurements results for any frequency listed in measReselectionCarrierListNR in VarMeasRelectionConfig: 6> set the measResultReselectionNR in the RRCResumeComplete message to the valid NR measurement results, if available for any frequency listed in measReselectionCarrierListNR in VarMeasReselectionConfig and set validityStatus to the value of measReselectionValidityDuration in VarMeasReselectionConfig; 4> else: 5> if the UE has valid NR cell reselection measurements results: 6> set the measResultReselectionNR in the RRCResumeComplete message to any available valid NR measurement results, if available; (omitted) 3> else: (omitted) 3> else: 4> if the SIB1 contains reselectionMeasurementsNR: 5> if measReselectionCarrierListNR is present in VarMeasReselectionConfig and the UE has NR reselection measurements available for any frequency listed in measReselectionCarrierListNR in VarMeasReselectionConfig; or 5> if measReselectionCarrierListNR is not present in VarMeasReselectionConfig and if the UE has NR reselection measurements available: 6> include the reselectionMeasAvailable;

Whenever valid NR reselection measurements are reported to the network, the UE should set validityStatus accordingly. Otherwise, the network has no idea how validity check is performed by the UE.

Observation 2: Network has no idea how validity check is performed by the UE if validityStatus is not included in corresponding NR reselecion measurement results.

As highlighted in green above, the UE is allowed to include the reselectionMeasAvailable in RRCResumeComplete only if the reselectionMeasurementsNR is included in RRCResume. It is quite signalling inefficient in the sense that (1) network always needs to include the reselectionMeasurementsNR in RRCResume to check whether the UE has NR reselection measurements available and (2) the UE always reports valid NR reselection measurements, if available, together with the reselectionMeasAvailable to the network.

Observation 3: It is signalling inefficient to include the reselectionMeasurementsNR in RRCResume to check whether the UE has available NR reselection measurements.

To address both observation 2 and 3, we propose to update the existing procedure text as in the TP in Annex 2.

Proposal 2: RAN2 to adopt the TP provided in Annex 2.

2.3 Update of measurement configuration for eEMR and IMR

According to the clause 5.7.8.1a in TS 38.331 [1], the UE updates measIdleValidityDuration based on SIB11 as long as network does not configure dedicated frequency(ies) to be measured during RRC_IDLE or RRC_INACITVE in RRCRelease (relevant parts are copied and pasted below).

While in RRC_IDLE or RRC_INACTIVE, and T331 is running, the UE shall: 1> if VarMeasIdleConfig includes neither a measIdleCarrierListEUTRA nor a measIdleCarrierListNR received from the RRCRelease message: (omitted) 2> if SIB11 includes the measIdleConfigSIB and contains measIdleValidityDuration: 3> store or replace the measIdleValidityDuration of measIdleConfigSIB of SIB11 within VarEnhMeasIdleConfig; 2> else: 3> remove the measIdleValidityDuration in VarEnhMeasIdleConfig, if stored;

Observation 4: The UE updates the measIdleValidityDuration based on SIB11 if network does not configure both measIdleCarrierListEUTRA and measIdleCarrierListNR in RRCRelease.

Since the UE can be configured with the measIdleValidityDuration in RRCRelease or SIB11, we think that the update of measIdleValidityDuration should not be subject to the configuration of dedicated frequency(ies). Instead, it should be based on whether measIdleValidityDuration is included in RRCRelease.

According to the clauses 5.7.8.1a and 5.7.8.1b in TS 38.331 [1], it is observed that some procedure texts of updating measurement configuration related are not subject to UE capability (relevant parts are copied and pasted below).

5.7.8.1a Measurement configuration (omitted) 2> if SIB11 includes the measIdleConfigSIB and contains measIdleValidityDuration: 3> store or replace the measIdleValidityDuration of measIdleConfigSIB of SIB11 within VarEnhMeasIdleConfig; 2> else: 3> remove the measIdleValidityDuration in VarEnhMeasIdleConfig, if stored; 5.7.8.1b Measurement configuration (reselection measurements) (omitted) 1> if VarMeasReselectionConfig does not include measReselectionCarrierListNR received from the RRCRelease message: 2> if the UE supports reselection measurement reporting: 3> if SIB11 includes the measIdleConfigSIB and contains measReselectionCarrierListNR: 4> store or replace the measReselectionCarrierListNR of measIdleConfigSIB of SIB11 within VarMeasReselectionConfig; 3> else: 4> remove the measReselectionCarrierListNR in VarMeasReselectionConfig, if stored; 1> if VarMeasReselectionConfig does not include measReselectionValidityDuration received from the RRCRelease message: 2> if SIB11 includes the measIdleConfigSIB and contains measReselectionValidityDuration: 3> store or replace the measReselectionValidityDuration of measIdleConfigSIB of SIB11 within VarMeasReselectionConfig; 2> else: 3> remove the measurementValidityDuration in VarMeasReselectionConfig, if stored.

Since both enhanced early measurement report (eEMR) and idle/inactive cell reselection measurement report (IMR) features are introduced in Release 18, we think that each relevant measurement configuration needs to be updated based on UE capability. Otherwise, it could be misleading that any UE is required to update such configuration.

Observation 5: There is no UE requirement of updating measIdleValidityDuration and/or measReselectionValidityDuration based on SIB11 if it does not support eEMR and/or IMR feature.

To address both observation 4 and 5, we propose to update the existing procedure text as in the TP in Annex 3.

Proposal 3: RAN2 to adopt the TP provided in Annex 3.

2.4 Removal of measIdleValidityDuration

Upon entering RRC_CONNECTED, the UE stops timer T331 if running and releases all idle/inactive measurement configuration i.e., VarMeasIdleConfig. On the other hand, the UE keeps measIdleValidityDuration in VarEnhMeasIdleConfig in order to perform validity check and report valid measurement results to the network if requested.

Upon request from the network, the UE reports (valid) measurement results to the network and discard them i.e., VarMeasIdleReport. But the UE still keeps the measIdleValidityDuration in VarEnhMeasIdleConfig.

Observation 6: Upon entering RRC_CONNECTED, the UE releases all idle/inactive measurement configuration except measIdleValidityDuration. After discarding the measurement results (i.e., VarMeasIdleReport) as result of their successful transmission, the UE still keeps measIdleValidityDuration.

If proposal 3 is agreed, the UE does not update the measIdleValidityDuration based on SIB11 if it is configured via RRCRelease. Note that the field measIdleConfig in RRCRelease is defined using the parameterized SetupRelease type with need code M.

RRCRelease-v1610-IEs ::=            SEQUENCE {

voiceFallbackIndication-r16        ENUMERATED {true}                             OPTIONAL, -- Need N

measIdleConfig-r16                 SetupRelease {MeasIdleConfigDedicated-r16}    OPTIONAL, -- Need M

nonCriticalExtension               RRCRelease-v1650-IEs                          OPTIONAL

}

MeasIdleConfigDedicated-r16 ::= SEQUENCE {

measIdleCarrierListNR-r16       SEQUENCE (SIZE (1..maxFreqIdle-r16)) OF MeasIdleCarrierNR-r16          OPTIONAL,     -- Need N

measIdleCarrierListEUTRA-r16    SEQUENCE (SIZE (1..maxFreqIdle-r16)) OF MeasIdleCarrierEUTRA-r16       OPTIONAL,     -- Need N

measIdleDuration-r16            ENUMERATED{sec10, sec30, sec60, sec120, sec180, sec240, sec300, spare},

validityAreaList-r16            ValidityAreaList-r16                                                   OPTIONAL,     -- Need N

...,

[[

measReselectionCarrierListNR-r18     SEQUENCE (SIZE (1..maxFreqIdle-r16)) OF MeasReselectionCarrierNR-r18    OPTIONAL,    -- Need S

measIdleValidityDuration-r18         MeasurementValidityDuration-r18                                         OPTIONAL,    -- Need S

measReselectionValidityDuration-r18  MeasurementValidityDuration-r18                                         OPTIONAL     -- Need S

]]

}

It means that network should instruct the UE to release it explicitly in RRCRelease i.e. the measIdleConfig in RRCRelease is set to release or set to setup without containing measIdleValidityDuration.

Observation 7: If measIdleValidityDuration is configured in RRCRelease, the UE keeps it forever in VarEnhMeasIdleConfig unless network explicitly releases it in RRCRelease later on.

When sending the RRCRelease to the UE, it is quite challenging to know whether the UE was configured with measIdleConfig in RRCRelease or not because there will be no exchange of such information between gNBs. Besides, the UE needs to keep it even though it enters in RRC_CONNECTED to E-UTRA. Thus, we propose to discuss whether to release the measIdleValidityDuration in VarEnhMeasIdleConfig after reporting measurement results in VarMeasIdleReport to the network.

Proposal 4: RAN2 to discuss whether to remove the measIdleValidityDuration in VarEnhMeasIdleConfig, if stored, after discarding the VarMeasIdleReport.

2.5 Clarification for ServingCellNoResetID and ServingCellUE-MeasuredTA-ID

In Rel-18, the following two IEs are defined as below:

–                      VarLTM-ServingCellNoResetID

The IE VarLTM-ServingCellNoResetID is used to store the serving cell ID based on which the UE determines whether a L2 reset is needed or not upon an LTM cell switch procedure.

–                      VarLTM-ServingCellUE-MeasuredTA-ID

The IE VarLTM-ServingCellUE-MeasuredTA-ID is used to store the serving cell ID based on which the UE determines whether UE-based TA measurements are needed or not.

The terminology of “the serving cell ID” is also used for other IEs, which is used to indicate the serving cell index, e.g.,

servCellIndex Serving cell ID of a PSCell. The PCell of the Master Cell Group uses ID = 0.

servingCellId The serving Cell ID of the source SSB, CSI-RS, or SRS for the spatial relation of the target SRS resource. If this field is absent the SSB, the CSI-RS, or the SRS is from the same serving cell where the SRS is configured.

referenceCarrierIndex The serving cell ID of the carrier which is to be used as the reference for interpreting the Tx Direction Current location as reported using txDirectCurrentLocation-r16. The numerology of the uplink BWP ID reported with bwp-Id-r16 for this serving cell is the numerology used for interpreting the reported subcarrier location

Technically, it is understandable that the above two “Serving cell ID” are referring to different things. However, from specification point of view, it is better to have differentiation between them; otherwise, the “Serving cell ID” used for LTM feature may be misinterpreted as serving cell index.

Proposal 5: RAN2 to agree to change the serving cell IDs under VarLTM-ServingCellNoResetID and VarLTM-ServingCellUE-MeasuredTA-ID as “ID associated with the serving cell”.

3 Conclusion

In section 2, the following observations are made:

Observation 1: The UE does not include the reselectionMeasAvailable in RRCSetupComplete since whether available NR reselection measurement results are valid or not is determined at the timing of reporting them.

Observation 2: Network has no idea how validity check is performed by the UE if validityStatus is not included in corresponding NR reselecion measurement results.

Observation 3: It is signalling inefficient to include the reselectionMeasurementsNR in RRCResume to check whether the UE has available NR reselection measurements.

Observation 4: The UE updates the measIdleValidityDuration based on SIB11 if network does not configure both measIdleCarrierListEUTRA and measIdleCarrierListNR in RRCRelease.

Observation 5: There is no UE requirement of updating measIdleValidityDuration and/or measReselectionValidityDuration based on SIB11 if it does not support eEMR and/or IMR feature.

Observation 6: Upon entering RRC_CONNECTED, the UE releases all idle/inactive measurement configuration except measIdleValidityDuration. After discarding the measurement results (i.e., VarMeasIdleReport) as result of their successful transmission, the UE still keeps measIdleValidityDuration.

Observation 7: If measIdleValidityDuration is configured in RRCRelease, the UE keeps it forever in VarEnhMeasIdleConfig unless network explicitly releases it in RRCRelease later on.

Based on above, RAN2 is requested to discuss and agree on the following proposals:

Proposal 1: RAN2 to adopt the TP provided in Annex 1.

Proposal 2: RAN2 to adopt the TP provided in Annex 2.

Proposal 3: RAN2 to adopt the TP provided in Annex 3.

Proposal 4: RAN2 to discuss whether to remove the measIdleValidityDuration in VarEnhMeasIdleConfig, if stored, after discarding the VarMeasIdleReport.

Proposal 5: RAN2 to agree to change the serving cell IDs under VarLTM-ServingCellNoResetID and VarLTM-ServingCellUE-MeasuredTA-ID as “ID associated with the serving cell”.

4 Reference

[1] 3GPP TS 38.331 v18.5.1	Radio Resource Control (RRC) protocol specification

Annex 1 – TP for Proposal 1

5.3.3.4	Reception of the RRCSetup by the UE

The UE shall perform the following actions upon reception of the RRCSetup:

1>	if the RRCSetup is received in response to an RRCReestablishmentRequest; or

1>	if the RRCSetup is received in response to an RRCResumeRequest or RRCResumeRequest1:

2>	if the UE is NCR-MT:

3>	indicate to NCR-Fwd to cease forwarding;

2>	if sdt-MAC-PHY-CG-Config is configured:

3>	instruct the MAC entity to stop the cg-SDT-TimeAlignmentTimer, if it is running;

3>	instruct the MAC entity to start the timeAlignmentTimer associated with the PTAG indicated by tag-Id, if it is not running;

2>	if srs-PosRRC-Inactive is configured:

3>	instruct the MAC entity to stop the inactivePosSRS-TimeAlignmentTimer, if it is running;

2>	if srs-PosRRC-InactiveValidityAreaPreConfigList or srs-PosRRC-InactiveValidityAreaNonPreConfig is configured:

3>	instruct the MAC entity to stop the inactivePosSRS-ValidityAreaTAT, if it is running;

2>	if the UE is configured to receive MBS multicast in RRC_INACTIVE:

3>	reset MAC;

2>	discard any stored UE Inactive AS context and suspendConfig;

2>	discard any current AS security context including the KRRCenc key, the KRRCint key, the KUPint key and the KUPenc key;

2>	release radio resources for all established RBs except SRB0 and broadcast MRBs, including release of the RLC entities, of the associated PDCP entities and of SDAP;

2>	release the RRC configuration except for the default L1 parameter values, default MAC Cell Group configuration, CCCH configuration and broadcast MRBs;

2>	indicate to upper layers fallback of the RRC connection;

2>	for each application layer measurement configuration with appLayerIdleInactiveConfig absent:

3>	forward the measConfigAppLayerId and inform upper layers about the release of the application layer measurement configuration;

3>	release the application layer measurement configuration;

3>	discard any application layer measurement reports which were not yet fully submitted to lower layers for transmission;

3>	consider itself not to be configured to send application layer measurement reports for the measConfigAppLayerId;

2>	stop timer T380, if running;

1>	perform the cell group configuration procedure in accordance with the received masterCellGroup and as specified in 5.3.5.5;

1>	perform the radio bearer configuration procedure in accordance with the received radioBearerConfig and as specified in 5.3.5.6;

1>	if stored, discard the cell reselection priority information provided by the cellReselectionPriorities or inherited from another RAT;

1>	stop timer T300, T301, T319;

1>	if T319a is running:

2>	stop T319a;

2>	consider SDT procedure is not ongoing;

1>	if T390 is running:

2>	stop timer T390 for all access categories;

2>	perform the actions as specified in 5.3.14.4;

1>	if T302 is running:

2>	stop timer T302;

2>	perform the actions as specified in 5.3.14.4;

1>	stop timer T320, if running;

1>	if the RRCSetup is received in response to an RRCResumeRequest, RRCResumeRequest1 or RRCSetupRequest:

2>	if T331 is running:

3>	stop timer T331;

3>	perform the actions as specified in 5.7.8.3;

2>	enter RRC_CONNECTED;

2>	stop the cell re-selection procedure;

2>	stop relay (re)selection procedure if any for L2 U2N Remote UE;

1>	consider the current cell to be the PCell;

1>	perform the L2 U2N Remote UE configuration procedure in accordance with the received sl-L2RemoteUE-Config as specified in 5.3.5.16;

1>	perform the sidelink dedicated configuration procedure in accordance with the received sl-ConfigDedicatedNR as specified in 5.3.5.14;

1>	if the UE has radio link failure or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report; or

1>	if the UE has radio link failure or handover failure information available in VarRLF-Report and if the current registered SNPN identity is included in snpn-IdentityList stored in VarRLF-Report:

2>	if reconnectCellId in VarRLF-Report is not set after failing to perform reestablishment and if this is the first RRCSetup received by the UE after declaring the failure:

3>	if the UE supports RLF-Report for conditional handover and if choCellId in VarRLF-Report is set:

4>	set timeUntilReconnection in VarRLF-Report to the time that elapsed since the radio link failure or handover failure experienced in the failedPCellId stored in VarRLF-Report;

3>	else:

4>	set timeUntilReconnection in VarRLF-Report to the time that elapsed since the last radio link failure or handover failure;

3>	set nrReconnectCellId in reconnectCellId in VarRLF-Report to the global cell identity and the tracking area code of the PCell;

1>	if the UE supports RLF report for inter-RAT MRO NR as defined in TS 36.306 [62], and if the UE has radio link failure or handover failure information available in VarRLF-Report of TS 36.331 [10] and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report of TS 36.331 [10]:

2>	if reconnectCellId in VarRLF-Report of TS 36.331[10] is not set after failing to perform reestablishment and if this is the first RRCSetup received by the UE after declaring the failure:

3>	set timeUntilReconnection in VarRLF-Report of TS 36.331[10] to the time that elapsed since the last radio link failure or handover failure in LTE;

3>	set nrReconnectCellId in reconnectCellId in VarRLF-Report of TS 36.331[10] to the global cell identity and the tracking area code of the PCell;

1>	for each application layer measurement configuration with appLayerIdleInactiveConfig configured:

2>	if the RPLMN is not included in plmn-IdentityList in VarAppLayerPLMN-ListConfig:

3>	forward the measConfigAppLayerId and inform upper layers about the release of the application layer measurement configuration;

3>	release the application layer measurement configuration including its fields in the UE variables VarAppLayerIdleConfig and VarAppLayerPLMN-ListConfig;

3>	discard any application layer measurement reports which were not yet fully submitted to lower layers for transmission;

3>	consider itself not to be configured to send application layer measurement reports for the measConfigAppLayerId;

1>	set the content of RRCSetupComplete message as follows:

2>	if upper layers provide a 5G-S-TMSI:

3>	if the RRCSetup is received in response to an RRCSetupRequest:

4>	set the ng-5G-S-TMSI-Value to ng-5G-S-TMSI-Part2;

3>	else:

4>	set the ng-5G-S-TMSI-Value to ng-5G-S-TMSI;

2>	if upper layers selected an SNPN or a PLMN and in case of PLMN UE is either allowed or instructed to access the PLMN via a cell for which at least one CAG ID is broadcast:

3>	set the selectedPLMN-Identity from the npn-IdentityInfoList;

2>	else:

3>	set the selectedPLMN-Identity to the PLMN selected by upper layers from the plmn-IdentityInfoList;

2>	if upper layers provide the 'Registered AMF':

3>	include and set the registeredAMF as follows:

4>	if the PLMN identity of the 'Registered AMF' is different from the PLMN selected by the upper layers:

5>	include the plmnIdentity in the registeredAMF and set it to the value of the PLMN identity in the 'Registered AMF' received from upper layers;

4>	set the amf-Identifier to the value received from upper layers;

3>	include and set the guami-Type to the value provided by the upper layers;

2>	if upper layers provide one or more S-NSSAI (see TS 23.003 [21]):

3>	include the s-NSSAI-List and set the content to the values provided by the upper layers;

2>	if upper layers provide onboarding request indication:

3>	include the onboardingRequest;

2>	set the dedicatedNAS-Message to include the information received from upper layers;

2>	if connecting as an IAB-node but not as a mobile IAB-node:

3>	include the iab-NodeIndication;

2>	else if connecting as a mobile IAB-node:

3>	include the mobileIAB-NodeIndication;

2>	if connecting as an NCR-node:

3>	include the ncr-NodeIndication;

2>	if the SIB1 contains idleModeMeasurementsNR and the UE has NR idle/inactive measurement information concerning cells other than the PCell available in VarMeasIdleReport; or

2>	if the SIB1 contains idleModeMeasurementsEUTRA and the UE has E-UTRA idle/inactive measurement information available in VarMeasIdleReport:

3>	include the idleMeasAvailable;

2>	if the SIB1 contains reselectionMeasurementsNR:

3>	if measReselectionCarrierListNR is present in VarMeasReselectionConfig and the UE has NR reselection measurements available for any frequency listed in measReselectionCarrierListNR in VarMeasReselectionConfig; or

3>	if measReselectionCarrierListNR is not present in VarMeasReselectionConfig and if the UE has NR reselection measurements available:

4>	include the reselectionMeasAvailable;

2>	if the UE has logged measurements available for NR and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport; or

2>	if the UE has logged measurements available for NR and if the current registered SNPN identity is included in snpn-ConfigID-List stored in VarLogMeasReport:

3>	include the logMeasAvailable in the RRCSetupComplete message;

3>	if Bluetooth measurement results are included in the logged measurements the UE has available for NR:

4>	include the logMeasAvailableBT in the RRCSetupComplete message;

3>	if WLAN measurement results are included in the logged measurements the UE has available for NR:

4>	include the logMeasAvailableWLAN in the RRCSetupComplete message;

2>	if the sigLoggedMeasType in VarLogMeasReport is included; or

2>	if the UE supports the override protection of the signalling based logged MDT for inter-RAT (i.e. LTE to NR), and if the sigLoggedMeasType in VarLogMeasReport of TS 36.331 [10] is included:

3>	if T330 timer is running (associated to the logged measurement configuration for NR or for LTE):

4>	set sigLogMeasConfigAvailable to true in the RRCSetupComplete message;

3>	else:

4>	if the UE has logged measurements in VarLogMeasReport or in VarLogMeasReport of TS 36.331 [10]:

5>	set sigLogMeasConfigAvailable to false in the RRCSetupComplete message;

2>	if the UE has connection establishment failure or connection resume failure information available in VarConnEstFailReport or VarConnEstFailReportList and if the RPLMN is equal to plmn-Identity stored in VarConnEstFailReport or in at least one of the entries of VarConnEstFailReportList; or

2>	if the UE has connection establishment failure information or connection resume failure information available in VarConnEstFailReport or VarConnEstFailReportList and if the current registered SNPN identity is equal to snpn-Identity stored in VarConnEstFailReport or any entry of VarConnEstFailReportList:

3>	include connEstFailInfoAvailable in the RRCSetupComplete message;

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report, or

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report of TS 36.331 [10], and if the UE is capable of cross-RAT RLF reporting and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report of TS 36.331 [10]; or

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report and if the current registered SNPN identity is included in snpn-IdentityList stored in the VarRLF-Report:

3>	include rlf-InfoAvailable in the RRCSetupComplete message;

2>	if the UE has successful handover information available in VarSuccessHO-Report and if the RPLMN is included in plmn-IdentityList stored in VarSuccessHO-Report; or

2>	if the UE has successful handover information available in VarSuccessHO-Report and if the current registered SNPN identity is included in snpn-IdentityList stored in the VarSuccessHO-Report:

3>	include successHO-InfoAvailable in the RRCSetupComplete message;

2>	if the UE has successful PSCell change or addition information available in VarSuccessPSCell-Report and if the RPLMN is included in plmn-IdentityList stored in VarSuccessPSCell-Report; or

2>	if the UE has successful PSCell change or addition information available in VarSuccessPSCell-Report and if the current registered SNPN identity is included in snpn-IdentityList stored in the VarSuccessPSCell-Report:

3>	include successPSCell-InfoAvailable in the RRCSetupComplete message;

2>	if the UE supports storage of mobility history information and the UE has mobility history information available in VarMobilityHistoryReport:

3>	include the mobilityHistoryAvail in the RRCSetupComplete message;

2>	if the UE has at least one stored application layer measurement configuration with appLayerIdleInactiveConfig configured:

3>	include measConfigReportAppLayerAvailable in the RRCSetupComplete message;

2>	if the UE supports uplink RRC message segmentation of UECapabilityInformation according to the network indication rrc-SegAllowed:

3>	may include the ul-RRC-Segmentation in the RRCSetupComplete message;

2>	if the UE supports uplink RRC message segmentation of UECapabilityInformation according to the network indication rrc-MaxCapaSegAllowed:

3>	include the ul-RRC-MaxCapaSegments in the RRCSetupComplete message;

2>	if the RRCSetup is received in response to an RRCResumeRequest, RRCResumeRequest1 or RRCSetupRequest:

3>	if speedStateReselectionPars is configured in the SIB2:

4>	include the mobilityState in the RRCSetupComplete message and set it to the mobility state (as specified in TS 38.304 [20]) of the UE just prior to entering RRC_CONNECTED state;

2>	if SIB1 contains musim-CapRestrictionAllowed:

3>	if supported, include the musim-CapRestrictionInd in the RRCSetupComplete message upon determining it has temporary capability restriction;

2>	if the UE has flight path information available:

3>	include flightPathInfoAvailable;

1>	submit the RRCSetupComplete message to lower layers for transmission, upon which the procedure ends.

NOTE:	Upon reception of musim-CapRestrictionInd in RRCSetupComplete, it is up to network implementation to configure the UE with a limited configuration that is used until network sends RRCReconfiguration based on the actual restricted UE capabilities included in UEAssistanceInformation.

Annex 2 – TP for Proposal 2

5.3.13.4	Reception of the RRCResume by the UE

The UE shall:

1>	stop timer T319, if running;

1>	stop timer T319a, if running and consider SDT procedure is not ongoing;

1>	stop timer T380, if running;

1>	if T331 is running:

2>	stop timer T331;

2>	perform the actions as specified in 5.7.8.3;

1>	if the RRCResume includes the fullConfig:

2>	perform the full configuration procedure as specified in 5.3.5.11;

1>	else:

2>	if the RRCResume does not include the restoreMCG-SCells:

3>	release the MCG SCell(s) from the UE Inactive AS context, if stored;

2>	if the RRCResume does not include the restoreSCG:

3>	release the MR-DC related configurations (i.e., as specified in 5.3.5.10) from the UE Inactive AS context, if stored;

2>	restore the masterCellGroup, mrdc-SecondaryCellGroup, if stored, and pdcp-Config from the UE Inactive AS context;

2>	configure lower layers to consider the restored MCG and SCG SCell(s) (if any) to be in deactivated state;

1>	discard the UE Inactive AS context;

1>	store the used nextHopChainingCount value associated to the current KgNB;

1>	if the UE is configured to receive MBS multicast in RRC_INACTIVE:

2>	reset MAC;

1>	if sdt-MAC-PHY-CG-Config is configured:

2>	instruct the MAC entity to stop the cg-SDT-TimeAlignmentTimer, if it is running;

2>	instruct the MAC entity to start the timeAlignmentTimer associated with the PTAG indicated by tag-Id, if it is not running;

1>	if srs-PosRRC-Inactive is configured:

2>	instruct the MAC entity to stop inactivePosSRS-TimeAlignmentTimer, if it is running;

1>	if srs-PosRRC-InactiveValidityAreaNonPreConfig is configured; or

1>	if srs-PosRRC-InactiveValidityAreaPreConfigList is configured and if the cell is not listed in srs-PosConfigValidityArea:

2>	instruct the MAC entity to stop inactivePosSRS-ValidityAreaTAT, if it is running;

1>	release the suspendConfig except the ran-NotificationAreaInfo;

1>	if the RRCResume includes the masterCellGroup:

2>	perform the cell group configuration for the received masterCellGroup according to 5.3.5.5;

1>	if the RRCResume includes the mrdc-SecondaryCellGroup:

2>	if the received mrdc-SecondaryCellGroup is set to nr-SCG:

3>	perform the RRC reconfiguration according to 5.3.5.3 for the RRCReconfiguration message included in nr-SCG;

2>	if the received mrdc-SecondaryCellGroup is set to eutra-SCG:

3>	perform the RRC connection reconfiguration as specified in TS 36.331 [10], clause 5.3.5.3 for the RRCConnectionReconfiguration message included in eutra-SCG;

1>	if the RRCResume includes the radioBearerConfig:

2>	perform the radio bearer configuration according to 5.3.5.6;

1>	if the RRCResume message includes the sk-Counter:

2>	perform security key update procedure as specified in 5.3.5.7;

1>	if the RRCResume message includes the radioBearerConfig2:

2>	perform the radio bearer configuration according to 5.3.5.6;

1>	if the RRCResume message includes the needForGapsConfigNR:

2>	if needForGapsConfigNR is set to setup:

3>	consider itself to be configured to provide the measurement gap requirement information of NR target bands;

2>	else:

3>	consider itself not to be configured to provide the measurement gap requirement information of NR target bands;

1>	if the RRCResume message includes the needForGapNCSG-ConfigNR:

2>	if needForGapNCSG-ConfigNR is set to setup:

3>	consider itself to be configured to provide the measurement gap and NCSG requirement information of NR target bands;

2>	else:

3>	consider itself not to be configured to provide the measurement gap and NCSG requirement information of NR target bands;

1>	if the RRCResume message includes the needForGapNCSG-ConfigEUTRA:

2>	if needForGapNCSG-ConfigEUTRA is set to setup:

3>	consider itself to be configured to provide the measurement gap and NCSG requirement information of E-UTRA target bands;

2>	else:

3>	consider itself not to be configured to provide the measurement gap and NCSG requirement information of E-UTRA target bands;

1>	for each application layer measurement configuration with appLayerIdleInactiveConfig configured:

2>	if the RPLMN is not included in plmn-IdentityList in VarAppLayerPLMN-ListConfig:

3>	forward the measConfigAppLayerId and inform upper layers about the release of the application layer measurement configuration;

3>	release the application layer measurement configuration including its fields in the UE variables VarAppLayerIdleConfig and VarAppLayerPLMN-ListConfig;

3>	discard any application layer measurement reports which were not yet fully submitted to lower layers for transmission;

3>	consider itself not to be configured to send application layer measurement reports for the measConfigAppLayerId;

1>	if the RRCResume message includes the appLayerMeasConfig:

2>	if idleInactiveReportAllowed is included in the RRCResume message:

3>	if the UE is configured with at least one application layer measurement configuration with appLayerIdleInactiveConfig configured:

4>	initiate the procedure in 5.7.16.2 after the RRCResumeComplete has been transmitted;

2>	else:

3>	for each application layer measurement configuration with appLayerIdleInactiveConfig configured:

4>	forward the measConfigAppLayerId and inform upper layers about the release of the application layer measurement configuration;

4>	release the application layer measurement configuration including its fields in the UE variables VarAppLayerIdleConfig and VarAppLayerPLMN-ListConfig, if stored;

4>	discard any application layer measurement reports which were not yet fully submitted to lower layers for transmission;

4>	consider itself not to be configured to send application layer measurement reports for the measConfigAppLayerId;

2>	perform the application layer measurement configuration procedure as specified in 5.3.5.13d;

1>	if the RRCResume message includes the sl-L2RemoteUE-Config (i.e. the UE is a L2 U2N Remote UE):

2>	perform the L2 U2N Remote UE configuration procedure as specified in 5.3.5.16;

1>	if the RRCResume message includes the sl-ConfigDedicatedNR:

2>	perform the sidelink dedicated configuration procedure as specified in 5.3.5.14;

1>	resume SRB2 (if suspended), SRB3 (if configured), SRB4 (if configured), SRB5 (if configured), all DRBs (that are suspended) and multicast MRBs (that are suspended);

NOTE 1:	If the SCG is deactivated, resuming SRB3 and all DRBs does not imply that PDCP or RRC PDUs can be transmitted or received on SCG RLC bearers.

1>	if stored, discard the cell reselection priority information provided by the cellReselectionPriorities or inherited from another RAT;

1>	stop timer T320, if running;

1>	if the RRCResume message includes the measConfig:

2>	perform the measurement configuration procedure as specified in 5.5.2;

1>	resume measurements if suspended;

1>	if T390 is running:

2>	stop timer T390 for all access categories;

2>	perform the actions as specified in 5.3.14.4;

1>	if T302 is running:

2>	stop timer T302;

2>	perform the actions as specified in 5.3.14.4;

1>	enter RRC_CONNECTED;

1>	indicate to upper layers that the suspended RRC connection has been resumed;

1>	stop the cell re-selection procedure;

1>	stop relay reselection procedure if any for L2 U2N Remote UE;

1>	consider the current cell to be the PCell;

1>	set the content of the of RRCResumeComplete message as follows:

2>	if the upper layer provides NAS PDU, set the dedicatedNAS-Message to include the information received from upper layers;

2>	if upper layers provides a PLMN:

3>	if the UE is either allowed or instructed to access the PLMN via a cell for which at least one CAG ID is broadcast:

4>	set the selectedPLMN-Identity from the npn-IdentityInfoList;

3>	else:

4>	set the selectedPLMN-Identity to the PLMN selected by upper layers from the plmn-IdentityInfoList;

2>	if the masterCellGroup contains the reportUplinkTxDirectCurrent:

3>	include the uplinkTxDirectCurrentList for each MCG serving cell with UL;

3>	include uplinkDirectCurrentBWP-SUL for each MCG serving cell configured with SUL carrier, if any, within the uplinkTxDirectCurrentList;

2>	if the masterCellGroup contains the reportUplinkTxDirectCurrentTwoCarrier:

3>	include in the uplinkTxDirectCurrentTwoCarrierList the list of uplink Tx DC locations for the configured uplink carrier aggregation in the MCG;

2>	if the masterCellGroup contains the reportUplinkTxDirectCurrentMoreCarrier:

3>	include in the uplinkTxDirectCurrentMoreCarrierList the list of uplink Tx DC locations for the configured uplink carrier aggregation in the MCG;

2>	if the UE has idle/inactive measurement information concerning cells other than the PCell available in VarMeasIdleReport:

3>	if the idleModeMeasurementReq is included in the RRCResume message:

4>	if validatedMeasurementsReq is included in the RRCResume and measIdleValidityDuration is included in VarEnhMeasIdleConfig;

5>	set the measResultIdleEUTRA in the RRCResumeComplete message to the value of measReportIdleEUTRA in the VarMeasIdleReport for any valid measurement results, if available, and set validityStatus to the value of measIdleValidityDuration in VarEnhMeasIdleConfig;

5>	set the measResultIdleNR in the RRCResumeComplete message to the value of measReportIdleNR in the VarMeasIdleReport for any valid measurement results, if available, and set validityStatus to the value of measIdleValidityDuration in VarEnhMeasIdleConfig;

5>	discard the VarMeasIdleReport upon successful delivery of the RRCResumeComplete message is confirmed by lower layers;

4>	else:

5>	set the measResultIdleEUTRA in the RRCResumeComplete message to the value of measReportIdleEUTRA in the VarMeasIdleReport, if available;

5>	set the measResultIdleNR in the RRCResumeComplete message to the value of measReportIdleNR in the VarMeasIdleReport, if available;

5>	discard the VarMeasIdleReport upon successful delivery of the RRCResumeComplete message is confirmed by lower layers;

3>	else:

4>	if the SIB1 contains idleModeMeasurementsNR and the UE has NR idle/inactive measurement information concerning cells other than the PCell available in VarMeasIdleReport; or

4>	if the SIB1 contains idleModeMeasurementsEUTRA and the UE has E-UTRA idle/inactive measurement information available in VarMeasIdleReport:

5>	include the idleMeasAvailable;

2>	if the reselectionMeasurementReq is included in the RRCResume message:

3> if validatedMeasurementsReq is included in the RRCResume and measReselectionValidityDuration is included in VarMeasReselectionConfig:

4>	if measReselectionCarrierListNR is present in VarMeasReselectionConfig:

5>	if the UE has valid cell reselection measurements results for any frequency listed in measReselectionCarrierListNR in VarMeasRelectionConfig:

6>	set the measResultReselectionNR in the RRCResumeComplete message to the valid NR measurement results, if available for any frequency listed in measReselectionCarrierListNR in VarMeasReselectionConfig and set validityStatus to the value of measReselectionValidityDuration in VarMeasReselectionConfig;

4>	else:

5>	if the UE has valid NR cell reselection measurements results:

6>	set the measResultReselectionNR in the RRCResumeComplete message to any available valid NR measurement results, if available, and set validityStatus to the value of measReselectionValidityDuration in VarMeasReselectionConfig;

3> else:

4>	if measReselectionCarrierListNR is present in VarMeasReselectionConfig:

5>	if the UE has cell reselection measurements results for any frequency listed in measReselectionCarrierListNR in VarMeasRelectionConfig:

6>	set the measResultReselectionNR in the RRCResumeComplete message to the NR measurement results, if available for any frequency listed in measReselectionCarrierListNR in VarMeasReselectionConfig;

4>	else:

5>	if the UE has NR cell reselection measurements results:

6>	set the measResultReselectionNR in the RRCResumeComplete message to any available NR measurement results, if available;

2>	else:

3>	if the SIB1 contains reselectionMeasurementsNR:

4>	if measReselectionCarrierListNR is present in VarMeasReselectionConfig and the UE has NR reselection measurements available for any frequency listed in measReselectionCarrierListNR in VarMeasReselectionConfig; or

4>	if measReselectionCarrierListNR is not present in VarMeasReselectionConfig and if the UE has NR reselection measurements available:

5>	include the reselectionMeasAvailable;

2>	if the RRCResume message includes mrdc-SecondaryCellGroup set to eutra-SCG:

3>	include in the eutra-SCG-Response the E-UTRA RRCConnectionReconfigurationComplete message in accordance with TS 36.331 [10] clause 5.3.5.3;

2>	if the RRCResume message includes mrdc-SecondaryCellGroup set to nr-SCG:

3>	include in the nr-SCG-Response the SCG RRCReconfigurationComplete message;

2>	if the UE has logged measurements available for NR and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport; or

2>	if the UE has logged measurements available for NR and if the current registered SNPN identity is included in snpn-ConfigID-List stored in VarLogMeasReport:

3>	include the logMeasAvailable in the RRCResumeComplete message;

3>	if Bluetooth measurement results are included in the logged measurements the UE has available for NR:

4>	include the logMeasAvailableBT in the RRCResumeComplete message;

3>	if WLAN measurement results are included in the logged measurements the UE has available for NR:

4>	include the logMeasAvailableWLAN in the RRCResumeComplete message;

2>	if the sigLoggedMeasType in VarLogMeasReport is included; or

2>	if the UE supports the override protection of the signalling based logged MDT for inter-RAT (i.e. LTE to NR), and if the sigLoggedMeasType in VarLogMeasReport of TS 36.331 [10] is included:

3>	if T330 timer is running (associated to the logged measurement configuration for NR or for LTE):

4>	set sigLogMeasConfigAvailable to true in the RRCResumeComplete message;

3>	else:

4>	if the UE has logged measurements in VarLogMeasReport or in VarLogMeasReport of TS 36.331 [10]:

5>	set sigLogMeasConfigAvailable to false in the RRCResumeComplete message;

2>	if the UE has connection establishment failure or connection resume failure information available in VarConnEstFailReport or VarConnEstFailReportList and if the RPLMN is equal to plmn-Identity stored in VarConnEstFailReport or in at least one of the entries of VarConnEstFailReportList; or

2>	if the UE has connection establishment failure information or connection resume failure information available in VarConnEstFailReport or VarConnEstFailReportList and if the registered SNPN identity is equal to snpn-Identity in networkIdentity stored in VarConnEstFailReport or any entry of VarConnEstFailReportList:

3>	include connEstFailInfoAvailable in the RRCResumeComplete message;

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report; or

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report of TS 36.331 [10] and if the UE is capable of cross-RAT RLF reporting and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report of TS 36.331 [10]; or

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report and if the current registered SNPN identity are included in snpn-IdentityList stored in VarRLF-Report; or

3>	include rlf-InfoAvailable in the RRCResumeComplete message;

2>	if the UE has successful PSCell change or addition related information available in VarSuccessPSCell-Report and if the RPLMN is included in plmn-IdentityList stored in VarSuccessPSCell-Report; or

2>	if the UE has successful PSCell change or addition information available in VarSuccessPSCell-Report and if the current registered SNPN identity is included in snpn-IdentityList stored in the VarSuccessPSCell-Report:

3>	include successPSCell-InfoAvailable in the RRCResumeComplete message;

2>	if the UE has successful handover information available in VarSuccessHO-Report and if the RPLMN is included in plmn-IdentityList stored in VarSuccessHO-Report; or

2>	if the UE has successful handover information available in VarSuccessHO-Report and if the current registered SNPN identity is included in snpn-IdentityList stored in the VarSuccessHO-Report:

3>	include successHO-InfoAvailable in the RRCResumeComplete message;

2>	if the UE supports storage of mobility history information and the UE has mobility history information available in VarMobilityHistoryReport:

3>	include the mobilityHistoryAvail in the RRCResumeComplete message;

2>	if speedStateReselectionPars is configured in the SIB2:

3>	include the mobilityState in the RRCResumeComplete message and set it to the mobility state (as specified in TS 38.304 [20]) of the UE just prior to entering RRC_CONNECTED state;

2>	if the UE has at least one stored application layer measurement configuration with appLayerIdleInactiveConfig configured:

3>	include measConfigReportAppLayerAvailable in the RRCResumeComplete message;

2>	if the UE is configured to provide the measurement gap requirement information of NR target bands:

3>	include the NeedForGapsInfoNR and set the contents as follows:

4> include intraFreq-needForGap and set the gap requirement information of intra-frequency measurement for each NR serving cell;

4>	if requestedTargetBandFilterNR is configured, for each supported NR band that is also included in requestedTargetBandFilterNR, include an entry in interFreq-needForGap and set the gap requirement information for that band; otherwise, include an entry in interFreq-needForGap and set the corresponding gap requirement information for each supported NR band;

3>	if the needForInterruptionConfigNR is enabled:

4>	include the needForInterruptionInfoNR and set the contents as follows:

5>	include intraFreq-needForInterruption with the same number of entries, and listed in the same order, as in intraFreq-needForGap;

5> for each entry in intraFreq-needForInterruption:

6>	include interruptionIndication and set the interruption requirement information if the corresponding entry in intraFreq-needForGap is set to no-gap;

5>	include interFreq-needForInterruption with the same number of entries, and listed in the same order, as in interFreq-needForGap;

5>	for each entry in interFreq-needForInterruption:

6>	include interruptionIndication and set the interruption requirement information if the corresponding entry in interFreq-needForGap is set to no-gap;

2>	if the UE is configured to provide the measurement gap and NCSG requirement information of NR target bands:

3>	include the NeedForGapNCSG-InfoNR and set the contents as follows:

4> include intraFreq-needForNCSG and set the gap and NCSG requirement information of intra-frequency measurement for each NR serving cell;

4>	if requestedTargetBandFilterNCSG-NR is configured:

5>	for each supported NR band included in requestedTargetBandFilterNCSG-NR, include an entry in interFreq-needForNCSG and set the NCSG requirement information for that band;

4>	else:

5>	include an entry for each supported NR band in interFreq-needForNCSG and set the corresponding NCSG requirement information;

2>	if the UE is configured to provide the measurement gap and NCSG requirement information of E-UTRA target bands:

3>	include the NeedForGapNCSG-InfoEUTRA and set the contents as follows:

4>	if requestedTargetBandFilterNCSG-EUTRA is configured:

5>	for each supported E-UTRA band included in requestedTargetBandFilterNCSG-EUTRA, include an entry in needForNCSG-EUTRA and set the NCSG requirement information for that band;

4>	else:

5>	include an entry for each supported E-UTRA band in needForNCSG-EUTRA and set the corresponding NCSG requirement information;

2>	if SIB1 contains musim-CapRestrictionAllowed:

3>	if supported, include the musim-CapRestrictionInd in the RRCResumeComplete message upon determining it has temporary capability restriction;

2>	if the UE has flight path information available:

3>	include flightPathInfoAvailable;

1>	submit the RRCResumeComplete message to lower layers for transmission;

1>	the procedure ends.

NOTE 2:	Network only configures at most one of reportUplinkTxDirectCurrent, reportUplinkTxDirectCurrentTwoCarrier or reportUplinkTxDirectCurrentMoreCarrier in one RRC message.

NOTE 3:	Upon reception of musim-CapRestrictionInd in RRCResumeComplete, it is up to network implementation to configure the UE with a limited configuration that is used until network sends RRCReconfiguration based on the actual restricted UE capabilities included in UEAssistanceInformation.

Annex 3 – TP for Proposal 3

5.7.8.1a	Measurement configuration

The purpose of this procedure is to update the idle/inactive measurement configuration.

The UE initiates this procedure while T331 is running and SDT procedure is not ongoing and one of the following conditions is met:

1>	upon selecting a cell when entering RRC_IDLE or RRC-INACTIVE from RRC_CONNECTED or RRC_INACTIVE; or

1>	upon update of system information (SIB4, or SIB11), e.g. due to intra-RAT cell (re)selection;

While in RRC_IDLE or RRC_INACTIVE, and T331 is running, the UE shall:

1>	if VarMeasIdleConfig includes neither a measIdleCarrierListEUTRA nor a measIdleCarrierListNR received from the RRCRelease message:

2>	if the UE supports idleInactiveEUTRA-MeasReport:

3>	if the SIB11 includes the measIdleConfigSIB and contains measIdleCarrierListEUTRA:

4>	store or replace the measIdleCarrierListEUTRA of measIdleConfigSIB of SIB11 within VarMeasIdleConfig;

3>	else:

4>	remove the measIdleCarrierListEUTRA in VarMeasIdleConfig, if stored;

2>	if the UE supports idleInactiveNR-MeasReport:

3>	if SIB11 includes the measIdleConfigSIB and contains measIdleCarrierListNR:

4>	store or replace the measIdleCarrierListNR of measIdleConfigSIB of SIB11 within VarMeasIdleConfig;

3>	else:

4>	remove the measIdleCarrierListNR in VarMeasIdleConfig, if stored;

1>	if VarEnhMeasIdleConfig does not include measIdleValidityDuration received from the RRCRelease message:

2>	if the UE supports measValidationReportEMR:

3>	if SIB11 includes the measIdleConfigSIB and contains measIdleValidityDuration:

4>	store or replace the measIdleValidityDuration of measIdleConfigSIB of SIB11 within VarEnhMeasIdleConfig;

3>	else:

4>	remove the measIdleValidityDuration in VarEnhMeasIdleConfig, if stored;

1>	for each entry in the measIdleCarrierListNR within VarMeasIdleConfig that does not contain an ssb-MeasConfig received from the RRCRelease message:

2>	if there is an entry in measIdleCarrierListNR in measIdleConfigSIB of SIB11 that has the same carrier frequency and subcarrier spacing as the entry in the measIdleCarrierListNR within VarMeasIdleConfig and that contains ssb-MeasConfig:

3>	delete the ssb-MeasConfig of the corresponding entry in the measIdleCarrierListNR within VarMeasIdleConfig;

3>	store the SSB measurement configuration from SIB11 into nrofSS-BlocksToAverage, absThreshSS-BlocksConsolidation, smtc, ssb-ToMeasure, deriveSSB-IndexFromCell, and ss-RSSI-Measurement within ssb-MeasConfig of the corresponding entry in the measIdleCarrierListNR within VarMeasIdleConfig;

2>	else if there is an entry in interFreqCarrierFreqList of SIB4 with the same carrier frequency and subcarrier spacing as the entry in measIdleCarrierListNR within VarMeasIdleConfig:

3>	delete the ssb-MeasConfig of the corresponding entry in the measIdleCarrierListNR within VarMeasIdleConfig;

3>	store the SSB measurement configuration from SIB4 into nrofSS-BlocksToAverage, absThreshSS-BlocksConsolidation, smtc, ssb-ToMeasure, deriveSSB-IndexFromCell, and ss-RSSI-Measurement within ssb-MeasConfig of the corresponding entry in the measIdleCarrierListNR within VarMeasIdleConfig;

2>	else:

3>	remove the ssb-MeasConfig of the corresponding entry in the measIdleCarrierListNR within VarMeasIdleConfig, if stored;

1>	perform measurements according to 5.7.8.2a.

5.7.8.1b	Measurement configuration (reselection measurements)

The purpose of this procedure is to update the reselection measurement configuration.

The UE initiates this procedure while SDT procedure is not ongoing and one of the following conditions is met:

1>	upon selecting a cell when entering RRC_IDLE or RRC_INACTIVE from RRC_CONNECTED or RRC_INACTIVE; or

1>	upon update of system information (SIB11), e.g., due to intra-RAT cell (re)selection;

While in RRC_IDLE or RRC_INACTIVE, the UE shall:

1>	if VarMeasReselectionConfig does not include measReselectionCarrierListNR received from the RRCRelease message:

2>	if the UE supports reselection measurement reporting:

3>	if SIB11 includes the measIdleConfigSIB and contains measReselectionCarrierListNR:

4>	store or replace the measReselectionCarrierListNR of measIdleConfigSIB of SIB11 within VarMeasReselectionConfig;

3>	else:

4>	remove the measReselectionCarrierListNR in VarMeasReselectionConfig, if stored;

1>	if VarMeasReselectionConfig does not include measReselectionValidityDuration received from the RRCRelease message:

2>	if the UE supports reselection measurement reporting:

3>	if SIB11 includes the measIdleConfigSIB and contains measReselectionValidityDuration:

4>	store or replace the measReselectionValidityDuration of measIdleConfigSIB of SIB11 within VarMeasReselectionConfig;

3>	else:

4>	remove the measReselectionValidityDuration in VarMeasReselectionConfig, if stored.

Annex 4 – TP for Proposal 5

#### –	VarLTM-ServingCellNoResetID

The IE VarLTM-ServingCellNoResetID is used to store the ID associated with the serving cell based on which the UE determines whether a L2 reset is needed or not upon an LTM cell switch procedure.

VarLTM-ServingCellNoResetID UE variable

-- ASN1START

-- TAG-VARLTM-SERVINGCELLNORESETID-START

VarLTM-ServingCellNoResetID-r18 ::=     SEQUENCE {

ltm-ServingCellNoResetID-r18            INTEGER (1..maxNrofLTM-Configs-plus1-r18)                         OPTIONAL

}

-- TAG-VARLTM-SERVINGCELLNORESETID-STOP

-- ASN1STOP

#### –	VarLTM-ServingCellUE-MeasuredTA-ID

The IE VarLTM-ServingCellUE-MeasuredTA-ID is used to store the ID associated with the serving cell based on which the UE determines whether UE-based TA measurements are needed or not.

VarLTM-ServingCellUE-MeasuredTA-ID UE variable

-- ASN1START

-- TAG-VARLTM-SERVINGCELLUE-MEASUREDTA-ID-START

VarLTM-ServingCellUE-MeasuredTA-ID-r18 ::=    SEQUENCE {

ltm-ServingCellUE-MeasuredTA-ID-r18           INTEGER (1..maxNrofLTM-Configs-plus1-r18)                    OPTIONAL

}

-- TAG-VARLTM-SERVINGCELLUE-MEASUREDTA-ID-STOP

-- ASN1STOP