3GPP TSG RAN WG2#129bis	R2-2501755

Wuhan, China, 7th - 11th April 2025

3GPP TSG-SA2 Meeting #167	S2-2502787

Athens, Greece, February 17 – 21, 2025

Title:	LS on NR Femto node shared by PLMN and PNI-NPN

Response to:	-

Release:	Rel-19

Work Item:	5G_Femto, NR_WAB_5GFemto-Core, eNPN

Source:	SA2

To:	RAN3

Cc:	RAN2

Contact person:	Myungjune Youn

m.youn@lge.com

Send any reply LS to: 3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

# Overall description

SA2 discussed access control description agreed in RAN3, as captured in RAN3 BL CR to TS 38.300 (R3-247936).

- The NR Femto node may activate a PLMN cell, which can be accessed by legacy UE without access control of CAG. - The NR Femto may activate a cell shared by both PLMN and PNI-NPN, through broadcasting both the plmn-IdentityInfoList and the npn-IdentityInfoList-r16 in the SIB1, but without the cellReservedForOtherUse. Then, this cell is accessible to UEs which have the allowed CAG list including a CAG-ID broadcasted by the cell. For the legacy UE not supporting CAG, this cell is viewed as a normal PLMN cell. - The NR Femto node may activate an NPN-only cell by broadcasting the cellReservedForOtherUse IE with value “true”, then this cell can only be accessed by the UEs whose allowed CAG list includes a CAG-ID broadcasted by the cell.

Among above three options, SA2 seeks some clarifications regarding the second option.

As described in TS 23.501 clause 5.30.3.4 for PNI-NPN, the AMF and NG-RAN would reject the UE if none of the CAG ID of the cell are part of the UE's Allowed CAG list.

As described in TS 23.501 clause 5.18, the MOCN sharing between the PLMN and PNI-NPN is achieved by using different cell IDs with separate configuration.

Therefore, SA2 would like to understand whether the second option in the RAN3 BL CR for NR Femto contradicts with the existing SA2 specifications.

# Actions:

To RAN3:

SA2 kindly asks RAN3 to clarify whether the second option in the RAN3 BL CR has any contradictions with the existing definitions in SA2 specifications regarding PNI-NPN and Network Sharing.

# Date of Next TSG-SA2 Meetings:

SA WG2#168	07 – 11, April 2025	Goteborg, Sweden

SA WG2#169	19 – 23, May 2025	Fukuoka, Japan