3GPP TSG-SA5 Meeting #159	S5-250343

Sophia-Antipolis, FR, 17 - 21 February 2025

# Source:	China Telecom, Huawei, China Unicom, CATT, Ericsson, ZTE, Intel

# Title:	DP on number of UEs in RRC_INACTIVE state with data transmission

# Document for:	Endorsement

Agenda Item:	5.3

1	Decision/action requested

The group is asked to endorse the detailed proposal in section 4.

2	References

[1]	S5-245138(R2-2409280): “LS on Number of UEs in RRC_INACTIVE state with data transmission”.

[2]	S5-246405: “Reply to SA5 LS on Number of UEs in RRC_INACTIVE state with data transmission”.

[3]	3GPP TS 38.314: “NG-RAN; NG Application Protocol (NGAP)”.

[4]	RP-222364 Work Item on NR small data transmissions in INACTIVE state

3	Rationale

3.1	Background

In the meeting SA5 #156, SA5 discussed whether ‘active UEs’ could include UEs in RRC_INACTIVE state (in S5-243676) and sent the liaison S5-245138 to RAN2 to evaluate whether there is a possibility to specify the number of UEs in RRC_INACTIVE state with data transmission.

RAN2 confirms in R2-2409280(S5-245138) that the network knows the number of UEs in RRC_INACTIVE state in a cell with an ongoing SDT session as the connection requests from such UEs are distinguished either based on dedicated RACH resources or based on a dedicated RRC resume cause. Meanwhile, RAN2 respectfully asks SA5 to take the above into consideration in their further work and inform RAN2 if and which L2 metrics SA5 would like RAN2 to specify.

3.2	Anlysis and observations of requirement

Perspective of specification update

NR supports RRC_INACTIVE state and UEs with small data transmission are generally maintained by the network in the RRC_INACTIVE state. SDT can transmit UL SDT data and/or UL SDT signalling by MO-SDT or DL user data and/or DL NAS signalling by MT-SDT. And it is used for user data over DRBs or for signalling over SRBs. It is understood that the qualifier ‘in RRC_CONNECTED’ is added to the title of 4.1.3.2 in TS 38.314 because UEs can only transmit data in the RRC_CONNECTED state until Rel-16. To keep up with SDT feature updates, the "number of active users" needs to be expanded to include the RRC_INACTIVE state.

Perspective of industry requirement

According to GSMA 2024, while cellular networks currently handle around 15% of total IoT connections, the rapid expansion of the IoT market provides significant room for growth in the cellular IoT space. Accordingly, the number of licensed cellular IoT connections is expected to reach 5.8 billion globally by 2030. IoT terminals usually use SDT feature to transmit data, so the number of UEs in RRC_INACTIVE state with data transmission cannot be ignored.

Limited impact of occasional unsuccessful SDT procedure

Regarding the statement mentioned in R2-2409280 that occasionally an SDT procedure may be unsuccessfully completed, we consider that as long as the SDT is initiated, the network allocates resources to the user and the resources are occupied. As a result, regardless of whether the SDT process is completed or not, it will have an impact on the network congestion and the resource occupancy. Furthermore, most of the non-smartphone IoT terminals rarely move, so the "occasionally unsuccessful completion of SDT procedure" scenario described in R2-2409280 is virtually impossible to occur.

Besides, measurements can be implemented based on the statistics of a specific area consisting of some cells. Therefore, “occasionally an SDT procedure may be unsuccessfully completed” mentioned in R2-2409280 will have no impact on the use cases in SA5.

If still concerned, to address the pitfalls caused by the SDT process occasionally not be completed successfully, one solution is to add the following description to the new defined measurement: “For this measurement, the expected accuracy is dependent on application scenario, cell load, UE configuration and how DRBs are distributed over logical channel groups” in TS 38.314.

3.3	Usecase

To reduce power consumption and signaling overhead, NR supports RRC_INACTIVE state and UEs with infrequent (periodic and/or non-periodic) data transmission are generally maintained by the network in the RRC_INACTIVE state. Specific examples of small and infrequent data traffic include the following applications in RP-222364:

- Smartphone applications:

  - Traffic from Instant Messaging services (whatsapp, QQ, wechat etc)

  - Heart-beat/keep-alive traffic from IM/email clients and other apps

  - Push notifications from various applications

- Non-smartphone applications:

  - Traffic from wearables (periodic positioning information etc)

  - sensors (Industrial Wireless Sensor Networks transmitting temperature, pressure readings periodically or in an event triggered manner etc)

  - smart meters and smart meter networks sending periodic meter readings

- usecase 1：The number of UEs with infrequent and non-periodic data transmission can predict the number of users that may be converted to RRC_CONNECTED state in the future, helping to analyse network capacity.

- usecase 2：Non-smartphone usually use SDT procedure with periodic data transmission in RRC_INACTIVE state. If UEs are accessing the network in large numbers when the cell is busy and the resources are being heavily utilised, then this is indicative of RACH congestion leading to increased latency. This metrics can judge the propriety of the periodicity time.

- usecase 3：Monitoring the percentage level of the number of UEs in RRC_CONNECTED state and RRC_INACTIVE state (with data transmission) during the observation period (e.g., daily, weekly, monthly) is useful for operators to obtain congestion information to balance the cell load for UEs in different RRC states.

3.4 Summary

In the current 5G network, the number of UEs using SDT can be huge, especially in IoT scenarios, where the total amount of SDT data transmission can be very high, which may lead to potential network congestion or contribute to network congestion to some extent. There are benefits for operators to monitor the dynamics of the network state caused by these UEs with SDT transmitting, to help analyze the network conditions. For instance, the percentage of the network workload contributed by SDT transmission, and the SDT traffic volume of UEs in a cell given that handover is not considered in most IoT scenarios.

As for the occasional unsuccessful SDT procedure, the impact is very small given that the likelihood of a inter-cell move occurring before the SDT is complete is very small, and can be overcome in SA5 by area statistics. Moreover, the specific definition in the tables of TS 38.314 clause 4.2.1.3 does not identify and restrict that the UE is in the RRC_CONNECTED state, but instead can be used generically for both the connected state and inactive state. The wording ‘in RRC_CONNECTED’ in the title of clause 4.2.1.3 may cause confusion.

4	Detailed proposal

Proposal #1: It is useful to define counters/subcounters to monitor the number of UEs in RRC_INACTIVE state with data transmission in TS 28.552.

Proposal #2: Reply LS R2-2409280(S5-246405) to RAN2 asking for cooperation, and take the analysis above into consideration when drafting the reply LS.