3GPP TSG-RAN WG2 #72	R2- 106618

Jacksonville, USA, November 15th – 19th 2010

Agenda Item:	7.2.2

Source: 	Samsung

Title:  	Handling layer1 parameters in RNReconfiguration message

Document for:	Discussion and decision

# Introduction

RAN1 has sent an LS listing the layer 1 parameters that needs to be configured in the RNReconfiguration message [3]. The list of parameters also contains a “Rate-of-change” indication providing how often the parameter is assumed to be updated. In this document we provide the ASN structure for the RNReconfiguration message incorporating the layer 1 parameters.

# Discussion

Based on the Layer 1 parameters the RN-SubframeConfig message could be as indicated below:

#### –	RN-SubframeConfig

The IE RN-SubframeConfig is used to specify the subframe configuration for RN.

RN-SubframeConfig information element

-- ASN1START

RN-SubframeConfig-r10 ::=		SEQUENCE {

phySubframeConfig-r10		PhySubframeConfigList-r10,	        OPTIONAL,	    -- FFS

rpdcch-Config-r10	               Rpdcch-Config-r10		        OPTIONAL,      ---FFS

pucch-Config-r10		               Rpucch-Config-r10		        OPTIONAL,     --- FFS

backhaul-DL-StartSymbol-r10           Backhaul-DL-StartSymbol-r10                 OPTIONAL,     --- FFS

...

}

PhySubframeConfigList-r10 ::=		SEQUENCE (SIZE (1..maxRN-Config-r10)) OF

PhySubframeConfig			    –- FFS

PhySubframeConfig ::=       		 CHOICE {

backhaul-TDD-SubframeConfig	INTEGER(0..18) ,

backhaul-FDD-DlSubframeConfig	INTEGER(0..127)

}

Rpdcch-Config-r10::=		SEQUENCE {

rPDCCH-resourceAllocation-type    			ENUMERATED(type0, type1, type2)	--FFS

rPDCCH-resourceAllocation-VrbToPrbMapping	          ENUMERATED(lvrb, dvrb)	               --FFS

rPDCCH-resourceAllocation-ResourceBlockAssignment       BIT STRING FFS                                   --FFS,

rPDCCH-interleaving				          ENUMERATED(interleaving, no interleaving) --FFS

rPDCCH-demodulationRs				          ENUMERATED(crs, dmrs)                 --FFS

}

Rpucch-Config-r10::=		SEQUENCE {

backhaul-PUCCH-ackNackResource-port0			INTEGER(0…2047)		--FFS

backhaul-PUCCH-ackNackResource-port1		 	INTEGER(0…2047)		--FFS

}

Backhaul-DL-StartSymbol-r10::=		SEQUENCE {

backhaul-DL-StartSymbol-r10                                              ENUMERATED(type1, type2, type3)  ---FFS

}

-- ASN1STOP

# Conclusion

From the LS from RAN 1, it is clear that some parameters are expected to change much frequently compared to others. Some parameters are a function of the number of users in an RN cell. An RNSubframeconfiguration structure has been proposed to initiate further discussions.

Proposal: Finalize the RNReconfiguration message structure based on the LS from RAN1 and discuss possible optionality issues further.

# References

RAN2#71bis Meeting Notes, Chairman

3GPP TS 36.216, Physical layer for relaying operation

R1-105819, LS on physical layer parameters for relay-specific advancements to be configured by higher layers

# ANNEX A:

The following is the layer1 parameter table provide in the LS

## Resource partitioning

| Parameter name | Description | Size [bits] | Values | Rate-of-change |
| --- | --- | --- | --- | --- |
| Backhaul-TDD-SubframeConfig | Backhaul subframe configuration (TDD) | 5 | 0…18 | Low |
| Backhaul-FDD-DlSubframeConfig | DL Backhaul subframe configuration (FDD) , | 8 | 8-bit bitmap | Low |

## Timing

| Parameter name | Description | Size [bits] | Values | Rate-of-change |
| --- | --- | --- | --- | --- |
| Backhaul-DL-StartSymbol | Start symbol index of DL backhaul subframes, | 2 | 1,2,3 |  |

## R-PDCCH

| Parameter name | Description | Size [bits] | Values | Rate-of-change |
| --- | --- | --- | --- | --- |
| RPDCCH-resourceAllocation-Type | Resource allocation type for configuration of the set of RBs for potential R-PDCCH transmission, | 2 | 0,1,2 |  |
| RPDCCH-resourceAllocation-VrbToPrbMapping | Virtual to physical resource block mapping for configuration of the set of RBs for potential R-PDCCH transmission. Applicable to RPDCCH-resourceAllocation-Type = 2 only. | 1 | LVRB / DVRB |  |
| RPDCCH-resourceAllocation-ResourceBlockAssignment | Resource block assignment for configuration of the set of RBs for potential R-PDCCH transmission | see [2] | bitmap equivalent to corresponding field in DCI, |  |
| RPDCCH-interleaving | Interleaving mode of R-PDCCH, | 1 | interleaving / no interleaving | Low |
| RPDCCH-demodulationRs | RS used for R-PDCCH demodulation. Applicable to RPDCCH-interleaving = no interleaving only. | 1 | CRS / DMRS | Low |

## PUCCH

| Parameter name | Description | Size [bits] | Values | Rate-of-change |
| --- | --- | --- | --- | --- |
| Backhaul-PUCCH-ackNackResource-port0 | PUCCH resource for ACK/NACK feedback on antenna port p=0, | 11 | 0…2047 |  |
| Backhaul-PUCCH-ackNackResource-port1 | PUCCH resource for ACK/NACK feedback on antenna port p=1, | 11 | 0…2047 |  |