3GPP TSG-RAN2#73 BIS	R2-112396

# Shanghai, China, 11th ~ 15th April 2011

| CR-Form-v9.7 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 25.331 | CR | CRNum | rev | - | Current version: | 10.2.0 |  |
| For HELP on using this form look at the pop-up text over the symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Optionality of Enhanced Security Mode procedure handling in case of delayed L2 ACK |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI-10 |  |  |  |  |  |  | Date: |  | 28/03/2011 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-10 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) |  |
| Reason for change: |  | As agreed in R2-106774, CR4427 to 25.331 introduces two subfeatures: - Integrity check on SRB3 messages when the UE misses the L2 ACK for RRC SECURITY MODE COMPLETE message: - Optional at the UE - No impact on the network (if the network suspends messages while L2 Ack is pending) - Report of aborting an ongoing security mode procedure via the IE "Security Revert Status Indicator” - Mandatory at the UE - The network needs either a network workaround or to understand the flag. - Making it optional at the UE will not break the interoperability Also, as agreed in RP-110459, the extended SMC feature should not be mandatory for Rel-10. RAN2 can discuss detailed mechanisms to avoid IOT issue (e.g., through addition of a capability bit or modification of the signalling). |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR introduces the IE "Security Revert Status Capability" in "Security capability". When this IE is set to TRUE, it means that the UE has the capability to report when it reverts back to the old security configuration due to abortion of the ongoing security mode command. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The network may not know which UEs are capable of this indication. If the network understand the IE Security Revert Status Indicator”, but the UE does not implement it, the network may still choose to run alternative workarounds. Even though a workaround has its drawbacks, but it is still better than guessing whether the UE has not included the IE because it did not revert, or because it hasn't implemented this new feature. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.3.3.37, 11.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
|  |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | This CR is based on v10.2 of TS25.331. The latest version of the specification was not available at the time of writing of this CR. |  |  |  |  |  |  |  |  |

#### 10.3.3.37	Security capability

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Ciphering algorithm capability | MP |  |  |  |  |
| >UEA0 | MP |  | Boolean |  |  |
| >UEA1 | MP |  | Boolean |  |  |
| >UEA2 | MP |  | Boolean | The value TRUE means that UEA2 is fully interoperability tested. If fully interoperability tested, also "Security capability indication" of "UE radio access capability compressed" shall be set to TRUE. | REL-7 |
| >Spare | MP | 13 | Boolean | Shall be set to FALSE by UEs complying with this version of the protocol. |  |
| Integrity protection algorithm capability | MP |  |  |  |  |
| >UIA1 | MP |  | Boolean | The value TRUE means that UIA1, Kasumi, is supported |  |
| >UIA2 | MP |  | Boolean | The value TRUE means that UIA2 is fully interoperability tested. If fully interoperability tested, also "Security capability indication" of "UE radio access capability compressed" shall be set to TRUE. | REL-7 |
| >Spare | MP | 14 | Boolean | Shall be set to FALSE by UEs complying with this version of the protocol. |  |
| Security Revert Status Capability | CV-NotInSMC |  | Enumerated (TRUE) | The value TRUE means that the UE is able to report when it has reverted back to the old security configuration by aborting the ongoing security mode command | REL-10 |

| Condition | Explanation |
| --- | --- |
| CV-NotInSMC | This IE is not needed in the Security Mode Command, otherwise, it is optionally present. |

[… next change…]

## 11.3	Information element definitions

InformationElements DEFINITIONS AUTOMATIC TAGS ::=

[… next change…]

UE-RadioAccessCapability-vaxyext ::= SEQUENCE {

ue-RadioAccessCapabBandFDDList4			UE-RadioAccessCapabBandFDDList4,

ue-securityRevertStatusCapability			ENUMERATED { true }					OPTIONAL

}

[… end of changes…]