3GPP TSG RAN WG2 Meeting #77Bis	R2-121536

Jeju, South Korea, 26-30 March 2012

Agenda Item:		7.10.4

Source: 	Samsung

Title: 	Mobility State Estimation Enhancements

Document for:	Discussion and Decision

# Introduction

One of the objectives of the SI “Study on HetNet mobility enhancements for LTE” [1] is: Evaluate performance benefits of enhanced UE mobility state estimation and related functionalities, and other possible mobility solutions to take different cell-sizes into account.

In this contribution, potential issues in HetNet scenario are first analysed for the current mobility state estimation (MSE) mechanism. Possible enhancements are discussed along with simulation results.

# Discussion

In the release 8 MSE, UEs count handovers/cell re-selections during time period T_Evaluation and T_HystNormal. A UE detects its mobility state based on the number of handovers/cell re-selections. If the number of handovers/cell re-selections during the time period T_Evaluation exceeds NCR_H then the UE enters High-mobility state. If the number of handovers/cell re-selections during the time period T_Evaluation exceeds NCR_M but does not exceed NCR_H, then the UE enters Medium-mobility state. If either Medium or High-mobility state is not detected during time period T_HystNormal, then the UE enters Normal-mobility state. For connected mode UE scales the timeToTrigger (TTT) or Treselection (idle mode) based on its mobility state and if High-mobility state is detected, TTT is scaled by a SF_H value. For Medium-mobility state, TTT is scaled by SF_M and for normal mobility no scaling is applied.

The expected behaviour of Rel 8 MSE in HETNET with increased density of pico cell deployments is that it would increase the handover count resulting in wrong mobility state estimation (increased medium or high state) and the related scaling of TTT would result in increase in ping-pong rates (unnecessary handover). Scaling of TTT would also result in increase in short Time of Stay (TOS) rates.

In this contribution we have considered a simple enhancement to the Rel 8 MSE (applicable only for Hetnet) where Handovers to and from pico cells are not counted. Only the macro to macro handover counts are used for mobility state estimation [2]. The expected behaviour of this enhancement is that the mobility state estimation would be similar to homogeneous network and the Ping-pong rates and Short TOS rates are expected to reduce.

## Simulation

The performance results with counting only the Macro to Macro handovers for the mobility state estimation in a HETNET scenario is provided in this section.

For the simulations, the Evaluation Metrics are as per TR 36.839 and the all radio parameters are according to TR 36.814. The mobility specific parameters are listed in Table1.

Table 1: Configurations for the HetNet mobility simulation

| Profile | Value |
| --- | --- |
| UE speed [km/h] | 30 or 60 |
| TTT [ms] | 200 |
| A3 offset [dB] | 1 |
| L1 to L3 period [ms] | 100 |
| RSRP L3 Filter K | 1 |
| T_Evaluation (s) | 60 |
| NCR_M (Medium mobility count) | 5 |
| NCR_H (High mobility count) | 10 |
| Sf_H = Sf_M | 0.5 |

Figure 1: Performance Result Enhanced MSE (30 km/hr)

Figure 2: Performance Result Enhanced MSE (60 km/hr)

From the results we see the following:

Observation 1: As expected (with the enhanced MSE) the ping pong rates and short TOS rates reduce for all simulated cases.

Observation 2: Enhanced MSE slightly worsens the Pico to Macro (P2M) handover failure rates.

As we consider the P2M handover failures as an all together another problem area which will need additional mechanisms to mitigate, it is handled in another contribution [3].

Figure 3-A: HOF with weighted counting (4 picos, 30 km/hr)

Figure 3-B: HOF with weighted counting (4 picos, 30 km/hr)

Figure 4-A: PP and short ToS with weighted counting (4 picos, 30 km/hr)

Figure 4-B: PP and short ToS with weighted counting (6 picos, 30 km/hr)

Figures 3 and 4 provide the comparison of different counting schemes on the HoF rates and the Ping Pong (Short ToS) rates. It can be seen that the weighted counting helps reduce the Ping Pongs and the Short ToS when the Pico density is smaller. Further simulations will be needed to finalise any conclusions.

# Conclusion

Simulations are performed to evaluate the potential impact of Release 8 MSE on HETNET mobility performance. Simulation results demonstrate that there is performance gain by enhancing MSE mechanism for HetNet deployment specifically for the ping pong rates and short TOS rates

Proposal 1: RAN2 is requested to agree on the necessity for enhancements to the MSE in TR 36.839 for further studies.

# Reference

RP-110709, Study on HetNet mobility enhancements for LTE, Alcatel-Lucent.

R2-116273	 Issues related to UE mobility state estimation in Hetnet; Samsung; Disc;

R2-121537,  Pico to Macro Handover Failures Improvements, Samsung.

# Appendix

Figure 3: Network Layout

# Simulation parameters

| Feature/Parameter | Notes | Value/Description |
| --- | --- | --- |
| Bandwidth |  | 10 MHz |
| 3GPP Macro Cell Scenario | Cell layout | 57 sectors/19 BSs |
|  | Inter site distance (ISD) | 0.5 km |
| Pico cell layout | Number of picos per macro cell | 4 or 6 |
| Macro-pico deployment type |  | Co-channel |
| Distance-dependent path loss | Macro cell model (TS 36.814, Model 1) | 128.1 + 37.6log10(r) [r in kms] |
|  | Pico cell model (TS 36.814, Model 1) | 140.7 + 36.7log10(r) [r in kms] |
| BS Tx power | Macro Pico | 41.2288dBm 30 dBm |
| Shadowing standard deviation | Macro Pico | 8 dB 10 dB |
| Shadowing correlation between cells/sectors |  | 0.5 / 1.0 |
| Shadowing correlation distance | Macro Pico | 50 m 50 m |
| Multipath delay profile |  | Typical Urban |
| UE velocity |  | 30 or 60 km/h |
| UE movement | How do the UEs move in the cell? | Straight line throughout the call |
| RSRP Measurement | L1 measurement period Measurement bandwidth L1 sliding window size | 40 ms 6 RBs 5 |
| Handover preparation time | Time from reception of UL A3 measurement report to sending HO command | 50 ms |
| Radio link failure monitoring | Qout threshold Qin threshold T310 N310 | -8 dB -6 dB 1000 ms 1 |
| Cell identification |  | Ideal |
| DL Interference load | Macro, Pico | 100% RBs loaded |