3GPP TSG-RAN WG2#79 meeting	R2-123951

Qingdao, P. R. China, 13 - 17 Aug 2012

Agenda Item:	7.7

Souce:		Samsung (rapporteur)

Title:	Email discussion report on [78#52b] LTE/COMP: CSI-RS Measurement Framework

Document for:	Discussion and decision

# Introduction

This document captures the email discussions on CSI-RS measurement framework as requested by the Chairman.

[78#52] LTE/COMP: CSI-RS Measurement Framework [Samsung]

-	Progress the work on stage-2 issues but also how to configure events, how to configure CSI-RS resources, how to report measurements.

-	Try to provide an as complete as possible stage-3 CR.

=>	Intended outcome: Email discussion report and draft 36.331 CR

A total of 19 companies contributed to the email discussions: InterDigital, Ericsson, Intel, Panasonic, New Postcom, Nokia, NSN, Samsung, ZTE, Huawei, Alcatel-Lucent, Kyocera, LGE, NEC, ITRI, Fujitsu, Hitachi, Acer, Motorola Mobility.

# Discussion

In this section, companies are invited to discuss the open issues listed below. Discussion should take into account the latest agreements from RAN1/2 [1][2][3][5][6]. Companies are also welcome to raise other issues if necessary.

## General

### Measurement model

To what extent the general measurement model as described in Section 10.6 of TS36.300 [7] is applicable for CRM. In particular, is Layer 3 filtering required for CRM?

Alt 1: No L3 filtering for CRM.

- This may save implementation and testing effort.

Alt 2: L3 filtering is still applicable for CRM.

- This may be beneficial to avoid measurement spike.

Figure 1: Measurement model [7]

| Company name | Proposal/comment |
| --- | --- |
| InterDigital | Similar to normal RSRP measurements layer 3 filtering should also be applicable for CSI-RSRP measurements. Layer 3 filtering is used to further average out the layer 1 results and the same reasoning as to why it was introduced for normal RSRP measurements should apply for CSI-RSRP measurements. The filtering coefficient, k, should also remain configurable. |
| Ericsson | Although we would like to reuse the existing functionality from CRS measurements, we also note that there are some differences. One difference between the CSI-RS and the CRS based RSRP measurements that should be taken into account for this decision is that the CSI-RS is only transmitted at most every 5 ms (and the largest periodicity is 80 ms). This is in contrast to CRS that is transmitted every ms. Hence, the number of L1 samples that can be measured within a time window is less for the CSI-RSRP. This does not mean that the CSI-RSRP should not be L3 filtered, but we need to allow a quite short filter to be configured. Hence we support Alt 2, but we think we might need to look more into filter lengths. |
| Intel | We also believe L3 filtering is needed (Alt. 2) with parameters adjusted to account for less frequent CSI-RS transmission. |
| Panasonic | We prefer Alt 2. The L3 filter parameter should be independent from those for CRS configuration. We agree with Ericsson on the need to check filter parameters. |
| New Postcom | We believe that L3 filter should be also used for CSI-RS RSRP measurement, However, it should be noted that the number of L1 samples measured for the CSI-RSRP is less than that for CRS-RSRP, which may impact the accuracy of L1/L3 filter for CSI-RS RSRP measurement. Therefore, L1/L3 filter need to be enhanced (e.g. need to adjust measurement period, or change factors and formula for L3 filter). |
| Nokia & NSN | Alt2 to avoid spikes and too many inaccurate and unnecessary reports. |
| Samsung | Alt 2. L3 filtering is useful to avoid potential measurement spike. Existing procedure as defined in Sec 5.5.3.2 of TS36.331 can be reused. Existing filterCoefficient parameter range can be reused. Responding to ZTE’s comment, RAN4 assumes longer measurement period because of the relatively low CSI-RS density compared to CRS. |
| ZTE | Currently RAN4 is investigating the accuracy of CSI-RS RSRP and the used L1 measurement period is 200/400/600/800ms. And it has not been decided which periodic will be eventually chosen by RAN4. Assuming longer period 800ms is chosen by RAN4 then it is not likely there will be measurement spike in such long time. So our view is RAN2 need further input from RAN4. |
| Huawei | Alt 2. The existing range of filterCoefficent seems sufficient to meet the needs of configuring various smoothing intervals. |
| Alcatel-Lucent | Support Alt 2. We do not see the problem with reusing the existing L3 filter coefficient range |
| Kyocera | We support Alt 2. L3 filtering should be adopted for CSI-RS RSRP measurements as CRS RSRP measurements. We agree with ZTE that further input from RAN4 is helpful. |
| LGE | We think L3 filtering needs to be applied for CSI-RS RSRP measurements, and reusing the existing L3 filter coefficients can work. |
| NEC | Alt.2. The filter coefficient parameter would need to be re-considered and we think RAN4 guidance will be necessary. |
| ITRI | Alt 2. We prefer to have L3 filtering. |
| Fujitsu | Alt.2 L3 filtering but with some further consideration from RAN4 |
| Hitachi | We support Alt 2. The existing range could be reused as the baseline. |
| acer | We think L3 filtering is still needed to filter L1 measurement results. |

Summary of discussion:

Majority of companies agree that L3 filtering should still be applied on CRM measurement. Several companies proposed that the existing L3 filter parameter values should be revisited to ensure it is still appropriate for CRM measurement.

Proposed agreement:

- L3 filtering is applicable for CRM measurement.

  - FFS on the need to modify the existing L3 filter parameter values.

### CRM measurement on SCell

According to RAN1 agreed way forward [6], CSI-RSRP is only applicable for RRC_CONNECTED intra-frequency.  It is assumed that CoMP can be configured in conjunction with carrier aggregation where CoMP can also operate on an SCell. Given this assumption, companies are invited to discuss CRM measurement /reporting behaviour on an SCell. In particular, which of the following alternatives is true?

Alt 1: If CRM for an SCell is configured, CRM measurement as well as reporting can only be performed when the SCell is activated.

Alt 2: If CRM for an SCell is configured, CRM measurement can also be performed when the SCell is deactivated. No CRM reporting can be performed.

Alt 3: If CRM for an SCell is configured, CRM measurement as well as reporting can also be performed when the SCell is deactivated.

Note: For RRM measurement today, SCell can be measured /reported in deactivated state according to measCycleSCell-r10 configuration.

| Company name | Proposal/comment |
| --- | --- |
| InterDigital | We believe alternative 3 is the best option as it is in line with carrier aggregation frame work. The UE can continue measurements based on the measurement frequency configuration which is not related to the activation/deactivation. When deactivated, the UE can use the measCycleScell-r10 configuration similar to carrier aggregation. If the network doesn’t want the UE to measure on the Scell frequency then it can remove the CSI-RS configuration for that frequency. |
| Ericsson | We believe RAN1 has not yet concluded on the use of COMP and CA in combination, hence we believe it might be premature to decide upon this in RAN2 at this time. We propose to wait for RAN1 before concluding on this. |
| Intel | We also prefer Alt. 3 assuming that CA and CoMP can be used together. We also think the combination of CoMP and CA should be allowed as they are independent features unless there are compelling reasons to exclude such combination. We can wait for RAN1 to also discuss this but prefer to assume support for their combination as working assumption to make progress given our short timelines. |
| Panasonic | We agree with Ericsson. Although we see the combination is quite useful, taking into account the time frame, how much really should be supported in Rel. 11 itself needs to be discussed. |
| New Postcom | We prefer Alt 1 to be adopted in CoMP combining with CA. Since activation/deactivation of SCell is motivated by UE power saving. Performing CRM measurement and reporting for CSI-RS RSRP on deactivated SCell would result in unnecessary power consumption of UE, as well as increased signalling overhead, which is not aligned with the motivation of introducing activation/deactivation of SCell |
| Nokia and NSN | Even if RAN1 concludes to support CoMP and CA combination, we believe Alt1 is enough. CRS could be used for SCell activation, no need to perform CRM measurement when the SCell is deactivated concerning UE power consumption. |
| Samsung | We agree that RAN1 has yet to confirm the applicability of CA+CoMP. RAN4 workload also need to be considered. Nevertheless, preference from RAN2’s point of view could still be discussed assuming CA+CoMP is supported, while waiting for RAN1/4 to provide further information. Alt 1 allows maximum power saving for the UE as the UE is not required to measure CSI-RS when the SCell is deactivated. However it is questionable how much power can be saved since the UE is required to perform RRM measurement for deactivated SCell. Moreover, long measurement period (potentially as long as 800ms) may be needed before accurate measurement can be generated by the UE, thus resulting in excessive delay in properly configuring CoMP measurement set. Alt 2 improves the accuracy of the measurement when reporting resumes after the SCell is reactivated. This method also has the benefit that performance requirement for deactivated SCell may not need to be defined. Delay in configuring the CoMP measurement set may also be much less compared to Alt 1 (e.g. in order of 10s of ms). However, there are still open questions (for RAN4) e.g. how to make sure UE has performed measurement correctly during SCell deactivation period, should there be a different performance requirement for period immediately after SCell reactivation etc. Alt 3 allows fast configuration/reconfiguration of CoMP measurement set by the network after the SCell is reactivated based on the measurement report obtained during the deactivated state of the SCell. A main concern seems to be RAN4 workload as RAN4 may not be able to complete the performance requirement in time for Rel-11. From CoMP performance perspective, Alt 3 is preferred. RAN4 should be consulted if Alt3 is fine from their perspective. |
| ZTE | We also prefer to wait for the further input from RAN1. |
| Huawei | There is currently no RAN1 agreement to jointly support CoMP and CA in Rel-11. Therefore, we prefer to await more solid outcome from RAN1 before prematurely deciding CRM measurement/reporting behaviour on an SCell. |
| Alcatel-Lucent | If CoMP is supported on SCell, we prefer Alt3 to ensure that an up-to-date CoMP Measurement Set is available at SCell activation. If UE power is the main concern, the measSCellCycle should also be applied for CSI RSRP measurement. |
| Kyocera | We prefer Alt 1. We think fast CoMP measurement set configuration/reconfiguration is not needed. Therefore CSI-RS RSRP measurement is not needed during SCell deactivation and should be reconfigured after SCell is reactivated. Also from UE’s point of view, Alt. 1 is the best alternative for saving power consumption. |
| LGE | This issue should be discussed when support of the scenario of using CA and CoMP in mixture is decided. |
| NEC | We agree with Nokia and NSN. In addition, we do not see a scenario such that an eNB triggers to perform CoMP on deactivated SCell where an activation has not been triggered based on CRS measurements. Alt.1 is preferable. |
| ITRI | We prefer to wait for RAN1. |
| Fujitsu | We also prefer to wait for RAN1 decision on the support for CoMP and CA in REL-11 |
| Hitachi | We prefer Alt 3, but we suggest waiting for the further input from RAN1. |
| acer | If RAN1 agrees CoMP for SCCs, we prefer alt. 1. |
| Motorola Mobility | We don’t think RAN2 is in a position to discuss CoMP+CA yet. |

Summary of discussion:

Many companies propose to wait for RAN1’s decision on the support of CA+CoMP before discussing this issue. Assuming CA+CoMP is supported, there is no consensus on the behavior of CRM measurement on SCell:

- 8 companies prefer to wait for further input from RAN1 on whether CA+CoMP is supported before making a decision on this issue.

- Assuming CA+CoMP is supported in Rel-11:

  - 5 companies prefer Alt 1 out of concern on UE power consumption

  - 5 companies prefer Alt 3. Reasons include commonality with CA measurement framework, maintaining independence of CA and CoMP (no compelling reason to preclude such combination) and better CoMP performance due to reduced latency of CoMP measurement set configuration.

Proposed agreement:

- The decision on CRM measurement behavior on SCell is postponed until there is a clear RAN1 decision on the support of CA+CoMP scenario.

## CSI-RS resource configuration details

In RAN2#78, agreements relevant to CSI-RS resource configuration for CRM were:

1	CSI-RS resources on which the UE is supposed to measure are configured explicitly by the NW, i.e., blind detection of CSI-RS resources in the UE is not required

2	CSI-RS resource specific configurations are configured as part of the MeasurementObject (FFS how to associate them with measurement configuration and how to refer to them in measurement reports)

We assume we shall continue with the Rel-8/9/10 RRM principle that there is one measurement object per frequency and all TPs’ related measurement reporting configurations for a frequency will be linked to the same measurement object.

Further discussions on the agreement to extend the existing MeasObject:

| Company Name | Proposal/ Comment |
| --- | --- |
| Panasonic | We think this is not so straight forward. If we simply extend, the UE must always look into the events first (inside ReportingConfiguration) to decide which part of the MeasObject is applicable. Also, the network cannot then only reconfigure part of the CoMP configuration; it must also send the non-CoMP information always. As for the lower layer configuration, in some implementation it may lead to un-necessary configuration of lower layers. From this perspective, an extension to the current MeasObject is rather contradicting the RAN1 agreements in R2-121077_R1-120929: "Note that this decision does not have any impact on inter-cell mobility handling" and in R2-121995_R1-121912 “Note that this is independent of existing mobility measurements and procedures” …since it seems to have some impact on legacy mobility handling also the new feature is not really independent?? |
| New Postcom | We think it is simply to extend the existing MeasObject for CSI-RS resource configuration for CRM. We believe that CoMP configuration will not changed frequently, then the un-necessary configuration of lower layers is acceptable. |
| Samsung | We don’t see a good reason to change the existing agreement. We agree that the event would define which fields of the measObject the UE would apply, but we don’t think this is a problem. We don’t think the current agreement strictly contradicts RAN1 agreement which is more of a high level concept. CRM configuration is also not expected to change frequently. |
| ZTE | We also think RAN2 should re-consider this issue. RAN4 will still discuss the measurement bandwidth issue. And RAN1 will continue to discuss antenna port 16~20 at next RAN1 meeting. NeighCellConfig , Q-OffsetRange and cellForWhichToReportCGI will be useless for CSI-RS RSRP measurement. Most likely the only overlapped IE is ARFCN-ValueEUTRA. Based on this, we don’t see a justification to reusing current object IE. Considering there are few mandatory IE in the object IE, it may result in more signalling overhead considering reconfiguration of parameters related to CSI-RS resources. If we introduce an independent IE to describe CSI-RS resource, then the description of the object including all parameters related to CSI-RS resource will be decoupled from existing object IE. And we believe it will make the specification much clean and clear. In addition , UE’s implementation is also simple e.g. UE doesn’t have to pick part of the parameter based on the report configuration. |
| Huawei | Our understanding is points 1 & 2 together are related to the configuration of CSI-RS resources for CRM set. We haven’t seen big issues in extending MeasurementObject to configure CSI-RS resources into CRM set. |
| Alcatel-Lucent | We also prefer to stick to the agreement. |
| LGE | We do not see a real issue regarding the reuse of existing measurement/reporting procedure with small extension within MeasObject to support CoMP. |
| NEC | So far, we do not see a big problem for the current agreement and thus prefer to keep it at this moment. |
| ITRI | We also prefer to stick the current agreement. |
| Fujitsu | We also prefer to stick to the current agreement. |
| Hitachi | We also prefer to stick to the current agreement. |
| acer | We would like to follow existing RAN2 agreement. |

Summary of discussion:

Two companies proposed to revisit the RAN2#78 agreement: CSI-RS resource specific configurations are configured as part of the MeasurementObject. One company thinks that the existing agreement violates RAN1 agreement that CRM is to be independent of existing mobility measurements and reporting procedure. Another company thinks that introducing new IE for CSI-RS resource configuration can simplify specification and UE implementation. However, the proposal to revisit the agreement does not seem to have enough support.

Proposed agreement:

- No change to the existing agreement that CSI-RS resources, on which UE is supposed to measure, are configured as part of the MeasurementObject.

  - Companies that wish to revisit the agreement are encouraged to input contributions to RAN2#79.

We also note that RAN1 has agreed recently ‎[6] that the maximum size of the CRM Set is at least 8, and the UE shall be capable of reporting at least 8 CSI-RSRP measurements. RAN1 has also agreed that the CRM set represents the maximum number of measurements the network can request rather than the UE capability. RAN1 is to request RAN4 to define the maximum size of the CRM Set and the UE capability and then inform RAN2 of the final decision so that the appropriate signalling can be defined. Considering the RAN1 development, the rapporteur would like to propose the discussions related to the maximum size of the CRM set and the related signalling be deferred until further agreements are made by RAN4/1.

Other open issues related to configuration of CSI-RS resource in CRM proposed to be discussed are given below.

### CSI-RS resource parameters

RAN1 has agreed on a set of CSI-RS resource parameters for CRM ‎[6]. The list and the (assumed) value range are given below:

- AntennaPortsCount,

  - ENUMERATED {an1, an2, an4, an8}

- ResourceConfig,

  - INTEGER (0..31)

- SubframeConfig,

  - INTEGER (0..154)

- Scrambling initialization parameter

  - INTEGER (0..503)

Note also the following statement from RAN1’s recently agreed WF:

“Note to RAN2 in a LS: RAN1 considers that it is FFS whether additional information should be given to the UE with respect to the transmission power or power boosting of CSI-RS in order to improve the UE’s comparison of CSI-RSRP measurements for the case of event-triggered reporting. RAN1 noted that the difference of CSI-RSRP measurements may not be representative of the difference of expected received powers of PDSCHs due to different boosting values applied to the CSI-RS at different transmission points (which includes the case where one transmission point may have one CSI-RS port and another transmission point may have two CSI-RS ports).”

Apart from the list of parameters above, are there other parameters required for a CSI-RS resource? In particular, is there a need for CSI-RS resource specific offset (analogous to cellIndividualOffset for RRM)?

Companies are invited to discuss the need for additional parameters as well as their value ranges.

| Company name | Proposal/comment |
| --- | --- |
| InterDigital | We think that a CSI-RS individual offset may be useful in order to allow the network to favour one TP over another, based on different CSI-RS settings, transmission point loading, and different TPs in different sectors/cells. |
| Ericsson | We think the list presented above is sufficient at the moment and adding extra parameters later on does not seem very hard. Should RAN1 conclude on the issue of power boosting a “CSI-RS specific offset” might be needed. |
| Intel | We also think the list is a good starting point and prefer to add CSI-RS specific offset. |
| Panasonic | We support the need for cellIndividualOffset. This is useful for the case of different transmission power of CSI-RS like heterogeneous network, different antenna port setting and so on. The offset is also useful to control the load management. |
| New Postcom | We also agree that CSI-RS resource specific offset should be introduced as CSI-RS resource parameter, especially in HetNet scenarios. |
| Nokia & NSN | We agreed with the listed parameters. In addition, CSI-RS resource specific offset is needed to help resolve Tx power difference of 1Tx and 2Tx, also the Pc difference of the TPs. It is very similar to the existing cell specific offset, so in our view could be discussed and decided in RAN2. |
| Samsung | According to RAN1’s observation, the CSI-RS EPRE from a one-port TP can be 3dB higher than the CSI-RS EPRE from a two-port TP even though the received powers of PDSCHs from the two TPs (at the same distance) are the same. This is because of the CDM structure of the CSI-RS ports. In general, CSI-RS ports of different TPs may be applied different power boosting values with respect to the PDSCH powers by the network. To allow flexibility for the network to configure TP-specific reporting bias as needed, CSI-RS resource specific offset can be configured and included in the event trigger definition. The definition of the strongest TP also needs to be clarified in light of this observation. |
| ZTE | It is not clear for us how can the CSI-RS specific offset can compensate the difference of CSI-RS EPRE because it based on the assumption that only port15 is measured. However in latest RAN1 LS R1-123077 it is also mentioned that RAN1 will discuss whether port 16 “should” be measured or not i.e. there is some uncertainty in the UE which eNB doesn’t know. So how can eNB set the offset without the knowledge? Nevertheless we still think CSI-RS specific offset parameter is useful for other use case e.g. offload between TPs. |
| Huawei | We agree that the list of parameters is needed to configure CSI-RS resource. Since there is potential difference in power boosting from PDSCH to CSI-RS among transmission ports, some kind of equalization offset would be needed to make measurement on different CSI-RS resources comparable. Hence CSI-RS resource specific offset can be useful. |
| Alcatel-Lucent | Like ZTE, we are also not clear how this will help resolve the TX power difference between 1TX and 2TX. Anyway, if the number of antenna ports is signalled as listed, then UE should know the power difference between 1TX and 2TX and compensate the CSI-RS RSRP accordingly without the need of the CSI-RS specific offset. Since the definition for CSI-RS RSRP, particularly on the number of antenna ports to measure and the power boosting (Pc) is still being discussed by RAN 1, we prefer that the usage of CSI-RS specific offset for these cases are discussed later. On the other hand, we see the benefits for having such offset for TP specific bias and this can be discussed further in RAN 2 irrespective of the RAN 1 decision. |
| Kyocera | We agree the listed parameters are needed for the configuration of CSI-RS resource. And we also support to have a specific offset for building trust in measurements. |
| LGE | Since the listed parameters are asked by RAN1, RAN2 should implement those as the CSI-RS resource configuration parameters. Regarding the CSI-RS specific offset, we would like to follow explicit RAN1 guidance. That is, if the CSI-RS specific offset is needed, we assume that RAN1 need to indicate this explicitly. |
| NEC | We agree the list of parameters above and also support the CRS-RS specific offset. |
| ITRI | We think the set of CSI-RS resource parameters related to the CRM should be discussed and decided in RAN1. We do not see any reason to object RAN1’s agreement. Regarding the CSI-RS specific offset, we also believe that it should be discussed and decided in RAN1. |
| Fujitsu | We support the need for a CSI-RS resource specific offset. This may be useful to help the management of load control. |
| Hitachi | We are fine with the listed parameters and prefer to include CSI-RS individual offset as CSI-RS resource parameter. |
| acer | We support to assign a cell specific offset for a CSI-RS resource. |

Summary of discussion:

Apart from the existing list of parameters indicated by RAN1, majority of companies prefer to also add CSI-RS resource specific offset as part of the CSI-RS resource parameters for CRM. Reasons mentioned include better load control and to account for Tx power difference between 1Tx and 2Tx TPs.

Proposed agreement:

- List of parameters for CSI-RS resource parameters are:

  - AntennaPortsCount,

    - ENUMERATED {an1, an2, an4, an8}

    - Note: The range may be changed depending on the RAN1/4 decision on the measurement ports.

  - ResourceConfig,

    - INTEGER (0..31)

  - SubframeConfig,

    - INTEGER (0..154)

  - Scrambling initialization parameter

    - INTEGER (0..503)

  - CSI-RS resource specific offsets

    - Note: Range is assumed given by Q-OffsetRange

### The need for configuring separate id for each CSI-RS resource in CRM

Discuss the need for configuring separate id for each CSI-RS resource in CRM. Potential use cases of the id are:

- Specific CSI-RS resource configuration release: id can be used to indicate which CSI-RS resource is to be released by the UE

- Reporting: id can be used to indicate which CSI-RS resource is reported.

- Others?

If the need for an id is confirmed, what can be used as the id? Possible alternatives are:

Alt 1: New id is defined and configurable by the network

- The range of ids can be the same as the max size of CRM set configurable.

Alt 2: Id is defined as a function of the CSI-RS configuration parameters.

- Companies are encouraged to provide concrete proposals on the function if Alt 2 is proposed.

Alt 3: Other?

| Company name | Proposal/comment |
| --- | --- |
| InterDigital | We prefer alternative 1 as it leaves no room for ambiguity in the future. |
| Ericsson | We think the ID must uniquely identify a configured CSI-RS resource among all the configured CSI-RS resources in the UE. We also believe the size of the set of all possible CSI-RS resources is much bigger than the size of the set of the configured CSI-RS resources in the UE. Hence an explicit ID would be much shorter than an ID based on a function. The function may potentially also need be updated, should more parameters be needed to configure the CSI-RS resource. Based on this we think there is no need to specify a separate function. We note that the range of the ID must be at least the max size of the CRM set plus the max size of the COMP measurement set. We think the ID should be configured by the network prior to configuring the measurement. This means that this ID can be used when configuring measurements and reporting results. We think there is a benefit in keeping the range of the ID a couple of times larger than the max size of the CRM set. If the number of CSI-RS resources that the network would like the UE to measure is bigger than the number of CSI-RS resources the UE actually can measure, then all of the CSI-RS resources can be configured once, with unique IDs. It is then easy for the network to configure measurements sequentially with various subsets of the configured CSI-RS resources using the CSI-RS resource ID. We support alt 1; the NW configures the ID for a CSI-RS resource. |
| Intel | We also support Alt. 1 as having such indexes can be used to simplify subsequent CSI-RS based RRM measurement reports. Such indexing is UE specific and may be assigned explicitly or implicitly as part of CRM/Measurement configuration. Also the use/scope of such indexing if defined should not be mixed with those defined/used in CSI-RS configurations for CoMP measurement set. |
| Panasonic | We prefer Alt. 1 and agree with InterDigital that no room for ambiguity in the future should be possible. |
| New Postcom | We believe Alt 1 is more suitable to be supported for each CSI-RS resource in CRM, which seems less complexity than Alt 2. We also agreed with Ericsson that the range of ID should be larger than the size of the CRM set, more details should be discussed. |
| Nokia & NSN | Alt 1 as it is simple and clean. |
| Samsung | We agree there is a need for defining id for each CSI-RS resource in CRM and Alt 1 is sufficient. |
| ZTE | We also support alt1. |
| Huawei | We prefer Alt 1 as it is generic and straightforward. CSI-RS ID can be used to configure both CRM set and CoMP measurement set, to fully take advantage of the existing configuration of CSI-RS resource parameters (i.e., to avoid sending the same CSI-RS resource parameters in multiple signaling). The number of ID can be larger than the maximum size of CRM, and it can be determined based on the typical number of CSI-RS resources a UE may detect/experience during an updating cycle of CSI-RS resource parameters. |
| Alcatel-Lucent | Alt1 |
| Kyocera | We also support Alt. 1. Regarding the ID’s range, we have the similar view with Ericsson’s view that the range of the ID should be the max size of the CRM set plus the max size of the CoMP measurement set. |
| LGE | We do not fully understand the Alt2 about how it is formulated with the existing RRC. For now we believe Alt1 is a cleaner approach. Regarding the max range of IDs, we do not think there is something critical issue to discuss in RAN2 as it is only related to how many CSI-RSs resources the UE can detect and measure with desired accuracy (i.e. can be decided in/based on RAN4) . Even with quite large max value, the network can configure smaller value for which the UE is required to measure, whenever needed. |
| NEC | We support Alt.1 for simplicity, while the size of the new IDs should be discussed further. |
| ITRI | We prefer Alt. 1. |
| Fujitsu | We prefer Alt1. Where a new id is defined and is configurable by the network, the range of ids should be FFS |
| Hitachi | We also support Alt.1. |
| acer | We think a CSI-RS resource should have a unique ID for management and reporting purposes. and the ID should be defined and assigned by the network (alt.1). |

Summary of discussion:

All companies agree there is a need to assign/configure separate id to each CSI-RS resource configured for CRM. Majority (eleven) of companies seem to agree that range of ids can be the same as the max size of CRM set configurable. But four companies think that the range of ids should be larger than the max size of CRM set, while two companies prefer the range to be FFS.

Proposed agreement:

- New id is defined and configurable by the network

  - The range of ids is FFS.

## New Events

Events agreed to be supported in the last meeting are:

- Event triggers comparing a CSI-RS resource to a fixed threshold

- Event trigger that compares CSI-RS resources to each other is supported (e.g. a resource becomes less than x dB worse than the best resource in the CRM).

To clarify the meaning of the events agreed, example event definitions are given below:

Event C1: CSI-RS resource becomes better than threshold

Entering condition:

Leaving condition:

The variables in the formula are defined as follows:

- Mcp is the measurement result of the target CSI-RS resource (dBm for CSI-RSRP).

- Hys is the hysteresis parameter for this event (dB)

- Thresh is the offset parameter for this event (dB)

Event C2: CSI-RS resource becomes offset better than the best CSI-RS resource in the CRM

Entering condition:

Leaving condition:

The variables in the formula are defined as follows:

- Mcp is the measurement result of the target CSI-RS resource (dBm for CSI-RSRP).

- Mrp is the measurement result of the strongest CSI-RS resource (dBm for CSI-RSRP).

- Hys is the hysteresis parameter for this event (dB)

- Off is the offset parameter for this event (dB)

Note: CSI-RS resource specific offset can be included in the event definition pending conclusion from Sec 2.2.1.

Companies are invited to present views on the definition of new events above to align understanding on the meaning of the new events agreed.

| Company name | Proposal/comment |
| --- | --- |
| InterDigital | Event C1 We think event C1 can be effectively used for the initial configuration of the CoMP set to determine the transmission points that are above an acceptable threshold for CoMP operation. If it is agreed that the transmission point individual offset is included in the TP configuration, then the formula above should be updated. One open issue to consider is minimizing the amount of events triggered due to C1. There can be scenarios where two TP are already part of the CoMP set and are the best detected TPs. In the mean time additional TPs may meet the absolute threshold criteria, but are still worst than the two best TPs already in the CoMP set. This will trigger a measurement report that may not be very useful to the network as the best two TPs are already in the CoMP set. Similarly, if the event is no longer met for the same TP, we would unnecessarily report that the TP doesn’t meet the threshold criteria. We think that this may significantly increase layer 3 reporting overhead and may be inefficient. RAN2 should discuss the benefits and needs of always triggering this criterion, even though the best TPs are already configured. The signalling can be reduced, for example, by not triggering the C1 report (e.g. cell going above a threshold) once the CoMP set is full. Once the CoMP set is full the relative comparison event, C2, should be sufficient to allow the UE to report when neighboring transmission point’s CSI-RSRP change. [Samsung] We don’t think reporting overhead is an issue since it is a long term measurement report. Besides, it is still beneficial for network to know which TPs has signal strength above a certain threshold even though the best TPs are already part of the CoMP set since those TPs can be involved in interference coordination scheme (coordinated scheduling) that doesn’t require CSI feedback. Event C2 We agree with the concept that an event used to relatively compare CSI-RS RSRP measurements should be introduced. However, we think that the relative comparison of CSI-RSRP measurements should be done between the TPs in the CRM set that are not in the CoMP set and the TPs that are in the CoMP set. This would allow the network to know when a TP outside the CoMP set has become better than any point in the CoMP set rather than the best point in the CoMP set and might be therefore replaced. Therefore, we suggest defining C2 the following way: CSI-RS resource not in the CoMP set becomes offset better than the worst CSI-RS resource in the CoMP set. If we keep the current proposed definition of C2 we run into the problem that the best TP may remain that best TP for a very long time especially if the macro cell is one of the TPs and many small RRHs are under the macro coverage. If we compare against the best TP, we may not end up triggering event C2 and the network may not become aware that a new TP has become better than the second best TP in its CoMP set and may not properly update its CoMP set. Note that HSPA mobility principles of maintaining an active set work in similar way as described above for event C1 and C2. [Samsung] The network may only want to know the exact ordering of the TPs if there are more than enough TP candidates that are sufficiently good to be included in the CoMP measurement set. RAN1 agreed that the CoMP measurement set size is 3 and according to our evaluation (R2-122739), only a small percentage of UEs would detect more than 3 TPs. Therefore, the majority of the UE is fine. For the UEs that see too many suitable TPs, the network can always reconfigure a tighter offset value to filter out the weaker TPs. The network can also configure multiple offset parameters (multiple report config linked to the same measurementObject). For example, assuming a TP can be included in the CoMP measurement set if it is within 6dB of the best TP (suitable TPs), the 1st offset configured can be 3dB and the 2nd offset can be 6dB, enable detection of weaker TPs among the suitable TPs. Moreover, as mentioned in Sec 2.4.1 and 2.4.3, all TPs in the triggered list are reported, so the network would know the order of TPs whenever a report is triggered. |
| Ericsson | The proposed events are useful. We would like to add two more. Event C3: CSI-RS resource becomes offset better than a configured reference CSI-RS resource Entering condition: Leaving condition: The variables in the formula are defined as follows: - Mcp is the measurement result of the target CSI-RS resource (dBm for CSI-RSRP). - Mrp is the measurement result of the reference CSI-RS resource (dBm for CSI-RSRP). - Hys is the hysteresis parameter for this event (dB) - Off is the offset parameter for this event (dB) This event is quite similar to Event C2, with the difference that the “target” CSI-RS reference does not change over time, as it does with Event C2. We believe this event can be useful in a situation where one CSI-RS resource is expected to dominate over others, for example, in a hetnet deployment the reference CSI-RS resource could be the macro node, and the UE would trigger this event when coming close to the pico nodes. [Samsung] The benefit of C3 is unclear. Why C2 can’t be triggered when UE comes close to a pico node? [Ericsson July 31] We agree with Samsung that Event C2 can be used to trigger a measurement report when the UE comes close to a pico node. However, we are concerned about the definition of “the strongest CSI-RS resource in the CRM set”. How is this determined? Upon triggering of C2, does this imply that the new stronger CSI-RS resource is now the strongest one? We think Event C3 is better in that sense since a clear reference CSI-RS resource is used to compare the other CSI-RS resources. Event C4: CSI-RS resource becomes worse than threshold Entering condition: Leaving condition: The variables in the formula are defined as follows: - Mcp is the measurement result of the target CSI-RS resource (dBm for CSI-RSRP). - Hys is the hysteresis parameter for this event (dB) - Thresh is the offset parameter for this event (dB) This event is the “opposite” of Event C1. We think this can be useful to help the NW select which CSI-RS resources can be excluded from the CRM set. [Samsung] Event C1/C2 can already help the network to determine which TPs can be excluded: TPs that never trigger the entry condition or TPs that satisfy the leave condition. It is unclear what additional benefit C4 brings. [Ericsson July 31] This depends what Event C1 is used for. If Event C1 is used for identifying which CSI-RS resources should be part of the COMP measurement set, it will be configured with a fairly high threshold. CSI-RS resources triggering a reportOnLeave with that configuration do not imply that these resources can be excluded from the CRM set we think. We agree that it would be possible to use that fact that some CSI-RS resources never trigger the entry condition as a sign that these could be excluded from the CRM set, but with our proposed Event C4 these “bad” resources can be found faster, which would improve the efficiency of the whole solution. Regarding InterDigital’s proposal on Event C2, we think the problem identified by InterDigital can be solved by the use of our proposed Event C3. [Samsung] Further clarification would be appreciated. [Ericsson July 31] The problem identified by InterDigital is that one CSI-RS resource (e.g. the macro resource) may be the best CSI-RS resource over a long period of time. This means that the network would fail to get any notice that the second best CSI-RS resource have changed. We believe this problem can be solved with our Event C3, as it allows for the use of various reference CSI-RS resources. Also see our previous comment on the ambiguous definition of “strongest CSI-RS resource”. |
| Intel | Our preference is C2 as proposed. We also find event C3 proposed by Ericsson, useful as a generalized version of C2. So that C3 can be used and when no CSI-RS reference is configured the strongest should be used as a reference by default. |
| Panasonic | - We don't see the need for distinction between CSI-RS resource used for CSI reporting and CSI-RS resource used for CSI RSRP measurements. One TP may have multiple antenna ports and the antenna port used for CSI reporting and the antenna port used for the event evaluation could be different. In addition, the antenna port used for CSI report could be transmitted differently like multiple TP in JT (combined in the air). Therefore, in general, UE doesn't know the TP and actual CSI-RS resource linkage since it is solely network knowledge. Therefore, whether CSI-RS is within CoMP measurement set or not should not be used for the event evaluation. - We support both C1 and C2 with the clarification that comparison in C2 is with the UE´s best CSI-RS at the time of evaluation and not with the best CSI-RS that was signalled previously to the network. - C1 would be used mainly to trigger single TP operation where one CSI-RS is sufficiently good. C2 would be used mainly to trigger CoMP operation where multiple CSI-RS are relatively close together. If neither C1 nor C2 would be triggered, the network would be operated as Rel-8-10 type operation. - Removal of CSI-RS resource for CSI reporting needs some discussion. One possible way could be to use (average of) CSI reports; another could be to use ReportOnLeave. If these are not sufficient (e.g. if we need separate offset, hysteresis etc. for leaving condition), separate event(s) need(s) to be defined. These would be opposite of C1 and C2. - We don't see InterDigital's problem of strong macro cell on event C2. This is solved by setting cellIndividualOffset properly. We also don't see the problem raised by Ericsson. This is also solved by cellIndividualOffset properly. If it is difficult to reach an agreement on what events need to be supported, we should consider having only periodic measurements for Rel.11 [Samsung] We agree with the proposed clarification for C2 that the best resource is at the time of evaluation. |
| New Postcom | We think Event C1 should be supported as a baseline event for CSI-RS RSRP measurement, since it can be used for eNB to collect measurement results of all potential CSI-RS resources and then select suitable CSI-RS resources for CoMP. Event C2 and C3 are beneficial and suitable in some typical scenarios of CoMP. Therefore, we think both Event C2 and C3 should be considered in CRM measurement. For Event C4, although CRM measurement is independent with mobility measurement, we think it is necessary to add A1 like event for CRM measurement, in order to allow different condition for entering and leaving CRM set. Therefore, we prefer to introduce Event C4 in CRM Measurement. [Samsung] Similar to comments for Ericsson, it is unclear to us the benefits of C3 and C4. |
| Nokia and NSN | Regarding to Event C2: we agree with Panasonic that the best should be the best at the time of evaluation. As it is impossible to have something better than the best, it should be CSI-RS resource becomes less than offset worse than the best CSI-RS resource as captured in the agreement from last RAN2 meeting, to define the x dB window for trigger and report. i.e. Entering condition: Leaving condition: Ocp is the CSI-RS resource specific offset as commented in section 2.2.1. Comparing to C2, C3 proposed above would need more frequent reconfiguration to reconfigure the reference resource, thus not preferred. It was agreed that the CRM set management would be leave to eNB implementation, thus C4 would not be needed for removing CSI-RS resources from the set. C4 would introduce a lot of unnecessary reports as typically very weak TPs could be included in the CRM set to avoid frequent reconfiguration. |
| Samsung | Given the observation mentioned in Section 2.2.1, we propose that the events are defined as follows: Event C1 Entering condition: Leaving condition: Where Opcp is the offset of the target CSI-RS resource. Event C2 Entering condition: Leaving condition: Where Oprp is the offset of the strongest CSI-RS resource. To accurately determine the strongest CSI-RS resource for Event C2, the definition of the strongest CSI-RS resource needs to include the CSI-RS resource-specific offset. i.e. the effective CSI-RSRP of the TP is determined as measured CSI-RSRP+.TPIndividualOffset. |
| ZTE | We also agree to introduce event-based report. Event C1 is useful to filter out weak TPs which is useless for CoMP. As for C2 we intend agree with NSN that the Off parameter must be negative otherwise it is bit strange to say one CSI-RS resource is stronger than current best CSI-RS resource. But we also think C2 might not so efficient. The maximum CoMP measurement set is 3. If there are more than 3 CSI-RS resource can trigger event C2 then more signalling is needed to report the event. Actually eNB only care about best N CSI-RS resource (N<=3), so the most important information is which CSI-RS resources are the best N CSI-RS resource. Based on this we propose another measurement event: C5: best N CSI-RS resources are changed, N is a preconfigured number By the way, if C2 like event is agreed we don’t think C3 is needed since it is coved by C2. And we think leaving event corresponding to C2 is helpful. [Samsung] Similar comment to InterDigital, if more than 3 resources can trigger C2, the offset parameter can be adjusted to filter out more TPs. |
| Huawei | Wouldn’t C2, as it is defined now, be always triggered at the time of evaluation, since there is always a best measurement to meet the condition? Hence, we think specifying reference, as proposed by Ericsson, is needed. To address Nokia/NSN’s concern of the need of frequent update of the reference, a set of reference CSI-RS resources can be defined. Event C6: The measurement result of a candidate CSI-RS resource becomes offset better than that of the worst reference CSI-RS resource. Entering condition: Leaving condition: The variables in the formula are defined as follows: - Mcp is the measurement result of the candidate CSI-RS resource (dBm for CSI-RSRP), including its specific equalization offset. - Mrp is the measurement result of the worst reference CSI-RS resource (dBm for CSI-RSRP), including its specific equalization offset. - Hys is the hysteresis parameter for this event (dB) - Off is the offset parameter for this event (dB) C1 can be used to configure the initial CoMP measurement set and the reference set. C6 can be used to continuously monitor the need of updating CoMP measurement set (and the reference set). |
| Alcatel-Lucent | We prefer having Event C1 and C2 for CoMP RSRP measurement. Event C1 is particularly useful when the interference from a TP is strong but falls outside of the relative offset (e.g. when the RRH is closed to the macro or intra-site CoMP). Furthermore, it can also provide the network with knowledge of interfering TPs even though it cannot be added to the CoMP Measurement Set as mentioned by Samsung. The signalling overhead will depend on the absolute threshold being set. |
| Kyocera | We prefer to have Event C1 and C5. We think that C1 is useful for Network to configure the initial CoMP measurement set. We also think a C2-like event needs to be introduced for further evaluation after establishing the initial CoMP measurement set. To properly manage the CoMP measurement set, we agree with InterDegital that the relative comparison of CSI-RSRP measurements should be done between the TPs in the CRM set that are not in the CoMP measurement set and the TPs that are in the CoMP measurement set. We also agree with ZTE’s comment that “actually eNB only care about best N CSI-RS resource (N<=3), so the most important information is which CSI-RS resources are the best N CSI-RS resource”. It’s enough for the UE to only trigger the measurement report when best N CSI-RS resources are changed. Therefore we support having event C5. |
| LGE | For the event based on comparison between the measured value and the absolute threshold, - We think event C1 and C4, which are CSI-RS version of event A1 and A2 respectively, are quite useful and essential as they are applicable for a variety of purpose. Thus the support of C1 and C4 is essential For the event based on comparison between/among the measured value, - We think C5 provides more general usage than C2 because in general network is interested in best N CSI-RS resources, and the combined use of C5 and C1/C4 (e.g., C5+C4) enables many variants for reporting condition. So we support the C5. - The C2 is targeting a specific CoMP operation scenario and as a result the applicable scope of C2 is too limited. Note that the combined use of C1 and C4 can achieve the similar result that is intended by event C2 in normal cases. So we think the C2 is less essential than C5. |
| NEC | We think the event C1 and C2 with modification above by Samsung are necessary, but for other we do not have strong motivation to have as mentioned by Nokia and NSN. |
| ITRI | We agree with NSN that the Event C2 proposed by rapporteur is not correct and it does not reflect the agreement of the last meeting. So far, we only see the benefits of Event C1 to configure the CoMP measurement set. For the other events, i.e., Event C2~C6, we do not understand the benefits clearly. Since CoMP mainly focuses on the throughput of the cell edge, too much report should be avoided especially with non-clear benefits. |
| Ericsson (July 31) | Event C2 seems to spur most discussion. We think that Nokia/NSN’s proposal to Event C2 (denoted as Event C2N in this text) provides some clarification to our concerns about how to define the best resource. However, as we understand the proposed Event C2N, it can also be called, “the difference in signal strength between the top two CSI-RS resources is less than offset”. Is this a correct understanding? If our understanding of Event C2N is correct, we think this limitation of top two CSI-RS resources is somewhat unfortunate. Is it possible to extend this somehow? |
| Fujitsu | We see some advantages to defining both C1 and C2 events but the details of the Event C2 need to be studied further. |
| Hitachi | We think Event C1 and C2 are sufficient. Event C1 is useful for NW to collect the knowledge of candidate TPs. Event C2 is the simple way to maintain the best CSI-RS combination. |
| acer | We support C1 for its simplicity and applicability to many scenarios. For C4, if reportOnLeave can be configured for C1, it would not be an essential event trigger. We think C5 is a generalization of C2 and it can be applied in more scenarios. Compared to C1, no threshold is specified in C5. So CSI-RS resources with very low received signalled power are still reported. In that case, the benefit is unclear. So we propose to revise the C5 as: C5a: best N CSI-RS resources which are better than a threshold are changed , N is a preconfigured number |
| InterDigital (Aug 2nd) | [InterDigital] For the scenario we discussed above, of the strong macro being much better than the RRHs, we think the Event C2 will not be sufficient as the Event C2 will never be triggered. In essence, to be able to properly manage the CoMP set, the network needs a notification whenever an outside resource becomes better than any one of the resources in the measurement set. However, as per current RAN1 discussions, is no mapping available at the UE to figure out which CSI-RS resource in the CRM set maps to which resource in the measurement set, so, it is not easy to decide how the reference resources should be identified for comparison. We agree that one possible solution would be Event C3 which is defined to compare against a configured reference CSI-RS resource. With this solution, in order to compare against all resources in the measurement set, separate event would need to be configured considering each element from the CoMP set as a reference. However, this may cause some unnecessary reporting, for e.g., if a new resource becomes better than 2 out of 3 resources, two events would be generated when one report should be sufficient to detect a new resource. We think that the signaling overhead and unnecessary reporting could be minimized by slightly re-defining Event C3 in the following way: Event C7: A CSI-RS resource becomes better than any one of the CSI-RS resources in the reference set Entering condition: Leaving condition: The variables in the formula are defined as follows: - Mcp is the measurement result of the target CSI-RS resource (dBm for CSI-RSRP). - Mrp is the measurement result of the any CSI-RS resource in the configured Set S (dBm for CSI-RSRP). - Hys is the hysteresis parameter for this event (dB) - Off is the offset parameter for this event (dB) - Set S is configured set of CSI-RS resources to be used for comparison. |
| Motorola Mobility | We have some concerns about event C2 as defined above. It is not clear what it means for a CSI-RS resource to be better than the best CSI-RS resource. Assume UE is configured with CSI-RS resource 1 (Res1) and CSI-RS resource 2 (Res2) for CSI-RSRP measurements and the offset ‘off’ is 3 db (event is triggered when target is at least 3db better than the best). First Res1 is the “best CSI-RS resource”. At the next measurement UE finds that Res2 has become better than Res1. So Res2 is now the “best CSI-RS resource”. However, at this time, Res2 is not 3db better than Res1. So the event is not triggered. Later Res2 becomes more than 3 db better than Res1. However the event is not triggered because the “best CSI-RS resource” is Res2. ‘Off’ could be restricted to only negative values (as some companies have proposed) to avoid this problem. But then it is not clear what happens if the best CSI-RS resource changes rather quickly without triggering the C2 event. Given this, we think event C3 is preferable over C2. Note also that the agreement in the chairman notes is simply to have an event comparing between CSI-RS resources (“We will progress on supporting event trigger that compare CSI-RS resources to each other (e.g. a resource[s] becomes less than x dB worse than the best resource in the CRM).”). Event C2 as described above is trying to capture the example (but not quite). In summary, we think C2 needs more discussion. We propose to leave the details of the event for comparing CSI-RS resources FFS in this email discussion. |

Summary of discussion:

From Sec 2.2.1 (and also reiterated by some companies here), definition of Event C1/C2 should be modified to include CSI-RS resource-specific offset.

It seems that no company objects to Event C1 can definition including the CSI-RS resource-specific offset can be agreed.

Majority of companies (9) supports Event C2. But several companies are unclear about how Event C2 would work, e.g. how the best resource is determined by the UE, how the triggering works. Therefore, further clarifications/discussions on Event C2 are needed.

More new events are also proposed by companies:

Event C3: CSI-RS resource becomes offset better than a configured reference CSI-RS resource (See Ericsson’s comment)

  - Supported by 4 companies

Event C4: CSI-RS resource becomes worse than threshold (See Ericsson’s comment)

  - Supported by 3 companies

Event C5: best N CSI-RS resources are changed, N is a preconfigured number (See ZTE’s comment)

  - Supported by 3 companies

Event C5a: best N CSI-RS resources which are better than a threshold are changed, N is a preconfigured number (see Acer’s comment)

  - Supported by 1 company

Event C6: The measurement result of a candidate CSI-RS resource becomes offset better than that of the worst reference CSI-RS resource. (See Huawei’s comment)

  - Supported by 1 company

Event C7: A CSI-RS resource becomes better than any one of the CSI-RS resources in the reference set (see InterDigital’s comment)

  - Supported by 1 company

More discussions are needed on the exact formulation of event C2 and the need for the new events. Proponents are encouraged to provide exact definitions in RAN2#79.

Proposed agreements:

- Event C1 as defined below is agreed:

Entering condition:

Leaving condition:

The variables in the formula are defined as follows:

- Mcp is the measurement result of the target CSI-RS resource (dBm for CSI-RSRP).

- Hys is the hysteresis parameter for this event (dB)

- Thresh is the offset parameter for this event (dB)

- Opcp is the offset of the target CSI-RS resource.

- Event C2 is supported. Exact definition is FFS.

- The needs for Event C3, C4, C5, C5a, C6 and C7 are FFS.

## Measurement reporting

### Measurement report triggering framework

It seems logical to re-use the current measurement report triggering framework (section 5.5.4.1 of 36.331) for CSI-RS related report triggering. More specifically:

a) It is assumed there is a CSI-RSsTriggeredList (analogous to cellsTriggeredList)

b) A report is triggered when a new TP meets the event entering condition. All TPs that passed the entering condition of an event are stored by the UE in CSI-RSsTriggeredList until they pass the leaving condition

c) The report contains all CSI-RSs in the CSI-RSsTriggeredList up to a configured maximum number of TPs.

Some open issues identified are:

Issue 1: Should a report be triggered on Leave?

Issue 2: Is there a need to support periodic / event triggered periodic reporting?

Issue 3: Others?

Companies are invited to comment on stated assumptions/open issues.

| Company name | Proposal/comment |
| --- | --- |
| InterDigital | To keep in line with the existing reporting events, the network should be allowed to configure reportOnLeave for the new introduced events. We think event triggered reporting should be sufficient for CoMP RRM procedures. |
| Ericsson | We believe a field called csi-rs-triggeredList containing references to all CSI-RS resources which fulfilled the event is useful. Its function is similar to cellsTriggeredList. Issue 1: We believe reportOnLeave is useful, but we think clarifications might be needed regarding which CSI-RS resources are “leaving” and which ones are “entering”. Issue 2: We believe there could be a need to support periodic and event triggered periodic reporting. |
| Intel | We also agree to have a csi-rs-triggeredList similar to cellsTriggeredList. We see benefit of reportOnLeave but do not see strong need for periodic or event periodic triggered reporting. |
| Panasonic | We agree that a separate ´leaving condition´ and ´reportOnLeave´ might be required but only based on the outcome of 2.3 (“How the network would remove cell from the CSI reporting including CSI measurement set…”). We don't see the need of the periodic/ event-triggered report configuration. On the other hand, if it is difficult to reach an agreement on what events need to be supported in Rel11, just to support periodic only is also one approach. |
| New Postcom | Issue 1: We think “reportOnLeave” should be supported in some events (e.g. Event C2 or C3) for CSI-RS resource, especially in the scenarios that Event C1 and C4 are not configured to provide the leaving condition of CRM set. Issue 2: It seems that periodic/event triggered periodic reporting may need to be supported in CRM measurement. |
| Nokia and NSN | Issue 1: Yes; Issue 2: No strong opinion. No big need foresee so far. |
| Samsung | Comment on Issue 1: To allow the network to exclude a TP from the CoMP measurement set, report should always be triggered on leave. We think there is no need to make this configurable, i.e. reportOnLeave parameter is not required. Comment on Issue 2: We currently do not see the motivation to introduce periodic / event triggered periodic reporting. |
| ZTE | Issue1: yes, we also think it is useful Issue2: we support to introduce periodic/event triggered period report. These event can provide more information to eNB. So it would be useful at the initial stage of CoMP implementation. People may have concern on the signalling overhead. But specification just provides kind of tool kits. Network can decide whether it want to configure it or not. [Samsung] Introducing feature with no strong need increase implementation and testing effort. |
| Huawei | Instead of TP, CSI-RS resource should be used as the basic unit in measurement report. Issue 1: we also think reportOnLeave can be useful. Issue 2: we haven’t see strong use case for periodic and event triggered periodic reporting. |
| Alcatel-Lucent | We think that reportOnLeave should be supported for both C1 and C2 to inform the network that a particular CSI-RS resource is no longer valid for CoMP Measurement Set. It is also useful to have event triggering periodic reporting to allow the network to know the RSRP order of the reported CSI-RS resources to maintain the right set of CSI-RS resources in the CoMP Measurement Set in the case where there are more than 3 strong candidates. As on periodic reporting, this may be useful for maintaining a suitable CoMP Resource Management Set. |
| Kyocera | Issue 1: We support to introduce reportOnLeave. Issue 2: No strong opinion to support periodic and/or event periodic triggered reporting. |
| LGE | Issue1: We think reporting upon meeting leaving condition for event C1 and C2 is useful. Using ‘ReportOnLeave’ is most straightforward approach for this. Issue2: Our understanding is that current RRM structure already can support event triggered periodical reporting if reportInterval and reportAmount are properly configured. Issue3, we think that the default inclusion of available CSI-RS measured results, in addition to the CSI-RS result that triggers the reporting, can be useful for network to capture the whole picture regarding the CSI-RS measured results. This is similar to default inclusion of measured results of all SCells in measurement report in case of CA. |
| NEC | We also agree to support a) to c) with renaming to CSI-RSsTriggeredList. Issue 1: Yes, Issue 2: No, but it is not a strong opinion. |
| ITRI | Since CSI-RS resource is not exactly related to one TP based on the definition of CRM, we prefer Ericsson’s naming, i.e., csi-rs-triggeredList. Issue 1: Yes. Issue 2: No much benefit is foreseen so far. |
| Fujitsu | We also agree to support a) to c) with renaming to CSI-RSsTriggeredList and that a report be triggered on Leave. |
| Hitachi | Issue 1: We also think reportOnLeave is useful. Issue 2: We don’t see strong need for periodic reporting so far. |
| acer | Issue 1: We think reportOnLeave should be configurable for at least C1. As to rapporteur’s proposed agreement 2 below, we think most companies support to have a reportOnLeave field for a trigger event instead of that a reporting should be triggered on leave. Issue 2: We have not seen a need to have periodic reporting. |

Summary of discussion:

All companies seem to agree to reuse the current measurement report triggering framework (section 5.5.4.1 of 36.331) for CSI-RS related report triggering.

Issue 1: All companies support report to be triggered on Leave (one company’s support is conditioned on the outcome of Sec 2.3).

Issue 2:  Regarding the need for periodic / event triggered periodic reporting, 10 companies don’t see the (strong) need or don’t have a strong opinion; 4 companies think that there may/could be a need and one company supports periodic / event triggered periodic reporting.

Proposed agreements:

- Re-use the current measurement report triggering framework (section 5.5.4.1 of 36.331) for CSI-RS related report triggering. More specifically:

a) It is assumed there is a CSI-RSsTriggeredList (analogous to cellsTriggeredList)

b) A report is triggered when the measurement of a candidate CSI-RS resource meets the event entering condition. All CSI-RSs that passed the entering condition of an event are stored by the UE in CSI-RSsTriggeredList until they pass the leaving condition

c) The report contains all CSI-RS resources in the CSI-RSsTriggeredList up to a configured maximum number of CSI-RS resources.

- Report should be triggered on leave.

- Periodic / event triggered periodic reporting is not supported unless a strong motivation is identified.

### Measurement report configuration

Companies are invited to provide views on what are the parameters that are required for measurement report configuration. Parameters that are necessary seem to be:

- Event Id

- Event threshold/offset

- Hysteresis

- Time to trigger

- Others?

Note that RAN1 has agreed only CSI-RSRP is supported, hence it is assumed that triggerQuantity and reportQuantity are not needed.

Companies are also invited to provide views with respect to the ASN.1 design:

Alt 1: Create the new events as part of the current IE ReportConfigEUTRA

- Alt 1 might result in less deviating behaviour in the future compared to the cell based reporting

- Alt 1 might result in more changes to existing IE, e.g. updating with respect to what parts of the configuration are not applicable to TP related reporting

Alt 2: Create the new events as part of the new IE specific for TP reporting

- Only IEs relevant for TP related reporting can be included in this new IE

- Alt 2 might result in more deviations/less consistency between cell and TP related reporting

| Company name | Proposal/comment |
| --- | --- |
| InterDigital | We prefer Alternative 1, where the new events can be created with the current ReportConfigEUTRA. The existing IEs such as triggerQuantity can potentially be extended to include CSI-RSRP. |
| Ericsson | We prefer Alternative 1, to extend the current IE ReportConfigEUTRA. With respect to our Event C3, a reference CSI-RS Id is also needed. triggerQuantity may need to be extended to include csi-rs-rsrp, but it is also possible to assume that all events related to CSI-RS are based on this metric. |
| Intel | Alternative 1 is preferred and list of parameters can be extended to add reference CSI-RS for proposed event 3 and CSI-RS Specific Offset. We also have slight preference of keeping triggerQuantity to be extended to include csi-rsrp . |
| Panasonic | We prefer Alt. 2 The answer to this question also depends on the outcome of 2.4.1 (whether to have triggerType other than event triggered). Besides, the following points should be considered: - one of the main content ´events´ shall be different from legacy, - some other mandatory IEs are no more required, and - clarity/ readability esp. in future when the mixed operation of CRS and CSI-RS are expected. Also, since for all CoMP scenarios we talk about the CSI RSRP reporting, there is not necessarily a need to also have a cell related reporting; i.e. TP related reporting only is sufficient. |
| New Postcom | We prefer Alt 1 to create the new events as part of the current IE ReportConfigEUTRA. |
| Nokia and NSN | Slightly prefer Alt 1 since we already agreed to reuse the measureObject to configure the CRM while there are a lot of parameters/IEs not applicable to CRM, should reuse current IE ReportConfigEUTRA for new event as well to make it possible to reuse the whole existing framework. Otherwise, should define new IEs for all, including CRM configuration, even trigger configuration and reporting. |
| Samsung | For other cases e.g. resource configuration for SCells, we agreed to create a new IE dedicated for SCells if there was a significant difference (e.g. when only 1 or 2 small fields are not applicable for SCells we would still re-use the existing IE). We should follow a similar principle here also. First we have to conclude the parameters that are needed. |
| ZTE | We basically agree with Panasonic and prefer alt2 |
| Huawei | We slightly prefer Alt 1 to maintain a consistent measurement reporting configuration. Similar to “maxReportCells”, a parameter like “maxReportResources” may also be need. |
| Alcatel-Lucent | It depends on the commonality between CRS RSRP reporting configuration and CSI RSRP reporting configuration. We think this should be discussed after agreeing on the other things (e.g. event triggered periodic reporting, TTT etc.) |
| Kyocera | We slightly prefer Alt 1. |
| LGE | We prefer to reuse existing measurement structure as much as possible, and thus prefer Alt 1. |
| NEC | We tend to agree with Panasonic and our mild preference is Alt2. |
| ITRI | We slightly prefer Alt. 1 if we agree to reuse the measureObject to configure the CSI-RS resource of CRM set. |
| Fujitsu | We prefer Alt 1: Create the new events as part of the current IE ReportConfigEUTRA |
| Hitachi | We slightly prefer Alt.1. |
| acer | We prefer alt.1. |

Summary of discussion:

12 companies express preference for Alt 1 while 3 companies prefer Alt 2. For Alt 1, several companies think it is possible to extend triggerQuantity to include CSI-RSRP. It is also pointed out that the decision also depends on the outcome of Sec 2.4.1 (see Panasonic’s comment for details).

Proposed agreements:

- Alt 1 is the baseline: Create the new events as part of the current IE ReportConfigEUTRA.

  - Can be revisited if needed

- At least the following parameters are relevant to CRM measurement reporting:

  - Event Id

  - Event threshold/offset

  - Hysteresis

  - Time to trigger

  - maxReportResources (reflecting agreement 1c in 2.4.1) (note: maxReportCells may be reused)

  - reportOnLeave

FFS on extending triggerQuantity to include CSI-RSRP.

### Measurement report content

As mentioned before, it is assumed that the list of TPs from the TPsTriggeredList is reported. The report could be added in the current measurement report (as part of MeasResults) or in a new report IE. Note that extending MeasResults implies due to ASN.1 coding that PCell related measurements are always to be included in the TP measurement report.

Companies are requested to provide views on detailed measurement report contents.

| Company name | Proposal/comment |
| --- | --- |
| InterDigital | We think that the measurement results should be added as part of MeasResult. We do not see the Pcell reporting as a big concern or big signalling overhead. |
| Ericsson | We think the reuse of measReasultListEUTRA could be done. We note that the field cgi-Info does not seem to have a correspondence in the CSI-RS context, but is anyway optional. We think the field physCellId corresponds to the CSI-RS resource Id discussed in section 2.2.2. We are not concerned about the overhead from PCell reporting. |
| Intel | Reporting as part of measResultListEUTRA should be feasible and extra overhead for PCell reporting is not big concern. |
| Panasonic | Our preference is to have new MeasResults for CoMP containing only CSI RSRP values for reported TPs (section 2.2.2) for each serving cell since most other Mandatory information (like measId, measResultPCell, physCellId, cgi-Info etc.) are not relevant anymore and the efforts in trying to ´fit in´ will be at the cost of overhead and readability (and perhaps also affect the existing UE mobility procedure/ implementation as commented in 2.2). |
| New Postcom | We think CRM measurement results should be included in measResultListEUTRA, and additional overhead is acceptable. |
| Nokia and NSN | Could be added to MeasResult. Index and RSRP of the TPs meet the event trigger should be reported, e.g.: For C1: Index and RSRP of all (or configurable N strongest) TPs above threshold (for both entering and leaving); For C2: Index and RSRP of all (or configurable N strongest) TPs between best and best-offset (for both entering and leaving). |
| Samsung | We prefer to extend the existing measurement report message. PCell measurements are not really needed, but given the limited size it is no real problem if this information is included. |
| ZTE | We have no strong position on this issue but slightly to prefer to have a new IE just to save signalling overhead and to decouple from existing measurement result IE. |
| Huawei | We think MeasResults can be extended with a new IE for CSI-RS measurement, which includes the CSI-RS resource index and RSRP result. |
| Alcatel-Lucent | MeasResults will always be present as long as the Measurement Report is used. We prefer using the Measurement Report for the CSI RSRP measurement (i.e. extending MeasResults to include CSI RSRP reporting). |
| Kyocera | We think signalling overhead could be acceptable level and prefer to extend MeasResults for CSI-RS RSRP reporting. |
| LGE | We prefer to reuse with existing RRM measurement procedure, where the MeasResults can be extended to further include CSI-RS measurement results. Similar to reporting of measured results of all configured SCells by default, it is useful to provide measured results of all available CSI-RS resources by default once CoMP is configured. |
| NEC | The IE MeasResults could be extended to include CSI-RS based measurements. Regarding the unnecessary (or less important) measurement results of PCell, we agree with Samsung that the overhead will not cause a big problem especially in the case that we do not support periodic or event-triggered periodic reporting. On the other hand, if the new IE is agreed for 2.4.2, then the new IE would be preferred. |
| ITRI | We think the measurement result could be part of MeasResult. In addition, we prefer using a new IE to carry the measurement result instead of using MeasResultEUTRA. A CSI-RS resource ID should be included in this new IE. |
| Fujitsu | We also prefer using the existing Measurement Report for the CSI RSRP measurement (i.e. extending MeasResults to include CSI RSRP reporting). |
| Hitachi | We prefer to extend the existing measurement report IE. |
| acer | We also support to include the measurement result in MeasResult, but doesn’t have strong opinion on whether a new IE should be introduced. |

Summary of discussion:

11 companies prefer to include the report in the IE MeasResults, while 2 companies prefer to introduce a new IE.

Proposed agreement:

- The IE MeasResults is extended to include CRM measurement report.

# Conclusions

Proposed agreements based on the email discussions are summarised below.

Measurement Model:

- L3 filtering is applicable for CRM measurement.

  - FFS on the need to modify the existing L3 filter parameter values.

CRM measurement on SCell:

- The decision on CRM measurement behavior on SCell is postponed until there is a clear RAN1 decision on the support of CA+CoMP scenario.

CSI-RS resource configuration:

- No change to the existing agreement that CSI-RS resources, on which UE is supposed to measure, are configured as part of the MeasurementObject.

CSI-RS resource parameters:

- List of parameters for CSI-RS resource parameters are:

  - AntennaPortsCount,

    - ENUMERATED {an1, an2, an4, an8}

    - Note: The range may be changed depending on the RAN1/4 decision on the measurement ports.

  - ResourceConfig,

    - INTEGER (0..31)

  - SubframeConfig,

    - INTEGER (0..154)

  - Scrambling initialization parameter

    - INTEGER (0..503)

  - CSI-RS resource specific offsets

    - Note: Range is assumed given by Q-OffsetRange

The need for id for CSI-RS resource:

- New id is defined and configurable by the network

  - The range of ids is FFS.

New Events:

- Event C1 as defined below is agreed:

Entering condition:

Leaving condition:

The variables in the formula are defined as follows:

- Mcp is the measurement result of the target CSI-RS resource (dBm for CSI-RSRP).

- Hys is the hysteresis parameter for this event (dB)

- Thresh is the offset parameter for this event (dB)

- Opcp is the offset of the target CSI-RS resource.

- Event C2 is supported. Exact definition is FFS.

- The needs for Event C3, C4, C5, C5a, C6 and C7 are FFS.

Measurement reporting triggering framework:

- Re-use the current measurement report triggering framework (section 5.5.4.1 of 36.331) for CSI-RS related report triggering. More specifically:

a) It is assumed there is a CSI-RSsTriggeredList (analogous to cellsTriggeredList)

b) A report is triggered when the measurement of a candidate CSI-RS resource meets the event entering condition. All CSI-RSs that passed the entering condition of an event are stored by the UE in CSI-RSsTriggeredList until they pass the leaving condition

c) The report contains all CSI-RS resources in the CSI-RSsTriggeredList up to a configured maximum number of CSI-RS resources.

- Report should be triggered on leave.

- Periodic / event triggered periodic reporting is not supported unless a strong motivation is identified.

Measurement report configuration:

- Create the new events as part of the current IE ReportConfigEUTRA is the baseline.

  - Can be revisited if needed

- At least the following parameters are relevant to CRM measurement reporting:

  - Event Id

  - Event threshold/offset

  - Hysteresis

  - Time to trigger

  - maxReportResources (reflecting agreement 1c in 2.4.1) (note: maxReportCells may be reused)

  - reportOnLeave

FFS on extending triggerQuantity to include CSI-RSRP.

Measurement report content:

- The IE MeasResults is extended to include CRM measurement report.

# Reference

- RAN2#78 Chairman’s note

- R2-121077		LS on CSI-RS based measurement for CoMP,	RAN1

- R2-121995		LS on the Definition of the CoMP Resource Management Set,	RAN1

- R1-123071		LS response on clarifications on CSI-RS based measurement for CoMP,	RAN1

- [78#52a] LTE/COMP: CSI-RS Measurement Framework (stage-2 text proposal)

- RAN1 email discussion: [69-13] R1-123028 WF on CoMP Resource Management Set and CSI-RSRP definition

- TS 36.300 V11.1.0

- R1-123077	LS on CSI-RSRP and CoMP Resource Management Set,	RAN1