3GPP TSG RAN WG2 Meeting #79-BIS	R2-124477

# Bratislava, SK, 8th – 12th Oct 2012

Source:	QUALCOMM Incorporated

Title:	UE Requirements for determination of Common E-RGCH Radio Links in FE-FACH

Agenda item:			10.1.6

Document for:	Discussion

# 1. Introduction

An uplink related improvement listed in the WI [1] for further enhancements to CELL_FACH is on signaling based interference control. In this contribution, we propose UE requirements on determination of common E-RGCH radio link in CELL_FACH.

# 2. Common E-RGCH based Interference Control

Common E-RGCH based interference control was introduced to handle the following uplink related problems in

- In the interest of conserving UE energy, cell reselection procedures in the non-cell DCH states (URA_PCH, CELL_PCH, CELL_FACH) are not that fast compared to serving cell change procedures in CELL_DCH. Hence, it is quite likely that when a UE initiates a transmission on the uplink in CELL_FACH, the serving cell need not be the strongest cell on the downlink. In the absence of any uplink imbalance between the serving and neighbor cell, the UE could cause interference to the neighbor cells.

- Furthermore, even if the serving cell is strongest on the downlink, if an uplink imbalance were to exist i.e. uplink to neighbor cell is stronger than uplink to serving cell, an uncontrollable interference condition could happen at the neighbor cell.

- If we were to operate CELL_FACH in heterogeneous networks (mix of different transmit power cells), the interference issue can only worsen due to severe uplink imbalance between the macro and small cells

- The lack of macro diversity or soft handover in CELL_FACH state can potentially lead to uplink interference at neighbor cells, especially as an increased number of smartphones begin to camp in CELL_FACH.

# 3. Factors affecting performance of common E-RGCH monitoring in CELL_FACH

One important factor that can affect the performance of this feature is the filtering duration that is used to decide if a Rel-11 CELL_FACH UE will listen to a neighbor cell’s common E-RGCH. Since it might not be practical to filter over a long observation interval in CELL_FACH state, the use of a very short filtering duration or no filtering can affect performance in  two cases as below:

- Failure to monitor Common E-RGCH from a strongly interfered neighboring  cell

A strongly interfered cell (cell where the UE is causing uncontrolled uplink interference) well within the reporting range can instantaneously leave the reporting range due to fading and the UE would not monitor this cell’s common E-RGCH. In this case, the Rel-11 UE is causing uncontrolled interference to the neighbor cell if the below hold true.

- Filtered EcIo of a strongly interfered cell faded down to leave the event 1a reporting range

- The uplink of the UE to the strongly interfered cell did not also fade down by the same amount as downlink

In Figure 1, the downlink of a strongly interfered cell EcIo fades down by more than y dB and leaves the event 1a reporting range. The time duration for which this lasts is indicated in Figure 1 as tfadedown . If at anytime during this tfadedown, if the UE were to make the decision as to which cells to monitor common E-RGCH from, then this strongly interfered cell would not be included. Although the downlink faded down by y dB, the UE can still be causing interference on the uplink to this cell as long as uplink to this cell did not fade down by y dB. When such a situation happens, the interference from this UE to this cell cannot be controlled by common E-RGCH for the entire duration of uplink data transfer while in CELL_FACH.

Figure 1: Circumstance leading to failure to monitor Common E-RGCH

- False monitoring of Common E-RGCH from a weakly interfered cell

A weakly interfered cell (cell where UE is not causing substantial interference) outside the reporting range can enter the reporting range due to fading and thus cause the UE to listen to its common E-RGCH. In this case, the UE may reduce its transmit power based on common E-RGCH commands from this cell thereby impacting its own user experience. This can happen if the below holds true.

- Filtered DL EcIo of a weakly interfered neighbor cell enters the event 1a reporting range (due to an upfade) and

- The uplink of the UE to the weakly interfered cell did not also fade up by the same amount as downlink

In Figure 2, a weakly interfered cell EcIo fades up by more than x dB on the downlink to enter the event 1a reporting range. The time duration for which this even lasts is indicated in Figure 2 as tfadeup . If at anytime during this tfadeup, if the UE were to make the decision as to which cells to monitor common E-RGCH from, then this weakly interfered cell will be included. Although the downlink faded up by x dB, the UE will not be causing interference on the uplink to this cell as long as uplink to this cell did not also fade up by x dB. In such a case, the UE monitoring E-RGCH from this cell will reduce its uplink power unnecessarily (i.e. the UE could limit its serving grant by listening to common E-RGCH without perceptibly reducing interference to this neighbor cell). This will affect the UE for the entire duration of uplink data transfer while in CELL_FACH.

Figure 2: Circumstance leading to false monitoring of Common E-RGCH

To minimize the occurance of failure to monitor common E-RGCH as well as false monitoring of common E-RGCH, requirements need to defined for common E-RGCH monitoring in CELL_FACH.

# 4. Requirements for common E-RGCH in CELL_FACH

In [2], minimum requirements on missed down probability of NS E-RGCH in CELL_DCH is defined. For CELL_FACH, since common E-RGCH channel has similar characteristics as NS E-RGCH in CELL_DCH, we can reuse the performance requirement tests for NS E-RGCH from [2]. In the case of CELL_FACH, the missed down probability will have to include the probability of failure to monitor common E-RGCH due to neighboring cell not satisfying the Event 1a criteria. If the probability of failure to monitor common E-RGCH is “x” due to event 1a criteria not being satisfied, and the minimum requirements on the missed down probability for NS E-RGCH in CELL_DCH is <=5% (as per [2]), the effective missed down probability will increase to PMD = x + (1-x)*5%.

For the parameters specified in Table 1 , parameters the effective missed down probability is specified in Table 2.

Table 1: Parameters for common E-RGCH in CELL_FACH for Missed down probability testing

| Parameter | Unit | Value |
| --- | --- | --- |
|  | dBm/3.84 MHz | -60 |
| Phase reference | - | P-CPICH |
| P-CPICH | dB | -10 |
| Reporting Range (Event 1a) | dB | 4.5 |
| Hysteresis (Event 1a) | dB | 0 |
| E-AGCH information |  | Fixed SG2 |
| E-RGCH signalling pattern for Non-serving E-DCH RL |  | 100% DOWN |

Table 2 : Effective Missed down probability for parameters specified in Table1

| Propagation Conditions | Reference value |  |  |
| --- | --- | --- | --- |
|  | E-RGCH (dB) | Îor1/Ioc and Îor2/Ioc (dB) | Missed DOWN probability |
| VA30 | -27.3 | 0 | 0.31 |

The filtering of CPICH EcpIo should be performed to meet the above defined requirements. Since it’s a current agreement in RAN2 that a filter time constant for filtering serving as well as neighboring cell EcpIo will be signaled for common E-RGCH feature in cell_FACH, it might not be possible to meet the above requirement with the signaled time constant.

Proposal 1: We propose to remove filter time constant from the list of parameters that are signaled to the UE for common E-RGCH monitoring in CELL_FACH. The filtering should be implementation dependent to meet the above defined requirements. RAN4 to provide input to RAN2 accordingly so that RAN2 can update their specifications.

The start time time for monitoring common E-RGCH from a neighboring cell should be based on the filtering time required to meet the above missed down probability.

Proposal 2: RAN4 to give input to RAN2 with respect to the start time of monitoring common E-RGCH when the UE is transmitting on a common E-DCH resource in CELL_FACH state. Accordingly, RAN2 can update their specifications.

# 5. Conclusion

In this document, we have motivated the need for defining minimum requirements for missed down probability while monitoring common E-RGCH in CELL_FACH. We have also additionally laid out simulation parameters and the missed down probability of common E-RGCH while in CELL_FACH.  Based on the requirements, we propose the following

Proposal 1: We propose to remove filter time constant from the list of parameters that are signaled to the UE for common E-RGCH monitoring in CELL_FACH. The filtering should be implementation dependent to meet the above defined requirements. RAN4 to provide input to RAN2 accordingly so that RAN2 can update their specifications.

Proposal 2: RAN4 to give input to RAN2 with respect to the start time of monitoring common E-RGCH when the UE is transmitting on a common E-DCH resource in CELL_FACH state. Accordingly, RAN2 can update their specifications.

# 6.	References

RP-110436, “Further Enhancements to CELL_FACH”, WID

3GPP TS 25.101 “User Equipment (UE) radio transmission and reception (FDD)”