3GPP TSG-RAN WG2 #81b	 R2-130975

Chicago, USA, 15th – 19th April, 2013

Agenda Item:	5.2

Source: 	CMCC

Title:  	Discussion on SA2 SDDTE Solutions from RAN2 Perspective

Document for:	Discussion and Decision

Introduction

In RAN #59, a new SI: Study on RAN aspects of Machine-Type and other mobile data applications Communications enhancements, was approved. One objective listed in SID [1] is:

- RAN2 should provide feedback to SA2 identified solutions with RAN impact by the required timeframe.

In this contribution, we discuss the impacts of SA2 solutions for small data and device triggering on RAN side.

Discussion

In TR 23.887, 8 solutions (solutions in 5.1.1.3.1~5.1.1.3.7) are presented for small data and device triggering enhancements, and we classify them into six categories:

NAS signaling based solutions

In these solutions (e.g. solutions in 5.1.1.3.1~5.1.1.3.3), small data is piggybacked in NAS message and transmitted through RRCConnectionSetup message (as shown in Fig 1). For transmitting a UL data and a DL ack, 6 steps in RAN side are required.

The main advantage of this solution, compared to the normal RAN side procedure, is that it reuses SRB1 and NAS encryption to transmit the data, so it has little impact on RAN side, and it doesn’t require eNB to activate RRC security, and to reconfigure UE to establish data radio bearers.

Several disadvantages of this solution are:

- It is only suitable for infrequent small data. For frequent small data (e.g. one packet per several seconds), it may lead to too much signalling overhead, especially considering that application layer may abuse this ‘small data’ NAS procedure.

- RRCConnectionSetupComplete message typically carries NAS PDU of around 80 bytes. Thus carrying small data of around 1KB may challenge L1/L2 mechanism.

- Because AS security is not activated, eNB will not configure UE for mobility measurements. Therefore, if UE is moving to another cell when waiting to receive the DL ACK, RLF may happen.

UL small data transmission                DL small data transmission

Fig 1 RRC signalling of small data transmitted through NAS PDU

Stateless gateway solution

It (e.g. solution in 5.1.1.3.4) combines SGW and PGW into one CGW to reduce signalling as well as gateway state information retention. This solution has no change on radio interface, but rather has some modification on S1 interface. It might be beneficial to reduce core network signalling, but doesn’t reduce AS signaling.

Downlink small data transfer using RRC message

This solution (e.g. solution in 5.1.1.3.5) is an optimization of the transmission procedure of downlink small data using NAS Signalling based solution. As shown in figure 2, small data is transferred through RRC connection setup message instead of DLInformationTransfer message. Compared with NAS siganlling based solution, two more messages in RAN and two more messages between eNB and MME are reduced.

The weaknesses of this solution are:

- All the eNBs in the corresponding Tracking area have to buffer the small data for a certain time duration. If the tracking area is large, the memory consumption will be severe.

- RRC Connection Setup message is transmitted in TM mode rather than AM mode, therefore it is impossible for RAN to detect the NACK-to-ACK inversion error of HARQ. So the reliable transmission of small data cannot be guaranteed.

- It increases the possibility of RRC connection setup failure due to the increased message size.

- It has the same mobility problem as NAS signalling based solution.

Fig 2 Downlink small data transfer using RRC message

Fast path/connectionless solution

In small data fast path/Connectionless solution (e.g. solutions in 5.1.1.3.6), small data is not piggybacked in NAS message, but rather transmitted directly between eNB and SGW, without going to MME.

The potential advantages of these solutions are:

- There is no upper limit of small data volume except for constrain of radio design.

- Less signaling is consumed than NAS signaling based solution because MME will not be involved.

However, both of the solutions require new design of RAN side transmission mechanism in idle state, and have similar mobility problem as NAS signalling based solution. For fast path solution, security is in a different way than current RAN security mechanism. It is between UE and SGW, so additional work is needed. For connectionless solution, current RAN security strategy can be reused, but eNB and UE need to store the security key in idle.

Service request signaling reduction solution

This solution (e.g. solution in 5.1.1.3.7) uses RRCConnectionRequest message to transmit service request as illustrated in Fig 3. One possible advantage is for the mobility. Because AS security is activated, it is possible for eNB to configure the mobility measurement, so there would be no mobility problems at the cell border.

Several disadvantages are listed as follows:

- New mechanism should be introduced for eNB to allocate enough UL resources due to the increased size of RRCConnectioRequest.

- It requires additional security mechanism to protect the DRB config IE.

- The delay between RRCConnectionRequest and RRCConnectionSetup will be increased due to the interaction between MME and eNB.

- It consumes more signalling than NAS based signalling and fast path/connectionless solution.

- It only combines several steps, and nearly doesn’t reduce the signalling overhead.

Fig 3 Service request signalling reduction by RRC combining

Keep UE in connected mode solution

For this solution (e.g. solution in 5.1.2.3.1), eNB adjusts user inactivity timer to a longer value. The main benefits of the solution are: it doesn’t have specification change to the protocol, and greatly reduces the signalling overhead.

However, it faces several challenges:

- Keeping UE in connected mode will consume considerable amount of UE power and how to reduce it is challengeable.

- eNB needs to support a large amount of UEs in connected state, which requires large memory for UE contexts. However whether this is a real problem needs network vendor’s clarification.

- It may be only suitable for delay tolerant traffic if longer DRX is introduced.

The following is a summary of the solutions mentioned above:

Table 1 a summary of the SA2 solutions for small data transmission

| Solution | RAN events (Data + ack) [NOTE 1] | Core signalling (Data + ack) [NOTE 2] | Supported Data frequency | Regular traffic or not | Data transmission Reliability | Traffic Delay in RAN side | Mobility problem | RAN Protocol impact |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2.1 | MO: 6 MT: 9 | MO: 4 MT: 6 | Infrequent | Regular and irregular | Reliable | MO: tens of ms MT: depends on paging cycle | Yes | Little |
| 2.2 | MO: 12 MT: 15 | MO: 6 MT: 8 | Infrequent | Regular and irregular | Reliable | MO: tens of ms MT: depends on paging cycle | Yes | No |
| 2.3 | MT: 7 | MT: 2 | Infrequent | Regular and irregular | Unreliable | MT: depends on paging cycle | Yes | Medium |
| 2.4 | Equal or smaller than solution 1(Depends on RAN design) | MO: 2 MT: 4 | Depends on the RAN design | Regular and irregular | Depends on RAN design | MO: tens of ms MT: depends on paging cycle | Yes | Depends on RAN design |
| 2.5 | MO: 8 MT: 9 | MO: 8 MT: 10 | Infrequent | Regular and irregular | Reliable | MO: tens of ms MT: depends on paging cycle | No | Large |
| 2.6 | MO: 0 MT: 0 | MO: 0 MT: 0 | Frequent | Regular and irregular | Reliable | MO: tens of ms MT: depends on DRX cycle | No | Little |
|  | Note 1: events include RRC signalling, preamble transmission and RAR. Besides, RRC connection release is also counted. Note 2: only signalling between eNB and MME, and signalling between MME and SGW are counted. Besides, signalling for RAN side to release the connection is also put into account. |  |  |  |  |  |  |  |

From the table we can see that most of the solutions require more than 9 steps for the transmission of one packet pair. For small data transmission whose inter arrival time is in the order of seconds, these solutions may still result in considerable signaling overhead, therefore some optimized solutions, e.g. taking advantage of the traffic feature, are desirable:

Proposal 1: Optimized solution based on specific traffic feature is needed to further reduce signaling overhead, especially for data inter arrival time in the order of seconds.

Besides, most of the solutions have more or less mobility problem, whether this is negligible needs to be evaluated:

Proposal 2: RAN2 is kindly asked to evaluate whether mobility problem of SA2 solutions is negligible.

Some of the solutions use RRC signalling to transmit small data, which is against the initial design principle of RRC message. As we know, the size of RRC connection request message will affect the design of random access, and the size of RRC connection setup message may impact the success probability of RRC connection setup. As to the size of RRC connection setup complete, it may have less impact, but we still need to evaluate.

Proposal 3: RAN2 is kindly asked to evaluate the potential impact of increased size of RRC message (e.g. RRC connection request, RRC connection setup, RRC connection setup complete).

Service request signaling reduction solution needs too much specification change in RAN side, and the benefit is very disappointing. Therefore, we propose:

Proposal 4: Service request signaling reduction solution should not be considered as a solution for SDDTE.

Conclusions and Proposal

In this contribution, we discuss the impacts of SA2 solutions for small data and device triggering on RAN side, and propose that:

Proposal 1: Optimized solution based on specific traffic feature is needed to further reduce signaling overhead, especially for data inter arrival time in the order of seconds.

Proposal 2: RAN2 is kindly asked to evaluate whether mobility problem of SA2 solutions is negligible.

Proposal 3: RAN2 is kindly asked to evaluate the potential impact of increased size of RRC message (e.g. RRC connection request, RRC connection setup, RRC connection setup complete).

Proposal 4: Service request signaling reduction solution should not be considered as a solution for SDDTE.

References

RP-130396, “Study on RAN aspects of Machine-Type and other mobile data applications Communications enhancements,” 3GPP TSG RAN Meeting #59, Vienna, Austria, Feb 26th – Mar 1st, 2013