3GPP TSG-RAN2 Meeting #81bis	R2-131014

# Chicago, USA, April 15th – 19th, 2013

| CR-Form-v10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.302 | CR | CRNum | rev | - | Current version: | 11.2.0 |  |
| For HELP on using this form look at the pop-up text over the symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on downlink reception type combinations for UEs supporting multiple TAGs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_CA_enh-Core |  |  |  |  |  |  | Date: |  | 2013-3-27 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-11 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) |  |
| Reason for change: |  | According to current Table 8.2-2 in TS36.302, simultaneous reception of DL-SCH with C-RNTI/SPS-CNTI and PDCCH order for SCell on different cell is not allowed for UE supporting multiple TAGs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Include entries in Table 8.2-2 corresponding to simultaneous reception of DL-SCH with C-RNTI/SPS-CNTI and PDCCH order for SCell on different cell for UEs supporting multiple TAGs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE supporting multiple TAGs cannot simultaneously receive DL-SCH with C-RNTI/SPS-CNTI and PDCCH order for SCell on different cell. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

8.2	Downlink

The table describes the possible combinations of physical channels that can be received in parallel in the downlink in the same subframe by one UE. In one subframe, the UE shall be able to receive all TBs according to the indication on PDCCH.

Table 8.2-1: Downlink "Reception Types"

| "Reception Type" | Physical Channel(s) | Monitored RNTI | Associated Transport Channel |
| --- | --- | --- | --- |
| A | PBCH | N/A | BCH |
| B | PDCCH+PDSCH | SI-RNTI | DL-SCH |
| C | PDCCH+PDSCH | P-RNTI | PCH |
| D | PDCCH+PDSCH | RA-RNTI (Note 3) | DL-SCH |
|  |  | Temporary C-RNTI (Note 3) (Note 4) | DL-SCH |
|  | (PDCCH/EPDCCH) +PDSCH | C-RNTI and Semi-Persistent Scheduling C-RNTI | DL-SCH |
| D1 | (PDCCH/EPDCCH) +PDSCH (Note 9) | C-RNTI | DL-SCH |
| E | PDCCH/EPDCCH (Note 1) | C-RNTI | N/A |
| F | PDCCH | Temporary C-RNTI (Note 5) | UL-SCH |
|  | PDCCH/EPDCCH | C-RNTI and Semi-Persistent Scheduling C-RNTI | UL-SCH |
| F1 | PDCCH/EPDCCH (Note 9) | C-RNTI | UL-SCH |
| G | PDCCH | TPC-PUCCH-RNTI | N/A |
| H | PDCCH | TPC-PUSCH-RNTI | N/A |
| I | PDCCH/EPDCCH | Semi-Persistent Scheduling C-RNTI (Note 6) | N/A |
| J | PDCCH/EPDCCH | Semi-Persistent Scheduling C-RNTI (Note 7) | N/A |
| K | PDCCH | M-RNTI (Note 8) | N/A |
| L | PMCH | N/A (Note 8) | MCH |
| Note 1: PDCCH or EPDCCH is used to convey PDCCH order for Random Access. Note 2: Void. Note 3: RA-RNTI and Temporary C-RNTI are mutually exclusive and only applicable during Random Access procedure. Note 4: Temporary C-RNTI is only applicable when no valid C-RNTI is available. Note 5: Temporary C-RNTI is only applicable during contention-based Random Access procedure. Note 6: Semi-Persistent Scheduling C-RNTI is used for DL Semi-Persistent Scheduling release. Note 7: Semi-Persistent Scheduling C-RNTI is used for UL Semi-Persistent Scheduling release. Note 8: In MBSFN subframes only Note 9: DL-SCH reception corresponding to D1, and UL-SCH transmission corresponding to F1, are only applicable to SCells. |  |  |  |

Table 8.2-2: Downlink "Reception Type" Combinations

| Combination | Mandatory/Optional | Comment |
| --- | --- | --- |
| 1xA + 1xB + 1xC | Mandatory | RRC_IDLE |
| 1xK + 1xL | Mandatory for MBMS UEs | RRC_IDLE |
| 1xA + 1xB + 1x(D or E or G or I) + (p-1-m)xD1 + mxE + 1x(F or H or J) + (q-1)xF1 | Mandatory. (NOTE 3) (NOTE 4) | RRC_CONNECTED |
| 1xA + 1xB + 1x(D or E or G or I) + 1x(F or H or J) + 1xF+ (p-1-m)xD1 + mxE + 2x(q-1)xF1 | Mandatory for UEs supporting FS2. (NOTE 3) (NOTE 4) | RRC_CONNECTED (NOTE 1) |
| ((1x(E or G or I) + 1xL) or 1xD) + 1x(F or H or J) + 1xK + (p-1-m)xD1+ mxE + (q-1)xF1 | Mandatory for MBMS UEs. (NOTE 3) (NOTE 4) | RRC_CONNECTED (NOTE 2) |
| ((1x(E or G or I) +1xL) or 1xD) + 1x(F or H or J) + 1xF + 1xK + (p-1-m)xD1+ mxE + 2x(q-1)xF1 | Mandatory for MBMS UEs supporting FS2. (NOTE 3) (NOTE 4) | RRC_CONNECTED (NOTE 1) (NOTE 2) |
| 1xA + 1xB + 1xC + 1x(D or E or G or I)+(p-1-m)xD1+ mxE + 1x(F or H or J)+(q-1)xF1 | Mandatory for ETWS and CMAS UEs Optional for all other UEs. (NOTE 3) (NOTE 4) | RRC_CONNECTED |
| 1xA + 1xB + 1xC + 1x(D or E or G or I) + 1x(F or H or J) + 1xF + (p-1-m)xD1+ mxE + 2x(q-1)xF1 | Mandatory for ETWS and CMAS UEs supporting FS2 Optional for all other UEs. (NOTE 3) (NOTE 4) | RRC_CONNECTED (NOTE 1) |
| NOTE 1: For TDD UL/DL configuration 0, two PDCCHs or EPDCCHs can be received in the same subframe for UL-SCH in two different uplink subframes. NOTE 2: The combination is the requirement when MBMS reception is on PCell. If the UE is capable to receive MBMS on any other cell, it is not required to simultaneously receive MBMS on PCell. NOTE: p is the number of DL CCs supported by the UE. q is the number of UL CCs supported by the UE. q = p = 1 implies non-CA capable UE. NOTE: The UE is only required to receive one PDSCH, pertaining to D or D1, per DL CC. NOTE 3: Combination involving EPDCCH is optional and required only for UE supporting EPDCCH. NOTE 4: m = 0 or 1 for UE supporting multiple TAGs, otherwise m=0. The UE is not allowed to simultaneously receive more than one PDCCH order for Random Access. |  |  |

NOTE:	Any subset of the combinations specified in table 8.2-2 is also supported.