# 3GPP TSG RAN WG2 Meeting #81 BIS                                                        	R2-131371

# Chicago, USA, April 15 – April 19, 2013

# Title: 	Improving HO performance by multicasting HO Command

# Source: 	Research In Motion UK Limited

# Agenda Item:	7.1.1

# Document for: 	Discussion

# Introduction

HetNet mobility enhancement for LTE has been approved as a Work Item for Rel-12 [1]. The WI will focus on the aspects or problems already studied in the Rel-11 HetNet Mobility SI and documented in TR36.839. One objective of the WI is to improve overall HO performance with regard to HO failure and ping-pong in HetNet environments. .

In this contribution, we will discuss possible solutions to improve the successful reception of HO command at the UE.

As noted in earlier discussions, this work is closely related to the work described in Small cell study item for release 12.

# HO command multicasting

It has been concluded during the study phase that the mobility performance with the currently defined HO mechanism is not as good in a typical heterogeneous deployement compared to the performance in a homogeneous network deployment. In particular, when the UE is moving from a low-power cell to a macro cell, the probability of UE successfully receiving the HO command (RRCConnectionReconfiguration message with the Mobility controlInfo IE) is low [1]. The probability of successful reception further reduces by increasing the cell range expansion (CRE) bias.

In these situations, the successful reception of HO command can be improved if the message is transmitted by multiple cells. Assuming that the HO command is available at the neighbouring cells (same or different eNB), the cells with respect to which the UE has acceptable DL SINR can transmit the message. For example if the probability of message error with respect to the transmission from cell-i is pi; if N cells transmit the same message, then the probability of message error, pN becomes:

pN=j=0N-1pj

As shown in Figure 1, when UE-a is moving away from a pico cell towards a macro cell, the HO command can be transmitted by both the serving cell, i.e. pico cell, P0, and the target cell, i.e. macro cell (M0). In another scenario, as illustarted in Figure 2, when UE-b is moving away from a pico cell, P0 towards another pico cell, P1, the HO command can be transmitted by the serving cell, i.e. the pico cell, P0, the target cell, i.e. the pico cell, P1 and the macro cell, M0.

Based on the deployment scenario, the UE may be asked to monitor other neighbouring cells for HO command in case the HO command is not received from the serving cell. The potential neighbour cells for transmitting the HO command can be configured by the serving eNB by defining a threshold on the neighbour list measurements. For example all the neighbour cells with respect which the measured RSRQ is above certain threshold can be monitored for the HO command in case of a suspected RLF. The transmission of HO command at each cell can be individually scheduled to reduce the coordination complexity.

Figure 1

Figure 2

# Simulation results

The mobility performance of HO command multicasting has been evaluated using the large area HetNet simulation. In the simulation the HO command is sent from both the source and target cell. The HO command is assumed to be successfully received at the UE if the filtered SINR over the past 200msec from either the source cell or the target cells is above Qout (-8dB). The SINR is estimated once every 10msec and averaged over a sliding window of 200msec.  We follow the simulation assumptions in TR36.839 and TR 36.814. We assumed 0dB CRE without eICIC. Other simulation assumptions are described in Table 1.

Table 1 Simulation assumptions

| Items | Descriptions |
| --- | --- |
| Configuration parameter set | Set 3 in TR36.839 with UE speed 60km/h |
| Pico palcement | 1 pico per macro cell, the pico is placed at boresight 1/2 ISD |

Table 2 to Table 4 compare the mobility performance with and without HO command multicasting. We observe that for UE speed of 60km/h HO command multicasting greatly reduces the number of HOFs/UE/sec and significantly improves the HOF rate. The overall HOF rate is reduced from 9% to 0.1%. The HO command multicasting also significantly reduces the number of RLFs. At higher UE speeds and/or larger CRE bias, much larger gain are expected with multicasting HO command.

Table 2 Handover performance

|  | Handover state | Handover metrics | Handover performance in HetNets |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | macro-pico | pico-macro | macro-macro | pico-pico | Overall |
| w/o HO command multicasting | 2 | HOFs/UE/s | 0.002395 | 0.003554 | 0.007185 | 0.000026 | 0.013159 |
|  |  | HO failure rate [%] | 8.848716 | 13.622902 | 6.959341 | 100.000000 | 8.412907 |
|  | 3 | HOFs/UE/s | 0.000026 | 0.000052 | 0.000129 | 0.000000 | 0.000206 |
|  |  | HO failure rate [%] | 0.095147 | 0.197433 | 0.124719 | 0.000000 | 0.131709 |
|  | Total | Successful HOs/UE/s | 0.024644 | 0.022481 | 0.095922 | 0.000000 | 0.143047 |
|  |  | HOFs/UE/s | 0.002421 | 0.003605 | 0.007313 | 0.000026 | 0.013365 |
|  |  | HO failure rate [%] | 8.943863 | 13.820336 | 7.084061 | 100.000000 | 8.544616 |
| with HO command multicasting | 2 | HOFs/UE/s | 0.000013 | 0.000013 | 0.000000 | 0.000000 | 0.000025 |
|  |  | HO failure rate [%] | 0.049925 | 0.050226 | 0.000000 | 0.000000 | 0.016724 |
|  | 3 | HOFs/UE/s | 0.000025 | 0.000025 | 0.000114 | 0.000000 | 0.000165 |
|  |  | HO failure rate [%] | 0.099850 | 0.100452 | 0.113023 | 0.000000 | 0.108705 |
|  | Total | Successful HOs/UE/s | 0.025368 | 0.025216 | 0.100889 | 0.000025 | 0.151499 |
|  |  | HOFs/UE/s | 0.000038 | 0.000038 | 0.000114 | 0.000000 | 0.000190 |
|  |  | HO failure rate [%] | 0.149775 | 0.150678 | 0.113023 | 0.000000 | 0.125429 |

Table 3 RLF performance

|  | Average number of RLFs/UE/second with HetNet |  |  |  |
| --- | --- | --- | --- | --- |
|  | State 1 | State 2_Normal | State 2_HOF | Overall |
| w/o HO command multicasting | 0.000000 | 0.000000 | 0.007957 | 0.007957 |
| w/ HO command multicasting | 0.000000 | 0.000000 | 0.000025 | 0.000025 |

Table 4 Short time-of-stay (ToS) performance

|  | ToS metrics | Overall |
| --- | --- | --- |
| w/o HO command multicasting | Short ToS/UE/s | 0.022584 |
|  | Short ToS rate [%] | 15.787579 |
| w/ HO command multicasting | Short ToS/UE/s | 0.021550 |
|  | Short ToS rate [%] | 14.224715 |

# Conclusion

In this paper, we discussed HO-command multicasting mechanisms to improve HetNet mobility performance.

# Reference

RP-122007, New WI proposal: Hetnet mobility enhancement for LTE

TR 36.839, “Mobility Enhancements in Heterogeneous Networks”, v11.1.0, Dec. 2012.

The handover failure rates in this column should be ignored as they are not significant statistical results due to the extremely few samples.