# 3GPP TSG RAN WG2 Meeting #81bis                                                         	R2-131455

# Chicago, Apr. 15 – 19, 2013

# Title: 	Mobility Performance of Inter-frequency Small Cells

# Source: 	Research In Motion UK Limited

# Agenda Item:	7.2

# Document for: 	Discussion

# Introduction

During RAN2#81, three deployment scenarios were identified for small cell enhancements:

Scenario #1: Macro and small cells on the same carrier frequency (intra frequency) connected via non-ideal backhaul

Scenario #2: Macro and small cells on different carrier frequencies (inter frequency) connected via non-ideal backhaul

Scenario #3: Only small cells on one or more carrier frequencies connected via non-ideal backhaul typically low and medium UE mobility

The mobility performance for Scenario #2 (small cells and macro cells on different carriers) was not studied in the Rel-11 HetNet mobility SI. It is unclear whether the handover failure would increase in this case and if it increases by how much. In addition, the mobility performance of Scenario #1 (small cells and macro cells on the same carrier) for the small cell cluster deployment was not studied in the Rel-11 HetNet mobility SI. In this contribution, we will show the mobility performance for Scenario #1 and #2 assuming that small cells are deployed in a cluster.

# Simulation Assumptions

Mobility performance has been evaluated using the large area HetNet simulations. There are 57 macro cells in the system with an ISD of 500m. Small cells are placed in the macro area in a cluster. We use the small cell cluster model agreed in RAN1#72 [1]. As shown in Figure 1, in the simulation we assume one small cell cluster per macro cell. In each cluster, 10 small cells are randomly dropped within a radius of 50m. The minimum distance between two small cells is 20m. The small cell cluster is placed at the boresight 1/2 ISD (i.e., the small cell cluster is deployed at the macro cell boundary).

In case of Scenario #2, we assume that the macro cell and the small cell are on 2GHz and 3.5GHz, respectively. The pathloss in the small cell at 3.5GHz is derived based on the ITU UMi (Urban Micro) NLOS pathloss model:

PL=36.7log10d+ 22.7+26log10fc=36.8+36.7log10d, d in m, fc in GHz

For Scenario #2 of inter-frequency small cells, the A3 event is configured for both intra-frequency and inter-frequency. For intra-frequency cells the A3 event is triggered based on RSRP and for inter-frequency cells the trigger is based on RSRQ.

The remaining simulation assumptions follow from TR36.839 and TR36.814. Table 1 summarizes our simulation assumptions.

Figure 1 small cell cluster deployment

Table 1 Simulation assumptions

| Items | Descriptions |
| --- | --- |
| Macro cell ISD | 500m |
| Carrier frequency | Scenario 1 of intra-freq small cells: 2GHz for both macro and small cells Scenario 2 of inter-freq small cells: 2GHz for macro cell, 3.5GHz for small cell |
| Small cell placement | One small cell cluster per macro cell. In each cluster, 10 small cells are randomly dropped within a radius of 50m. The cluster center is at boresight 1/2 ISD. The minimum distance between two small cells is 20m. |
| Target cell selection | Based on RSRP for intra-frequency cells; based on RSRQ for inter-frequency cells |
| Intra-freq RSRP measurement | measurement period 200ms; L3 filter K=1 |
| Inter-freq RSRQ measurement | measurement period 480ms; L3 filter K=1; |
| A3 offset | 2dB for both intra-frequency A3 event (RSRP based) and inter-frequency A3 event (RSRQ based) |
| TTT | 160ms |
| UE speed | 3km/h, 30km/h, and 60km/h |
| Pathloss | Macro cell: PL=15.3+37.6log10d, d in m Small cell at 2GHz: PL=30.5+36.7log10d, d in m Small cell at 3.5GHz: PL=36.8+36.7log10d, d in m |
| Fast fading | On |
| Cell loading | 100% |

# Simulation Results

Figure 2 to Figure 4 show the HOF and RLF performance of intra-frequency and inter-frequency small cell clusters (the complete set of the results are shown in Appendix). Here are the observations:

- Figure 2 and Figure 3 show the number of HOFs and the number of RLFs per UE per second, respectively. We observe that the macro only deployment has a better HOF/RLF performance than the intra-frequency and inter-frequency small cell deployments.

- Figure 2 and Figure 3 also show that the inter-frequency small cell deployment has a better HOF/RLF performance than the intra-frequency deployment due to the less interference as the small and macro cells are deployed on separate carriers. Although in inter-frequency small cell case the long measurement period for inter-frequency cells may delay the handover and degrade the mobility performance, it seems that the gain from the reduced interference is more than the degradation from the long measurement period.

- Figure 4 shows the HOF rates of different HO types in small cell deployments with UE speed of 30km/h. Due to reduced interference, the HOF rates in inter-frequency small cell deployment are much improved compared to the case of intra-frequency small cell. However we observe a moderate improvement for pico-to-pico case. This could be due to the fact that the small cell cluster is deployed at the macro boundary. The amount of macro interference reduced by the inter-frequency deployment is limited and hence a moderate gain is observed.

- We also note that in inter-frequency small cell deployment due to the long measurement period of inter-frequency cells, the system tends to perform more intra-frequency HOs (macro-to-macro and pico-to-pico).

- If dual connectivity is used to anchor the UE’s control plane on the macro cell (i.e., the HO related signals are sent from/to the macro cell), the HOF/RLF performance of the small cell deployment (inter or intra-frequency small cells) can be improved and can be as good as that of the macro only deployment.

Figure 2 HO performance

Figure 3 RLF performance

Figure 4 HOF rates of different HO types (UE speed 30km/h)

# Conclusion

In this paper, we discussed the mobility performance of HetNets with inter-frequency and intra-frequency small cell clusters. We have the following observations:

Observation 1: The macro only deployment has a better HOF/RLF performance than the intra-frequency and inter-frequency small cell deployments.

Observation 2: The inter-frequency small cell deployment has a better HOF/RLF performance than the intra-frequency deployment.

Observation 3: If dual connectivity is used to anchor the UE’s control plane on the macro cell (i.e., the HO related signals are sent from/to the macro cell), the HOF/RLF performance of the small cell deployment can be as good as that of the macro only deployment.

# Reference

R1-130856, Evaluation assumptions for small cell enhancements – physical layer

TR36.839, Mobility enhancements in heterogeneous networks

TR36.814, Further advancements for E-UTRA physical layer aspects

# Appendix

Table 2 Handover performance

|  |  |  | Handover performance in HetNets |  |  |  |  | macro only system |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Handover state | Handover metrics | macro-pico | pico-macro | macro-macro | pico-pico | Overall | macro-macro |
| 3km/h, inter-freq | 2 | HOFs/UE/s | 0.000000 | 0.000040 | 0.000241 | 0.000415 | 0.000696 | 0.000280 |
|  |  | HO failure rate [%] | 0.000000 | 0.551471 | 4.838710 | 4.696970 | 2.386416 | 2.169625 |
|  | 3 | HOFs/UE/s | 0.000000 | 0.000000 | 0.000000 | 0.000000 | 0.000000 | 0.000000 |
|  |  | HO failure rate [%] | 0.000000 | 0.000000 | 0.000000 | 0.000000 | 0.000000 | 0.000000 |
|  | Overall | Successful HOs/UE/s | 0.008072 | 0.007242 | 0.004739 | 0.008420 | 0.028474 | 0.012638 |
|  |  | HOFs/UE/s | 0.000000 | 0.000040 | 0.000241 | 0.000415 | 0.000696 | 0.000280 |
|  |  | HO failure rate [%] | 0.000000 | 0.551471 | 4.838710 | 4.696970 | 2.386416 | 2.169625 |
| 30km/h, inter-freq | 2 | HOFs/UE/s | 0.000051 | 0.001361 | 0.001618 | 0.007653 | 0.010683 | 0.003478 |
|  |  | HO failure rate [%] | 0.156986 | 4.809437 | 4.571843 | 12.979094 | 6.876033 | 4.567117 |
|  | 3 | HOFs/UE/s | 0.000103 | 0.000000 | 0.000000 | 0.000077 | 0.000180 | 0.000015 |
|  |  | HO failure rate [%] | 0.313972 | 0.000000 | 0.000000 | 0.130662 | 0.115702 | 0.019857 |
|  | Total | Successful HOs/UE/s | 0.032564 | 0.026939 | 0.033771 | 0.051234 | 0.144507 | 0.072656 |
|  |  | HOFs/UE/s | 0.000154 | 0.001361 | 0.001618 | 0.007730 | 0.010863 | 0.003493 |
|  |  | HO failure rate [%] | 0.470958 | 4.809437 | 4.571843 | 13.109756 | 6.991736 | 4.586974 |
| 60km/h, inter-freq | 2 | HOFs/UE/s | 0.000229 | 0.003508 | 0.003661 | 0.032259 | 0.039657 | 0.008287 |
|  |  | HO failure rate [%] | 0.455927 | 11.500000 | 5.568445 | 37.040280 | 16.982364 | 6.467877 |
|  | 3 | HOFs/UE/s | 0.001144 | 0.000000 | 0.000000 | 0.000229 | 0.001373 | 0.000018 |
|  |  | HO failure rate [%] | 2.279635 | 0.000000 | 0.000000 | 0.262697 | 0.587851 | 0.014405 |
|  | Total | Successful HOs/UE/s | 0.048809 | 0.026997 | 0.062078 | 0.054605 | 0.192489 | 0.119826 |
|  |  | HOFs/UE/s | 0.001373 | 0.003508 | 0.003661 | 0.032488 | 0.041030 | 0.008306 |
|  |  | HO failure rate [%] | 2.735562 | 11.500000 | 5.568445 | 37.302977 | 17.570216 | 6.482282 |
| 3km/h, intra-freq | 2 | HOFs/UE/s | 0.000237 | 0.000306 | 0.000320 | 0.000557 | 0.001420 | 0.000280 |
|  |  | HO failure rate [%] | 4.314721 | 5.699482 | 5.943152 | 5.291005 | 5.304212 | 2.169625 |
|  | 3 | HOFs/UE/s | 0.000000 | 0.000000 | 0.000000 | 0.000000 | 0.000000 | 0.000000 |
|  |  | HO failure rate [%] | 0.000000 | 0.000000 | 0.000000 | 0.000000 | 0.000000 | 0.000000 |
|  | Overall | Successful HOs/UE/s | 0.005247 | 0.005066 | 0.005066 | 0.009965 | 0.025345 | 0.012638 |
|  |  | HOFs/UE/s | 0.000237 | 0.000306 | 0.000320 | 0.000557 | 0.001420 | 0.000280 |
|  |  | HO failure rate [%] | 4.314721 | 5.699482 | 5.943152 | 5.291005 | 5.304212 | 2.169625 |
| 30km/h, intra-freq | 2 | HOFs/UE/s | 0.003732 | 0.005851 | 0.003271 | 0.009583 | 0.022437 | 0.003478 |
|  |  | HO failure rate [%] | 11.654676 | 21.746575 | 9.428951 | 18.309859 | 15.372475 | 4.567117 |
|  | 3 | HOFs/UE/s | 0.000092 | 0.000000 | 0.000046 | 0.000092 | 0.000230 | 0.000015 |
|  |  | HO failure rate [%] | 0.287770 | 0.000000 | 0.132802 | 0.176056 | 0.157828 | 0.019857 |
|  | Total | Successful HOs/UE/s | 0.028196 | 0.021055 | 0.031375 | 0.042663 | 0.123288 | 0.072656 |
|  |  | HOFs/UE/s | 0.003824 | 0.005851 | 0.003317 | 0.009675 | 0.022667 | 0.003493 |
|  |  | HO failure rate [%] | 11.942446 | 21.746575 | 9.561753 | 18.485915 | 15.530303 | 4.586974 |
| 60km/h, intra-freq | 2 | HOFs/UE/s | 0.013165 | 0.013165 | 0.008776 | 0.027792 | 0.062898 | 0.008287 |
|  |  | HO failure rate [%] | 23.126338 | 35.409836 | 13.333333 | 39.790576 | 27.374005 | 6.467877 |
|  | 3 | HOFs/UE/s | 0.000244 | 0.000122 | 0.000000 | 0.000122 | 0.000488 | 0.000018 |
|  |  | HO failure rate [%] | 0.428266 | 0.327869 | 0.000000 | 0.174520 | 0.212202 | 0.014405 |
|  | Total | Successful HOs/UE/s | 0.043516 | 0.023891 | 0.057047 | 0.041932 | 0.166386 | 0.119826 |
|  |  | HOFs/UE/s | 0.013408 | 0.013287 | 0.008776 | 0.027914 | 0.063385 | 0.008306 |
|  |  | HO failure rate [%] | 23.554604 | 35.737705 | 13.333333 | 39.965096 | 27.586207 | 6.482282 |

Table 3 RLF performance

|  | Average number of RLFs/UE/second |  |  |  |
| --- | --- | --- | --- | --- |
|  | State 1 | State 2_Normal | State 2_HOF | Overall |
| 3km/h, inter-freq | 0.000000 | 0.000000 | 0.000616 | 0.000616 |
| 30km/h, inter-freq | 0.000000 | 0.000000 | 0.008141 | 0.008141 |
| 60km/h, inter-freq | 0.000000 | 0.000000 | 0.028904 | 0.028904 |
| 3km/h, intra-freq | 0.000000 | 0.000000 | 0.001016 | 0.001016 |
| 30km/h, intra-freq | 0.000000 | 0.000000 | 0.017461 | 0.017461 |
| 60km/h, intra-freq | 0.000000 | 0.000000 | 0.048027 | 0.048027 |
| 3km/h, macro only | 0.000000 | 0.000000 | 0.000229 | 0.000229 |
| 30km/h, macro only | 0.000000 | 0.000000 | 0.002011 | 0.002011 |
| 60km/h, macro only | 0.000000 | 0.000000 | 0.004134 | 0.004134 |