3GPP TSG-RAN WG2 Meeting #82	 R2-132184

Fukuoka, Japan, 21th - 24th May 2013

| CR-Form-v10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.321 | CR | 0667 | rev | - | Current version: | 11.2.0 |  |
| For HELP on using this form look at the pop-up text over the symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Further issues on removing optionality of CSI/SRS transmission during transient state |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, Panasonic, LGE, CATT, ASUSTek, Intel, Ericsson, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI11, LTE-L23 |  |  |  |  |  |  | Date: |  | 2013-05-08 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel- 11 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) |  |
| Reason for change: |  | According to current specification UE estimates based on received grants/assignments the DRX status of a subframe 4ms ahead and consequently determines whether to send periodic CSI on PUCCH/SRS. UE behavior w.r.t CSI/SRS reporting is not clear for the case when Active Time is stopped by DRX Command MAC CEs. Considering also DRX MAC CEs which are received until and including N-4 for determining periodic CSI on PUCCH/SRS reporting in subframe N will cause some unpredictable UE behaviour since eNB knows only after reception of the associated HARQ feedback for the PDSCH carrying DRX MAC CE (which e.g. comes in subframe N for a DRX MAC CE received in subframe N-4), whether DRX MAC CE was correctly received by the UE and hence considered in the determination step. Impact analysis Impacted functionality: DRX functionality & CSI/SRS transmission functionality Inter-operability: If the UE implements the change but the eNB does not, UE may send CSI/SRS while ENB doesn’t expect it. If the eNB implements the change but the UE does not, UE may not send periodic CSI/SRS while eNB would expect the reception of CSI/SRS. In both cases decoding of PUCCH might fail if CSI on PUCCH/SRS coincides with other UL control signalling. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For the evaluation of all the DRX Active Time conditions for subframe N, e.g. whether onDurationTimer or drx-InactivityTimer or drx-RetransmissionTimer are running in subframe N, and consequently for the determination whether to send CSI on PUCCH/SRS, UE also considers DRX Command MAC CEs which are received until and including subframe N-5. In order to have a single UE behaviour for grants/assignments and DRX Command MAC CEs as well as Scheduling Request, UE considers grants/assignments/DRX Command MAC CEs received and Scheduling Request sent until and including subframe N-5 when evaluating all DRX Active Time conditions in order to determine whether to send periodic CSI on PUCCH/SRS in subframe N. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE behaviour is not predictable w.r.t CSI/SRS reporting when DRX Command MAC CE is received, hence double decoding is necessary in eNB. Furthermore exact conditions UE is checking for determining whether to send CSI/SRS are not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

5.7	Discontinuous Reception (DRX)

The UE may be configured by RRC with a DRX functionality that controls the UE’s PDCCH monitoring activity for the UE’s C-RNTI, TPC-PUCCH-RNTI, TPC-PUSCH-RNTI and Semi-Persistent Scheduling C-RNTI (if configured). When in RRC_CONNECTED, if DRX is configured, the UE is allowed to monitor the PDCCH discontinuously using the DRX operation specified in this subclause; otherwise the UE monitors the PDCCH continuously. When using DRX operation, the UE shall also monitor PDCCH according to requirements found in other subclauses of this specification. RRC controls DRX operation by configuring the timers onDurationTimer, drx-InactivityTimer, drx-RetransmissionTimer (one per DL HARQ process except for the broadcast process), the longDRX-Cycle, the value of the drxStartOffset and optionally the drxShortCycleTimer and shortDRX-Cycle. A HARQ RTT timer per DL HARQ process (except for the broadcast process) is also defined (see subclause 7.7).

When a DRX cycle is configured, the Active Time includes the time while:

-	onDurationTimer or drx-InactivityTimer or drx-RetransmissionTimer or mac-ContentionResolutionTimer (as described in subclause 5.1.5) is running; or

-	a Scheduling Request is sent on PUCCH and is pending (as described in subclause 5.4.4); or

-	an uplink grant for a pending HARQ retransmission can occur and there is data in the corresponding HARQ buffer; or

-	a PDCCH indicating a new transmission addressed to the C-RNTI of the UE has not been received after successful reception of a Random Access Response for the preamble not selected by the UE (as described in subclause 5.1.4).

When DRX is configured, the UE shall for each subframe:

-	if a HARQ RTT Timer expires in this subframe and the data of the corresponding HARQ process was not successfully decoded:

-	start the drx-RetransmissionTimer for the corresponding HARQ process.

-	if a DRX Command MAC control element is received:

-	stop onDurationTimer;

-	stop drx-InactivityTimer.

-	if drx-InactivityTimer expires or a DRX Command MAC control element is received in this subframe:

-	if the Short DRX cycle is configured:

-	start or restart drxShortCycleTimer;

-	use the Short DRX Cycle.

-	else:

-	use the Long DRX cycle.

-	if drxShortCycleTimer  expires in this subframe:

-	use the Long DRX cycle.

-	If the Short DRX Cycle is used and [(SFN * 10) + subframe number] modulo (shortDRX-Cycle) = (drxStartOffset) modulo (shortDRX-Cycle); or

-	if the Long DRX Cycle is used and [(SFN * 10) + subframe number] modulo (longDRX-Cycle) = drxStartOffset:

-	start onDurationTimer.

-	during the Active Time, for a PDCCH-subframe, if the subframe is not required for uplink transmission for half-duplex FDD UE operation and if the subframe is not part of a configured measurement gap:

-	monitor the PDCCH;

-	if the PDCCH indicates a DL transmission or if a DL assignment has been configured for this subframe:

-	start the HARQ RTT Timer for the corresponding HARQ process;

-	stop the drx-RetransmissionTimer for the corresponding HARQ process.

-	if the PDCCH indicates a new transmission (DL or UL):

-	start or restart drx-InactivityTimer.

-	in current subframe n, if the UE would not be in Active Time considering  grants/assignments/DRX Command MAC control elements received and Scheduling Request sent until and including subframe n-5 when evaluating all DRX Active Time conditions as specified in this subclause, type-0-triggered SRS [2] shall not be reported.

-	if CQI masking (cqi-Mask) is setup by upper layers:

-	in current subframe n, if onDurationTimer would not be running considering  grants/assignments/DRX Command MAC control elements received until and including subframe n-5 when evaluating all DRX Active Time conditions as specified in this subclause, CQI/PMI/RI/PTI on PUCCH shall not be reported.

-	else:

-	in current subframe n, if the UE would not be in Active Time considering  grants/assignments/DRX Command MAC control elements received and Scheduling Request sent until and including subframe n-5 when evaluating all DRX Active Time conditions as specified in this subclause, CQI/PMI/RI/PTI on PUCCH shall not be reported.

Regardless of whether the UE is monitoring PDCCH or not, the UE receives and transmits HARQ feedback and transmits type-1-triggered SRS [2] when such is expected.

NOTE:	The same active time applies to all activated serving cell(s).