3GPP TSG-RAN WG2 Meeting #88 	R2-144822

San Francisco, USA, Nov. 17 – 21, 2014

| CR-Form-v11 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 25.331 | CR |  | rev | - | Current version: | 12.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections about CELL_FACH with Second DRX to 3G Logged MDT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Acer |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI12 |  |  |  |  |  |  | Date: |  | 2014-11-17 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-12 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) |  |
| Reason for change: |  | In Rel-12, 3G Logged MDT capable UE is allowed to make MDT measurement logging of neighbour cells in idle mode, CELL_PCH, URA_PCH states and CELL_FACH state when second DRX cycle is used. But some sections miss the enhancement for CELL_FACH when second DRX cycle is used. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In two places, two normative statements are added to CELL_FACH state for 3G Logged MDT. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The description for R-12 3G Logged MDT is inconsistent in some sections. The UE in CELL_FACH state when second DRX cycle is used may not perform logged measurement. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.2.2, 8.5.63.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CRxxxx, TS/TR ... CRxxxx |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

// Start//

#### 7.2.2.2	CELL_FACH state

In the CELL_FACH state the UE shall perform the following actions:

NOTE:	DCCH and, if configured, DTCH are available in this state.

1>	if the UE is "in service area":

2>	maintain up-to-date system information as broadcast by the serving cell as specified in subclause 8.1.1;

2>	perform cell reselection process as specified in [4];

2>	perform measurements process according to measurement control information as specified in subclause 8.4 and in subclause 14.4;

2>	run timer T305 (periodical cell update);

2>	select and configure the RB multiplexing options applicable for the transport channels to be used in this RRC state;

2>	store available measurements if logged measurement is configured as specified in [4] when second DRX cycle is used;

2>	for 3.84 Mcps and 7.68 Mcps TDD; or

2>	for FDD and 1.28 Mcps TDD, if the UE does not support HS-DSCH reception in CELL_FACH state; or

2>	if the IE "HS-DSCH common system information" is not included in System Information Block type 5 or System Information Block type 5bis; or

2>	for 1.28 Mcps TDD, if the IE "common E-DCH system info" is not included in System Information Block type 5:

3>	listen to all FACH transport channels mapped on the S-CCPCH selected by the UE according to the procedure in subclause 8.5.19.

2>	else:

3>	if variable H_RNTI is set:

4>	receive physical channels HS-SCCH(s) using the value of the variable H_RNTI as UE identity and parameters given by the IE(s) "HS-DSCH common system information" according to the procedure in subclause 8.5.36.

3>	else:

4>	if the variable HS_DSCH_RECEPTION_OF_CCCH_ENABLED is set to TRUE:

5>	receive physical channel(s) of type HS-SCCH with selected common H-RNTI using parameters given by the IE(s) "HS-DSCH common system information" according to the procedure in subclause 8.5.37.

2>	act on RRC messages received on BCCH, CCCH and DCCH;

2>	act on RRC messages received on MCCH if it supports MBMS and has activated an MBMS service as specified in subclause 8.7;

2>	act on RRC messages received on, if available, SHCCH (TDD only).

// Next//

#### 8.5.63.1	General

Figure 8.5.63.1-1: Logged measurements configuration

The purpose of this procedure is to configure the UE to perform logging of measurement results while in IDLE mode, CELL_PCH, URA_PCH states and CELL_FACH state when second DRX cycle is used.

NOTE:	UTRAN may retrieve stored logged measurement information by means of the UE Information procedure.

// End//