3GPP TSG-RAN WG2 Meeting #88	R2-144827

San Francisco, USA, 17 - 21 November 2014

Agenda item:		7.4.2

Source:	Nokia Networks

Title:	Clarification on p0-Persistent-SubframeSet2 Handling

WID/SID:	LTE_TDD_eIMTA-Core / Release 12

Document for:		Discussion and Decision

# 1	Introduction

p0-Persistent-SubframeSet2-r12 is added by R2-142934 [2] as an outcome of eIMTA. This contribution tries to clarify how UE handles this parameter.

# 2	Discussion

The following is extracted from TS36.331 v12.3.0;

SPS-ConfigUL ::=	CHOICE {

release							NULL,

setup							SEQUENCE {

semiPersistSchedIntervalUL			ENUMERATED {

sf10, sf20, sf32, sf40, sf64, sf80,

sf128, sf160, sf320, sf640, spare6,

spare5, spare4, spare3, spare2,

spare1},

implicitReleaseAfter				ENUMERATED {e2, e3, e4, e8},

p0-Persistent						SEQUENCE {

p0-NominalPUSCH-Persistent			INTEGER (-126..24),

p0-UE-PUSCH-Persistent				INTEGER (-8..7)

}		OPTIONAL,												-- Need OP

twoIntervalsConfig					ENUMERATED {true}			OPTIONAL,	-- Cond TDD

...,

[[	p0-Persistent-SubframeSet2-r12		SEQUENCE {

p0-NominalPUSCH-Persistent-SubframeSet2-r12			INTEGER (-126..24),

p0-UE-PUSCH-Persistent-SubframeSet2-r12				INTEGER (-8..7)

} 															OPTIONAL	-- Cond TwoSetsUL

]]

}

}

| Conditional presence | Explanation |
| --- | --- |
| TwoSetsUL | The field is optional present, need ON, if the field tpc-SubframeSet-r12 is configured. Otherwise the field is not present. |

As highlighte above p0-Persistent-SubframeSet2-r12 can be present only if tpc-SubframeSet-r12 is configured.

And tpc-SubframeSet-r12 can be configured if set2PowerControlParameter is setup as below;

UplinkPowerControlDedicated-v12x0 ::=	SEQUENCE {

set2PowerControlParameter		CHOICE {

release							NULL,

setup							SEQUENCE {

tpc-SubframeSet-r12					BIT STRING (SIZE(10)),

p0-NominalPUSCH-SubframeSet2-r12		INTEGER (-126..24),

alpha-SubframeSet2-r12				ENUMERATED {al0, al04, al05, al06, al07, al08, al09, al1},

p0-UE-PUSCH-SubframeSet2-r12			INTEGER (-8..7)

}

}

}

The condition “TwoSetsUL” says the field is ON if tpc-SubframeSet-r12 is configured. However the specification does not say what UE should do if tpc-SubframeSet-r12 is released. For instance, if network configures set2PowerControlParameter to “setup” and configures p0-Persistent-SubframeSet2 first but later nework removes set2PowerControlParameter by setting “release”. According to the current specification, it is expected that UE shall delete p0-Persistent-SubframeSet2-r12 autonomously because there is no mean for the network to remove it.

Proposal: It is propose to confirm that if configured earlier, UE shall delete p0-Persistent-SubframeSet2-r12 autonomosly if set2PowerControlParameter is set to “release” and clarify this in the specification.

# 3	Conclusion and Proposal

This contribution highlighted p0-Persistent-SubframeSet2-12 handling and proposed the following;

Proposal 1: It is proposed to confirm that if configured earlier, UE shall delete p0-Persistent-SubframeSet2-r12 autonomosly if set2PowerControlParameter is set to “release” and clarify this in the specification.

It is also proposed to agree on the CR to clarify the case.

Proposal 2: It is proposed to agree on the CR in [3]

# References

- 3GPP TS 36.331 Radio Resource Control (RRC); Protocol specification

- R2-142934 Introduction of TDD eIMTA – CATT

- R2-144828 Correction for p0-Persistent-SubframeSet2 Handling – Nokia Networks