3GPP TSG-RAN WG2 #89		 		                                                                 R2-150175

Athens, Greece, 9th – 13th, February 2015

Agenda item:	6.2.3.1

Source:	Coolpad

Title: 	Discussion on signaling procedures for ProSe transmitting UE

Document for:		Discussion and approval

Introduction

ProSe communication issues have been discussed in Rel-12 and so far two resource allocation modes have been agreed including scheduled resource allocation and UE-selected resource allocation.  For both two modes, there is no feedback from receiving UE to transmitting UE.  In this paper, we discuss the related signalling procedures for ProSe communication including cell selection/reselection and handover.  For handover procedure, we observe that during handover procedure, it is beneficial if the target eNB can take into account the ongoing ProSe communication about the UE.  The potential standard impact for RAN2 is identified and we propose RAN2 to consider this in Rel-12. This paper was previously submitted to RAN2 #88 and we submit to RAN2#89 with some revisions.

Discussion on related signalling procedures

In Release 12, resource allocation for ProSe communication is mainly tailored for the transmitting UEs.  For receiving UEs, the resource pool for reception from SIB or dedicated signalling is common for all the UEs. Thus, to improve the continuity of ProSe communication, the major mission is to ensure the transmitting UEs can keep transmitting during handover, cell selection/reselection and even RLF (i.e. exceptional case handling).

For normal cell selection/reselection procedures except for reestablishment case [1], we think Rel-12 solution is complete already.  And we assume that when UE is RRC_Idle mode, when UE starts to setup RRC connection and when T300 is running, UE can use common resource pool.  For RLF procedures, it is part of exceptional case handling which has been captured in TS 36.331 running CR[1].  In this paper, we would like to analyze the handover procedures.  We consider there might be some cases happen during handover procedures as follows.

- Case 1: One ProSe transmitting UE is in scheduled resource allocation mode under cell1, handover happens to cell2 but cell2 cannot provide the schedule resource to continue the ProSe communication in a timely manner, i.e., the ProSe transmitting UE doesn’t get the resource in handover command or even after it established RRC connection with cell2.   This might be caused by the reason that eNB has allocated the ProSe communication resources to other ProSe UEs and WAN services and thus scheduled resource allocation is not available for UE in Cell2.  In this case, if there is another neighbour cell which also meet the condition of handover and can provide ProSe communication resource, UE should be handed over to that cell instead of cell2.  But, current solution doesn’t enable this as the source eNB is unaware whether target cell can provide ProSe communication resources to the UE or not.

- Case 2: One transmitting UE is in UE-selected resource allocation mode in previous cell cannot continue the ProSe communication in this mode shortly after handover because it has to wait for the resource pool configuration by the new serving cell.   This might be caused by the reason that the corresponding eNB of cell2 has not allocated the resource pool for UE-selected resource allocation yet or the UE has to spend more time to acquire the configuration of resource pool which is to be broadcasted by SIB or delivered via dedicated RRC signalling.  If the target cell cannot provide the resource pool for UE-selected resource allocation but another neighbour cell can and the source doesn’t know this, it may cause improper handover for the ProSe transmitting UE which may impact the service continuity during handover.

To handle the above cases, we think firstly the source eNB should be aware of the ongoing ProSe transmission.  This is enabled for the UEs in scheduled resource allocation mode but not eanbled for the UEs in UE-selected resource allocation mode in current RAN2 agreements.  Secondly, the source cell can indicate the ProSe communication mode information of the UE to target side and target side can take this into consideration.  Thirdly, the target cell may inform the source cell whether it can allocate scheduled or UE-selected/common resource pool for the UE or not.  This can enables the source cell to decide whether target cell is a suitable cell for handover when there are more than one cells which can meet handover condition for the UE.

For further illustration, an example signalling procedure is shown in Figure 1.

Serving eNB

Target eNB

UE1

UE2

1.    Indication of on-going ProSe communication in UE-selected mode

On-going ProSe transmission

2. Handover Request < ProSe communication context>

3. Handover Request Acknowledge<ProSe resource indication>

4.RRCConnectionReconfiguration

Figure 1  Handover Procedure for ProSe transmitting UE

Referring to Figure 1, In Step 1, UE indicates that  the on-going ProSe communication is in UE-selected resource allocation mode.   This means one bit indication in RRC message.  As has been explained above, the reason to introduce this bit is that eNB is actually unaware of whether the UE has ongoing ProSe transmission if it ProSe communication is in UE-selected mode.  In step 2, in Handover Request message, serving eNB indicates the ongoing ProSe communication of the UE in the X2 message e.g. via AS context.  Target eNB conducts admission control for handover with the indication being taken into consideration.  In Step 3, target eNB sends the Handover Request Acknowledgement message and indicate the ProSe resource configuration to source eNB.  Finally in step 4, in RRCConnectionReconfiguration, serving eNB may indicate the ProSe resource configuration in target cell if necessary. When there are more than one handover target cells for the UE and the NW side can select one cell which can provide the desired ProSe resource for the UE.  From RAN2 perspective, we think step 1 is not supported yet.  Thus we have the following Proposal.

Proposal1: For ProSe UEs in RRC_Connected mode, when handover hppens, UE may indicate its ongoing ProSe communication in UE-selected resource allocation mode to the serving eNB with one bit in RRC message.

The indication can be supported by ProSeUEInformation, impact to TS 36.331 is provided in Annex part.

Conclusions

This document discussed the related signalling procedures for ProSe transmitting UE and we have the following proposal.

Proposal1: For ProSe UEs in RRC_Connected mode, when handover happens, UE may indicate its ongoing ProSe communication in UE-selected resource allocation mode to the serving eNB with one bit in RRC message.

References

- R2-145302  TS 36.331 Running CR

Annex: Changes to R2-145302

<start of change>

–	ProseUEInformation

The ProseUEInformation message is used for the indication of Prose information to the eNB.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to E-UTRAN

ProseUEInformation message

-- ASN1START

ProseUEInformation-r12 ::=		SEQUENCE {

criticalExtensions				CHOICE {

c1								CHOICE {

proseUEInformation-r12			ProseUEInformation-r12-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

ProseUEInformation-r12-IEs ::=	SEQUENCE {

commRxInterestedFreq-r12		ARFCN-ValueEUTRA-r9			OPTIONAL,

commTxResourceReq-r12			ProseCommTxResourceReq-r12	OPTIONAL,

discRxInterest-r12				ENUMERATED {true}			OPTIONAL,

discTxResourceReq-r12			INTEGER (1..63)				OPTIONAL,

modeIndication-r12              ENUMERATED {true}           OPTIONAL,

lateNonCriticalExtension		OCTET STRING				OPTIONAL,

nonCriticalExtension			SEQUENCE {}					OPTIONAL

}

ProseCommTxResourceReq-r12 ::=		SEQUENCE {

carrierFreq-r12					ARFCN-ValueEUTRA-r9			OPTIONAL,

proseDestinationInfoList-r12	ProseDestinationInfoList-r12

}

ProseDestinationInfoList-r12 ::=	SEQUENCE (SIZE (1..maxProseDest-r12)) OF ProseDestinationIdentity-r12

ProseDestinationIdentity-r12 ::=	BIT STRING (SIZE (24))

-- ASN1STOP

ProseUEInformation field descriptions

commRxInterestedFreq Indicates the frequency on which the UE is interested to receive Prose Direct Communication.

discTxResourceReq Indicates the number of resources requested by the UE for Prose Direct Discovery announcement.

discInterest Indicates that the UE is interested to monitor Prose Direct Discovery announcements.

modeIndication Indicates that the UE has on-going ProSe communication in UE-selected resource allocation mode

<end of change>