3GPP TSG RAN WG2 Meeting #89		R2-150183

Athens, Greece, 9th – 13th February 2015

Source: 	Sony

Title:	Considerations on Reduced Mobility Support for MTC

Agenda Item:	7.4.1

Document for:	Discussion

# Introduction

Complexity reduction and coverage enhancement are the main objectives of Rel.13 WID “Further LTE Physical Layer Enhancements for MTC” [1]. It is also stated that reduced mobility support can be considered if this is needed to fulfil the objectives.

In WG1 #79 Meeting in San Francisco, there have been some discussions related to the MTC mobility support. The outcome so far is that RAN1 recommends RAN2 to consider limiting support of mobility for Rel-13 low complexity UEs to reduce SIB size at least in enhanced coverage [2].

Sony’s opinion [3] is that MTC Release-13 should support two broad classes of device:

- Smart meter type devices: SM-LTE devices

- Consumer electronic type devices: CE-LTE devices

MTC should not be limited to a smart meter or stationary device. But, it should also support non-stationary devices which may require more mobility consideration. In our previous contribution [4], it has been shown that the intra RAT mobility functionalities of radio resource management (e.g. idle mode and connected mode) only introduce negligible UE power consumption.

This contribution describes Sony’s view on reduced mobility support for MTC devices.

# Discussion

## SM-LTE and CE-LTE Devices

Table 1 lists the high level characteristics of CE-LTE and SM-LTE devices. Considering that at least these two types of device will be supported by MTC, MTC should be able to support very different mobility aspects from static / low mobility (SM-LTE) to high mobility (CE-LTE) device.

Table 1 – High Level Characteristics of CE-LTE and SM-LTE devices

| Requirement | CE-LTE device | SM-LTE device |
| --- | --- | --- |
| Cost | Low cost | Very low cost |
| Data rate | 100kbps – 1Mbps range | 10 – 100bps |
| Latency | Approx. 250ms to 5 seconds | 24 hours |
| Frequency of UL Data Transmissions | Every 10 seconds to 30 minutes | 24 hours |
| Coverage | Standard LTE coverage | Coverage extension to basements |
| Form factor | Very small | Standard |
| Battery capacity | Up to 300 mAh = 1Wh | Up to 7000mAh (2 AA cells) = 10Wh |
| Battery lifetime | 1 to 2 weeks | Up to 10 years |
| Mobility | Required | Slow mobility acceptable |
| Roaming | Required | Required |

Table 2 (derived from [3]) identifies some communicating consumer electronics devices that could benefit from the Release 13 LTE MTC specifications, considering the mobility aspects of these devices. It can be identified that some CE-LTE devices can require high mobility. For example, a user with a smart watch riding a car or train would be in a high mobility situation.

Table 2 – Example Consumer Electronics Devices

| Consumer electronics device | Mobility Classifications |
| --- | --- |
| Smart watch | High mobility (user in vehicle) |
| Fitness tracker | Medium mobility (running, cycling) |
| Health monitor | High mobility (monitored patient on transport) |
| Baby monitor | Low mobility |
| Panic alarm | High mobility (user on public transport) |
| Pet tracker | Medium mobility (pet running away from owner) |
| Headphones | High mobility (user in vehicle) |
| Toy car | Low mobility (local use) |
| Robot lawn mower | Low mobility (local use) |

The usage of SM-LTE is well known. Even tough, the device can be semi-permanently placed in a location, it may still require mobility. As an example, an SM-LTE device in a parking meter can occasionally be hidden behind a bus which obstructs the radio wave propagation from its normal serving cell. In such situations, an SM-LTE device may perform cell-reselection to a neighbour cell. Another example, SM-LTE device in the train vending machine also obviously requires mobility support.

## Reduced Mobility Support for MTC devices

The reduced mobility support as written in the WI proposal [1] has not been clearly defined. Based on the latest discussion from NTT Docomo contribution [5], the SIB seems to be the limiting factor of mobility support for MTC devices. It also states one possible approach to make the new SIB be efficient is to focus on very low mobility scenario. If a support for mobility is limited, design for SIB can be simplified from viewpoint of both physical and higher layers. More specifically, if mobility support for the UE in the idle mode, i.e., cell reselection, is limited, SIB3 -SIB8 would be simplified [5].

A legacy LTE UE has a broad range of mobility support. For examples: roaming across countries/operators, hand-over to other cells, hand-over to other radio access technology (e.g. GSM, WCDMA, CDMA), or even the speed of UE physical movement. In this document, we discuss some potential mobility supports for MTC that can be removed, so that it can fulfil WI proposal objectives (i.e complexity reduction and coverage enhancement).

For complexity and cost reasons, a Release-13 LTE MTC UE may be designed with a single RAT. Single RAT operation will reduces the mobility support as the device is no longer required to perform inter-RAT mobility. Single-RAT operation will have the following impact:

- UE measurements related to other RATs are no longer needed.

- MTC UE is no longer required to decode the broadcast SIB information that related to other RATs (i.e. SIB6-8).

Proposal 1: Reduced complexity MTC device only needs to support intra-RAT mobility.

For the cost and power consumption reduction consideration, a Release-13 LTE MTC UE is expected to support fewer bands (potentially in the region of 2-4 bands) than a full capability Release-12 UE. A reduction in the number of bands reduces the number of inter-frequency measurements required at the UE. Consequently, less number of measurements brings benefit that MTC UE consumes less power than a legacy LTE UE.

Considering the supported bands are limited and there is on-going discussion to reduce the SIB size, one possibility is to reduce the number of instances of inter-frequency neighbour frequencies, cells and blacklisted cells. We can also consider reducing the number of instances of intra-frequency neighbour cells and blacklisted cells.

The current specification specifies the number of instances of inter-frequency neighbour cells is up to 8 cells on up to 8 frequencies, and intra-frequency neighbour cells is up to 16 cells. The number of instances can be reduced in an MTC SIB so that the total transport block size is less than 1000 bits.

Proposal 2: Considering the supported bands are limited and we need to reduce the SIB size, we can consider SIB with reduced size of intra-frequency and inter-frequency neighbour lists.

Our analysis indicates no evidence that we need to specifically constrain the MTC mobility in terms of velocity (e.g. MTC device for low mobility) as pointed out in [5]. This also applies to the UE in enhanced coverage but may depend on the level of coverage enhancement provided/needed. In an extreme case (i.e. 15 dB coverage enhancement) where a large number of SIB repetitions are required, we expect the target device is located in a basement (i.e. a stationary device) and no mobility support is needed for such a device. However, a device such as a smartwatch which may need coverage enhancement in order to compensate for a reduction in performance due to complexity reductions will require mobility support, for example to support coverage enhancement of 3dB.

Proposal 3: The number of SIB repetitions is configurable by the eNB depending on the intended coverage enhancement level.

# Conclusion

In this contribution, we have elaborated the definition of reduced mobility for MTC in order to meet MTC rel.13 objectives. MTC should not be limited to a smart meter or stationary device. It should also support non-stationary devices which may require more mobility consideration. This creates more use-cases for MTC devices.

Our proposals are summarized as follows:

Proposal 1: Reduced complexity MTC device only needs to support intra-RAT mobility.

Proposal 2: Considering the supported bands are limited and we need to reduce the SIB size, we can consider SIB with reduced size of intra-frequency and inter-frequency neighbour lists.

Proposal 3: The number of SIB repetitions is configurable by the eNB depending on the intended coverage enhancement level.

# References

RP-141660, “New WI Proposal: Further LTE Physical Layer Enhancements for MTC”

R2-150006, “LS on simultaneous reception requirements and SIBs for MTC UEs”

R1-144050, “Complexity Reduction Targets and General Considerations for MTC UE Types”

R1-145017, “Mobility support impact on power consumption for MTC devices”

R1-144962, “Features for Rel-13 Low Cost MTC with UE Complexity Reduction”